3GPP TSG-RAN WG4 Meeting #116bis	R4-2514137

# Prague, Czech, October 13th – 17th 2025

Source:	Ericsson

Title:	Discussion on Rel-20 NR demodulation with CDL models

Agenda item:	7.5.3

Document for:	Discussion

# Introduction

In Rel-20 WID NR demodulation performance: Phase 6 [1], the FR1 SU-MIMO demodulation requirements with CDL model are defined.

Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1

- Using rCDL-C1 channel model detailed in TR 38.753 as starting point

  - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case.

  - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding.

- Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias

- Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

In this contribution, we deliver our view on objectives listed in WID.

# Discussion

## 2.1 	General issue impact

As discussed in [2], we suggest that RAN4 should check the impact of new version 38.901 and CDL model generalization issue at the beginning of WI. It would impact the final model used for requirement definition.

New version 38.901 impact

The new version 38.901 might have two main impacts on agreed rCDL model derivation:

- New handheld UE and CPE antenna model. The UE antenna coordination and orientations are different from assumptions in rCDL-C1. The truncated model profile would be impacted by different spatial filters and the simulation results are expected to be quite different.

- New AS (Angular Spread) values for Uma scenario. rCDL-C1 take Table 7.2-4 in TR 38.827 as desired AS [3] for model derivation based on μlg values of Uma NLOS scenario in Table 7.5.6-part1 and Table 7.5-7 of 38.901 v18 [4], but new AS could be smaller especially ASD in v19 [5].

Table 2.1-1 Desired AS comparison for FR1 UMa NLOS

| ASdesired [deg] | ASD | ASA | ZSD | ZSA |
| --- | --- | --- | --- | --- |
| v18 | 25.7620 | 74.1138 | 4.8978 | 18.2050 |
| v19 | 12.3027 | 70.0587 | 4.8978 | 16.7017 |

Regarding all future CDL model derivation would depend on v19 specification, it is worthy to update rCDL-C1 model with v19 values and compare the performance gap.

- Scaling factor update for ZOA and ZOD. Adding 14 cluster configurations.

- New angular scaling formula and factors are introduced for CDL MIMO extension. The new formula could have similar as one used in TR38.753 to derive rCDL-C1 model. But the LUT for scaling factor in Table 7.7.5.1-1 in v19 is introduced. Whether to use this table or extend it to larger table for more desired AS values need discussion.

The updates in 38.901 v19 would highly impact rCDL model derivation and final performance.

Proposal 1	RAN4 address the impact of following aspects caused by 38.901 v19. Update rCDL model profile and common methodology if necessary.

- New handheld UE and CPE antenna model

- New AS values for Uma scenario.

- Scaling factors update for ZOA and ZOD.

- New angular scaling formula and factors are introduced for CDL MIMO extension.

Frequency generalization of channel model

As we discussed in Rel-19, parameter differences caused by frequency band are generally more like scaling based on same derivation formula, then the corresponding performance are comparable based on experience. For example, we use different TDL models in HST for FDD band and TDD band separately to differentiate Doppler impact. In CDL based model derivation, desired AS, Doppler shift and antenna configuration might be relevant to frequency band selection.

- In TS38.827 Table 7.2-4, it is indicated that the desired AS is evaluated with frequency 6GHz which could be enough to cover whole FR1 band (the difference is in deployment scenario, e.g., Uma or Umi). Current CDL based model profile is derived from CDLC model in TR38.901 with desired AS values in TR38.827.

- The delay spread selection in TR38.827 Table 7.2-1 is also appliable for whole FR1 band but different for deployment scenario.

- Companies agree to use one-by-one mapping from CSI-port to antenna-port, then there would be no difference among FR1 bands on antenna configurations.

- The current Doppler shift is evaluated from the center frequency and UE velocity vector.

In that case, the profile besides Doppler shift should be OK for whole FR1 band. It would not be very complicated to handle Doppler impact in simulation or TE implementation. In normal UE mobile conditions within one frequency range, one Doppler shift configuration (based on center frequency) could be applied for whole band demodulation requirements since the Doppler offset won’t be too large. It is reasonable to take 3.5GHz as center frequence for FR1 CDL model derivation in Rel-20 scope.

Proposal 2	Only consider 3.5GHz for Doppler evaluation for FR1 rCDL model derivation for normal UE mobile condition in Rel-20 scope.

In general, multiple channel models would be needed for different test cases conditions. For TDL models, we have TDLA, TDLB, TDLC and TDLD in existing demodulation requirements. It could be also useful to have more CDL models if demodulation requirements would be defined. For example, TDLA models with small delay spread and low Doppler shift are normally used for PMI tests. Now we only have rCDLC-C1 with 365ns rms delay spread for all test cases, it might be better to generate more CDL models for comparison regarding test case’s character. Of course, it depends on the discussion progress on above general issues and timeline.

Proposal 3	Generate more CDL models for performance comparison regarding test case’s character if timeline is allowed.

## 2.2	SU-MIMO PDSCH requirement

According to Rel-19 study output, SU-MIMO PDSCH with rCDL simulation results could get majority alignment on both 4 layers and 8 layers (per CW) normalized throughput. Although rCDL model might need some modifications and re-alignment, but it could be straightforward for requirement definition with rCDL model.

The scope of the new requirements could focus on normal PDSCH demodulation with 4Rx and 8Rx, e.g., PDSCH mapping type A. The existing 4Rx PDSCH requirements include many scenarios, the feasibility of applying rCDL model need verifications case by case, but it would lead to too large scope which can’t be accepted with short timeline in Rel-20.

Proposal 4	Only consider PDSCH mapping type A demodulation requirements with 4Rx and 8Rx.

## 2.3 	SU-MIMO PMI requirements

For PMI simulations with rCDL models in Rel-19 study, companies deliver simulation results for full throughput curves for comparison and alignment. According to alignment conclusion [3], majority companies could get alignment on 4 layers transmission with Type-I codebook on both 70% and 90% normalized throughputs, but can’t get similar alignment on eType-II code book. In that case, it could be possible to consider Type-I and eType-II as separate condition.

For Type-I codebook, it is not mentioned in WID might because the legacy PMI requirement with TDL model on single panel Type-I codebook is good enough, such as gamma value is 1.3 for 8Tx and 1.5 for 4Tx, while the corresponding gamma values with rCDL-C1 are 1.8 and 2.4 in our simulations. The low layer transmission (e.g., 2 layers) could get more robust test metric, but it might be so attractive for CDL model application. For 8Tx with 4 layers transmission, test metric is improved obviously. Regarding the potential rCDL model modification in Rel-20, it would be better to also take Type-I PMI cases as reference to get alignment among companies at first. In that case, 8Tx with 4 layers with Type-I PMI case is a good start point for PMI demodulation requirement definition for CDL model.

Proposal 5	Take 8Tx 4 layers Type-I PMI case for initial alignment if rCDL model is modified.

For eType-II codebook, the major issue from Rel-19 is how to get alignment among companies as mentioned above. Another checking point for eType-II is comparing performance with Type-I codebook and see if obvious performance gain could be seen. In our simulation results, 8Tx can only show limited gain (<1dB) at 70% and 90% normalized throughput while 32Tx could show higher gain (>2dB) due to smaller spatial resolution. The gamma value with 32Tx is also pretty high for both Type-I and eType-II. But some companies results show opposite relationship between Type-I and eType-II. There is no conclusion on this status in Rel-19 discussion, but the reason behind should be clarified and fixed if performance requirement would be introduced. Higher number of Tx ports could be also considered for comparison in Rel-20 regarding potential future implementation.

In Rel-19, the eType-II follow PMI with rCDL model results are not well aligned among companies.

Proposal 6	Companies to find out the reason of misalignment on simulation results with rCDL model before considering introduce requirement.

Proposal 7 	Consider 32Tx with eType-II PMI case for comparison regarding potential future implementation.

Once simulation results with single rCDL model could get alignment, companies could continue consider how to do angle rotation during PMI test to have robust performance verification. It could be tricky for CDL based model but it is very important if CDL models would be used for AI receiver CSI testing. As the start point for non-AI receiver, mathematical way (e.g., cluster steering matrix) or iteration with several model profiles might be considered for methodology discussion.

Proposal 8	Study CDL model angular variation methodology for PMI tests in Rel-20.

As for the final requirement metrices, legacy gamma value after averaging throughput gain during the test could be referred as start point. Regarding different companies’ implementation, gamma value could be quite variant. If it is observed from simulation results, normalized throughput could also be considered as metric. The percentile could be further discussed.

Proposal 9	Check PMI simulation results on normalized throughput and gamma values, then decide which one could be used as test metric.

# Conclusions

In the previous sections we made the following observations:

Observation 1	The updates in 38.901 v19 would highly impact rCDL model derivation and final performance.

Observation 2	In Rel-19, the eType-II follow PMI with rCDL model results are not well aligned among companies.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 address the impact of following aspects caused by 38.901 v19. Update rCDL model profile and common methodology if necessary.

	New handheld UE and CPE antenna model

	New AS values for Uma scenario.

	Scaling factors update for ZOA and ZOD.

	New angular scaling formula and factors are introduced for CDL MIMO extension.

Proposal 2	Only consider 3.5GHz for Doppler evaluation for FR1 rCDL model derivation for normal UE mobile condition in Rel-20 scope.

Proposal 3	Generate more CDL models for performance comparison regarding test case’s character if timeline is allowed.

Proposal 4	Only consider PDSCH mapping type A demodulation requirements with 4Rx and 8Rx.

Proposal 5	Take 8Tx 4 layers Type-I PMI case for initial alignment if rCDL model is modified.

Proposal 6	Companies to find out the reason of misalignment on simulation results with rCDL model before considering introduce requirement.

Proposal 7 	Consider 32Tx with eType-II PMI case for comparison regarding potential future implementation.

Proposal 8	Study CDL model angular variation methodology for PMI tests in Rel-20.

Proposal 9	Check PMI simulation results on normalized throughput and gamma values, then decide which one could be used as test metric.

# References

[1] 	RP-252957: WID on NR demodulation performance: Phase 6, China Telecom, BT, RAN#109, Beijing, China

[2]	R4-2513955, Discussion on Rel-20 general issue of NR demodulation enhancement phase 6, Ericsson

[3]	TR 38.753 v2.0.0

[4]	TR 38.901 v18.0.0

[5]	TR 38.901 v19.0.0