3GPP TSG-RAN WG4 Meeting #116bis	R4-2514138

# Prague, Czech, October 13th – 17th 2025

Source:	Ericsson

Title:	Discussion on Rel-19 NR SBFD demodulation requirements

Agenda item:	6.13.4

Document for:	Discussion

# Introduction

In RAN4#116, companies achieved a good progress on demodulation part [1].

Issue 1-1-1 whether to introduce UE demodulation with SBFD carrier (with DL sub-band and UL sub-suband configured)

- Agreement

  - Not to introduce UE demodulation requirements with SBFD carrier (with DL sub-band and UL sub-band)

Issue 2-1-1 Types of interference modeling for demodulation requirement with UL muting

- Agreement

  - Evaluate PUSCH performance with UL muting under inter-site gNB interference modelling

    - FFS on self-interference and intra-site inter sector interference

Issue 2-1-4 Baseline Receiver Assumption

- agreement

  - MMSE-IRC with the same assumption in Rel-19 performance enhanced WI

In this contribution, further analysis on interference modelling and simulation results are delivered.

# Discussion

Issue 2-1-2 how to model interference modeling for performance evaluation

- Agreement

  - Considering TDL+INR type of interference model for evaluation

    - Note: the RF assumption can be considered for interference modelling

  - The following candidate value for INR can be considered for evaluation as starting point

    - Option 1: based on link budget analysis

| Conditions | FR1 |
| --- | --- |
| Interf1 > Interf2 | INR1 <= 17.2dB, INR2 <= 7.2dB Trial simulation could try following sets: [INR1, INR2] = {[5, -5], [9, -1], [13, 3], [17, 7]} dB |
| Interf1 = Interf2 | INR1 = INR2 <=11.2dB Trial simulation could try following sets: [INR1, INR2] = {[-1, -1], [3, 3], [7, 7], [11, 11]} dB |
| Note: The interference wave form could use PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup. |  |

    - Other options are not concluded

  - Interesting companies can provide the system level simulation to derive the INR value

For modelling of inter-site cochannel interference, we delivered our proposal based on simple link budget regarding to the worst case (boresight pointing from aggressing gNB) [2]. Some companies propose taking RF discussion assumptions into account to calibrate the model.

By checking RF agreement [3], the in-channel adjacent subband blocking requirement for FR1 WA BS is -28dBm which could reflect the inter-site CLI impact. The corresponding RF assumptions are that aggressing gNB Tx power is 53dBm and the coupling loss is 80dBc. In that case, 1dB more margin is added on top of 53dBm – (80dBc+ 1dB) = -28dBm.

In [2], we also assume gNB Tx power is 53dBm, but the coupling loss is only 73.5dBc according to 4GHz centre frequency, 500m ISD, gNB EIRP is 78dBm and UE antenna gain is 0dBi. There are 7.5dB difference compared to RF coupling loss plus margin. If we would align with RF agreement, the derived INR values for trial simulations in [2] should be reduced with 7.5dB. To simplify, we can just reduce 8dB.

Table 2-1 Revised INR values for FR1

| Conditions | FR1 |
| --- | --- |
| Interf1 > Interf2 | INR1 <= 9.2dB, INR2 <= -0.8dB Trial simulation could try following sets: [INR1, INR2] = {[5, -5], [9, -1]} dB |
| Interf1 = Interf2 | INR1 = INR2 <=3.2dB Trial simulation could try following sets: [INR1, INR2] = {[-1, -1], [3, 3]} dB |
| Note: The interference wave form could use PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup. |  |

The coupling loss assumption for in-channel adjacent subband blocking requirement in RF discussion is ~8dB higher than in option 1 in WF.

Proposal 1	Reduce interference INR values as in Table 2-1 for initial simulations.

# Conclusions

In the previous sections we made the following observations:

Observation 1	The coupling loss assumption for in-channel adjacent subband blocking requirement in RF discussion is ~8dB higher than in option 1 in WF.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reduce interference INR values as in Table 2-1 for initial simulations.

# References

[1] 	R4-2512615, Way Forward for [116][328] NR_duplex_evo_demod, Samsung

[2]	R4-2510751, Discussion on SBFD demodulation requirements, Ericsson

[3]	R4-2512665, Way Forward for [116][306] NR_duplex_evo_General, Samsung, CATT, Charter, CMCC, Ericsson, Huawei, HiSilicon, Nokia, Qualcomm, Tejas Network, ZTE

[4]	R4-2514139, Simulation results for SBFD PUSCH UL muting, Ericsson