3GPP TSG-RAN WG4 Meeting #116bis	R4- 2514153

Prague, Czech Republic, 13-17 October, 2025

Source:	Nokia

Title:	Discussion on Rel-19 NR LP-WUS Performance requirements

Agenda item:	6.16.3

Document for:	Discussion

Introduction

In this contribution we discuss Rel-19 NR LP-WUS Performance requirements.

Discussion

Entry threshold alignment between companies

In the core specification LR is required to “meet the entry criterion by a margin of 6 dB”. Our understanding is that RAN4 defined accuracy margin describes how much better than (how far beyond) the threshold LR measured value is. When LR “meet the criterion by a margin of 6 dB” it means the measured value is 6 dB better than the configured threshold. For example, the following figures illustrates this:

Figure 2: Example of Entry threshold for MR relaxation or offloading based on RAN4 assumptions.

We don’t anticipate any specification impact on this, and the below proposal is more for aligning the understanding of the entry criteria in the performance part.

For the test case definition purpose, confirm the understanding that LR “meet the entry criterion by a margin of 6 dB” means UE can enter when LR measurement is 6dB better than the threshold.

About testability of LP-WUS requirements

Issue 3-1-2 General aspects

Based on the RAN1 design, the UE may monitor LP-WUS depending on whether the entry / exit conditions are fulfilled. This means that the UE requirements cannot be tested in such a way that the UE would be guaranteed to monitor LP-WUS signal and pass 90% of the test runs.

Alternatively, a softer option would be to define test mode for LP-WUS functionality which guarantees that the UE enters the right state before or during the test is performed. However, it’s unclear how it would be guaranteed that UE enters the correct state.

The downside of the test mode is that it does not necessarily test the runtime execution of the UE. In other words, the performance at the field may differ from the performance seen at the test cases.

Test mode may be required for LP-WUS testing.

Define test mode / test procedure for LP-WUS / WUR testing

Testing of entry threshold and margin

Issue x-x-x: Test case design for entry testing for case 1/case 3

Based on the offline discussions with companies in the last meeting, testing LP-WUS operation with test mode may not be the best option due to the reasons mentioned above. We understand this, but from the network point of view, it is important that a feature is tested in such extent that it does not break the existing services. Furthermore, there should not be major core requirements that do not have test case.

Therefore, we think that at least it should be testable that when UE is configured with LP-WUS it responds to paging normally when the tester signal is outside of the configured threshold. This means a test case where:

T1:

- UE is configured with entry threshold(s)

- Tester signal is set above (e.g. 0.5dB above the entry threshold)

- Tester signal is kept as the same, UE shall not enter  LP-WUS operation

T2:

- Network transmits paging without LP-WUS

- UE responds with preamble with normal delay

The benefit of this type of test case is that it guarantees for the network operator that UE respects the configured entry margin that the UE is able to respond to paging without LP-WUS (i.e., the UE implementation does not always expect LP-WUS signal before responding to paging, including MR delay).

For entry threshold and margin testing without test mode, define a test method where the signal level of the tester is set to 0.5dB above from the configured entry threshold. The tester signal level (serving cell) is maintained at the same value throughout T1 and T2. At the beginning of T2 the UE is paged without network transmitting LP-WUS first.  In this test case the UE shall not enter relaxation and respond to paging as legacy.

LP-WUR test case design

Issue 3-2-1: Test case scenarios

Summary from [1]

Issue 3-2-1: Test case scenarios - Proposals - P1: Consider test case for the following scenarios: 1: LP-WUR monitoring related test case; 2: RRM offloading related test case; 3: RRM relaxation related test case (CATT China telecom vivo) - P2: RAN4 to define the test cases to verify both MR RRM relaxation and MR RRM offloading scenarios.(Ericsson Nokia)

RAN4 to define the test cases to verify both MR RRM relaxation and MR RRM offloading scenarios.

OOK and OFDM differentiation in the test cases

Issue 3-2-2: Test case for OOK based LR and OFDM based LR

The purpose of this test is to verify UE’s ability to detect and decode LP-WUS using OOK based LR and OFDM based LR. As the underlying measurements and detection techniques differ between the two types of signals. We propose to define separate set of test cases for OOK based LR and OFDM based LR.

RAN4 to define separate test cases for PSS/SSS based LR and LP-SS based LR. (RAN4 to define separate test cases for entry and exit condition tests).

Testing exit for RRM relaxation / offloading

Issue 3-2-3: Test case design for exit conditions for case 1/case 3

For the Case 1 (offloading):

The purpose of this test is to evaluate the UE’s ability to enter and exit measurement relaxation in the fully offloading case when the serving cell quality changes.

For the Case 3 (MR relaxation):
The purpose of this test is to assess the UE’s performance when LR is ON, MR is ON with relaxed measurements for both serving and neighbouring cells. The test examines how quickly MR should resume full measurement activity when serving cell quality degrades.

RAN4 to test UE exist Case 1 to legacy and UE exit Case 3 to legacy.

Issue 3-2-4: Test case design for RRM relaxation

For cell reselection, both intra-frequency reselection and inter-frequency reselection should be supported to guarantee that both features are operating as expected with LP-WUS and reselection works regardless of UE’s LP-WUS operation. In intra-frequency test case, the UE exits relaxation after threshold is met and enters intra-frequency reselection. Correspondingly, in inter-frequency scenario UE enters inter-frequency reselection. The test cases can use the existing test cases as a baseline. RAN4 can further discuss how to initialise UE in the test case for MR relaxation.

We propose to support the scenarios in the below table:

| Test cases | MR relaxation |
| --- | --- |
| Cell reselection to FR1 intra-frequency NR case with LP-WUS | Test case |
| Cell reselection to FR1 inter-frequency NR case with LP-WUS | Test case |

A test case where, if the UE does apply relaxation, the UE will perform inter-frequency measurement (which can be according to the relaxed measurements).

An intra-frequency TC where UE still does reselection NOT applying relaxation (as the UE will exit relaxation early enough to trigger reselection using non-relaxed measurement.

RAN4 to discuss the test methodology about how to verify MR RRM offloading scenario, such as whether to introduce a new test mode.

Issue 3-2-6: Test case design for higher priority frequency layer search

In the fully offloading case (MR is OFF, LR is ON), only LR is performing serving cell measurements we propose to introduce test case as fully offloading case typically considers cell re-selection for high priority frequency layers.

RAN4 to introduce test case for high priority frequency layers.

Issue 3-2-6: Test case design for LP-WUS monitoring

In LP-WUS monitoring test case, the UE can be set to meet LP-WUS monitoring thresholds, and the UE is required to respond with the correct delay to the paging. This is part of the core-functionality of the LP-WUS, hence it should be tested. One of the challenges in this test case is how to ensure that the UE is monitoring LP-WUS as legacy UE can easily pass the test case if the only change is that the UE responds to paging at certain time.

RAN4 to introduce test case only for LP-WUS monitoring to test the delay between network transmitted LP-WUS to UE responding to paging message.

Issue 3-2-8: On FR1/FR2 for test case 

Clarify frequency range specific aspects of LP-WUS test cases and ensure that test configurations are appropriate for both FR1 and FR2.

RAN4 to introduce test cases related to FR1 and FR2

Conclusion

The following Observations and Proposals were made:

Proposal 1: For the test case definition purpose, confirm the understanding that LR “meet the entry criterion by a margin of 6 dB” means UE can enter when LR measurement is 6dB better than the threshold.

Observation 1: Test mode may be required for LP-WUS testing.

Proposal 2: Define test mode / test procedure for LP-WUS / WUR testing

Proposal 3: For entry threshold and margin testing without test mode, define a test method where the signal level of the tester is set to 0.5dB above from the configured entry threshold. The tester signal level (serving cell) is maintained at the same value throughout T1 and T2. At the beginning of T2 the UE is paged without network transmitting LP-WUS first.  In this test case the UE shall not enter relaxation and respond to paging as legacy.

Proposal 4: RAN4 to define the test cases to verify both MR RRM relaxation and MR RRM offloading scenarios.

Proposal 5: RAN4 to define separate test cases for PSS/SSS based LR and LP-SS based LR. (RAN4 to define separate test cases for entry and exit condition tests).

Proposal 6: RAN4 to test UE exist Case 1 to legacy and UE exit Case 3 to legacy.

Proposal 7: A test case where, if the UE does apply relaxation, the UE will perform inter-frequency measurement (which can be according to the relaxed measurements).

Proposal 8: An intra-frequency TC where UE still does reselection NOT applying relaxation (as the UE will exit relaxation early enough to trigger reselection using non-relaxed measurement.

Proposal 9: RAN4 to discuss the test methodology about how to verify MR RRM offloading scenario, such as whether to introduce a new test mode.  Issue 3-2-6: Test case design for higher priority frequency layer search

Proposal 10: RAN4 to introduce test case for high priority frequency layers.

Proposal 11: RAN4 to introduce test case only for LP-WUS monitoring to test the delay between network transmitted LP-WUS to UE responding to paging message.

Proposal 12: RAN4 to introduce test cases related to FR1 and FR2

References

[1] R4-2509065 Topic summary for [116][222] NR_LPWUS, vivo, RAN4#116