3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514159

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on core requirements for NTN in Ku band

Source: 	Huawei, HiSilicon

Agenda item:	4.2.7.4

Document for:	Discussion

Introduction

RRM core requirements for NTN in Ku band are discussed in RAN4#116, and the outcomes are captured in WF [1]. During CR review, we find one issue on scheduling restriction for soft satellite switching that needs to be further discussed.

In this paper, we will provide our views on remaining issues in RRM core requirements for NTN in Ku.

Discussion

In the agreed big CR [2], the scheduling restriction for soft satellite switching is defined as

In addition, following scheduling restriction applies during the time period from t-serviceStart to t-service, - If deriveSSB-IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and KRX_switch data symbol before and after each consecutive SSB symbols to be measured within SMTC window duration. - If deriveSSB-IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration, and KRX_switch data symbol before and after SMTC window duration. - where KRX_switch is for the switch period of UE to steer the downlink spatial domain reception filter toward a target cell. KRX_switch = 1.

The scheduling restriction is defined based on SMTC window. However, the location of SSB of the target satellite is indicated via ssb-TimeOffset-r18, and SSB of the target satellite is not necessarily in an SMTC configured by the source satellite. For this case, our view is that the scheduling restriction should be based on a virtual window that covers the SSB of the target satellite and resembles SMTC, i.e. the periodicity, offset and duration and the window are in subframe or ms.

The offset of the window should be the latest subframe that is no later than the SSB of the target satellite. It is note that the location of SSB of the target satellite is already defined in existing requirements. For example, if the SSB of the source satellite is transmitted in the first half frame of SFN0, and ssb-TimeOffset-r18 is 10, after accounting for the propagation delay difference, the SSB of the target satellite starts within the second subframe of SFN1. In this case, the offset of the window is 11ms.

- Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, and if ssb-TimeOffset is configured, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the SSB-TimeOffset reference point as defined in TS 38.331 [2] to UE.

The duration of the window should be the smallest number in subframe that is no smaller than the burst length of the SSB of the target satellite. For example, if 64 SSB index are transmitted by the target satellite, one SSB burst will span over 32 slots which equals to 4ms for 120kHz SCS.

The periodicity of the window should be the periodicity of the SSB of the source satellite. It is aligned with the definition of Tsat_beam, and the assumption is that UE will measure the target satellite for sync and beam steering based on the SSB periodicity which is common for source and target satellites.

Proposal: For soft satellite switch, when SSB of the target satellite does not fall in any SMTC, the scheduling restriction is defined based on a window:

- The offset of the window is the latest subframe that is no later than the SSB of the target satellite,

- The duration of the window is the smallest number in subframe that is no smaller than the burst length of the SSB of the target satellite,

- The periodicity of the window is the periodicity of the SSB of the source satellite.

Conclusions

In this paper we provided our views on remaining issues in RRM core requirements for NTN in Ku.

Proposal: For soft satellite switch, when SSB of the target satellite does not fall in any SMTC, the scheduling restriction is defined based on a window:

- The offset of the window is the latest subframe that is no later than the SSB of the target satellite,

- The duration of the window is the smallest number in subframe that is no smaller than the burst length of the SSB of the target satellite,

- The periodicity of the window is the periodicity of the SSB of the source satellite.

Reference

- R4-2512121, WF on RRM requirements for NR_NTN_Ku_bands, Apple

- R4-2512351, Big CR on RRM requirements of Ku band for NR NTN, Apple