3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514175

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on core requirements for R19 A-IoT

Source: 	Huawei, HiSilicon

Agenda item:	4.20.2

Document for:	Discussion

Introduction

RRM core requirements for Rel-19 A-IoT are discussed in RAN4#116, and the outcomes are captured in WF [1]. Meanwhile, the bit TP for 38.191 is approved in [2]. There are some issues related to the random access requirements that need to be corrected.

In this paper, we will provide our views on remaining issues in RRM core requirements for A-IoT.

Discussion

In the latest version of TS 38.191, the random access requirements for CBRA are defined as follows.

### 9.1.2 Requierments for contention based random access #### 9.1.2.1 Correct behaviour when transmitting A-IoT Msg1 When an A-IoT device is indicated to perform CBRA by the A-IoT Paging message according to the procedure described in clause 5.2 in TS 38.391 [6], the A-IoT device shall have the capability to randomly select an access occasion among access occasions configured in the A-IoT paging message according to the procedure described in clause 5.3.1.1 and shall have the capability to transmit the A-IoT Msg1 (i.e., random ID message) on this selected access occasion according to the procedure described in clause 5.3.1.2 in TS 38.391[6]. #### 9.1.2.2 CBRA: Correct behaviour when receiving A-IoT MSG2 The UE shall again perform the Random Access Resource selection procedure defined clause 5.3.1 in TS 38.391 [6]. #### 9.1.2.3 CBRA: Correct behaviour when not receiving A-IoT MSG2 The UE shall again perform the random access selection procedure defined in clause 5.3.1 in TS 38.391 [6].

The 2 highlighted requirements are not aligned with the UE behavior in 38.391.

1> if the device has no stored AS ID (i.e., initial reception of Random ID Response message): 2> for each ID entry in Random ID Response message: 3> if the value indicated by Echoed Random ID field is identical to the value of the Random ID field in the transmitted Access Random ID message; and 3> if the Frequency Index field is present (i.e., Frequency Index Present Indication is set to 1), and the small frequency shift factor indicated by the Frequency Index field matches the value of the small frequency shift factor used for the transmission of Access Random ID message: 4> consider this CBRA procedure is successful; 4> if the Assigned AS ID field corresponding to the Echoed Random ID field is included (i.e., AS ID Present Indication field is set to 1): 5> set AS ID to the value indicated by the Assigned AS ID field and store the AS ID; 4> else: 5> set AS ID to the value indicated by the Echoed Random ID field and store the AS ID; 4> initiate the D2R message transmission as specified in clause 5.4.1, upon which the procedure of processing this Random ID Response message ends;

In 39.391 it is specified that when device receives Msg2 containing the same Random ID it transmitted in Msg1, it should consider the CBRA as successful and initiate the D2R message transmission. It is clear that RAN4 requirements highlighted in yellow that the device should again perform the Random Access Resource selection are not correct.

In 38.391 it is not specified what happens when UE does not receive Msg2 containing the same Random ID it transmitted in Msg1. Our understanding is that the device would not transmit anything until it receives A-IoT Paging message next time, which is covered by requirements in 9.1.2.1. It is noted that there is no Msg1 re-transmission as in NR, and all the D2R transmission by the device should be triggered by a R2D message. It is clear that the cyan highlighted requirements are not correct.

In our view, there are 2 options to handle the cyan highlighted requirements:

- Option 1: remove the requirements, as there is no UE behaviour defined in 38.391

- Option 2: change the requirements to “the UE shall not initiate any D2R transmission”

We slightly prefer option 2 to keep the requirements.

Proposal 1: Update the requirements for “when receiving A-IoT MSG2” to that “UE shall initiate the D2R message transmission as defined in clause 5.3.1”.

Proposal 2: Update the requirements for “when not receiving A-IoT MSG2” to that “UE shall not initiate any D2R transmission”.

Conclusions

In this paper we provided our views on remaining issues in RRM core requirements for A-IoT.

Proposal 1: Update the requirements for “when receiving A-IoT MSG2” to that “UE shall initiate the D2R message transmission as defined in clause 5.3.1”.

Proposal 2: Update the requirements for “when not receiving A-IoT MSG2” to that “UE shall not initiate any D2R transmission”.

Reference

- R4-2512134, WF on RRM for Ambient_IoT_Solutions, Moderator (CMCC)

- R4-2512353, Big TP on RRM requirements for Ambient IoT, CMCC