3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514180

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on core requirements for IoT NTN TDD

Source: 	Huawei, HiSilicon

Agenda item:	4.27.4

Document for:	Discussion

Introduction

RRM core requirements for Rel-19 IoT NTN TDD are discussed in RAN4#116, and the outcomes are captured in WF [1]. In our view, the following issues need to be further discussed.

- RRC re-establishment (6.5A)

- RRC Connection redirection to Non-Anchor Carrier (6.9A)

In this paper, we will provide our views on remaining issues in RRM core requirements for IoT NTN TDD.

Discussion

RRC re-establishment (6.5A)

## Issue 3: RRC re-establishment (6.5A) Agreement (online - Wed): - Tsearch_NB1-NC,i in RRC re-establishment requirement is - If the target cell is known, then Tsearch_NB1-NC,i = 0 ms. If the target cell is unknown, Tsearch_NB1-NC,i = Ksatellite,i *2.52 sec. - FFS the definition of Ksatellite,i

For re-establishment the value of Tsearch has been agreed as 2.52s, and what remains open is the definition of Ksatellite. The definition in current requirement is copied as below.

- If the target cell is known, then Tsearch_NB1-NC,i = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch_NB1-NC,i = Ksatellite *80 ms. Otherwise, Tsearch_NB1-NC,i = Ksatellite,i *1400 ms. Where Ksatellite,i is defined as the number NGSO satellites to be measured on i-th frequency for RRC re-establishment. Ksatellite,i = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite,i equals to the number NGSO satellites to be measured on the carrier if NGSO satellites are monitored.

The reason to keep definition of Ksatellite open is that RAN4 has agreed the following for measurement.

- Intra-satellite neighbor cell measurement requirements - When DRX is not in use: - Tdetect_intra_NB1-NC and Tmeasure _intra_NB1-NC are - 1.89 sec and 800 ms - When DRX is in use: - Tdetect_intra_NB1-NC = 10 DRX cycles - Tmeasure _intra_NB1-NC = 2 DRX cycles - No requirement in Rel-19 when DRX cycle is 5.12s and larger. - Scale Tdetect_intra_NB1-NC and Tmeasure _intra_NB1-NC by a factor of 2, when the UE measures cells across 3 or more different patterns of DL opportunities

In our view, the question is whether scaling based on Ksatellite is same as the newly agreed scaling for the case UE measures cells across 3 or more different patterns of DL opportunities. Based on reasoning of the new scaling provided in [2], we understand the they are both addressing the case where UE measures multiple NGSO satellites on a single carrier because for a single target satellite, UE would only need to measure on 1 DL opportunity. Given above, we suggest to keep one of the scaling to avoid double scaling, and our preference is to keep the Ksatellite which is already there in the spec, and has been considered in the UE implementation.

In the latest spec, Ksatellite is defined as 1 in certain cases.

When the UE is performing measurements in a neighbor satellite, Ksatellite = 1 if the DL opportunities of the cells measured in the neighbor satellite are non-overlapping with the DL opportunities of the serving cell.

In our view, this requirement is also not aligned with R18 NB-IoT NTN assumption. It is going in a contradicting direction compared to the newly agreed scaling, i.e. it will increase the total wake up time which is not favourable for NB-IoT device. Besides, the overlapping and non-overlapping may change dynamically over time in NGSO scenario, and changing requirements over time will also increase UE complexity. Finally, when UE measures multiple neighbour satellites Ksatellite=1 may not work.

Based on above, we suggest to simply re-use the definition of Ksatellite as in existing requirements, and to revert the following agreement to introduce new scaling or new definition.

Proposal 1: Re-use the definition of Ksatellite as in existing requirements. RAN4 to revert the following agreement from RAN4#116, i.e. do not introduce the following scaling:

- Scale Tdetect_intra_NB1-NC and Tmeasure _intra_NB1-NC by a factor of 2, when the UE measures cells across 3 or more different patterns of DL opportunities

RRC Connection redirection to Non-Anchor Carrier (6.9A)

## Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A) Agreement (ad-hoc): - Tperiod_DL_bitmap: It is hardcoded as zero in RRM spec. To be further discussed and concluded in RAN4#116-bis: - Change ‘110 ms’ in the following bullets to an integer number of 90ms + DL-to-UL gap within the 90 ms TDD pattern, where the exact value is TBD upon completion of RAN1 spec. - TRRC_procedure_delay: It is the RRC procedure for processing the received message “CarrierConfigDedicated-NB”. It shall be less than 110 ms. - When the NPUSCH ACK transmission for the received RRC message takes longer than 110ms, the overall RRC connection redirection delay may be extended. - TBD for the values of 10 ms, 40 ms, and 8 ms in the following bullets, and whether to retain the DL-Bitmap-NB IE upon completion of RAN1 and RAN2 spec: - TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. - Option 1: TDL-UL switch is 8 ms. - Option 2: TDL-UL switch is 50 ms (THALES, Iridium)

The existing requirements for re-direction is copied below.

The time delay (Tconnection_redirect_non-anchor) is the time between the end of the last subframe in the repetition period of NPDSCH containing the IE, “CarrierConfigDedicated-NB” received on the anchor carrier and the end of the last subframe in the repetition period of NPUSCH transmitted on the target non-anchor carrier. The time delay (Tconnection_redirect_non-anchor) shall be less than: Tconnection_redirect_non-anchor = TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH - TRRC_procedure_delay: It is the RRC procedure for processing the received message “CarrierConfigDedicated-NB”. It shall be less than 110 ms. - Tperiod_DL_bitmap: It is the periodicity of the downlink subframe configuration for downlink transmission on the non-anchor carrier. It is configured via IE DL-Bitmap-NB [2] and can be 10 ms or 40 ms. - TUL_grant: It is the time required to acquire uplink grant from the non-anchor carrier for transmitting NPUSCH on the non-anchor carrier. The value of TUL_grant depends on Tperiod_DL_bitmap and the number of repetitions of NPDCCH used in the non-anchor carrier. - TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is 8 ms. - TNPUSCH: It is the time required to transmit NPUSCH on the non-anchor carrier. The value of TNPUSCH depends on the number of repetitions of NPUSCH used in the non-anchor carrier. When the NPUSCH ACK transmission for the received RRC message takes longer than 110ms, the overall RRC connection redirection delay may be extended.

For the RRC procedure delay, we suggest to change 110ms to 172ms. In our view, this should be the time period after which UE can start receiving UL grant on the non-anchor carrier, since the Tperiod_DL_bitmap is agreed to be zero. In theory, UE may be able to receive UL grant 82ms after the RRC message, but it is likely UE implementation may have already taken 110ms for processing, and 110ms after the RRC message is guard period, so UE needs to wait for another 62ms for the next DL opportunity.

For the DL-UL switch time, we suggest to change to 50ms based on RAN1 agreement.

Proposal 2: For re-direction requirements

- Change TRRC_procedure_delay from 110ms to 172ms

- Change TDL-UL switch from 8ms to 50ms.

Conclusions

In this paper we provided our views on remaining issues in RRM core requirements for IoT NTN TDD.

Proposal 1: Re-use the definition of Ksatellite as in existing requirements. RAN4 to revert the following agreement from RAN4#116, i.e. do not introduce the following scaling:

- Scale Tdetect_intra_NB1-NC and Tmeasure _intra_NB1-NC by a factor of 2, when the UE measures cells across 3 or more different patterns of DL opportunities

Proposal 2: For re-direction requirements

- Change TRRC_procedure_delay from 110ms to 172ms

- Change TDL-UL switch from 8ms to 50ms.

Reference

- R4-2512149, WF on RRM requirements for IoT_NTN_TDD, Moderator (Qualcomm Incorporated)

- R4-2511280, On the measurement framework for the new IOT NTN TDD Punctured frame, Nokia