3GPP TSG-RAN4 Meeting #116bis	R4-2514198

Prague, Czech Republic, 13 October – 17 October 2025

Source:	Ericsson, Odido

Title:	TP for TR 38.719-03-01 adding CA_n7A-n20A-n28A

Agenda item:			5.3.4

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.719-03-01 to include CA_n7A-n20A-n28A as defined in WID [1].

# ---Start of changes---

## 5.x	CA_n7-n20-n28

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n20A-n28A | CA_n7A-n20A CA_n7A-n28A CA_n20A-n28A | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |

#### 5.x.1.3	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c values are derived from CA_n5-n7-n28 and given in the tables below.

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n20-n28 | 0.5 | 0.3 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n7-n20-n28 | - | - | 0.2 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	UE co-existence studies

##### 5.x.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.x.2.1.1-1, Table 5.x.2.1.1-2 and Table 5.x.2.1.1-3 provide the two UL bands with one CC per band IMD interference analysis for CA_n7A-n20A-n28A with UL CA_n7A-n20A, UL CA_n7A-n28A and UL CA_n20A-n28A.

Table 5.x.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 832 | 862 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1738 | 1638 | 3332 | 3432 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4308 | 4138 | 906 | 776 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5832 | 6002 | 4164 | 4294 |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6638 | 6878 | 74 | 86 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3476 | 3276 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8332 | 8572 | 4996 | 5156 |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6664 | 6864 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 758 | 948 | 9138 | 9448 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2644 | 2414 | 6046 | 5776 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5828 | 6018 | 10832 | 11142 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7496 | 7726 | 9164 | 9434 |

Based on the above table, the 3rd or the 5th IMD generated by UL CA_n7A-n20A may fall into own Rx of Band n28.

Table 5.x.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1867 | 1752 | 3203 | 3318 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4437 | 4252 | 1164 | 1004 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5703 | 5888 | 3906 | 4066 |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6752 | 7007 | 461 | 256 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3734 | 3504 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8203 | 8458 | 4609 | 4814 |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6406 | 6636 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 242 | 492 | 9252 | 9577 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3031 | 2756 | 6304 | 6004 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5312 | 5562 | 10703 | 11028 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7109 | 7384 | 8906 | 9206 |

Based on the above table, there is no IMD issue generated by UL CA_n7A-n28A to Band n20.

Table 5.x.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 159 | 84 | 1535 | 1610 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1021 | 916 | 544 | 664 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2367 | 2472 | 2238 | 2358 |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1748 | 1883 | 1247 | 1412 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 318 | 168 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3199 | 3334 | 2941 | 3106 |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 3070 | 3220 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1950 | 2160 | 2580 | 2745 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 385 | 580 | 1180 | 1000 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3644 | 3854 | 4031 | 4196 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3773 | 3968 | 3902 | 4082 |

Based on the above table, the 5th IMD generated by UL CA_n20A-n28A may fall into own Rx of Band n7.

#### 5.x.2.2	REFSENS requirements

Based on Table 5.x.2.1.1-2, the 3rd and 5th IMD generated by UL CA_n7A-n20A may fall into own Rx of Band n28. MSD value is derived from DC_7A-28A_n20A.

Based on Table 5.x.2.1.1-3, the 5th IMD generated by UL CA_n20A-n28A may fall into own Rx of Band n7. MSD value is derived from DC_7A-28A_n20A-

| Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurationsBand / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n20-n28 | n7 | N/A | 5 | N/A | 2640 | 5.9 | FDD | IMD5 |
|  | n20 | 842 | 5 | 25 | 801 | N/A | FDD | N/A |
|  | n28 | 728 | 5 | 25 | 783 | N/A | FDD | N/A |
|  | n7 | 2505 | 5 | 25 | 2625 | N/A | FDD | N/A |
|  | n20 | 859 | 5 | 25 | 818 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 787 | 17.4 | FDD | IMD31 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

---End of changes---

# Reference

[1] 	 RP-252440. Revised WID NR_CADC_SUL_R19, Ericsson