3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2514207

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	General discussion on RRM requirements for LB CA switching enhancement

Agenda item:			7.8.4

Document for:	Discussion

- Introduction

In RAN #109, the following objectives for enhancement of low band (LB) carrier aggregation (CA) via switching were agreed in RP-252953, the details are copied as below

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

In this paper, we would like to give a recap from Rel-19 LB-CA switching, and share some preliminary understandings in terms of impact on RRM requirements for LB CA switching enhancement.

- Background on Rel-19 LB-CA switching

  - What we discussed in Rel-19

In Rel-19, we thoroughly discussed what RRM core requirements shall be impacted to allow switching between {case 1, case 2} to enable low band carrier aggregation via switching, wherein, the two cases refer to:

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1.

To analyse the impact on RRM requirements of SDL SCC and FDD PCC, the group achieved a consensus on the generic assumptions first, which are shown in Table 1. And then the applicabilities of RRM requirement and switching pattern were confirmed, which are shown in Table 2. Furthermore, regarding the applicability of switching pattern for the SCell which is in activation procedure, when UE to apply the RRC configured switching pattern for MAC-CE based SCell activation and RRC based direct SCell activation are pending on further discussion.

To be more specific, in order to successfully perform L1/L3 measurement complying with different time relationship of measurement occasions and LB-CA switching pattern no matter for FDD PCell or SDL SCell, the restriction of overlapped occasions of measurement resources (i.e., SMTC window and L1 RS) and FDD PCell/SDL SCell reception duration is defined.

Observation 1: In Rel-19 LB CA, no matter for FDD PCell or SDL SCell, the L1/L3 measurement requirement is applicable to the case that there are overlapped occasions of measurement resources (i.e., SMTC window and L1 RS) and reception duration of the switching pattern.

Table 1. Assumption for Rel-19 LB CA via switching

|  | Assumption |
| --- | --- |
| MRTD | 3us |
| SSB | SSB based requirement as baseline |
| Band combinations | - RAN4 only defines requirements for CA with FDD PCC and one SDL SCC - Example band combinations: CA_n5A-n29A, CA_n12A-n29A, CA_n14A-n29A, CA_n28A-n67A, CA_n29A-n71A |

Table 2. Applicability analysis

| Applicability of switching pattern | - SCell which has been activated - SCell which is in activation procedure |
| --- | --- |
| Applicability of RRM requirement | For gap-less based L3 measurement and L1 measurement of a serving carrier which can be either FDD PCC or activated SDL SCC: - If all {the SSB occasions in SMTC window}or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching patterNo delay extension is allowed - If a fraction of all {the SSB occasions in SMTC window}or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching patterThe delay is extended in accordance to the fraction of available occasions - If no {the SSB occasions in SMTC window}or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching patterThe corresponding requirements do not apply |

  - Comparison of Rel-19 LB CA and Rel-20 LB CA

The case switching of Rel-19 LB CA and Rel-20 LB CA can be illustrated in the following figure

- (b)

Fig. 1 Switching scenario for Rel-19 LB-CA switching (a) and Rel-20 LB-CA switching(b)

In comparison to (a), the most distinct part accounted in (b) is that UE can receive on DL CA (FDD DL carrier and SDL carrier) simultaneously. And to realize such reception scenario, the RF retuning is considered to cover the span of bandwidth across the carriers. Thanks to such dual DL consideration, UE can truly enjoy the higher throughput enabled by SDL SCell scheduling. In RF, the UE receiver architecture to receive the inter-band CA will be discussed in the future meetings.

Observation 2: Compare to Rel-19 LB CA that UE can receive/transmit on the FDD carrier or receive on the SDL carrier in a given slot, in Rel-20 LB CA, UE can receive on the FDD carrier and on the SDL carrier in a given slot simultaneously.

- Discussion

  - RRM working scopes for discussion

3.1.1 Which part of Rel-19 LB-CA RRM requirements can be reused in Rel-20

< Generic assumptions >

By comparing the two WIDs, the following conditions used to derive RRM requirements are the same:

- Deployment constraints except the limitation on no in-gap interference between DL carriers is considered in Rel-20 LB CA

- BC assumption that is only CA with FDD PCC and one SDL SCC is considered

- Semi-static switching pattern based on RRC configuration is considered to trigger the switching-based LB CA

In this sense, from our understanding, the generic assumptions defined in Rel-19 can be reused in Rel-20, including: The 3us maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching; CA with FDD PCC and one SDL SCC; SSB based SDL SCell operation

Proposal 1: To define Rel-20 LB CA RRM requirement, the following generic assumptions defined in Rel-19 can be reused:

- The 3us maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching

- CA with FDD PCC and one SDL SCC

- SSB based SDL SCell operation

< Requirements related to the switching pattern itself >

From our understanding, when UE to perform operation on the SDL carrier to complete a measurement/or data reception procedure, the essential functionality of the configured switching pattern is in principle the same as we discussed in Rel-19. In this sense, we think the following requirements related to the switching pattern itself can be reused in Rel-20 LB CA, including: Applicability of switching pattern and interruption/scheduling restriction requirement for CC switching

Proposal 2: For Rel-20 LB CA, the following RRM requirements related to the switching pattern itself can be inherited from Rel-19

- The switching pattern is applicable to SCell which has been activated and SCell which is in activation procedure

- No interruption/scheduling restriction requirement for CC switching

<SDL SCC related RRM requirement >

The SDL SCC related RRM requirement encompasses: Activated SCell L1/L3 measurement, SCell activation/deactivation and deactivated SCell measurement. To be more specific,

- The main requirement of SCell activation involves: The definition of known/unknown conditions, CSI measurement and reporting, activation time timeline, interruption or scheduling restriction

- The main requirement of deactivated SCell measurement involves: Deactivated SCell measurement delay requirement, interruption requirement including interruption length and interruption rate (i.e., missed ACK/NACK rate).

- The main requirement of activated SCell measurement involves: L1/L3 measurement

- Miscellaneous: TCI state switch delay requirement

From our understanding, from the rationale of switching pattern perspective, the L1/L3 underlying measurement procedure on activated SDL SCC is similar to what we discussed in Rel-19 as long as the key and essential design of the pattern is kept in RAN1. In this sense, the RRM requirement applicability to gap-less based L1/L3 measurement of an activated SDL SCC and the requirement design approach of activated SDL SCell L1/L3 measurement can be inherited from Rel-19. More fine-grained,

- The requirement design for L1 measurement including L1-RSRP/L1-SINR/BFD/CBD: P factor updated

- The requirement design for L3 intra-frequency identification/measurement requirement with MG: Reuse legacy intra-frequency identification/measurement requirement

- The requirement design for L3 inter-frequency measurement and inter-RAT measurement with MG: Reuse the existing measurement requirements with gap

- The requirement design for L3 intra-frequency identification/measurement requirement without MG: Kp factor updated

- The applicability of RRM requirement: Show in Table 2.

Proposal 3: For Rel-20 LB CA, the following RRM requirements on activated SDL SCell L1/L3 measurement can be inherited from Rel-19

- RRM requirement applicability to gap-less based L1/L3 measurement of an activated SDL SCC

- The requirement design for L1 measurement including L1-RSRP/L1-SINR/BFD/CBD

- The requirement design for L3 intra-frequency identification/measurement requirement with MG

- The requirement design for L3 inter-frequency measurement and inter-RAT measurement with MG

- The requirement design for L3 intra-frequency identification/measurement requirement without MG

For SCell activation procedure, the key agreement we achieved in Rel-19 LB CA in terms of known/unknown condition is:

- Known/unknown conditions are reused from existing SCell activation requirements,

from this, we think the same conclusion can be stretched into Rel-20 LB CA.

Proposal 4: For Rel-20 LB CA, for known/unknown conditions during SDL SCell activation, the known/unknown conditions from existing SCell activation requirements can be reused.

For SCell deactivated measurement, the legacy deactivated SCell measurement delay requirements can be applied for UE to perform deactivated SDL SCell measurement since the switching pattern is not applicable to deactivated SCell status.

Proposal 5: For Rel-20 LB CA, the legacy deactivated SCell measurement delay requirements can be applied for UE to perform deactivated SDL SCell measurement.

For TCI state switch delay requirement for SDL SCell, in Rel-19 LB CA, the following principle was agreed

RAN4 #115 Agreement: The L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

From our understanding, in Rel-20 LB CA, the L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier can also follow the agreed principle in Rel-19, the TL1-RSRP and Tfirst-SSB in the formula should to reflect the overlapped occasion restriction, accordingly. In this sense,

Proposal 6: For Rel-20 LB CA TCI state switch delay requirement for SDL SCell, the principle related to the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern can be reused to define the L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier.

3.1.2 Potential RRM requirement impact

<SDL SCC related RRM requirement—SDL SCell activation/deactivation >

Regarding SCell activation, not only SCell activation procedure itself, the activation of switching pattern is also involved, from this, there are four key aspects should be discussed carefully:

- The timeline of switching pattern application, that is the time point of when UE shall apply the RRC configured switching pattern no matter for MAC-CE based SCell activation or RRC based direct SCell activation, which is still suspending in Rel-19

- How/When UE switches to SDL SCell no matter for MAC-CE based SCell activation or RRC based direct SCell activation

- The timeline of SDL SCell activation itself (i.e., delay requirement) including the first available SSB reception and interruption

- CSI measurement and reporting/L1-RSRP reporting

At the same time, the requirements for deactivation of switching pattern is also not stable with the on-going TLBCA in Rel-19 LB CA

Based on such analysis, from our understanding, since the mechanism of reception is different between Rel-20 LB CA and Rel-19 LB CA, details of requirements corresponding to the two releases should have discrepancies even under the same pinciple/framework, for example, in Rel-20 SCell activation procedure, SMTC determination is needed to ensure the cell specific RSs from the active serving cell (FDD PCell) and the SDL SCell being activated or released are available for AGC adjustment, and the CSI measurement and reporting/L1-RSRP reporting is also different taking PCell into consideration. In this sense, for SDL SCell activation activation in Rel-20, we prefer

Proposal 7: For Rel-20 LB CA SDL SCell activation/deactivation requirement

- Wait for more Rel-19 LB CA conclusions on the timeline of switching pattern application

- RAN4 to discuss how to define the first available cell specific RS and CSI measurement/L1-RSRP reporting during SDL SCell activation

- The supported SCell activation scenario in Rel-19 LB CA can be taken as a baseline

<SDL SCC related RRM requirement—Deactivated SCell measurement >

Recap from Rel-19 LB CA regarding interruption requirement for deactivated SCell measurement, the requirements are presented in the following Table

Table 3. RRM requirements impact (Interruption requirement for deactivated SDL SCell measurement)

| Requirement | Impact/Definition |
| --- | --- |
| Interruption length on PCC due to measurements on deactivated SDL | Tuning time in slot level + TSMTC_duation + retuning time in slot level - Tuning time from PCell to SCell/ Returning time from SCell to Pcell is related to LBCA-SwitchingGap-Duration-PCelltoSCell-r19 |
| Missed ACK/NACK rate | Up to X= Y/measCycleSCell, where Y=2* interruption length |

However, from our understanding, in Rel-20 LB CA, no matter interruption length or missed ACK/NACK rate is not the same as what we concluded in Rel-19 LB CA anymore, since normal reception on FDD carrier is introduced.  In this sense, we think the interruption length requirement is to some extent the similar one as for legacy inter-band case, and with this understanding, the interruption length on PCC due to measurements on deactivated SDL in Rel-20 LB CA is irrelevant to TSMTC_duation. And if the definition of interruption length is updated, the missed ACK/NACK rate will be updated, accordingly. Besides, the differences between legacy inter-band case are that 1) in Rel-20 LB CA, actually from RF reception perspective, it is similar to intra-band CA case ; 2) when UE to receive on SDL SCC, uplink transmission on the FDD carrier is not permitted.

Observation 3 : Interruption requirement for deactivated SDL SCell measurement including interruption length and missed ACK/NACK rate defined in Rel-19 LB CA is not applicable to Rel-20 LB CA

Proposal 8: For Rel-20 LB CA interruption requirement of deactivated SDL SCell measurement,

- RAN4 to discuss interruption requirement design of deactivated SDL SCell measurement, including interruption length on PCC and missed ACK/NACK rate

  - The interruption requirement corresponding to legacy inter-band CA case can be taken as the baseline/starting point

<FDD PCC related RRM requirement>

Rel-20 LB CA dual DL reception mechanism presents a different challenge on defining RRM requirement for PCell, to be more specific, not to focus the time relationship of measurement resource occasions (SMTC or RSs for L1 measurement) and available reception duration of LB-CA switching pattern, that is fully overlapping, partially overlapping and non-overlapping, but to focus the case that loss of occasion (s) of reference signal reception on PCC caused by pattern switching, in this sense,  the Rel-19 RRM requirement applicability to gap-less based L1/L3 measurement of an activated FDD PCC is not applicable any more, and the discussion on new RRM requirement applicability to gap-less based L1/L3 measurement of an activated FDD PCC is needed.

For Rel-19 LB CA RLM, there is a FFS point regarding the necessity of introducing a priority rule for faster link recovery when the RLM or BFD is triggered, while in our Rel-20 LB CA case, since UE keeps operating on the FDD carrier, we think even when the RLM/BFD is triggered, there is no need for UE to discard the configured switching pattern and operate on the FDD carrier to receive RS for faster link recovery.

Observation 4: For Rel-20 LB CA FDD PCC related RRM requirement, how to treat the case that loss of occasion (s) of reference signal reception on FDD PCC caused by pattern switching becomes new challenge.

Proposal 9: For Rel-20 LB CA, RAN4 to discuss the RRM requirement applicability to L1/L3 measurement of a FDD PCC, and to define the corresponding RRM requirement design for FDD PCell.

Proposal 10: For Rel-20 LB CA, deprioritizing the discussion on switch pattern applicability requirement for the case that when the RLM or BFD is triggered on the PCell , UE should prioritize to receive RS for faster link recovery.

- Conclusion

In this contribution, we provided our initial viewpoints on RRM requirements for LB CA switching enhancement. The following observations and proposals are obtained:

Observation 1: In Rel-19 LB CA, no matter for FDD PCell or SDL SCell, the L1/L3 measurement requirement is applicable to the case that there are overlapped occasions of measurement resources (i.e., SMTC window and L1 RS) and reception duration of the switching pattern.

Observation 2: Compare to Rel-19 LB CA that UE can receive/transmit on the FDD carrier or receive on the SDL carrier in a given slot, in Rel-20 LB CA, UE can receive on the FDD carrier and on the SDL carrier in a given slot simultaneously.

Observation 3 : Interruption requirement for deactivated SDL SCell measurement including interruption length and missed ACK/NACK rate defined in Rel-19 LB CA is not applicable to Rel-20 LB CA

Observation 4: For Rel-20 LB CA FDD PCC related RRM requirement, how to treat the case that loss of occasion (s) of reference signal reception on FDD PCC caused by pattern switching becomes new challenge.

Proposal 1: To define Rel-20 LB CA RRM requirement, the following generic assumptions defined in Rel-19 can be reused:

- The 3us maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching

- CA with FDD PCC and one SDL SCC

- SSB based SDL SCell operation

Proposal 2: For Rel-20 LB CA, the following RRM requirements related to the switching pattern itself can be inherited from Rel-19

- The switching pattern is applicable to SCell which has been activated and SCell which is in activation procedure

- No interruption/scheduling restriction requirement for CC switching

Proposal 3: For Rel-20 LB CA, the following RRM requirements on activated SDL SCell L1/L3 measurement can be inherited from Rel-19

- RRM requirement applicability to gap-less based L1/L3 measurement of an activated SDL SCC

- The requirement design for L1 measurement including L1-RSRP/L1-SINR/BFD/CBD

- The requirement design for L3 intra-frequency identification/measurement requirement with MG

- The requirement design for L3 inter-frequency measurement and inter-RAT measurement with MG

- The requirement design for L3 intra-frequency identification/measurement requirement without MG

Proposal 4: For Rel-20 LB CA, for known/unknown conditions during SDL SCell activation, the known/unknown conditions from existing SCell activation requirements can be reused.

Proposal 5: For Rel-20 LB CA, the legacy deactivated SCell measurement delay requirements can be applied for UE to perform deactivated SDL SCell measurement.

Proposal 6: For Rel-20 LB CA TCI state switch delay requirement for SDL SCell, the principle related to the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern can be reused to define the L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier.

Proposal 7: For Rel-20 LB CA SDL SCell activation/deactivation requirement

- Wait for more Rel-19 LB CA conclusions on the timeline of switching pattern application

- RAN4 to discuss how to define the first available cell specific RS and CSI measurement/L1-RSRP reporting during SDL SCell activation

- The supported SCell activation scenario in Rel-19 LB CA can be taken as a baseline

Proposal 8: For Rel-20 LB CA interruption requirement of deactivated SDL SCell measurement,

- RAN4 to discuss interruption requirement design of deactivated SDL SCell measurement, including interruption length on PCC and missed ACK/NACK rate

  - The interruption requirement corresponding to legacy inter-band CA case can be taken as the baseline/starting point

Proposal 9: For Rel-20 LB CA, RAN4 to discuss the RRM requirement applicability to L1/L3 measurement of a FDD PCC, and to define the corresponding RRM requirement design for FDD PCell.

Proposal 10: For Rel-20 LB CA, deprioritizing the discussion on switch pattern applicability requirement for the case that when the RLM or BFD is triggered on the PCell and UE should prioritize to receive RS for faster link recovery.

5. Reference

[1] RP-252953, New WID on enhancement of low NR band carrier aggregation via switching

[2] R4-2502588, WF on RRM requirements for NR_LBCA_Sw

[3] R4-2504895, WF on RRM requirements for NR_LBCA_Sw

[4] R4-2508259, WF on RRM requirements for NR_LBCA_Sw

[5] R4-2512119, WF on RRM requirements for NR_LBCA_Sw