3GPP TSG-RAN WG4 Meeting #116-bis 	R4-2514244

Prague, CZ, October 13th – 17th, 2025

Agenda Item: 6.13.3.1

Source: MediaTek inc.

Title: Discussion on performance requirements for SBFD

Document for: Discussion

# Introduction

In the last meeting, RRM group discussed several issues on the impacted RRM specification and whether new RRM requirements are needed to handle the new SBFD operation and CLI handling mechanism, where the issues and agreements are captured in the way forward (WF) [1] and collection of companies proposals in Topic summary [2]. In this contribution, we provide further discussion on the performance part issues captured in the WF.

# Discussion on performance part

# Discussion on accuracy requirements

Issue 3-1-1: L1-SRS-RSRP – side condition on time offset:

The discussion on time offset between DL timing and SRS arrival timing included two options, where the first option is use R16 requirements as baseline, i.e. with cell phase error. While Option 2 is based on removing the cell phase error, resulting in better accuracy requirements. It should be mentioned that Option 2 is only applicable for intra-cell CLI measurements, also, it is worth highlighting that the UE is not aware of whether this measurement is intra-cell CLI or inter-cell CLI.

Our preference is that RAN4 RRM to define a single set of test cases for a specific system parameter, i.e. not to define two sets of test cases for the same parameters, which are with and without cell phase error.

- RAN4 RRM to define test cases either with or without cell phase error but not for both for the same set of parameters.

Issue 3-1-2: L1-SRS-RSRP – side condition on SRS BW:

The discussion on frequency resources should follow the common deployment scenario where a given subband has a BW of approximately 20MHz, hence, the number of PRB should be 48PRBs.

- RAN4 RRM to define side condition on SRS BW to be 48PRBs, same as in R16.

Issue 3-1-3: L1-SRS-RSRP - side condition on SRS Es/Iot:

Regarding the side condition of SRS Es/IoT, reusing R16 CLI SRS RSRP assumption seems to be sufficient, which is ≥1 dB.

- RAN4 RRM to define side condition on SRS Es/Iot to be ≥1 dB, same as in R16.

Issue 3-1-4: L1-SRS-RSRP - side condition on accuracy requirements:

Follow the same conclusion of issue 3-2-1 on time offset, i.e. for a given set of system parameters the test cases to be defined for either intra-cell or inter-cell (not for both). In addition, L1-SRS-RSRP measurements accuracy requirements are defined based on simulation results for the agreed side conditions.

- For a given set of system parameters, RAN4 to define TCs for either intra-cell or inter-cell (not for both).

- L1-SRS-RSRP measurements accuracy requirements are defined based on simulation results for the agreed side conditions.

Issue 3-1-5: L1-SRS-RSSI - side condition on accuracy requirements:

- Recommended WF - Suggest to agree the following: - RAN4 to define accuracy requirements for L1-CLI-RSSI measurements. - FFS on how to define the requirements - Option 1: Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3. - Option 2: other

Recommended WF from the previous meeting could be used as a starting point. Therefore, RAN4 to define accuracy requirements for L1-CLI-RSSI measurements. Also, re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3.

- RAN4 to define accuracy requirements for L1-CLI-RSSI measurements.

- Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and type#2, as well as for Method#3.

# Discussion on report mapping

Issue 3-2-1: Absolute report mapping:

The RRM group reached the following agreements in [R4-2504904]:

#### Issue 1-1-4: Report mapping - Agreement (online session on Monday) - Confirm that the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI, including both absolute and differential reporting, given by RAN1 LS R1-2501620, are reasonable. - Note: In RAN4 understanding, the “Infinite” in Table 10.1.22.1.2-1, TS38.133 is also reused for L1 SRS-RSRP. - RAN4 to define measurement report mapping for L1-SRS-RSRP and L1-CLI-RSSI, including both absolute reporting and differential reporting.

With RAN1 working assumptions:

Working assumption For L1 based CLI measurement and reporting, the following is adopted - For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value. For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value.

Therefore, for absolute report mapping, re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively. In addition, Table 1 and Table 2 (based on Table 10.1.6.1-2) for reporting mapping of differential L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Table 1: Reporting mapping of differential L1-SRS-RSRP

| Reported value | Measured quantity value (difference in measured SRS-RSRP from strongest SRS-RSRP) | Unit |
| --- | --- | --- |
| DIFF_SRS-RSRP_0 | 0≥ΔSRS-RSRP>-2 | dB |
| DIFF_SRS-RSRP_1 | -2≥ΔSRS-RSRP>-4 | dB |
| DIFF_SRS-RSRP_2 | -4≥ΔSRS-RSRP>-6 | dB |
| DIFF_SRS-RSRP_3 | -6≥ΔSRS-RSRP>-8 | dB |
| DIFF_SRS-RSRP_4 | -8≥ΔSRS-RSRP>-10 | dB |
| DIFF_SRS-RSRP_5 | -10≥ΔSRS-RSRP>-12 | dB |
| DIFF_SRS-RSRP_6 | -12≥ΔSRS-RSRP>-14 | dB |
| DIFF_SRS-RSRP_7 | -14≥ΔSRS-RSRP>-16 | dB |
| DIFF_SRS-RSRP_8 | -16≥ΔSRS-RSRP>-18 | dB |
| DIFF_SRS-RSRP_9 | -18≥ΔSRS-RSRP>-20 | dB |
| DIFF_SRS-RSRP_10 | -20≥ΔSRS-RSRP>-22 | dB |
| DIFF_SRS-RSRP_11 | -22≥ΔSRS-RSRP>-24 | dB |
| DIFF_SRS-RSRP_12 | -24≥ΔSRS-RSRP>-26 | dB |
| DIFF_SRS-RSRP_13 | -26≥ΔSRS-RSRP>-28 | dB |
| DIFF_SRS-RSRP_14 | -28≥ΔSRS-RSRP>-30 | dB |
| DIFF_SRS-RSRP_15 | -30≥ΔSRS-RSRP | dB |

Table 2: Reporting mapping of differential L1-CLI-RSSI

| Reported value | Measured quantity value (difference in measured CLI-RSSI from strongest CLI-RSSI) | Unit |
| --- | --- | --- |
| DIFF_CLI-RSSI_0 | 0≥ΔCLI-RSSI>-2 | dB |
| DIFF_CLI-RSSI_1 | -2≥ΔCLI-RSSI>-4 | dB |
| DIFF_CLI-RSSI_2 | -4≥ΔCLI-RSSI>-6 | dB |
| DIFF_CLI-RSSI_3 | -6≥ΔCLI-RSSI>-8 | dB |
| DIFF_CLI-RSSI_4 | -8≥ΔCLI-RSSI>-10 | dB |
| DIFF_CLI-RSSI_5 | -10≥ΔCLI-RSSI>-12 | dB |
| DIFF_CLI-RSSI_6 | -12≥ΔCLI-RSSI>-14 | dB |
| DIFF_CLI-RSSI_7 | -14≥ΔCLI-RSSI>-16 | dB |
| DIFF_CLI-RSSI_8 | -16≥ΔCLI-RSSI>-18 | dB |
| DIFF_CLI-RSSI_9 | -18≥ΔCLI-RSSI>-20 | dB |
| DIFF_CLI-RSSI_10 | -20≥ΔCLI-RSSI>-22 | dB |
| DIFF_CLI-RSSI_11 | -22≥ΔCLI-RSSI>-24 | dB |
| DIFF_CLI-RSSI_12 | -24≥ΔCLI-RSSI>-26 | dB |
| DIFF_CLI-RSSI_13 | -26≥ΔCLI-RSSI>-28 | dB |
| DIFF_CLI-RSSI_14 | -28≥ΔCLI-RSSI>-30 | dB |
| DIFF_CLI-RSSI_15 | -30≥ΔCLI-RSSI | dB |

- Re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively.

- Use Table 1 and Table 2 (based on Table 10.1.6.1-2 and attached in this document) for reporting mapping of differential L1-SRS-RSRP and L1-CLI-RSSI, respectively.

# Discussion on test cases list

Issue: Test case list:

The following RRM core requirements are impacted due to the work on SBFD:

- L1 CLI measurement: test cases are needed to verify the new defined CLI measurements.

  - L1-SRS-RSRP measurements

  - L1-CLI-RSSI measurements

- RLM/BFD/CBD requirements: test cases are needed to verify the collision between measurement resources and UL transmission.

- L1 SINR and L1 RSRP measurements and reporting: test cases are needed to verify the collision between measurement resources and UL transmission.

Furthermore, there is no need to define test cases for the following scenarios:

- SSB L1/L3 measurements: this is because SSB symbols are not impacted and hence legacy test cases can be applied with no changes needed.

- CSI-RS L1-SINR measurement: there is already a test case for CSI-RS L1-RSRP measurement and hence there is no need to have a semi-duplicate test case.

- RACH: There is no change in RRM requirements hence it is not clear why a new test case is needed.

Therefore, test cases should be defined for the following topics:

- L1-SRS-RSRP measurement (delay and accuracy)

- L1-CLI-RSSI measurement (delay and accuracy)

- CSI-RS RLM

- CSI-RS BFR

- CSI-RS L1-RSRP measurement (delay)

And to exclude the following test cases:

- SSB L1/L3 measurements

- CSI-RS L1-SINR measurement

- RACH

- RAN4 RRM to define test cases for the following topics: (i) L1-SRS-RSRP measurement (delay and accuracy), (ii) L1-CLI-RSSI measurement (delay and accuracy), (iii) CSI-RS RLM, (iv) CSI-RS BFR, and (v) CSI-RS L1-RSRP measurement (delay).

- RAN4 RRM to exclude defining test cases for the following topics: (i) SSB L1 and L3 measurements, (ii) CSI-RS L1-SINR measurement; and (iii) RACH.

# Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 RRM to define test cases either with or without cell phase error but not for both for the same set of parameters.

Proposal 2: RAN4 RRM to define side condition on SRS BW to be 48PRBs, same as in R16.

Proposal 3: RAN4 RRM to define side condition on SRS Es/Iot to be ≥1 dB, same as in R16.

Proposal 4: For a given set of system parameters, RAN4 to define TCs for either intra-cell or inter-cell (not for both).

Proposal 5: L1-SRS-RSRP measurements accuracy requirements are defined based on simulation results for the agreed side conditions.

Proposal 6: RAN4 to define accuracy requirements for L1-CLI-RSSI measurements.

Proposal 7: Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and type#2, as well as for Method#3.

Proposal 8: Re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Proposal 9: Use Table 1 and Table 2 (based on Table 10.1.6.1-2 and attached in this document) for reporting mapping of differential L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Proposal 10: RAN4 RRM to define test cases for the following topics: (i) L1-SRS-RSRP measurement (delay and accuracy), (ii) L1-CLI-RSSI measurement (delay and accuracy), (iii) CSI-RS RLM, (iv) CSI-RS BFR, and (v) CSI-RS L1-RSRP measurement (delay).

Proposal 11: RAN4 RRM to exclude defining test cases for the following topics: (i) SSB L1 and L3 measurements, (ii) CSI-RS L1-SINR measurement; and (iii) RACH.

# References

- R4-2512133, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, India, Aug 2025.

- R4-2509061, “Topic summary for [116][218] NR_duplex_evo”, Huawei, HiSilicon, India, Aug 2025.