3GPP TSG-RAN WG4 Meeting #116bis	R4-2514257

Prague, Czech Republic, 13-17 October, 2025

Source:	Nokia

Title:	On remaining core requirements for RRM Phase 5

Agenda item:	4.15.1

Document for:	Discussion

Introduction

In this contribution we discuss the remaining open issues related to L3 beam sweeping factor reduction.

Discussion

Fast beam sweeping

RAN4 concluded the core part of the RRM Ph5 work item in the last meeting. Regarding fast L3 beam sweeping, RAN4 agreed that the UE is required to perform FBS for L3 measurements for a duration of a timer of 90 s after the configured condition for FBS becomes met.

What remained open was whether there is a need for a re-entry condition after the timer has expired.

Issue 1-1-2: Re-evaluation mechanism Agreement: FFS following options in the maintenance stage: - Option 1: Introduce a predefined time duration T2, and UE re-evaluates the activation condition after this predefined time duration expires, where this time duration starts after UE quits from Multi-Rx mode. Value of T2 can be: - Alt 1: Use the values for T1 as a starting point. - Alt 2: 5 s, which is predefined in RAN4 spec. - Option 2: It is up to UE implementation when to re-enter the FBS after quitting the last FBS. - Option 3: It is up to UE implementation when to re-evaluation of the activation condition for FBS after quitting the last FBS. Agreement in Friday online: - No consensus to introduce a predefined time duration T2 in the WI core part in RAN4.

In our view, defining a requirement for re-evaluating the triggering condition does not seem needed, as the UE is anyways continuously performing measurements and knows whether the condition is met or not.

The following cases exist:

- UE stops FBS at timer expiration and the condition after some time is not met any longer (UE moves back to cell center)

- UE exits FBS at timer expiration and the condition continues to be met (UE stays at the cell edge)

In the first scenario, UE shall re-enter FBS when the condition becomes met again (UE moves back to cell edge). However, we do not think this counts as re-entry as it is the same scenario as entry condition becoming fulfilled (again), where the UE would naturally need to restart FBS. Hence, this scenario is already covered.

After timer expiration, if the FBS condition stops being met and is met again, the UE shall re-enter FBS. This is already covered and has no specification impact.

Regarding the second scenario, it has been proposed that after a second timer, the UE would re-evaluate the condition and re-enter FBS. In our view it is questionable how a timer would benefit the UE or network in this case. If the UE stays stationary and network does not send a handover command, it does not matter whether the UE performs fast or normal beam sweeping. UE re-entering FBS would be useful rather if the UE moves even closer to the cell edge and handover becomes even more urgent. However, a second timer does not help to define such event. If a re-entry condition is to be defined, we think a second signal level (threshold) would be more informative than a second timer. However, we think such condition is not mandatory.

After timer expiration, if the FBS condition continues to be met, we prefer not to define a re-entry condition for FBS, but if a rule for re-entry to FBS is needed, a second signal level based threshold would be more useful than defining a second timer.

A second issue that was raised regarding behaviour in different bands is shown below:

### Issue 1-1-4: Requirements when UE reports different BSF values for different bands Way forward: Further discuss this issue in the maintenance stage: - Option 1: - It is proposed to discuss how to specify the measurement requirements when multiple bands are involved and UE reports different BSF values for different bands. One simple option is to specify the requirement based on the largest BSF value reported over all the bands where an MO is configured.

The way we understand option 1 is that it is a relaxation of the measurement delay, where the worst BSF will dominate the required measurement delay. We don’t think this is necessary. In existing requirements, the measurement delay requirement is defined per measurement object. For example, if the SMTC periodicity for different measurement objects is different, the MO with larger SMTC period will have a longer measurement delay. It is clear that the requirements consider the SMTC periodicity of each measurement object individually, as it can be observed in the following paragraph from 38.133 clause 9.3.1:

Existing measurement delay requirements consider different measurement delay for each measurement object.

The same applies here. If the UE is configured with 2 MOs for different bands, and on the first band the UE reports support of BSF = 6, while in the second band it reports support of fastRx-BSF-MeasDelayReduction-r19 = 2, it is only natural that the measurement delay will be faster for the second band.

When multiple bands are involved, the measurement requirement is calculated as in legacy requirements, i.e. per measurement object.

The last open FFS point from previous meeting is the following:

### Issue 1-1-5: Revision of the agreement on applicability requirement Agreement in Tuesday ad-hoc: - The FBS enhancement applies for the case when the target and serving carrier frequency are same. - FFS in maintenance which CC to apply the FBS when the target and serving carrier frequency are on different. Agreement in Thursday online: When the target and serving carrier frequency are different, - FBS requirement does not apply to inter-frequency measurement without measurement gap. - FFS inter-frequency measurement with per-FR measurement gap. - Note: inter-RAT is not in the scope.

In the case of the FFS in issue 1-1-5, the UE will have a dedicated gap for performing the measurements. In this case, is it not clear what would be the limitation of performing inter-frequency measurement. Since the UE is not receiving data during the gap, it should have the multiple Rx available for performing the measurements within the gap. Furthermore, we already specified requirements for measurements with gaps, so it is not clear why a concern is raised specifically on measurements without gaps.

FBS applies for inter-frequency measurement with per-FR measurement gap.

  - Fast Scell activation with EMR

    - UE capability

Issue 2-1-1: UE capability for EMR-based fast SCell activation

Agreement: - Introduce one single UE capability for R19 SCell activation based on EMR, further discuss the detail.

What is currently left open is the capability details. From the agreements it’s clear that both EMR and reselection capabilities need to be a pre-requisite.

Agreement: - The fast SCell activation delay requirements are defined for the case when - the UE supports Rel-18 eEMR: - Including both EMR and cell reselection measurement,

For Fast Scell with EMR capability, add both measValidationReportEMR-r18 and measValidationReportReselectionMeasurements-r18 as pre-requisite.

To be able to report SSB index in the measurement report, the following capability needs to be supported.

idleInactiveNR-MeasBeamReport-r16 Indicates whether the UE supports beam level measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding beam measurement results upon network request as specified in TS 38.331 [9]. A UE supports this feature shall also support idleInactiveNR-MeasReport-r16. If this parameter is indicated for FR1 and FR2 differently, each indication corresponds to the frequency range of measured target cell.

The following is specified:

38.133 If the SCell belongs to FR1, Tactivation_time is TFirstSSB_MAX + Trs + 5ms provided that the side condition Ês/Iot ≥ -2dB is fulfilled, if the following conditions are met: - During the period equal to 5s for FR1 before the reception of the SCell activation command: - the UE has sent a measurement report with SSB index according to the reporting requirements in 4.7.3 or 5.8.3.

For Fast Scell with EMR capability, add idleInactiveNR-MeasBeamReport-r16 as pre-requisite for the capability for FR1 and FR2.

Conclusion

The following Observations and Proposals were made:

Proposal 1: After timer expiration, if the FBS condition stops being met and is met again, the UE shall re-enter FBS. This is already covered and has no specification impact.

Proposal 2: After timer expiration, if the FBS condition continues to be met, we prefer not to define a re-entry condition for FBS, but if a rule for re-entry to FBS is needed, a second signal level based threshold would be more useful than defining a second timer.

Observation 1: Existing measurement delay requirements consider different measurement delay for each measurement object.

Proposal 3: When multiple bands are involved, the measurement requirement is calculated as in legacy requirements, i.e. per measurement object.

Proposal 4: FBS applies for inter-frequency measurement with per-FR measurement gap.

Proposal 5: For Fast Scell with EMR capability, add both measValidationReportEMR-r18 and measValidationReportReselectionMeasurements-r18 as pre-requisite.

Proposal 6: For Fast Scell with EMR capability, add idleInactiveNR-MeasBeamReport-r16 as pre-requisite for the capability for FR1 and FR2.