3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514258

Prague, CZ, 13th – 17th October 2025

Source:	Nokia

Title:	Discussion on RRM performance requirements for Rel-19 LBCA-sw

Agenda item:	6.3.2

Document for:	Discussion

Introduction

In this contribution we discuss the RRM testing aspects of LBCA.

Discussion

The list of impacted requirements as per the WF from RAN4 #115 is shown below:

| Requirement | Whether or not RAN4 will have requirement | Note |
| --- | --- | --- |
| interruption requirement for CC switching | No (can be revisited if not aligned with RAN1 further agreement.) | Issue 1-2-1, Agreed on Tuesday online session |
| Activated SDL SCell L3 measurement | Yes | Issue 1-3-1, Agreed on Tuesday online session |
| Deactivated SDL SCell measurement [and corresponding interruption] | Yes | Issue 1-3-3, Agreed on Tuesday online session |
| SDL SCell activation/deactivation and corresponding interruption | Yes |  |
| [BFD/CBD] | Yes | Related with issue 1-3-10 |
| TCI state switch delay | Yes | Related with issue 1-3-11 |
| PL-RS switch delay | No | Related with issue 1-3-12 |
| L1-RSRP/SINR | Yes | Related with issue 1-3-13 |

Considering the list above, RAN4 should define a minimum set of test cases that guarantee enough test coverage for LBCA-sw.

Regarding measurement requirements, RAN4 should verify the measurement delays considering that a switching pattern is applied. As part of those requirements, RAN4 has to verify at least one L3 measurement case where LBCA-switching pattern is applied, and one case after LBCA-sw stops. The second case for measurement delay is the L1-RSRP, and the measurement delay extension due to LBCA switching pattern.

Regarding signalling characteristics, it is crucial to have performance requirements with the activation of the switching pattern after SCell activation. We have requirements for two main cases , MAC-CE SCell activation delay and RRC-based direct activation. When verifying that, we should also verify that the switching pattern is applied following the requirements.

When agreeing on a test case list, all the impacted requirements will need to be tested in a setup where the UE is configured with low band FDD PCell and low band SDL SCell with a switching pattern. For many test cases, as the assumption is that the pattern takes measurements on both cells into account, the impact compared to the existing test cases is mostly in the configuration. UE shall pass the applicable requirements with the LBCA setup.

RAN4 to consider the following list as the starting point for the discussion of test case list:

  - Intra-frequency measurements without gaps of PCell target with activated LBCA-sw SCell

  - SSB-based L1-RSRP

  - Scell activation delay

  - Direct SCell activation delay

  - SCell deactivation

  - Interruptions during deactivated SDL SCell measurements

Additionally, for the SCell activation requirements, RAN4 should verify the start of the switching pattern.

RAN4 to verify the start/end of the switching pattern on SCell activation/deactivation test cases.

- Conclusion

The paper has presented Nokia’s position regarding RRM performance requirements for LBCA switching in Rel 19. As part of this discussion, the following proposals are made:

Proposal 1: RAN4 to consider the following list as the starting point for the discussion of test case list:

a.	Intra-frequency measurements without gaps of PCell target with activated LBCA-sw SCell

b.	SSB-based L1-RSRP

c.	Scell activation delay

d.	Direct SCell activation delay

e.	SCell deactivation

f.	Interruptions during deactivated SDL SCell measurements

Proposal 2: RAN4 to verify the start/end of the switching pattern on SCell activation/deactivation test cases.