# 3GPP TSG RAN WG4 Meeting #116bis 														R4-2514302

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.8.4

Source: 	Ericsson

Title:	RRM requirements for low band carrier aggregation via switching

Document for:	Discussion

# Introduction

New WID [1] on Low band carrier aggregation via switching is agreed in RAN#106. In this contribution, we provide our views on the potential RRM requirements impact.

# Discussion

In Rel-19, switching occurs between the following cases (e.g., FDD = n5/n12; SDL = n29):

- Case 1: Tx/Rx on FDD carrier, no Rx on SDL carrier

- Case 2: Rx on SDL carrier, no Tx/Rx on FDD carrier

To enhance low NR band carrier aggregation, support for dual RX via switching was agreed in RAN#109. The corresponding WID is referenced below.

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

According to the WID, the key enhancement is dual RX reception, covering the following cases:

- Case 1: Tx/Rx on FDD carrier 1, no Rx on SDL carrier 2

- Case 3: Rx on both FDD carrier 1 and SDL carrier 2, no Tx on FDD carrier 1

The WID proposes reusing the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching. However, it remains unclear whether the existing pattern can be reused as it is.

In Rel-19, the semi-static switch pattern is 40 bits long, with each slot assigned to either FDD or SDL operation. In Rel-20, when the UE switches between Case 1 and Case 3, the pattern must indicate whether a slot is for:

- FDD Tx/Rx (Case 1), or

- FDD Rx + SDL Rx with no FDD Tx (Case 3)

This difference is illustrated in the tables below.

| Slot No# | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel-19 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX |  |  |  |  |  |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |
| Rel-20 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD RX | FDD RX | FDD RX | FDD RX | FDD RX |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |

| Slot No# | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel-19 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX |  |  |  |  |  |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |
| Rel-20 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD RX | FDD RX | FDD RX | FDD RX | FDD RX |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |

| Slot No# | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel-19 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX |  |  |  |  |  |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |
| Rel-20 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD RX | FDD RX | FDD RX | FDD RX | FDD RX |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |

| Slot No# | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel-19 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX |  |  |  |  |  |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |
| Rel-20 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD RX | FDD RX | FDD RX | FDD RX | FDD RX |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |

To check if the pattern can be used as is, we copy the RAN1 spec below for the reference.

A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot. A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the SCell in the second slot. The first slot of the bitmap is same as the first slot of a system frame with SFN 0 on the PCell. The SCS for all configured DL BWPs or UL BWPs on both the PCell and the SCell is 15 kHz and the periodicity of the switching pattern is 40 slots.

As shown in the tables above, the interpretation of the pattern from Rel-19 to Rel-20 varies at the UE. Therefore, we believe RAN1/2 should clearly define how the pattern is to be interpreted at the UE and specify where the switch should occur. To assess the impact of potential RAN1 changes, we illustrate how these changes might be reflected.

- The interpretation of the text highlighted in this color may need to be revised as: The UE can either transmit/receive on the PCell in a slot or receive on the PCell and SCell in the slot,

- The interpretation of the text highlighted in this color may need to be revised as:A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the PCell and SCell in the second slot.

RAN4 should discuss how to involve RAN1 in the work item. Given the limited impact to RAN1, we could identify the potential impact to RAN1 and send a LS to RAN1, allowing them to introduce the necessary changes accordingly. Alternatively, the scope of RAN1/2 could be added to the WID in the upcoming RAN meeting. As the changes from RAN1 are small, we think sending a LS may be a better option as per opur understanding.

RAN4 Requirements:

SDL SCell requirements:

Regarding RAN4 requirements for SDL SCell, when comparing the pattern design differences between Rel-19 and Rel-20 as shown in the table above, the SDL behavior remains unchanged. Specifically, there are slots during which the UE cannot receive on SDL SCell as per the switching pattern design. In Rel-19, RAN4 already captured these missed reception occasions in the SDL SCell requirements, and we believe these can be reused for Rel-20.

One SDL SCell requirement that may need more discussion is the SCell activation delay. In Rel-19, due to the restriction that the FDD PCell and SDL SCell cannot be received simultaneously, the activation delay requirements were based on SMTC and SSB, rather than SMTC_max and SSB_max. However, when simultaneous reception of FDD PCell and SDL SCell is possible, AGC may be performed during overlapping SMTC and SSB occasions, as per legacy behavior.

Now, we have two options for the Rel-20 LB CA via switching:

- Define both legacy and Rel-19-based SDL SCell activation requirements, considering both approaches.

- Simplify UE implementation for Rel-19 UEs which also may support Rel-20 by adopting the Rel-19 method of SDL SCell activation and use SSB and SMTC.

RAN4 can further discuss whether to introduce/retain legacy-based SDL SCell activation requirements or simply reuse the Rel-19 SDL SCell activation requirements based on SSB and SMTC.

- RAN4 to use the Rel-19 SDL SCell activation requirements which consider the SSB and SMTC as baseline. In addition to baseline requirements, RAN4 to discuss if it is necessary to also support the Rel-15 design based on SSB_MAX and SMTC_MAX.

FDD PCell requirements:

For FDD PCell requirements, in Rel-19, RAN4 mainly focused on DL reception and discussed the impact of switching patterns on FDD PCell requirements such as L3, L1 (L1-RSRP/L1-SINR/BFD/CBD), RLM, and interruptions caused by deactivated SDL SCell measurements on the PCell. Since the UE can receive simultaneously on FDD and SDL, the interruption requirements during deactivated SCell measurements may need to be revisited.

In the current requirements, it was assumed that the UE cannot receive PDSCH on FDD during deactivated SDL measurements, potentially leading to missed ACKs. However, with dual DL reception capability, the UE can maintain DL reception on FDD, and only missed UL transmission opportunities need to be considered. Therefore, the interruption requirement related to deactivated SDL SCell measurements may require further discussion.

- Interruption on PCell due to deactivated SCell measurements shall be revisited.

- Other SDL SCell requirements such as L3 and L1 measurements for the activated SCell, BFD/CBD, TCI state switching can be reused from the Rel-19.

Other requirements such as L3, L1-RSRP, L1-SINR, BFD, CBD, and RLM, which have been agreed upon in Rel-19, can be reused. In Rel-19, when there were missed RS occasions, we extended the delay requirements for the above-mentioned requirements. The scenarios considered to deine the requirements are listed in the table below for reference.

Agreement from RAN4#116: - for gap-less based L3 measurement and L1 measurement of a serving carrier which can be either FDD PCC or activated SDL SCC: a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed. b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions. c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

One may initially assume that there is no impact on FDD reception due to dual DL capability, and therefore no need to extend the requirements in any case. However, a closer examination suggests that switching gaps could still result in missed SSB or CSI-RS occasions, potentially leading to the reuse of the same requirements. To illustrate this, we discuss the switching gap for Rel-19 and the potential switching gap for Rel-20.

Switching Gap Mechanism (Rel-19):

Figure 1: Switching gap mechanism in Rel-19

The RAN1-agreed switching gap mechanism in Rel-19 is shown in Figure 1. The figure illustrates the switching location and the switching gap length, which is configurable by the NW. The configuration allows the following values for the N1 and N2:

- N1: Configurable between 1 to 3 symbols

- N2: Configurable between 1 to 30 symbols

Switching Gap Mechanism (Rel-20):

If we reproduce the same figure for dual DL, we observe that N2 applies to switching from Case 3 to Case 1 (i.e., dual DL to FDD). Since N2 can span up to 30 symbols (i.e., ~3 slots), RS occasions such as SSB and CSI-RS may be missed.

Figure 2: Potential switching gap mechanism in Rel-20

- The RAN4 RRM requirements for the FDD PCell, covering L3, L1-RSRP, L1-SINR, BFD, CBD, and RLM —can be reuse the requirements defined for the Rel-19 LB CA.

Based on the Figures 1 and 2, we think the switching behaviour or interpretation may need to be changed in the RAN1 spec for the UE supporting Rel-20 LB CA via switching. To look at the potential changes, we provide the RAN1 spec below for reference.

When the UE is provided LBCA-SwitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration

To assess the impact of potential RAN1 changes in the switching mechanism, we illustrate how these changes might be reflected in bold text. Please note that these are only for illustrative purposes and the actual changes are upto RAN1.

- The interpretation of the text highlighted in this color may need to be revised as: a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell DL carrier and  paired PCell UL carrier to the PCell DL carrier and SCell DL carrier over the first duration, and

- The interpretation of the text highlighted in this color may need to be revised as: a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the PCell DL carrier and SCell DL carrier to the PCell DL carrier and  paired PCell UL carrier over the second duration

- From RAN4 perspective, the following would need to be accounted for by RAN1 for completing the LBCA via switching feature with switching between Case 1 and Case 3:

- A ‘1’ in the switching pattern bitmap configured to the UE indicates that the UE is receiving on both FDD DL and SDL carriers instead of only on the SDL carriers.

- N1 represents the switching gap from FDD to dual DL reception

- N2 represents the switching gap from dual DL reception to FDD

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

- Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1

- RAN4 to discuss how to involve RAN1 in the work to complete the feature

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to use the Rel-19 SDL SCell activation requirements which consider the SSB and SMTC as baseline. In addition to baseline requirements, RAN4 to discuss if it is necessary to also support the Rel-15 design based on SSB_MAX and SMTC_MAX.

- Interruption on PCell due to deactivated SCell measurements shall be revisited.

- Other SDL SCell requirements such as L3 and L1 measurements for the activated SCell, BFD/CBD, can be reused from the Rel-19.

- The RAN4 RRM requirements for the FDD PCell, covering L3, L1-RSRP, L1-SINR, BFD, CBD, and RLM —can be reuse the requirements defined for the Rel-19 LB CA.

- From RAN4 perspective, the following would need to be accounted for by RAN1 for completing the LBCA via switching feature with switching between Case 1 and Case 3:

- A ‘1’ in the switching pattern bitmap configured to the UE indicates that the UE is receiving on both FDD DL and SDL carriers instead of only on the SDL carriers.

- N1 represents the switching gap from FDD to dual DL reception

- N2 represents the switching gap from dual DL reception to FDD

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

- Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1

- RAN4 to discuss how to involve RAN1 in the work to complete the feature

# References

[1]		RP-252953: New WID on low band CA via switching