3GPP TSG RAN4 Meeting #116bis		R4-2514334
Prague, Czech Republic, 13th – 18th October, 2025

Source:	Nokia

Title:	Test Cases for DL Coverage Enhancement in NTN

Agenda item:	6.19.3.2

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_PH3). The WID contain a handful of objectives and among them, there is one topic on Downlink Coverage Enhancements. One of the main features discussed to be implemented as part of this feature is an extended SSB periodicity for the main (SA) cell in NTN, to match the periodic “active” and “inactive” periods of the cell. The switching between “active” and “inactive” is created in NTN to handle the limited power availability in the satellite.

In previous RAN4 meeting, the initial discussion on the test cases of such feature was initiated. In the present contribution we discuss the open issues in the WF from previous meeting [1] as well as add our views and proposals for the DL Coverage Enhancement feature test cases.

Discussion

In the previous meeting, RAN4 has discussed the performance requirements and potential test cases for the DL coverage enhancement feature and has reached the following way forward:

Issue 2-2-1: Test scope for downlink coverage enhancement <Way Forward>: - Regarding RRC Re-establishment for the UE capable of coverage enhancement, - Proposal 1: For default SMTC core requirements updates for SSB periodicity extension, RAN4 not to introduce new TC in NTN. - Proposal 2: For the case of SSB Ês/Iot < -8dB and an unknown NR cell core requirements, - Option 1: RAN4 to consider adding a new test case. - Further discuss how to define the test case. - Proposal 1a: Define RRC Re-establishment test case for Serving cell SSB Ês/Iot (dB)<-8 and SSB periodicity 160ms case. - Proposal 1b: Downselect the intra-frequency RRC re-establishment test case to the scenario with SSB Ês/Iot < -8 dB and unknown NR cell. - Proposal 1c: Define test case for RRC Re-establishment with 160ms SSB periodicity and without serving cell timing based on existing test case in A.6.3.2.1.3. - Option 2: RAN4 not to define test case. - Regarding SMTC enhancement related test cases for the UE capable of coverage enhancement, RAN4 to wait for the core requirement progress firstly. - Except for the above, RAN4 not to define other new test cases for the UE capable of coverage enhancement. Issue 2-2-2: Test configuration for downlink coverage enhancement <Way Forward>: - Proposal 1: RAN4 need to discuss whether to introduced new SSB patterns for DL enhancement NTN UE. - Proposal 1a: Introduce new 160ms periodicity SSB pattern for DL CE.

In this document we provide our views on issues 2-2-1 and 2-2-2 above and thereafter we provide our own proposals for the performance test cases.

First of all, regarding issue 2-2-1, regarding the RRC Re-Establishment for UE capable of DL coverage enhancement, the new requirements for RRC Re-establishment only prevents the UE to spend unnecessary energy scanning for cells at 20 ms periods, whereas some cells might be configured with a larger periodicity (160 ms). Therefore, the UE will be aware that some cells are implemented with larger periodicity. On the other hand, there is a functional change that the UEs are required to identify cells under very stringent conditions in this scenario (below -8 dB), which we believe it is possible to investigate.

Introduce test case for intra-frequency RRC re-establishment to the scenario with SSB Ês/Iot < -8 dB and unknown NR cell.

In issue 2-2-1, it is also proposed to not introduce new test cases for SSB periodicity extension (proposal 1). In general, we can agree with the rationality behind this proposal, as they do not affect any functionality at the UE. However, it is interesting to observe whether the UE is capable of adapting its measurement periods and filters to the case where SMTC periodicity is much longer. So, from one point of view, there is no significant functional difference, but on the other hand, it is interesting to check the UE capability to adapt the measurements. In order to compromise between such conflicting views, we propose to select one single test case to be tested with higher SSB periodicity.

In case the UE supports the DL Coverage Enhancement features, at least one test in RRC IDLE and one test in RRC CONNECTED mode shall be performed using the largest SSB periodicity configuration.

Our view provided in the proposal above already indicate that we are supportive of introducing a new test configuration of SSB implementations for NTN that covers for the 160 ms periodicity.  We would like to confirm the proposal in the way forward.

Introduce the new 160 ms periodicity SSB pattern for DL CE in the test case configurations.

Another point that we would like to emphasize is that in this meeting RAN4 is discussing how to capture the adaptation of UE requirements when 2 SMTC periodicities are configured in the same frequency layer. As this is in practice a new functionality it has never been tested before. Besides, it is one of the core changes promoted by RAN2 in their feature design. Therefore, we support the introduction of a new test case with 2 different SMTC periodicities.

Introduce a new test case where there are two configured SMTC periodicities, 20 ms and 160 ms, with different offsets.

For proposal 4, a baseline scenario is provided in Figure 1, where 3 SMTC configurations are provided, using two different SMTC periodicities.

Figure 1 – Example of test configuration with 3 SMTCs with different periodicities and offset

Conclusion

In this paper we discussed the impact of the new measurement framework proposed by RAN2 and RAN1 to handle the extended SSB periodicity in NTN under the topic of DL coverage enhancements. Based on our discussion and analysis, we made the following list of proposals:

Proposal 1: Introduce test case for intra-frequency RRC re-establishment to the scenario with SSB Ês/Iot < -8 dB and unknown NR cell.

Proposal 2: In case the UE supports the DL Coverage Enhancement features, at least one test in RRC IDLE and one test in RRC CONNECTED mode shall be performed using the largest SSB periodicity configuration.

Proposal 3: Introduce the new 160 ms periodicity SSB pattern for DL CE in the test case configurations.

Proposal 4: Introduce a new test case where there are two configured SMTC periodicities, 20 ms and 160 ms, with different offsets.

- References

- R4-2512146, WF on RRM requirements for NR_NTN_Ph3_Part1, RAN WG4 Meeting #116, Bengaluru, India, 25th - 29thAugust, 2025

- on SMTC enhancements, RAN WG2 Meeting #129bis, Wuhan, China, 7 - 11 Apr, 2025