3GPP TSG RAN4 Meeting #116bis		R4-2514344
Prague, Czech Republic, 13th – 18th October, 2025

Source:	Nokia

Title:	Test case configurations for NTN IOT TDD

Agenda item:	6.21.2

Document for:	Discussion

Introduction

In the plenary meeting RAN #105, a new WI for Rel-19 was approved to introduce a TDD frame operation for NTN [1]. The work item is designed to provide extended support of 3GPP NB-IoT features leveraging existing satellite systems deployed in Non-Geostationary Satellite Orbit (NGSO). It also aims at enabling low-complexity payloads for satellite operations that are non-reliant on a diplexer.

To achieve the aforementioned goals, the WID was designed to introduce an NTN-compatible TDD frame, that while based on minimum necessary changes on the NB-IoT NTN FDD specification provides a new approach to TDD periodicity.  RAN4 has analysed the potential impacts and specified the requirements for the new frame structure. Upon the completion of the core requirements, RAN4 needs to create the specification of test cases regarding this new frame structure.

Discussion

The introduction of the IoT NTN TDD frame structure has led RAN4 to revise the applicability of several core requirements. While some of the values adopted for the RRM requirements can be maintained, others had to be revised or updated. But regardless of whether the RRM requirement has been updated or not, the UEs supporting this feature cannot be tested on old requirements in current specifications as there is no provision for the reference channels in the new frame structure in the test configurations.

Because of this, in our view, the focus should be on defining new reference channels utilizing the new frame structure in Annex A.3 of TS 36.133.

RAN4 shall focus on defining new reference channels for NPDSCH, NPDCCH, NPRACH and any other applicable channel in Annex A.3 of TS 36.133.

Assuming that a UE might support the new IOT TDD band while not supporting other FDD bands, this UE will need to be tested in all functionalities. This can be done by reusing the same tests procedures used for FDD bands. The easiest way to achieve this (and also the neatest in terms of speciation) is adding a new configuration – with the new reference channels to be introduced – in every existent test case, and reuse the test description.

For existing NB-IoT NTN test cases, create a new test configuration with the new reference channel models and reuse the test description. The test requirements when the new configuration is used might be adapted to reflect the agreements of the core requirements.

One issue might be raised on whether a UE that supports both frame structures needs to configured according to both of them (basically duplicating the test effort). In our view, this is not needed, as long as the UE is tested sufficiently for each configuration.

RAN4 to agree which tests needs to be tested for each configuration when the UE supports both TDD and FDD frame structures, to avoid the duplication of test effort.

Conclusion

In this paper we discussed the TDD punctured pattern for IoT NTN UEs. The following proposals were made:

Proposal 1: RAN4 shall focus on defining new reference channels for NPDSCH, NPDCCH, NPRACH and any other applicable channel in Annex A.3 of TS 36.133.

Proposal 2: For existing NB-IoT NTN test cases, create a new test configuration with the new reference channel models and reuse the test description. The test requirements when the new configuration is used might be adapted to reflect the agreements of the core requirements.

Proposal 3: RAN4 to agree which tests needs to be tested for each configuration when the UE supports both TDD and FDD frame structures, to avoid the duplication of test effort.

- References

- RP-242415, “New WID on introduction of IoT-NTN TDD mode”, September 2024

- R4-2512149,  WF on RRM requirements for IoT_NTN_TDD,   Moderator (Qualcomm Incorporated),  Bengaluru, India, August 25th – 29th, 2025