3GPP TSG-RAN WG4 Meeting #116bis	R4-2514388

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:	7.4.3.2

Source:	Qualcomm Incorporated

Title: 		RRM enhancements for RedCap UEs

Document for:	Discussion

# Introduction

3GPP has agreed to specify RRM requirements for (e)RedCap UEs as a part of the new WI on NR Radio Resource Management (RRM) Phase 6 [1], where the scope of discussion is limited to:

- RLM/BFD RRM requirements

  - Use R17 RLM/BFD relaxation for legacy UE as a baseline

- Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB)

  - RF requirements, considering example bands: n100, n106, n26, n28 and n85

  - RRM requirements including relaxed RLM/BFD RRM requirements

  - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline

- Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD

In this contribution paper, we provide our initial thoughts on the RRM scope of the WI.

# RLM/BFD measurement relaxations

In Release 17, RLM and BFD measurement relaxation requirements for NR were specified for both SSB and CSI-RS based evaluations, applicable to both PCell and PSCell, and only under DRX cycles shorter than 80 ms. However, RedCap UEs operate under a simplified single-cell scenario (PCell only), and only SSB-based evaluations are supported. So, the scope of the RLM/BFD measurement relaxations shall be limited only to SSB based evaluations for PCell.

Proposal 1: Limit the scope of RLM/BFD measurement relaxations for (e)RedCap UEs to SSB-based evaluations for PCell, and only when the DRX cycle is shorter than 80 ms.

Regarding the relaxation factors, for 2Rx RedCap UEs, the capabilities are comparable to legacy UEs with dual receiver chains. The relaxation factors defined in Release 17 are therefore directly applicable. However, for 1Rx RedCap UEs, evaluation delays are already doubled due to the single receiver chain. Introducing further relaxation may result in excessive delays, potentially impacting link reliability.

Proposal 2: For 2Rx RedCap UEs, reuse the R17 relaxation requirements for both FR1 and FR2.

Proposal 3: For 1Rx RedCap UEs, RAN4 should discuss whether measurement relaxation is necessary.

Lastly, R17 introduced the UE feature rlm-Relaxation-r17and network signalling goodServingCellEvaluationRLM and goodServingCellEvaluationBFD. We think the UE feature and the network signallings from R17 can be re-used for RedCap UEs and no need to define new signaling.

Proposal 4: Re-use the UE feature rlm-Relaxation-r17and network signalling goodServingCellEvaluationRLM and goodServingCellEvaluationBFD for RedCap UEs. No new signaling is needed.

# Support of less than 5MHz Channel BW

In Release 18, RRM requirements for non-RedCap UEs operating with less than 5 MHz channel bandwidth were defined. These can serve as a baseline for (e)RedCap UEs, with differentiation based on receiver configuration – R18 requirements can be directly applied to 2Rx RedCap UEs but simulations are necessary to specify the requirements for 1Rx RedCap UEs.

Proposal 5: For 2Rx RedCap UEs, the R18 RRM requirements for non-RedCap UEs operating with less than 5 MHz channel bandwidth can be directly applied.

Proposal 6: For 1Rx RedCap UEs, RAN4 shall define <5 MHz RRM requirements based on simulation results.

# Conclusion

Proposal 1: Limit the scope of RLM/BFD measurement relaxations for (e)RedCap UEs to SSB-based evaluations for PCell, and only when the DRX cycle is shorter than 80 ms.

Proposal 2: For 2Rx RedCap UEs, reuse the R17 relaxation requirements for both FR1 and FR2.

Proposal 3: For 1Rx RedCap UEs, RAN4 should discuss whether measurement relaxation is necessary.

Proposal 4: Re-use the UE feature rlm-Relaxation-r17and network signalling goodServingCellEvaluationRLM and goodServingCellEvaluationBFD for RedCap UEs. No new signaling is needed.

Proposal 5: For 2Rx RedCap UEs, the R18 RRM requirements for non-RedCap UEs operating with less than 5 MHz channel bandwidth can be directly applied.

Proposal 6: For 1Rx RedCap UEs, RAN4 shall define <5 MHz RRM requirements based on simulation results.

# References

[1] R4-252955, “WID: NR Radio Resource Management (RRM) Phase 6”, Apple, RAN#109