3GPP TSG-RAN WG4 Meeting #116bis                                         R4-2514418

Prague, Czech Republic, Oct 13 – 17, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on HPUE RF requirements for NR single carrier operation

Agenda Item:	7.1.4

Document for:	Approval

# Introduction

The phase 5 UE RF enhancements WID was approved [1]. One of the objectives is to introduce new power classes and/or new Tx architecture for HPUE. It’s worth noting that the work on two sub-objectives will not start until 2026 Q4.

High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25 - PC1.5 1Tx for TDD bands for FWA UE - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 1 x 29 dBm - Example bands: n41, n77/n78, n79 - This work is to start in 2026 Q4 - 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - This work is to start in 2026 Q4

# Discussion

## 4x26dBm for CPE/FWA UEs on TDD bands

By the end of R19, PC1 has been specified for many RF bands, including: n7, n14, n25, n31, n40, n41, n66, n71, n77, n78, n85, n100 and n101, which is targeted for Fixed Wireless/Vehicle Mounted (FWVM) usage. The power consumption as well as physical size of the PA and filters can be much larger than those for handheld devices.

The ALCR for PC1 inherited from LTE and set to 37dB. The MPR/A-MPR requirements are derived accordingly.

To address the growing adoption of Fixed Wireless Access (FWA) in certain markets and to maximize the use of the existing RF component ecosystem, a 4x26dBm PA architecture will be standardized in Rel-20 [1]. This architecture is designed with a target Maximum Output Power (MOP) of up to 32dBm. Furthermore, a study phase on a 2x29dBm power class will be initiated in Q4 2026, with standardization to follow if feasibility is confirmed.

Furthermore, the ACLR requirement for 32dBm MOP will be evaluated via coexistence simulations, which could be different from 37dB for PC1. Compound with RIMD between multiple Tx, the MPR/A-MPR will be affected.

Observation 1: For 4x26dBm with up to 32dBm MOP, new general requirements for MOP, ACLR and MPR will be needed.

As an option, it was proposed to reuse the existing definition of PC1, but with 1dB relaxation for the upper bound tolerance. However, the current PC1 requirements are derived for 1Tx, which cannot be reused for 4Tx or 2Tx. In addition, PC1 requirements for some bands have been in TS38.101-1 and PC1 has a specific UE capability. Normally, changing the requirements with a specific UE capability may lead to NBC, since network cannot distinguish the UE with old definition and the new definition. For more specific example, in certain regions of the world, the power tolerance might be built into local regulations. Hence, UEs with new definition may violate the local regulations.

Proposal 1: RAN4 to check if it’s feasible to reuse the existing PC1 power class definition with 1dB relaxation of the upper bound tolerance. Any NBC issue should be avoided.

Proposal 2: To define a new power class for 32dBm MOP should also be considered, since the underlying PA assumption as well as the requirements of MOP, ACLR and MPR will be different from the existing requirements for PC1 (31dBm).

The following example bands are included in the WID: n41, n77/n78, n79 and n104. Only band n41 needs to consider new A-MPR requirements (for NS_04 and possibly for NS_47).

Proposal 3: Pending on the conclusion of the ACLR requirement, simulations and/or measurements are needed to evaluate the MPR and A-MPR requirements. Only n41 needs to consider A-MPR requirement.

In Rel-18, the support of PC1.5 with 4Tx was specified. The conclusions on ULFPTx modes, UL-MIMO coherence and etc. are still valid or relevant to the current discussions. For example [2],

Issue 1-2-1: Whether 4Tx UE need to keep power class capability when configured with different antenna ports, i.e., 4/2/1 - WF - Confirm the following understanding: - The power class of a UE is a static value depending on reporting and not subject to change for different configurations - The maximum output power achievable and applicable requirements are related to configuration and implementation - How to accommodate the maximum achievable output power aspect into spec is FFS Issue 1-2-3: Other Fall back related issues - WF - In non-MIMO operation, when UE power class fallback to PC2/PC3 due to exceeding dutycycle capability or the output power is lower than the reported power class due to scheduling, whether to maintain 4Tx transmission is up to UE implementation.

Proposal 4: The Rel-18 4Tx framework is used as the basis for the 4x26dBm PA configuration targeting a 32dBm MOP, including ULFPTx modes, UL-MIMO coherence and power fallback implementation.

Proposal 5: Follow Rel-18 agreement that supporting 4Tx is release independent from Rel-18.

It’s noted that supporting 4Tx in Rel-18 WID includes both TDD and FDD bands, and the specified generic requirements are band-agnostic. And several operators have expressed the interest in supporting 4x26dBm for FDD bands.

Proposal 6: The general requirements for 4x26dBm shall be band-agnostic, applicable for both TDD and FDD bands, unless otherwise specified in band-specific requirements.

## 2Tx PC1.5 for both handheld and FWA UEs on FDD bands

Although specified in the standards, 1Tx PC2 on FDD bands remains challenging for implementation, especially for handheld UEs. The availability of suitable PAs and duplexers is the limiting factor for its wide adoption. On TDD bands, UE can benefit from certain duty-cycle for Tx, which is not guaranteed on FDD bands. Thermal management is also a big challenge for 2Tx PC1.5 with two 26dBm PAs.

Observation 2: Compared to TDD bands, achieving 2Tx PC1.5 in FDD bands—where each transmitter reaches 26 dBm—poses significant challenges in terms of UE thermal management and duplexer power handling capability.

Proposal 7: RAN4 to confirm if it’s feasible to reuse existing TDD 2Tx PC1.5 MPR requirements for FDD bands.

The rest of work include A-MPR and RSD requirements for the example bands (i.e., n1 and n25).

Proposal 8: Once MPR requirements are concluded, continue to study A-MPR for the example bands (n1 and n25).

Proposal 9: RSD for the example bands (n1 and n25) can be evaluated in parallel to the Tx requirements.

Proposal 10: Check whether release independence for FDD 2Tx PC1.5 could be the same as that for TDD bands.

# Conclusion

In this paper, we have shared our initial analysis on the HPUE requirements for 4x26dBm and 2Tx PC1.5. The following observations and proposals are made:

4Tx FWA with 32dBm MOP

Observation 1: For 4x26dBm with up to 32dBm MOP, new general requirements for MOP, ACLR and MPR will be needed.

Proposal 1: RAN4 to check if it’s feasible to reuse the existing PC1 power class definition with 1dB relaxation of the upper bound tolerance. Any NBC issue should be avoided.

Proposal 2:  To define a new power class for 32dBm MOP should also be considered, since the underlying PA assumption as well as the requirements of MOP, ACLR and MPR will be different from the existing requirements for PC1 (31dBm).

Proposal 3: Pending on the conclusion of the ACLR requirement, simulations and/or measurements are needed to evaluate the MPR and A-MPR requirements. Only n41 needs to consider A-MPR requirement.

Proposal 4: The Rel-18 4Tx framework is used as the basis for the 4x26dBm PA configuration targeting a 32dBm MOP, including ULFPTx modes, UL-MIMO coherence and power fallback implementation.

Proposal 5: Follow Rel-18 agreement that supporting 4Tx is release independent from Rel-18.

Proposal 6: The general requirements for 4x26dBm shall be band-agnostic, applicable for both TDD and FDD bands, unless otherwise specified in band-specific requirements.

2Tx PC 1.5 for FDD

Observation 2: Compared to TDD bands, achieving 2Tx PC1.5 in FDD bands—where each transmitter reaches 26 dBm—poses significant challenges in terms of UE thermal management and duplexer power handling capability.

Proposal 7: RAN4 to confirm if it’s feasible to reuse existing TDD 2Tx PC1.5 MPR requirements for FDD bands.

Proposal 8: Once MPR requirements are concluded, continue to study A-MPR for the example bands (n1 and n25).

Proposal 9: RSD for the example bands (n1 and n25) can be evaluated in parallel to the Tx requirements.

Proposal 10: Check whether release independence for FDD 2Tx PC1.5 could be the same as that for TDD bands.

# References

[1] RP-252952, “New WID: UE RF enhancements for NR FR1, Phase 5”, Moderator (vivo), TSG RAN#109

[2] R4-2303517 WF on FR1 4Tx UE RF requirements, vivo, RAN4#106