3GPP TSG-RAN WG4 Meeting #116-bis                                                Draft-R4-2514431

# Prague, CZ, 13th – 17th of October 2025

Agenda item:			6.23

Source:	Moderator (Iridium)

Title:	Topic summary for [116bis][308] Rel-19_BS_SAN_conformance_Part2

Document for:	Information

# Introduction

This document is a summary of the proposals made in the contributions submitted under AIs 5.10.4, 6.21.4 for the RAN4#116-bis meeting.
(the AI 6.20.2 was also under thread [308] however, there were no TDocs submitted into that AI).

With the following pre-meeting and meeting schedule:

Topic moderator will be designated to provide the summary for a topic before the meeting

- Before 06.10.2025 (Monday): Session chairs will provide the list of topics with moderator assignments.

- 08.10.2025 (Wednesday), 17:00 UTC: Moderators provide the initial summary for a topic.

- 09.10.2025 (Thursday), 12:00 UTC: Deadline for companies review of initial summary.

- 10.10.2025 (Friday), 17:00 UTC: Moderators submit the formal tdoc of summary for a topic.

- 12.10.2025 (Sunday): Session chairs share the initial meeting notes taking moderators summary in consideration.

- In online discussions, session chairs will handle topics based on the moderator summary.

The following documents considered for discussion in [116bis][308] Rel-19_BS_SAN_conformance_Part2 under AIs 5.10.4, 6.21.4 are:

| TDoc # | TDoc Type | Title | Company/ Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2513155 | Other | Discussion on SAN conformance tests for NR NTN Ku-band | CATT | Approval | 5.10.4 |
| R4-2514028 | Other | Discussion on SAN conformance testing for NR NTN Ku band | ZTE Corporation, Sanechips | Approval | 5.10.4 |
| R4-2514113 | Other | SAN RF Conformance testing for Ku-bands | Ericsson, Thales | Discussion | 5.10.4 |
| R4-2514029 | Other | Discussion on SAN conformance testing requirements for Iot-NTN TDD band | ZTE Corporation, Sanechips | Approval | 6.21.4 |
| R4-2514411 | draftCR | Draft CR on TS 36.181 - IoT_NTN_TDD | THALES, Iridium Satellite LLC | Endorsement | 6.21.4 |

Further split into:

- Topic #1: SAN conformance (TS 38.181) for Ku-band

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2513155 | Other | Discussion on SAN conformance tests for NR NTN Ku-band | CATT | Approval | 5.10.4 |
| R4-2514028 | Other | Discussion on SAN conformance testing for NR NTN Ku band | ZTE Corporation, Sanechips | Approval | 5.10.4 |
| R4-2514113 | Other | SAN RF Conformance testing for Ku-bands | Ericsson, Thales | Discussion | 5.10.4 |

- Topic #2: SAN conformance (TS 36.181) for 249 band

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2514029 | Other | Discussion on SAN conformance testing requirements for Iot-NTN TDD band | ZTE Corporation, Sanechips | Approval | 6.21.4 |
| R4-2514411 | draftCR | Draft CR on TS 36.181 - IoT_NTN_TDD | THALES, Iridium Satellite LLC | Endorsement | 6.21.4 |

List of topics/sub-topics/issues prior to the meeting is:

- Topic #1: SAN conformance (TS 38.181) for Ku-band

  - Sub-topic 1-1: General discussion on Ku-band SAN conformance testing

    - Issue 1-1-1: MU (to define)

    - Issue 1-1-2: Test Signal for SAN 1-H and 1-O test configurations
(Moderator calls on all TE vendors….)

  - Sub-topic 1-2: Manufacturer declaration Ku-band SAN

    - Issue 1-2-1: Manufacturer declaration for 1-O
(For Moderator clarification: is this proposal to create 2 tables, one for below, another for above 10 GHz?)

  - Sub-topic 1-3: Applicability of requirements for Ku-band SAN conformance testing

    - Issue 1-3-1: General requirements in TS 38.181 for NTN Ku-band

    - Issue 1-3-2: Tx requirements in TS 38.181 for NTN Ku-band

    - Issue 1-3-3: Rx requirements in TS 38.181 for NTN Ku-band

- Topic #2: SAN conformance (TS 36.181) for 249 band

  - Sub-topic 2-1: General discussion on 249 band (TDD NB-IoT NTN) SAN conformance testing

    - Issue 2-1-1: Selected impacted clauses under TS 36.181

    - Issue 2-1-2: Draft CR for endorsement

# Topic #1: SAN conformance (TS 38.181) for Ku-band

| TDoc # | Company | Proposals/Observations |
| --- | --- | --- |
| R4-2513155 | CATT | Proposal 1: The MU for 13.75GHz-14.5GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA receiver tests, and the MU for 10.7GHz-12.75GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA transmitter tests need inputs from TE vendors. Proposal 2: To define manufacturer declaration for SAN type 1-O operating below 10GHz and SAN type 1-O operating above 10GHz as following example table. Declaration identifier; Declaration; Description; Applicability (Note 1). SAN type 1-H (Note 2); SAN type 1-O operating below 10GHz; SAN type 1-O operating above 10GHz; SAN type 2-O. D.1; Coordinate system reference point; Location of coordinated system reference point in reference to an identifiable physical feature of the SAN enclosure.; x; x; x; x. D.2; Coordinate system orientation; Orientation of the coordinate system in reference to an identifiable physical feature of the SAN enclosure.; x; x; x; x. …; …; …; ..; …; …; …. D.19; OSDD identifier; A unique identifier for the OSDD.; x; x; n/a; n/a. D.20; OSDD operating band support; Operating band supported by the OSDD, declared for every OSDD (D.19). (Note 5); x; x; n/a; n/a. …; …; …; ..; …; …; …. Proposal 3: Update FDL_high – FDL_low <100 MHz to FDL_high – FDL_low ≤2050MHz for test signal used to build Test configurations for SAN type 1-H and SAN type 1-O as following table. Table 4.7.2-1: Signal to be used to build NR TCs for SAN type 1-H and SAN type 1-O Operating Band characteristics; FDL_high – FDL_low ≤2050 MHz. TC signal; BWchannel; 5 MHz (Note 1, 2 and 3). characteristics; Subcarrier spacing; Smallest supported subcarrier spacing. NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. NOTE 2: For NRTC1, if NB-IoT is not supported, the signal's channel bandwidth shall be the narrowest supported one. NOTE 3: It should use 3MHz channel bandwidth, if supported.. |
| R4-2514028 | ZTE Corporation, Sanechips | Proposal 1: We make the proposals as Table 2.1, 2.2 and 2.3 for SAN conformance testing for NTN Ku band. Table 2.2 2.1 General requirements for NTN Ku band Requirements in TS 38.181; Expected impact. 4 General test conditions and declarations. 4.1 Measurement uncertainties and test requirements; Need to update the test system uncertainty for carrier frequency f of Ku band in Table 4.1.2.2-2, 4.1.2.2-3, 4.1.2.3-2, 4.1.2.3-3.. 4.2 Requirement reference points; No specification impact.. 4.3 Satellite Access Node classes; No specification impact.. 4.4 Regional requirements; No specification impact.. 4.5 SAN configurations; No specification impact.. 4.6 Manufacturer declarations; No specification impact.. 4.7 Test configurations; Need to define the signal to be used to build NR TCs for SAN type 1-O for Ku band.. 4.8 Applicability of requirements; No specification impact.. 4.9 RF channels and test models; No specification impact.. 4.10 Void. 4.11 Reference coordinate system; No specification impact.. 4.12 Format and interpretation of tests; No specification impact.. Table 2.2 Tx requirements for NTN Ku band Requirements in TS 38.181; Expected impact. 9 Radiated transmitter characteristics. 9.1 General; No specification impact.. 9.2 Radiated transmit power; Need to update test requirement for Ku band in Table 9.2.5-1.. 9.3 OTA SAN output power; The TRP accuracy in section 9.3.5 needs to be updated for Ku band.. 9.4 OTA output dynamics. 9.4.1 General; No specification impact.. 9.4.2 OTA RE power control dynamic range; No specification impact.. 9.4.3 OTA total power dynamic range; OTA total power dynamic range requirement in 9.4.3.5 needs to be updated for Ku bands.. 9.4.4 OTA NB-IoT RB power dynamic range for in-band operation; No specification impact.. 9.5 OTA transmit ON/OFF power; No specification impact.. 9.6 OTA transmitted signal quality. 9.6.1 General; No specification impact.. 9.6.2 OTA frequency error; No specification impact.. 9.6.3 OTA modulation quality; EVM window length in section 9.6.3.5.1 for FR1-NTN needs to be updated.. 9.7 OTA unwanted emissions. 9.7.1 General; No specification impact.. 9.7.2 OTA occupied bandwidth; Span and number of measurement points for OBW measurements in Table 9.7.2.4.2-1 needs to be updated for FR1-NTN Ku bands.. 9.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR); SAN channel bandwidths in Table 9.7.3.5.1-1 and 9.7.3.5.1-2 need to be updated.. 9.7.4 OTA out-of-band emissions; No specification impact.. 9.7.5 OTA transmitter spurious emissions; OTA transmitter spurious emission limits for Ku bands needs to be defined.. 9.8 OTA transmitter intermodulation; No specification impact.. Table 2.3 Rx requirements for NTN Ku band Requirements in TS 38.181; Expected impact. 10 Radiated receiver characteristics. 10.1 General; OTA REFSENS RoAoA absolute requirement values for FR1-NTN operating above 10GHz need to be updated.. 10.2 OTA sensitivity; EIS levels needs to be updated for Ku bands.. 10.3 OTA reference sensitivity level; OTA reference sensitivity level needs to be updated for Ku bands.. 10.4 OTA dynamic range; Not applicable.. 10.5 OTA in-band selectivity and blocking. 10.5.1 OTA adjacent channel selectivity; OTA ACS requirement for Ku bands needs to be updated.. 10.5.2 OTA in-band blocking; Not applicable.. 10.6 OTA out-of-band blocking; OTA out-of-band blocking for SAN type 1-O operating above 10GHz needs to be updated.. 10.7 OTA receiver spurious emissions; Not applicable.. 10.8 OTA receiver intermodulation; Not applicable.. 10.9 OTA in-channel selectivity; OTA ICS for SAN type 1-O operating above 10GHz needs to be updated.. |
| R4-2514113 | Ericsson, Thales | Observation 1: SAN Type 1-O and SAN Type 2-O are not anymore directly related to the frequency ranges FR1-NTN and FR2-NTN, now sharing a common frequency range. This can be confusing and need to be carefully assessed. Observation 2: For SAN Type 1-O the MU above 10 GHz is not a straightforward choice, as the 10-15 GHz interval lies in the middle of previously defined frequency ranges. Reusing either of the MUs does not seem to be a good option. Proposal 1: Investigate what is the proper MU for 1-15 GHz and align also with BS specifications. |

### Sub-topic 1-1

Sub-topic description: General discussion on Ku-band SAN conformance testing

Issue 1-1-1: MU

- Proposals:

  - Proposal 1 (P1/R4-2513155): The MU for 13.75GHz-14.5GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA receiver tests, and the MU for 10.7GHz-12.75GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA transmitter tests need inputs from TE vendors.

  - Proposal 2 (P1/R4-2514113): Investigate what is the proper MU for 1-15 GHz and align also with BS specifications.

- Recommended WF

  - It seems that there is an alignment between Proposal 1 and Proposal 2. For SAN Type 1-O above 10 GHz, it is not a straightforward choice what MUs to use, since the range of 10-15 GHz was not studied before for TN and cannot be reused for NTN.

  - Inputs from TE vendors would be required with respect to Measurement Uncertainties (MUs) for:

    - 13.75GHz-14.5GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA receiver tests

    - 10.7GHz-12.75GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA transmitter tests.

Issue 1-1-2: Test Signal for SAN 1-H and 1-O test configurations

- Proposals:

  - Proposal 1 (P3/R4-2513155): Update FDL_high – FDL_low <100 MHz  to FDL_high – FDL_low ≤2050MHz  for test signal used to build Test configurations for SAN type 1-H and SAN type 1-O as following table.

Table 4.7.2-1: Signal to be used to build NR TCs for SAN type 1-H and SAN type 1-O

| Operating Band characteristics |  | FDL_high – FDL_low ≤2050 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note 1, 2 and 3) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. NOTE 2: For NRTC1, if NB-IoT is not supported, the signal's channel bandwidth shall be the narrowest supported one. NOTE 3: It should use 3MHz channel bandwidth, if supported. |  |  |

- Recommended WF

  - To be discussed, agree with proposal 1 if no objection.

### Sub-topic 1-2

Sub-topic description: Manufacturer declaration Ku-band SAN

Issue 1-2-1: Manufacturer declaration for 1-O

- Proposals:

  - Proposal 1 (P3/R4-2513155): To define manufacturer declaration for SAN type 1-O operating below 10GHz and SAN type 1-O operating above 10GHz as following example table.

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SAN type 1-H (Note 2) | SAN type 1-O operating below 10GHz | SAN type 1-O operating above 10GHz | SAN type 2-O |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the SAN enclosure. | x | x | x | x |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the SAN enclosure. | x | x | x | x |
| … | … | … | .. | … | … | … |
| D.19 | OSDD identifier | A unique identifier for the OSDD. | x | x | n/a | n/a |
| D.20 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD (D.19). (Note 5) | x | x | n/a | n/a |
| … | … | … | .. | … | … | … |

- Recommended WF

  - Agree with Proposal 1 if no other objection.

### Sub-topic 1-3

Sub-topic description: Applicability of requirements for Ku-band SAN conformance testing

Issue 1-3-1: General requirements in TS 38.181 for NTN Ku-band

- Proposals:

  - Proposal 1 (P1/ R4-2514028): General requirements for NTN Ku band

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| 4 General test conditions and declarations |  |
| 4.1 Measurement uncertainties and test requirements | Need to update the test system uncertainty for carrier frequency f of Ku band in Table 4.1.2.2-2, 4.1.2.2-3, 4.1.2.3-2, 4.1.2.3-3. |
| 4.2 Requirement reference points | No specification impact. |
| 4.3 Satellite Access Node classes | No specification impact. |
| 4.4 Regional requirements | No specification impact. |
| 4.5 SAN configurations | No specification impact. |
| 4.6 Manufacturer declarations | No specification impact. |
| 4.7 Test configurations | Need to define the signal to be used to build NR TCs for SAN type 1-O for Ku band. |
| 4.8 Applicability of requirements | No specification impact. |
| 4.9 RF channels and test models | No specification impact. |
| 4.10 Void |  |
| 4.11 Reference coordinate system | No specification impact. |
| 4.12 Format and interpretation of tests | No specification impact. |

- Recommended WF

  - Agree with Proposal 1 if no other objection.

Issue 1-3-2: Tx requirements in TS 38.181 for NTN Ku-band

- Proposals:

  - Proposal 1 (P1/ R4-2514028): Tx requirements for NTN Ku band

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| 9 Radiated transmitter characteristics |  |
| 9.1 General | No specification impact. |
| 9.2 Radiated transmit power | Need to update test requirement for Ku band in Table 9.2.5-1. |
| 9.3 OTA SAN output power | The TRP accuracy in section 9.3.5 needs to be updated for Ku band. |
| 9.4 OTA output dynamics |  |
| 9.4.1 General | No specification impact. |
| 9.4.2 OTA RE power control dynamic range | No specification impact. |
| 9.4.3 OTA total power dynamic range | OTA total power dynamic range requirement in 9.4.3.5 needs to be updated for Ku bands. |
| 9.4.4 OTA NB-IoT RB power dynamic range for in-band operation | No specification impact. |
| 9.5 OTA transmit ON/OFF power | No specification impact. |
| 9.6 OTA transmitted signal quality |  |
| 9.6.1 General | No specification impact. |
| 9.6.2 OTA frequency error | No specification impact. |
| 9.6.3 OTA modulation quality | EVM window length in section 9.6.3.5.1 for FR1-NTN needs to be updated. |
| 9.7 OTA unwanted emissions |  |
| 9.7.1 General | No specification impact. |
| 9.7.2 OTA occupied bandwidth | Span and number of measurement points for OBW measurements in Table 9.7.2.4.2-1 needs to be updated for FR1-NTN Ku bands. |
| 9.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | SAN channel bandwidths in Table 9.7.3.5.1-1 and 9.7.3.5.1-2 need to be updated. |
| 9.7.4 OTA out-of-band emissions | No specification impact. |
| 9.7.5 OTA transmitter spurious emissions | OTA transmitter spurious emission limits for Ku bands needs to be defined. |
| 9.8 OTA transmitter intermodulation | No specification impact. |

- Recommended WF

  - Agree with Proposal 1 if no other objection.

Issue 1-3-3: Rx requirements in TS 38.181 for NTN Ku-band

- Proposals:

  - Proposal 1 (P1/ R4-2514028): Rx requirements for NTN Ku band

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| 10 Radiated receiver characteristics |  |
| 10.1 General | OTA REFSENS RoAoA absolute requirement values for FR1-NTN operating above 10GHz need to be updated. |
| 10.2 OTA sensitivity | EIS levels needs to be updated for Ku bands. |
| 10.3 OTA reference sensitivity level | OTA reference sensitivity level needs to be updated for Ku bands. |
| 10.4 OTA dynamic range | Not applicable. |
| 10.5 OTA in-band selectivity and blocking |  |
| 10.5.1 OTA adjacent channel selectivity | OTA ACS requirement for Ku bands needs to be updated. |
| 10.5.2 OTA in-band blocking | Not applicable. |
| 10.6 OTA out-of-band blocking | OTA out-of-band blocking for SAN type 1-O operating above 10GHz needs to be updated. |
| 10.7 OTA receiver spurious emissions | Not applicable. |
| 10.8 OTA receiver intermodulation | Not applicable. |
| 10.9 OTA in-channel selectivity | OTA ICS for SAN type 1-O operating above 10GHz needs to be updated. |

- Recommended WF

  - Agree with Proposal 1 if no other objection.

# Topic #2: SAN conformance (TS 36.181) for 249 band

| TDoc # | Company | Proposals/Observations |
| --- | --- | --- |
| R4-2514029 | ZTE Corporation, Sanechips | Proposal 1: We make the proposals as Table 2.1, 2.2, 2.3, 2.4, 2.5 and 2.6 for SAN conformance testing requirements for IoT-NTN TDD band 249. Table 2.1 General requirements for IoT-NTN TDD band 249 Requirements in TS 36.181; Expected impact. 4 General test conditions and declarations. 4.1 Measurement uncertainties and test requirements; No specification impact.. 4.2 Requirement reference points; No specification impact.. 4.3 Satellite Access Node classes; No specification impact.. 4.4 Regional requirements; Need to add band 249 in Table 4.4-1.. 4.5 SAN configurations; No specification impact.. 4.6 Manufacturer declarations; No specification impact.. 4.7 Test configurations; The carrier bandwidth in section 4.7.3.1 needs to be updated.. 4.8 Applicability of requirements; Need to revise the applicability Transmit ON/OFF power in Table 4.8.2-1. Need to add “Transmit ON/OFF power (only applied for NB-IoT TDD SAN)” in Table 4.8.3-1.. 4.9 RF channels and test models; No specification impact.. 4.10 Requirements for contiguous and non-contiguous spectrum; No specification impact.. 4.11 Requirements for SAN capable of multi-band operation; No specification impact.. 4.12 Reference coordinate system; No specification impact.. 4.13 Format and interpretation of tests; No specification impact.. Table 2.2 Conducted Tx requirements for IoT-NTN TDD band 249 Requirements in TS 36.181; Expected impact. 6 Conducted transmitter characteristics. 6.1 General; No specification impact.. 6.2 Satellite Access Node output power; No specification impact.. 6.3 Output power dynamics. 6.3.1 General; No specification impact.. 6.3.2 RE power control dynamic range; No specification impact.. 6.3.3 Total power dynamic range; No specification impact.. 6.4 Transmit ON/OFF power; Need to update transmit ON/OFF power requirement for NB-IoT TDD SAN.. 6.5 Transmitted signal quality. 6.5.1 General; No specification impact.. 6.5.2 Frequency error; No specification impact.. 6.5.3 Modulation quality; No specification impact.. 6.5.4 Time alignment error; No specification impact.. 6.5.5 DL RS power; No specification impact.. 6.6 Unwanted emissions. 6.6.1 General; No specification impact.. 6.6.2 Occupied bandwidth; No specification impact.. 6.6.3 Adjacent Channel Leakage Power Ratio (ACLR); No specification impact.. 6.6.4 Out-of-band emissions; No specification impact.. 6.6.5 Transmitter spurious emissions; No specification impact.. 6.7 Transmitter intermodulation; No specification impact.. Table 2.3 Conducted Rx requirements for NTN IoT-NTN TDD band 249 Requirements in TS 38.181; Expected impact. 7 Conducted receiver characteristics. 7.1 General; No specification impact.. 7.2 Reference sensitivity level; Need to add the following REFSENS requirement for IoT-NTN TDD band. Table 7.2.5-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation and NB-IoT operation in NTN NR in-band in TDD operating bands (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 200; 15; FRC A14-3 in Annex A.14; -126.1.. 7.3 Dynamic range; Need to add the following dynamic range requirement for IoT-NTN TDD band. Table 7.3.5-4: Dynamic range of SAN supporting standalone NB-IoT operation in TDD bands (LEO class payload) SAN channel bandwidth (kHz); Reference measurement channel; Wanted signal mean power (dBm); Interfering signal mean power (dBm) / BWChannel; Type of interfering signal. 200; FRC A15-1 in Annex A.15; -105.1; -101.7; AWGN.. 7.4 In-band selectivity and blocking. 7.4.1 Adjacent channel selectivity (ACS); Need to add the following ACS requirement for IoT-NTN TDD band. Table 7.4.1.5-3: ACS requirement of SAN supporting standalone NB-IoT operation for TDD bands SAN channel bandwidth of the lowest/highest carrier received (kHz); Wanted signal mean power (dBm); Interfering signal mean power (dBm); Interfering signal centre frequency offset to the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz); Type of interfering signal. 200; PREFSENS + 19.5dB (Note); LEO SAN class: -85.5; ±100; 180 kHz NB-IoT signal. Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-5... 7.4.2 In-band blocking; No specification impact.. 7.5 Out-of-band blocking; No specification impact.. 7.6 Receiver spurious emissions; No specification impact.. 7.7 Receiver intermodulation; No specification impact.. 7.8 In-channel selectivity; No specification impact.. Table 2.4 Radiated Tx requirements for NTN IoT-NTN TDD band 249 Requirements in TS 38.181; Expected impact. 9 Radiated transmitter characteristics. 9.1 General; No specification impact.. 9.2 Radiated transmit power; No specification impact.. 9.3 OTA SAN output power; No specification impact.. 9.4 OTA output power dynamics; No specification impact.. 9.5 OTA transmit ON/OFF power; Need to update OTA transmit ON/OFF power requirement for NB-IoT TDD SAN.. 9.6 OTA transmitted signal quality; No specification impact.. 9.7 OTA unwanted emissions; No specification impact.. 9.8 OTA transmitter intermodulation; No specification impact.. Table 2.5 Radiated Rx requirements for NTN IoT-NTN TDD band 249 Requirements in TS 38.181; Expected impact. 10 Radiated receiver characteristics. 10.1 General; No specification impact.. 10.2 OTA sensitivity; Need to add the following OTA sensitivity requirement for IoT-NTN TDD band. Table10.2.5-3: SAN LEO class EIS levels for standalone NB-IoT operation and NB-IoT operation in NTN NR in-band SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel (NOTE); OTA sensitivity level, EIS (dBm). 200; 15; A14-3 in Annex A.14; Declared minimum EIS + 1.6. NOTE: EIS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth... 10.3 OTA reference sensitivity level; Need to add the following OTA reference sensitivity level requirement for IoT-NTN TDD band. Table10.3.5-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation and NB-IoT operation in NTN NR in-band for TDD frequency bands (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, EISREFSENS (dBm). 200; 15; FRC A14-3 in Annex A.14; -125.8 - ΔOTAREFSENS.. 10.4 OTA dynamic range; Need to add the following dynamic range requirement for IoT-NTN TDD band. Table 10.4.5.2-4: Dynamic range of SAN supporting standalone NB-IoT operation in TDD frequency bands (LEO class payload) SAN channel bandwidth (kHz); Reference measurement channel; Wanted signal mean power (dBm); Interfering signal mean power (dBm) / BWChannel; Type of interfering signal. 200; FRC A15-1 in annex A.15; -105.1 - ΔOTAREFSENS; -101.7 - ΔOTAREFSENS; AWGN.. 10.5 OTA in-band selectivity and blocking; Need to add the following ACS requirement for IoT-NTN TDD band. Table 10.5.1.5-3: ACS requirement of SAN supporting standalone NB-IoT operation for TDD frequency bands SAN channel bandwidth of the lowest/highest carrier received (kHz); Wanted signal mean power (dBm); Interfering signal mean power (dBm); Interfering signal centre frequency offset to the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz); Type of interfering signal. 200; PREFSENS + 19.5dB - ΔOTAREFSENS(Note); LEO SAN class: -85.5- ΔOTAREFSENS; ±100; 180 kHz NB-IoT signal. Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-5... 10.5.1 OTA adjacent channel selectivity; No specification impact.. 10.5.2 OTA in-band blocking; No specification impact.. 10.6 OTA out-of-band blocking; No specification impact.. 10.7 OTA receiver spurious emissions; No specification impact.. 10.8 OTA receiver intermodulation; No specification impact.. 10.9 OTA in-channel selectivity; No specification impact.. Table 2.6 Annex Requirements in TS 38.181; Expected impact. Annex A (normative): Reference measurement channel; Need to add new FRC A14-3 in Table A.14-1 for IoT-NTN TDD band. Need to add new FRC A15-3 in Table A.15-1 for IoT-NTN TDD band.. Annex B (informative): Environmental requirements for SAN equipment; No specification impact.. Annex C (informative): Test tolerances and derivation of test requirements; No specification impact.. Annex D (informative): Measurement system set-up; No specification impact.. Annex E (normative): Characteristics of the interfering signals; No specification impact.. Annex F (normative): Propagation conditions; No specification impact.. Annex G (normative): In-channel Tx test; Need to revise averaging length for the transmission time to 8ms in section G.7.3 Averaged EVM.. |
| R4-2514411 | THALES, Iridium Satellite LLC | Draft CR on TS 36.181 - IoT_NTN_TDD on clauses: 3, 4.4, 4.8, 4.9.2.3, 4.9.2.4.3, 6.4, 6.5.3, 7.1, 7.2, 7.3, 7.4, 9.4, 10.3, 10.4, 10.5, A.14, A.15, G.7.3 |

### Sub-topic 2-1

Sub-topic description: General discussion on 249 band (TDD NB-IoT NTN) SAN conformance testing

Issue 2-1-1: Selected impacted clauses under TS 36.181

- Proposals:

  - Proposal 1 (P1/R4-2514029):

i/ General requirements for IoT-NTN TDD band 249

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| 4 General test conditions and declarations |  |
| 4.1 Measurement uncertainties and test requirements | No specification impact. |
| 4.2 Requirement reference points | No specification impact. |
| 4.3 Satellite Access Node classes | No specification impact. |
| 4.4 Regional requirements | Need to add band 249 in Table 4.4-1. |
| 4.5 SAN configurations | No specification impact. |
| 4.6 Manufacturer declarations | No specification impact. |
| 4.7 Test configurations | The carrier bandwidth in section 4.7.3.1 needs to be updated. |
| 4.8 Applicability of requirements | Need to revise the applicability Transmit ON/OFF power in Table 4.8.2-1. Need to add “Transmit ON/OFF power (only applied for NB-IoT TDD SAN)” in Table 4.8.3-1. |
| 4.9 RF channels and test models | No specification impact. |
| 4.10 Requirements for contiguous and non-contiguous spectrum | No specification impact. |
| 4.11 Requirements for SAN capable of multi-band operation | No specification impact. |
| 4.12 Reference coordinate system | No specification impact. |
| 4.13 Format and interpretation of tests | No specification impact. |

ii/ Conducted Tx requirements for IoT-NTN TDD band 249

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| 6 Conducted transmitter characteristics |  |
| 6.1 General | No specification impact. |
| 6.2 Satellite Access Node output power | No specification impact. |
| 6.3 Output power dynamics |  |
| 6.3.1 General | No specification impact. |
| 6.3.2 RE power control dynamic range | No specification impact. |
| 6.3.3 Total power dynamic range | No specification impact. |
| 6.4 Transmit ON/OFF power | Need to update transmit ON/OFF power requirement for NB-IoT TDD SAN. |
| 6.5 Transmitted signal quality |  |
| 6.5.1 General | No specification impact. |
| 6.5.2 Frequency error | No specification impact. |
| 6.5.3 Modulation quality | No specification impact. |
| 6.5.4 Time alignment error | No specification impact. |
| 6.5.5 DL RS power | No specification impact. |
| 6.6 Unwanted emissions |  |
| 6.6.1 General | No specification impact. |
| 6.6.2 Occupied bandwidth | No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio (ACLR) | No specification impact. |
| 6.6.4 Out-of-band emissions | No specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | No specification impact. |

iii/ Conducted Rx requirements for NTN IoT-NTN TDD band 249

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| 7 Conducted receiver characteristics |  |
| 7.1 General | No specification impact. |
| 7.2 Reference sensitivity level | Need to add the following REFSENS requirement for IoT-NTN TDD band. Table 7.2.5-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation and NB-IoT operation in NTN NR in-band in TDD operating bands (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, PREFSENS (dBm). 200; 15; FRC A14-3 in Annex A.14; -126.1. |
| 7.3 Dynamic range | Need to add the following dynamic range requirement for IoT-NTN TDD band. Table 7.3.5-4: Dynamic range of SAN supporting standalone NB-IoT operation in TDD bands (LEO class payload) SAN channel bandwidth (kHz); Reference measurement channel; Wanted signal mean power (dBm); Interfering signal mean power (dBm) / BWChannel; Type of interfering signal. 200; FRC A15-1 in Annex A.15; -105.1; -101.7; AWGN. |
| 7.4 In-band selectivity and blocking |  |
| 7.4.1 Adjacent channel selectivity (ACS) | Need to add the following ACS requirement for IoT-NTN TDD band. Table 7.4.1.5-3: ACS requirement of SAN supporting standalone NB-IoT operation for TDD bands SAN channel bandwidth of the lowest/highest carrier received (kHz); Wanted signal mean power (dBm); Interfering signal mean power (dBm); Interfering signal centre frequency offset to the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz); Type of interfering signal. 200; PREFSENS + 19.5dB (Note); LEO SAN class: -85.5; ±100; 180 kHz NB-IoT signal. Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-5.. |
| 7.4.2 In-band blocking | No specification impact. |
| 7.5 Out-of-band blocking | No specification impact. |
| 7.6 Receiver spurious emissions | No specification impact. |
| 7.7 Receiver intermodulation | No specification impact. |
| 7.8 In-channel selectivity | No specification impact. |

iv/ Radiated Tx requirements for NTN IoT-NTN TDD band 249

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| 9 Radiated transmitter characteristics |  |
| 9.1 General | No specification impact. |
| 9.2 Radiated transmit power | No specification impact. |
| 9.3 OTA SAN output power | No specification impact. |
| 9.4 OTA output power dynamics | No specification impact. |
| 9.5 OTA transmit ON/OFF power | Need to update OTA transmit ON/OFF power requirement for NB-IoT TDD SAN. |
| 9.6 OTA transmitted signal quality | No specification impact. |
| 9.7 OTA unwanted emissions | No specification impact. |
| 9.8 OTA transmitter intermodulation | No specification impact. |

v/ Radiated Rx requirements for NTN IoT-NTN TDD band 249

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| 10 Radiated receiver characteristics |  |
| 10.1 General | No specification impact. |
| 10.2 OTA sensitivity | Need to add the following OTA sensitivity requirement for IoT-NTN TDD band. Table10.2.5-3: SAN LEO class EIS levels for standalone NB-IoT operation and NB-IoT operation in NTN NR in-band SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel (NOTE); OTA sensitivity level, EIS (dBm). 200; 15; A14-3 in Annex A.14; Declared minimum EIS + 1.6. NOTE: EIS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth.. |
| 10.3 OTA reference sensitivity level | Need to add the following OTA reference sensitivity level requirement for IoT-NTN TDD band. Table10.3.5-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation and NB-IoT operation in NTN NR in-band for TDD frequency bands (LEO class payload) SAN channel bandwidth (kHz); Sub-carrier spacing (kHz); Reference measurement channel; Reference sensitivity power level, EISREFSENS (dBm). 200; 15; FRC A14-3 in Annex A.14; -125.8 - ΔOTAREFSENS. |
| 10.4 OTA dynamic range | Need to add the following dynamic range requirement for IoT-NTN TDD band. Table 10.4.5.2-4: Dynamic range of SAN supporting standalone NB-IoT operation in TDD frequency bands (LEO class payload) SAN channel bandwidth (kHz); Reference measurement channel; Wanted signal mean power (dBm); Interfering signal mean power (dBm) / BWChannel; Type of interfering signal. 200; FRC A15-1 in annex A.15; -105.1 - ΔOTAREFSENS; -101.7 - ΔOTAREFSENS; AWGN. |
| 10.5 OTA in-band selectivity and blocking | Need to add the following ACS requirement for IoT-NTN TDD band. Table 10.5.1.5-3: ACS requirement of SAN supporting standalone NB-IoT operation for TDD frequency bands SAN channel bandwidth of the lowest/highest carrier received (kHz); Wanted signal mean power (dBm); Interfering signal mean power (dBm); Interfering signal centre frequency offset to the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz); Type of interfering signal. 200; PREFSENS + 19.5dB - ΔOTAREFSENS(Note); LEO SAN class: -85.5- ΔOTAREFSENS; ±100; 180 kHz NB-IoT signal. Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-5.. |
| 10.5.1 OTA adjacent channel selectivity | No specification impact. |
| 10.5.2 OTA in-band blocking | No specification impact. |
| 10.6 OTA out-of-band blocking | No specification impact. |
| 10.7 OTA receiver spurious emissions | No specification impact. |
| 10.8 OTA receiver intermodulation | No specification impact. |
| 10.9 OTA in-channel selectivity | No specification impact. |

vi/ Annex

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| Annex A (normative): Reference measurement channel | Need to add new FRC A14-3 in Table A.14-1 for IoT-NTN TDD band. Need to add new FRC A15-3 in Table A.15-1 for IoT-NTN TDD band. |
| Annex B (informative): Environmental requirements for SAN equipment | No specification impact. |
| Annex C (informative): Test tolerances and derivation of test requirements | No specification impact. |
| Annex D (informative): Measurement system set-up | No specification impact. |
| Annex E (normative): Characteristics of the interfering signals | No specification impact. |
| Annex F (normative): Propagation conditions | No specification impact. |
| Annex G (normative): In-channel Tx test | Need to revise averaging length for the transmission time to 8ms in section G.7.3 Averaged EVM. |

- Recommended WF

  - Agree with Proposal 1 (as starting point) if no objections, and endorse Draft CR R4-2514411

Issue 2-1-2: Draft CR for endorsement

- Proposals:

  - Proposal 1: Endorse Draft CR R4-2514411

- Recommended WF

  - Agree with proposal 1 if no objection, since draft CR R4-2514411 (THALES, Iridium) on TS 36.181 - IoT_NTN_TDD (clauses: 3, 4.4, 4.8, 4.9.2.3, 4.9.2.4.3, 6.4, 6.5.3, 7.1, 7.2, 7.3, 7.4, 9.4, 10.3, 10.4, 10.5, A.14, A.15, G.7.3) seems to be aligned with previous ZTE R4-2514029

  - Other companies are invited to co-sign (Let THALES know).

# Recommendations for TDocs

Existing tdocs

| Tdoc number | Type | Recommendation | Comments |
| --- | --- | --- | --- |
| R4-2514411 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |