3GPP TSG-RAN WG4 Meeting #116-bis														R4-2514440

Prague, EU, 13th to 17th October 2025

Agenda item:			7.2

Source:	Moderator (Ericsson)

Title:	Topic summary for [116bis][317] NR_BS_RF_Ph2

Document for:	Information

# Introduction

This summary collects the different proposals and main observations related to the AI 7.2 NR BS RF phase 2.

The WI have been divided into following topics:

- General aspects and workplan

- Enhancement of co-location requirements

- SBFD BS to SBFD BS adjacent channel coexistence

The goal in the first meeting is to establish workplan, identify the common views and early agreements, refine the scope of the WI and list the different areas for further investigations, setting priorities when needed.

Note that in the following, proposals are not necessarily exclusive, they could be complementary. Some major observations have been captured to help the proposal’s understanding.

# Topic #1: General asepcts and work plan

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513772 | Huawei, HiSilicon | Proposal 1: it is proposed to take the work plan in clause 2.2 into account for SBFD-SBFD co-existence. Proposal 2: For SBFD to SBFD BS adjacent channel co-existence TR, the following clauses are proposed: > 1 Scope > 2 References > 3 Definitions, symbols and abbreviations > 4 Objectives of study > 5 Deployment scenarios > 6 Co-existence study > 7 RF requirements |
| R4-2513943 | Ericsson | Observation 1: Both work packages include simulation activities which would require careful planning considering the limited TU allocation. Observation 2: For co-location enhancement work package RAN4 to be able to determine a reasonable coupling loss assumption per requirement a simulation campaign accomplished with some measurement would be required. Observation 3: For SBFD co-existence work package a simulation campaign is required to be able to establish relevant requirements. It is essential that relevant models and parameters are adopted for the simulation campaign. Observation 4: For SBFD co-existence work package it is also worth to align parameters and models with previous work in Rel-19, to maintain scope of the simulation campaign reasonable and result relevant for Rel-19 SBFD requirements. Observation 5: The work package related to SBFD co-existence may need input from the work package for improving co-location requirements. Proposal: Initially split meeting time equally between the two work packages. |
| R4-2513998 | ZTE Corporation, Sanechips | Table 1: Work plan for BS RF requirement evolution for FR1 and testing, phase 2 Timeline; Work plan. RAN4#116bis (0.5 TU) -Core part October, 2025; Agree on the work plan. Enhancement of co-location requirements - Start to discuss the impacts on the OTA co-location requirements based on the Rel-19. - Discuss the missing cases for MCL analysis from Rel-19 (e.g.band combination) - Start to discuss the OTA co-location requirements improvement including: - CLRA concept (e.g. alternative testing antenna or other feasible testing method) - The possibility to define requirements using the existing OTA metrics. SBFD BS to SBFD BS adjacent channel co-existence - Initial discuss scenarios/cases, simulation assumption, coupling loss assumption for co-site case and etc. - Approve the TR skeleton. RAN4#117 (0.5 TU) -Core part November, 2025; Enhancement of co-location requirements - Continue discussing the impacts on the OTA co-location requirements. - Continue the MCL analysis for missing cases from Rel-19 (e.g. Band combination). - Continue discussing OTA co-location requirements improvement including: - CLRA concept (e.g. alternative testing antenna or other feasible testing method) - The possibility to define requirements using the existing OTA metrics SBFD BS to SBFD BS adjacent channel co-existence - Initial simulation results for SBFD BS to SBFD BS adjacent channel coexistence - Initial discuss the necessity and feasibility of BS Tx unwanted emission and Rx in-band blocking RF requirements.. RAN4#118 (1 TU) -Core part February, 2026; Enhancement of co-location requirements - Continue discussing the impacts on the OTA co-location requirements. - Continue the MCL analysis for missing cases from Rel-19 (e.g.band combination). - Continue discussing OTA co-location requirements improvement including: - CLRA concept (e.g. alternative testing antenna or other feasible testing method) - The possibility to define requirements using the existing OTA metrics SBFD BS to SBFD BS adjacent channel co-existence - Updated simulation results for SBFD BS to SBFD BS adjacent channel coexistence - Continue discussing the necessity and feasibility of BS Tx unwanted emission and Rx in-band blocking RF requirements.. RAN4#118bis (1 TU) -Core part April, 2026; Enhancement of co-location requirements - Continue discussing the impacts on the OTA co-location requirements. - Continue the MCL analysis for missing cases from Rel-19 (e.g.band combination). - Continue discussing OTA co-location requirements improvement including: - CLRA concept (e.g. alternative testing antenna or other feasible testing method) - The possibility to define requirements using the existing OTA metrics SBFD BS to SBFD BS adjacent channel co-existence - Align the simulation results and try to draw conclusion on the simulation results for SBFD coexistence - Identify the necessity and feasibility of BS Tx unwanted emission and Rx in-band blocking RF requirements based on the co-existence simulation study.. RAN4#119 (1 TU) -Core part May, 2026; Enhancement of co-location requirements - Continue discussing the impacts on the OTA co-location requirements. - Continue the MCL analysis for missing cases from Rel-19 (e.g.band combination). - Continue discussing OTA co-location requirements improvement including: - CLRA concept (e.g. alternative testing antenna or other feasible testing method) - The possibility to define requirements using the existing OTA metrics SBFD BS to SBFD BS adjacent channel co-existence - Discuss how to define the RF requirements based on the the conclusion of co-existence simulation study. - TP to TR if needed. RAN4#120 (1 TU) -Core part August, 2026; Enhancement of co-location requirements - Draw the conclusion on the MCL analysis for missing cases from Rel-19 - Continue discussing OTA co-location requirements improvement including: - CLRA concept (e.g. alternative testing antenna or other feasible testing method) - The possibility to define requirements using the existing OTA metrics SBFD BS to SBFD BS adjacent channel co-existence - Continue discussing how to define the new RF requirements.. RAN4#120bis (1 TU) -Core part October, 2026; Enhancement of co-location requirements - Continue discussing OTA co-location requirements improvement including: - CLRA concept (e.g. alternative testing antenna or other feasible testing method) - The possibility to define requirements using the existing OTA metrics SBFD BS to SBFD BS adjacent channel co-existence - Continue discussing how to define the new RF requirements.. RAN4#121 (1 TU) -Core part November, 2026; Enhancement of co-location requirements - Draw the conclusion on the core requirement improvement - Discuss and Endorse all draft CRs SBFD BS to SBFD BS adjacent channel co-existence - Agree on the definition of the new RF requirements. - Discuss and Endorse all draft CRs.. RAN4#122 (0.5+0.5 TU)-Core part + Perf. part February, 2027; Core: Enhancement of co-location requirements - Complete the core requirement for co-location requirement improvement - Agree all formal CRs SBFD BS to SBFD BS adjacent channel co-existence - Complete the RF requirements for SBFD-SBFD BS adjacent channel co-existence. - Agree all formal CRs - Agree the final TR. Perf: Enhancement of co-location requirements - Initial discuss the conformance testing for the improved of co-location requirements. SBFD BS to SBFD BS adjacent channel co-existence - Initial discuss the conformance testing of the new RF requirements including MU, test configuration, test procedures, and etc.. RAN4#122bis (0.5 TU)-Perf. part April, 2027; Enhancement of co-location requirements - Continue discussing the conformance testing for the improved of co-location requirements. SBFD BS to SBFD BS adjacent channel co-existence - Continue discussing the conformance testing of the new RF requirements including MU, test configuration, test procedures, and etc.. RAN4#123 (0.5 TU) -Perf. part May, 2027; Enhancement of co-location requirements - Continue discussing the conformance testing for the improved of co-location - Discuss and Endorse all draft CRs. SBFD BS to SBFD BS adjacent channel co-existence - Agree on the conformance testing of the new RF requirements including MU, test configuration, test procedures, and etc. - Discuss and Endorse all draft CRs. RAN4#124 (0.5 TU) -Perf. part August, 2027; Enhancement of co-location requirements - Complete the discussion on the conformance testing for the improved of co-location - Agree all final CRs. SBFD BS to SBFD BS adjacent channel co-existence - Complete the discussion on the conformance testing of the new RF requirements - Agree all final CRs .. Proposal: Agreed on the work plan as in table 1. |
| R4-2513999 | ZTE Corporation, Sanechips | TR skeleton for SBFD to SBFD adjacent channel co-existence |
| R4-2514089 | Nokia | Proposal 1: RAN4 to agree on the proposed work plan for enhancement of co-location requirements. Proposal 2: RAN4 to agree on proposed work plan for SBFD BS to SBFD BS adjacent channel co-existence. |

## Open issues summary

### Sub-topic 1-1

Workplan for enhancement of co-location requirements and SBFD to SBFD adjacent channel co-existence.

Issue 1-1: Workplan

- Proposals

  - Proposal 1: It is proposed to take the work plan in clause 2.2 into account for SBFD-SBFD co-existence. (Huawei)

  - Proposal 2: Initially split meeting time equally between the two work packages. (Ericsson)

  - Proposal 3: Agree on the work plan as in table 1. (ZTE)

  - Proposal 4: RAN4 to agree on the proposed work plan for enhancement of co-location requirements. (Nokia)

  - Proposal 5: RAN4 to agree on proposed work plan for SBFD BS to SBFD BS adjacent channel co-existence. (Nokia)

- Recommended WF

  - Based on provided proposals establish workplan including both objectives and capture outcome in WF.

### Sub-topic 1-2

Need for TR to capture technical background information.

Issue 1-2: TR for SBFD to SBFD adjacent channel co-existence

- Proposals

  - Option 1: For SBFD to SBFD BS adjacent channel co-existence TR, the following clauses are proposed (Huawei):

    - 1 Scope

    - 2	References

    - 3	Definitions, symbols and abbreviations

    - 4	Objectives of study

    - 5	Deployment scenarios

    - 6	Co-existence study

    - 7	RF requirements

  - Option 2: Draft TR skeleton in R4-2513999. (ZTE)

- Recommended WF

  - Use draft TR skeleton in R4-2513999 as baseline.

# Topic #2: Enhancement of co-location requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513238 | CATT | Proposal 1: Evaluation of BS-to-BS isolation for OTA co-location requirements and improving core requirements for upper FR1 frequencies should be performed in parallel, which requires two sub agenda items under this objective. Proposal 2: RAN4 to agree a single unified approach to define enhanced OTA co-location requirements to avoid mixed definitions and reduce test costs. Proposal 3: Enhanced OTA co-location core requirements for upper FR1 frequencies should be manufacturer-declared once specified. |
| R4-2513942 | Ericsson | Observation 1: It can be noticed that the coupling loss evaluation from Rel-19 is not complete for all relevant situations. Observation 2: If the requirement background is changed using a coupling loss assumption larger than before also the testability aspects needs to be considered. Eventually, the requirement can be defined in such a way that regular OTA parameters can be used and tested in regular OTA test ranges avoiding any specific CLTA to be used. Observation 3: In Rel-19 it was concluded that there is no straightforward one-to-one translation to use wideband horn antenna as CLTA with the current CLTA arrangement. Observation 4: For Rel-20, Option 1 and Option 2 are not mutually exclusive. For both Option 1 and Option 2, simulation results will be needed. Observation 5: For tower sharing, transmitter spurious emission and receiver out-of-band blocking requirements are essential and should be prioritised. Observation 6: Transmitter OFF power requirement is not related to the side-by-side co-location scenario. To derive proper requirement levels relevant parameters considering two neighbouring BS in the same network should be considered. Observation 7: For transmitter intermodulation RAN4 should study impact due to intra-system coupling and inter-system coupling for relevant AAS BS architectures and conclude. Observation 8: To be able to determine relevant coupling loss values for requirement derivations a simulation campaign including near-field modelling of victim BS and aggressor BS is necessary. Observation 9: The coupling loss assumptions should be captured for all band combinations picked from a division in sub-ranges. Proposal 1: Establish simulation methodology required for side-by-side co-location scenario considering near-field effects. Proposal 2: As outcome from RAN4#116-bis agree on a way-forward including a work-plan for co-location requirements enhancements part of Rel-20. |
| R4-2514090 | Nokia | Proposal 1: Take the learnings from Rel-19 work and evaluate the results to see how much can be used and what else is needed. Proposal 2: Evaluate if more simulation or measurement results are needed for BS-to-BS isolation. Proposal 3: Study both options 1 and 2 to evaluate what method should be used for co-location requirements. |
| R4-2514278 | ZTE Corporation, Sanechips | Proposal 1: In Rel-20 BS RF enhancement, propose not to consider the FR1 unlicensed band for co-location requirement improvement. Proposal 2: In Rel-20 BS RF enhancement, propose to continue the investigation on the rest of co-located cases which are not covered in Rel-19. Proposal 3: For Rel-20 NR BS to BS co-located deployment, propose to consider more practical installation assumptions instead of reusing the legacy 10cm separation distance as default assumption which is used to mimic 30dBc coupling loss. Observation 1: For OTA transmitter OFF power improvement, don’t see too much implementation burden on the existing requirement except for the measurement for conformance testing. Proposal 4: For OTA transmitter OFF power in Rel-20, we propose to consider following aspects: 1) Step 1: to identify more appropriate MCL1 between two BSs according to the legacy deployment assumption from single operator and relevant BS’s antenna configuration to derive the OTA core requirement (no intention to change the conducted core requirements according to the WID objective), then check whether TRP based OFF power measurement in the OTA chamber is feasible or not. 2) Step 2: If TRP based OFF power measurement in step 1 is not feasible, then to define more appropriate/relevant co-location reference antenna for the OTA requirement improvement and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement. Observation 2: For OTA transmitter intermodulation requirement, if NR BS is implemented with circulator for protection of external interference signal, IMD impacts due to transmitter intermodulation requirement should be quite impacted. Proposal 5: For OTA transmitter intermodulation requirement in Rel-20, we propose to consider following aspects: 1) Step 1: Discuss more appropriate co-location reference antenna for FR1 high bands as alternative of the existing testing antenna to emulate the more realistic deployment. 2) Step 2: Discuss whether the transmitter intermodulation requirement for band n79 and n104 except for band n77/n78 could be optionally removed from regions which don’t mandate the Tx intermodulation testing. Proposal 6: For OTA transmitter spurious co-location emission in Rel-20, we propose to consider following aspects: 1) Step 1: To identify more appropriate MCL1 between co-located BS from different bands, then use the following equation to derive the corresponding TRP based transmitter spurious co-location emission requirement and check its feasibility of TRP based measurement in the OTA chamber firstly. = -174dBm/Hz + 5dB + 50dB－7dB + MCL1+9dB (scaling factor) 2) Step 2: If TRP based OTA transmitter spurious co-location emission in step 1 is not feasible, to discuss more appropriate co-location reference antenna and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement. Proposal 7: For OTA Receiver out-of-band blocking in Rel-20, we propose to consider following aspects: 1) Step 1: Discuss more appropriate co-location reference antenna for FR1 high bands as alternative of the existing testing antenna to emulate the more realistic deployment. a. NOTE: Larger inter-band MCL observed in TR 37.941 will cause less REFSENS degradation compared with the legacy 30dBc MCL assumption; 2) Step 2: Discuss whether we need to align the requirement for BS type 1-H and BS type 1-O. |
| R4-2514468 | Huawei, HiSilicon | Observation 1: Side by side coupling for the co-location scenario is still worst case. Observation 2: Side-by-side antenna coupling remains a reasonable practical limitation. Observation 3: the existing 10 cm edge-to-edge definition of the OTA co-location scenario may not need modifying. Observation 4: The single column co-located antenna is worst case. Observation 5: If the effects of multi-column CLRA are to be used, a minimum number of columns must be attributed to the CLRA. Observation 6: For high frequencies, multi-column antennas are likely to be the norm. Proposal 1: Further investigate the CLRA being defined with a minimum number of columns for high FR1 frequencies. Observation 7: The CLRA height definition is currently dependent on the DUT. Proposal 2: Investigate standardising the height of the CLRA for each operating band. |

## Open issues summary

### Sub-topic 2-1: Organizing technical disucssion

Sub-topic description: This sub-topic handles aspects related to planning and organizing the work related to enhancement of co-location requirements.

Issue 2-1: Workplan

- Proposals

  - Proposal 1: Evaluation of BS-to-BS isolation for OTA co-location requirements and improving core requirements for upper FR1 frequencies should be performed in parallel, which requires two sub agenda items under this objective. (CATT)

  - Proposal 2: As outcome from RAN4#116-bis agree on a way-forward including a work-plan for co-location requirements enhancements part of Rel-20. (Ericsson)

- Recommended WF

  - Consider proposals above in WF capturing workplan in sub-topic 1-1.

Issue 2-2: Requirement approach

- Proposals

  - Proposal 1: RAN4 to agree a single unified approach to define enhanced OTA co-location requirements to avoid mixed definitions and reduce test costs. (CATT)

  - Proposal 2: Enhanced OTA co-location core requirements for upper FR1 frequencies should be manufacturer-declared once specified. (CATT)

  - Proposal 3: Study both options 1 and 2 to evaluate what method should be used for co-location requirements. (Nokia)

  - Proposal 4: Further investigate the CLRA being defined with a minimum number of columns for high FR1 frequencies. (Huawei)

  - Proposal 5: Investigate standardising the height of the CLRA for each operating band. (Huawei)

- Recommended WF

  - The WID in includes to two options:

    - Option 1: Investigate and if possible, improve current CLRA requirement concept.

    - Option 2: Investigate possibility to define requirements using the existing OTA metrics (such as TRP, EIRP, EIS), if any.

At this point work related to both can continue in parallel.

### Sub-topic 2-3: Assumptions

Issue 2-3-1: Co-location scenario description

- Proposals

  - Proposal 1: For Rel-20 NR BS to BS co-located deployment, propose to consider more practical installation assumptions instead of reusing the legacy 10 cm separation distance as default assumption which is used to mimic 30dBc coupling loss. (ZTE)

- Recommended WF

  - TBA

Issue 2-3-2: Band to consider

- Proposals

  - Proposal 1: In Rel-20 BS RF enhancement, propose not to consider the FR1 unlicensed band for co-location requirement improvement. (ZTE)

- Recommended WF

  - TBA

### Sub-topic 2-4: Coupling loss evaluation

Issue 2-4-1: Re-use of Rel-19 findings

- Proposals

  - Proposal 1: Take the learnings from Rel-19 work and evaluate the results to see how much can be used and what else is needed. (Nokia)

  - Proposal 2: In Rel-20 BS RF enhancement, propose to continue the investigation on the rest of co-located cases which are not covered in Rel-19. (ZTE)

- Recommended WF

  - Re-use Rel-19 coupling findings and continue to evaluate missing cases required for requirement derivation.

Issue 2-4-2: Near-field simulation methodology

- Proposals

  - Proposal 1: Establish simulation methodology required for side-by-side co-location scenario considering near-field effects. (Ericsson)

  - Proposal 2: Evaluate if more simulation or measurement results are needed for BS-to-BS isolation. (Nokia)

- Recommended WF

  - Establish simulation methodology and simulation assumptions needed to model coupling loss between two base stations in the near-field region.

### Sub-topic 2-5: Requirment derivation

Issue 2-5-1: OTA transmitter OFF power

- Proposals

  - Proposal: For OTA transmitter OFF power in Rel-20, we propose to consider following aspects: (ZTE)

    - 1) Step 1: to identify more appropriate MCL1 between two BSs according to the legacy deployment assumption from single operator and relevant BS’s antenna configuration to derive the OTA core requirement (no intention to change the conducted core requirements according to the WID objective), then check whether TRP based OFF power measurement in the OTA chamber is feasible or not.

    - 2) Step 2: If TRP based OFF power measurement in step 1 is not feasible, then to define more appropriate/relevant co-location reference antenna for the OTA requirement improvement and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement.

- Recommended WF

  - TBA

Issue 2-5-2: OTA transmitter intermodulation

- Proposals

  - Proposal: For OTA transmitter intermodulation requirement in Rel-20, we propose to consider following aspects: (ZTE)

  - 1) Step 1: Discuss more appropriate co-location reference antenna for FR1 high bands as alternative of the existing testing antenna to emulate the more realistic deployment.

  - 2) Step 2: Discuss whether the transmitter intermodulation requirement for band n79 and n104 except for band n77/n78 could be optionally removed from regions which don’t mandate the Tx intermodulation testing.

- Recommended WF

  - TBA

Issue 2-5-3: OTA transmitter spurious emissions

- Proposals

  - Proposal: For OTA transmitter spurious co-location emission in Rel-20, we propose to consider following aspects: (ZTE)

    - 1) Step 1: To identify more appropriate MCL1 between co-located BS from different bands, then use the following equation to derive the corresponding TRP based transmitter spurious co-location emission requirement and check its feasibility of TRP based measurement in the OTA chamber firstly.

    - = -174dBm/Hz + 5dB + 50dB－7dB + MCL1+9dB (scaling factor)

    - 2) Step 2: If TRP based OTA transmitter spurious co-location emission in step 1 is not feasible, to discuss more appropriate co-location reference antenna and the relevant MCL2 assumption between DUT and improved co-location reference antenna to define the OTA core requirement.

- Recommended WF

  - TBA

Issue 2-5-4: OTA Receiver out-of-band blocking

- Proposals

  - Proposal: For OTA Receiver out-of-band blocking in Rel-20, we propose to consider following aspects: (ZTE)

    - 1) Step 1: Discuss more appropriate co-location reference antenna for FR1 high bands as alternative of the existing testing antenna to emulate the more realistic deployment.

    - 2) Step 2: Discuss whether we need to align the requirement for BS type 1-H and BS type 1-O.

- Recommended WF

  - TBA

# Topic #3: SBFD BS to SBFD BS adjacent channel coexistence

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513207 | CATT | Proposal 1: For two SBFD-capable BSs with adjacent operating bands, both BSs configured as {DU}, and two DL sub-bands are adjacent is the best configuration. Observation 2: For the BPF used after PA, the required suppression levels on the UL subband of the victimized BS are shown in table 2. Observation 3: The required suppression for the 2.6GHz, 4.9GHz, and 7GHz RF BPFs shown in table 2 are achievable. However, in practical applications, factors such as the volume and weight of BS also need to be taken into consideration. Proposal 3: The best configuration does not require the introduction of additional ACLR and/or OBUE requirements for FR1 WA SBFD BS. Observation 4: If the in-band blocking of victim SBFD-capable BS is -43dBm, the required total isolation between transmission of aggressor SBFD-capable BS and the reception of victim SBFD-capable BS is 92dB. Observation 5: The sum of the co-location coupling loss and the suppression of the interfering signal by the RF BPF used in the victim BS UL link must be greater than 92dB. Observation 6: For the RF BPF used in UL link of victim BS, the required suppression levels on the DL subband of the aggressor BS are shown in table 3. Observation 7: The required suppression for the 2.6GHz, 4.9GHz, and 7GHz RF BPFs shown in table 3 are achievable. However, in practical applications, factors such as the volume and weight of BS also need to be taken into consideration. Proposal 4: The best configuration does not require the introduction of additional in-band blocking for FR1 WA SBFD BS. |
| R4-2513744 | CMCC | Proposal 1: The priority of the co-located scenario could be higher. Observation 1: There is a possibility that in the n104 frequency band, three or even four operators may jointly deploy IMT services. Proposal 2: Scenarios where two operators are adjacent and three operators are adjacent need to be considered. Proposal 3: It is recommended to select the two typical bandwidth 100 MHz and 200 MHz for analysis. Proposal 4: It is suggested to analysis the coexistence of 100 MHz SBFD and 60 MHz SBFD. Proposal 5: During coexistence analysis, both DU configuration and DUD configuration should be considered. |
| R4-2513773 | Huawei, HiSilicon | Proposal 1: FR1 WA BS, it is proposed to focus on the scenario that operator's spectrum is larger or equal to 100 MHz. Observation 1: the coupling loss is band/frequency dependent, the higher frequency larger coupling loss is observed. Observation 2: The 0.1 m edge-to-edge separation is the worst-case assumption and it is not suitable for SBFD-SBFD co-existence. Proposal 2: It is proposed to further discuss on the typical co-location scenario and deployments for SBFD-SBFD co-existence. Observation 3: more than half of the base stations are more than 50 meters away from the interfering base stations in a real C band deployment. Observation 4: Interferer-victim separation of 70 metres is adopted to derive ECC BEM requirement for non-synchronised TDD in the 3400-3800 MHz band. Proposal 3: It is proposed to further discuss the interferer-victim separation or grid shift for SBFD-SBFD co-existence. Proposal 4: it is proposed to adopt the same deployment scenario for both TX unwanted emission and RX blocking requirements definition for adjacent SBFD-SBFD co-existence. Proposal 5: simulation assumptions as in Rel-18 SBFD SI can be used as starting point for SBFD-SBFD co-existence. > The scenarios to be studied are Scenario 1, defined in TR 38.858 > The case to be studied is Case 3 Proposal 6: it is proposed to run ACIR simulation to derive additional unwanted emission limit. Proposal 7: it is proposed to further discuss and decide on the percentage values to be used for in-band blocking limits. Observation 5: 80 MHz offset from the spectrum block edge for unwanted emission could be used as starting point |
| R4-2513791 | Samsung | Proposal 1: Only Scenario 1 should be adopted for the current SBFD-SBFD co-existence simulation. Observation 2: RAN4 should define new co-existence cases for SBFD-SBFD adjacent channel co-existence simulation. Proposal 2: RAN4 should consider SBFD configurations {U, D} and {D, U, D} in the SBFD-SBFD co-existence cases. Proposal 3: RAN4 could adopt Case 5 (SBFD UD – SBFD DU) and Case 7 (SBFD DUD – SBFD DUD) in SBFD-SBFD adjacent channel co-existence simulation for simplification. Proposal 4: RAN4 could follow simulation parameters in TR38.858 Annex E at the beginning stage of the SBFD-SBFD co-existence simulation. Proposal 5: RAN4 to adopt both co-location and non-co-location BS deployment assumption for the SBFD-SBFD co-existence study. Proposal 6: RAN4 to consider 7GHz as one of example bands in the future co-existence study. Proposal 7: RAN4 to consider the BS antenna assumption for 7GHz in TR 38.922 as starting point. |
| R4-2513962 | Tejas Network Limited | Observation 1: Full-site SBFD deployment and SBFD filtering for full-band suppression are mandatory to minimize inter-operator interference. Proposal 1: Recommend full-site SBFD deployment to prevent inter-site CLI and maximize SBFD benefits. Observation 2: For UMa deployments with SBFD as the aggressor network and a 100% GS, the 99th percentile of in-band blocking power is –30.47 dBm. With a 10% GS, the in-band blocking power is –3.82 dBm. Proposal 2: Confirm optional SBFD compliance (Note 1) and TDD fallback (Note 2) in TS 38.104/38.141-2, ensuring no legacy impact. |
| R4-2513969 | Ericsson | Observation 1: All operators in the band need to negotiate together to deploy SBFD with enhanced filtering dedicated to the SBFD-SBFD adjacent channel coexistence scenario. Observation 2: The proposed scenario will only suppress the inter-operator interference, meanwhile, the challenges related to other types of interference, e.g., self-interference, inter-sector interference, inter-site interference, remain and should be addressed separately. Observation 3: Operators need to deploy SBFD at every site on the same carrier to avoid severe inter-site interference that would degrade SBFD performance. Observation 4: Carrier and sub-band allocations within a band would need to be fixed due to hardware restrictions. Observation 5: Rel-20 work should ensure that all kinds of deployment scenarios can work, including re-use of existing sites, close (e.g., within 10 m of one another) and co-located BSs (tower sharing), without requiring operators’ collaboration on deploying sites. Proposal 1: RAN4 agrees that Rel-20 SBFD-SBFD co-existence work focuses on close (e.g., within 10 m of one another) and co-located BSs (tower sharing) to ensure operators to be able to reuse existing sites. Proposal 2: RAN4 to expand the Rel-20 technical work in a structured way: The work begins by establishing reference deployment scenarios, proceeding to system-level co-existence simulation to evaluate performance, co-location modelling, and if needed, derive necessary RF requirements based on the worst case of co-existence and co-location. |
| R4-2513994 | Qualcomm Germany | Observation 1: Work in Rel-18 and Rel-19 considered TDD to SBFD adjacent channel coexistence, which is in general limited by legacy TDD BS requirements, especially regarding ACIR enhancements. Proposal 1: RAN4 to discuss if adjacent channel coexistence framework in TR 38.858 could be leveraged to quantify expected level of ACIR enhancement needed to enable feasible SBFD-to-SBFD adjacent channel deployments. Proposal 2: RAN4 to identify which Tx/Rx requirements will be studied via adjacent channel coexistence for the Rel-20 SBFD-to-SBFD coexistence. |
| R4-2514279 | ZTE Corporation, Sanechips | Proposal 1: Prioritize SBFD BS coexisting with SBFD BS in the adjacent channel in band n104 in Rel-20; for band n79 and n41, propose to consider the SBFD BS coexisting with SBFD BS with the certain frequency gap taken into account. Proposal 2: For the SBFD BS vs SBFD coexistence study in the co-located deployment, propose to consider one typical value for the coexistence study which could be based on values from TR 37.941. Proposal 3: For BS to BS co-located deployment, propose to consider more practical installation assumptions instead of reusing the legacy 10cm separation distance as default assumption. Proposal 4: For SBFD BS coexisting with SBFD BS, propose to use the simulation assumptions in Table E.2.1-1 and E.2.3-1 in this contribution and other relevant simulation assumptions in TR 38.858 to conduct the coexistence study. Proposal 5: for the potential tightening unwanted emission and blocking requirement for SBFD BS in Rel-20, it is proposed to consider the victim’s system DL sub-band as guard band to minimize the victim system’s performance degradation; Proposal 6: For RF requirements of SBFD BS in Rel-20, propose to consider the following factors: 1) Potential more tightening emission and blocking requirements from coexistence study 2) DL sub-band size of the victim system; 3) Coupling loss between the aggressor and victim system for co-site case 4) Analog filter performance of aggressor’s transmitter 5) Analog filter performance of victim’s receiver; Proposal 7: propose to consider the work procedure in RAN4 for Rel-20 SBFD vs SBFD coexistence study. |
| R4-2514454 | Nokia | Observation 1: Both co-located and non-colocated scenarios are within the scope of the Rel-20 work item. Observation 2: Methodology for deriving the requirements can be based on coupling loss (MCL) and/or co-existence studies. The selection of one or another option can be dependent on the specific scenario (e.g. co-located vs non-colocated) Proposal 1: If coexistence simulations is to be used for the co-located scenario, the assumed grid-shift shall be no more than 10%. Observation 3: Considering the different propagation and BS beamforming characteristics, different simulations specific to each of the frequency bands listed in the WID (n104, n79, n41) may be required. Proposal 2: Proposal: It is proposed to reuse Rel-19 simulation assumptions for band n79 and n41. Proposal 3: RAN4 to discuss simulation assumptions for n104. Proposal 4: RAN4 shall also discuss whether it is feasible to achieve the identified BS TX and RX RF requirements considering practical BS implementation. |

## Open issues summary

### Sub-topic 3-1: Workplan

Issue 3-1: Workplan

- Proposals

  - Proposal 1: RAN4 to expand the Rel-20 technical work in a structured way: The work begins by establishing reference deployment scenarios, proceeding to system-level co-existence simulation to evaluate performance, co-location modelling, and if needed, derive necessary RF requirements based on the worst case of co-existence and co-location. (Ericsson)

  - Proposal 2: RAN4 to discuss if adjacent channel coexistence framework in TR 38.858 could be leveraged to quantify expected level of ACIR enhancement needed to enable feasible SBFD-to-SBFD adjacent channel deployments. (Qualcomm)

Proposal 3: Propose to consider the work procedure in RAN4 for Rel-20 SBFD vs SBFD coexistence study.

- Recommended WF

  - Establish a workplan for SBFD-to-SBFD adjacent co-existence including a simulation campaign.

### Sub-topic 3-2: SBFD configuration

Issue 3-1: SBFD Configuration

- Proposals

  - Proposal 1:  Proposal 1: For two SBFD-capable BSs with adjacent operating bands, both BSs configured as {DU}, and two DL sub-bands are adjacent is the best configuration. (CATT)

  - Proposal 2: During coexistence analysis, both DU configuration and DUD configuration should be considered. (CMCC)

  - Proposal 3: RAN4 should consider SBFD configurations {U, D} and {D, U, D} in the SBFD-SBFD co-existence cases. (Samsung)

  - Proposal 4: RAN4 could adopt Case 5 (SBFD UD – SBFD DU) and Case 7 (SBFD DUD – SBFD DUD) in SBFD-SBFD adjacent channel co-existence simulation for simplification. (Samsung)

- Recommended WF

  - TBA

### Sub-topic 3-3: Simulation parameters

Issue 3-3-1: Simulation assumptions

- Proposals

  - Proposal 1: Simulation assumptions as in Rel-18 SBFD SI can be used as starting point for SBFD-SBFD co-existence. (Huawei)

    - The scenarios to be studied are Scenario 1, defined in TR 38.858

    - The case to be studied is Case 3

  - Proposal 2 RAN4 could follow simulation parameters in TR38.858 Annex E at the beginning stage of the SBFD-SBFD co-existence simulation. (Samsung)

  - Proposal 3: For SBFD BS coexisting with SBFD BS, propose to use the simulation assumptions in Table E.2.1-1 and E.2.3-1 in this contribution and other relevant simulation assumptions in TR 38.858 to conduct the coexistence study. (ZTE)

  - Proposal 4: Only Scenario 1 should be adopted for the current SBFD-SBFD co-existence simulation. (Samsung)

- Recommended WF

  - As baseline use simulation parameters defined in TR 38.858.

Issue 3-3-2: Scenario

- Proposals

  - Proposal 1: The priority of the co-located scenario could be higher. (CMCC)

  - Proposal 2: Scenarios where two operators are adjacent, and three operators are adjacent need to be considered. (CMCC)

  - Proposal 3: It is proposed to further discuss on the typical co-location scenario and deployments for SBFD-SBFD co-existence. (Huawei)

  - Proposal 4: RAN4 to adopt both co-location and non-co-location BS deployment assumption for the SBFD-SBFD co-existence study. (Samsung)

  - Proposal 5: RAN4 agrees that Rel-20 SBFD-SBFD co-existence work focuses on close (e.g., within 10 m of one another) and co-located BSs (tower sharing) to ensure operators to be able to reuse existing sites. (Ericsson)

  - Proposal 6: For the SBFD BS vs SBFD coexistence study in the co-located deployment, propose to consider one typical value for the coexistence study which could be based on values from TR 37.941. (ZTE)

  - Proposal 7: For BS to BS co-located deployment, propose to consider more practical installation assumptions instead of reusing the legacy 10cm separation distance as default assumption. (ZTE)

  - Proposal 8: It is proposed to further discuss the interferer-victim separation or grid shift for SBFD-SBFD co-existence. (Huawei)

  - Proposal 9: If coexistence simulations is to be used for the co-located scenario, the assumed grid-shift shall be no more than 10%. (Nokia)

- Recommended WF

  - Define parameters for both co-existence and co-location scenarios. Initially focus on simulation parameters for the co-existence case. Postpone discussion on co-location modelling until discussions related to the other objective in WI is progressing.

Issue 3-3-3: Bands

- Proposals

  - Proposal 1: RAN4 to consider 7 GHz as one of example bands in the future co-existence study. (Samsung)

  - Proposal 2: RAN4 to consider the BS antenna assumption for 7GHz in TR 38.922 as starting point. (Samsung)

  - Proposal 3: Prioritize SBFD BS coexisting with SBFD BS in the adjacent channel in band n104 in Rel-20; for band n79 and n41, propose to consider the SBFD BS coexisting with SBFD BS with the certain frequency gap taken into account. (ZTE)

  - Proposal 4: Proposal: It is proposed to reuse Rel-19 simulation assumptions for band n79 and n41. (Nokia)

  - Proposal 5: RAN4 to discuss simulation assumptions for n104. (Nokia)

- Recommended WF

  - Start with band n79 to align simulation assumptions with TR 38.858.

  - Further discuss how to define parameters for band n104 and n41 considering alignment with simulation results produced in Rel-18 and Rel-19.

Issue 3-3-4: SBFD bandwidth

- Proposals

  - Proposal 1: It is recommended to select the two typical bandwidth 100 MHz and 200 MHz for analysis. (CMCC)

  - Proposal 2: It is suggested to analysis the coexistence of 100 MHz SBFD and 60 MHz SBFD. (CMCC)

  - Proposal 3: FR1 WA BS, it is proposed to focus on the scenario that operator's spectrum is larger or equal to 100 MHz. (Huawei)

- Recommended WF

  - 100 MHz

### Sub-topic 3-4: Requirment derivation

Issue 3-4: TBA

- Proposals

  - Proposal 1: it is proposed to adopt the same deployment scenario for both TX unwanted emission and RX blocking requirements definition for adjacent SBFD-SBFD co-existence. (Huawei)

  - Proposal 2: it is proposed to run ACIR simulation to derive additional unwanted emission limit. (Huawei)

  - Proposal 3: RAN4 to consider the BS antenna assumption for 7GHz in TR 38.922 as starting point. (Huawei)

  - Proposal 4: RAN4 to identify which Tx/Rx requirements will be studied via adjacent channel coexistence for the Rel-20 SBFD-to-SBFD coexistence. (Qualcomm)

  - Proposal 5: for the potential tightening unwanted emission and blocking requirement for SBFD BS in Rel-20, it is proposed to consider the victim’s system DL sub-band as guard band to minimize the victim system’s performance degradation; (ZTE)

  - Proposal 6: For RF requirements of SBFD BS in Rel-20, propose to consider the following factors: (ZTE)

    - 1) Potential more tightening emission and blocking requirements from coexistence study

    - 2) DL sub-band size of the victim system;

    - 3) Coupling loss between the aggressor and victim system for co-site case

    - 4) Analog filter performance of aggressor’s transmitter

    - 5) Analog filter performance of victim’s receiver;

  - Proposal 7:	RAN4 shall also discuss whether it is feasible to achieve the identified BS TX and RX RF requirements considering practical BS implementation. (Nokia)

- Recommended WF

  - TBA

### Sub-topic 3-5: Other

Issue 3-5: Other

- Proposals

  - Proposal 1: Recommend full-site SBFD deployment to prevent inter-site CLI and maximize SBFD benefits. (Tejas)

  - Proposal 2: Confirm optional SBFD compliance (Note 1) and TDD fallback (Note 2) in TS 38.104/38.141-2, ensuring no legacy impact. (Tejas)

- Recommended WF

  - TBA