# 3GPP TSG RAN WG4 Meeting#116bis	R4-2514483

# Prague, Czech Republic, Oct 13 – 17, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for NTN testing for NGSO

Agenda item:	6.6.3.3

Document for:	Discussion

# 1. Introduction

During the last meeting, RAN4 discussed issues related to the demodulation performance for NTN testing for NGSO [1]. In this paper, we share our perspectives on the remaining open issues of analysing the impact of sampling frequency offset (SFO) on demodulation performance and test applicability.

2. Impact of Sampling frequency offset

Sampling frequency offset impact for testing

- Agreement

  - Interesting companies including UE and TE vendors are encouraged to provide analysis and evaluation for the impact of sampling frequency offset in related to both the Doppler frequency and its rate of change (i.e., Doppler drift) caused by the time-varying Doppler shift model for NGSO.

  - FFS how and where to capture the impact of sapling frequency offset if identified

In the previous meeting, RAN4 discused the issue of sampling frequency offset due to time-varying Doppler shift and its potential impact on demodulation performance. The way forward is outlined below.

We note that both Doppler frequency and its rate of change, commonly referred to as Doppler drift, are directly influenced by satellite motion, which in turn impacts the sampling frequency offset (SFO) observed at the receiver. To demonstrate the impact of residual SFO, below we consider a unidirectional drift model with constant rate of change, as illustrated in Fig. 1.

Figure 1: A unidirectional drift model with constant rate of change

Based on the drift model presented above in Fig. 1, we consider various delay amplitudes (Delay Amp) values over a simulation duration (SIM_DUR) of 10 seconds, resulting in different levels of constant drifts in one direction and residual sample frequency offset (SFO, measured in ppm). We assumed tracking loop is synchronized at t=0sec. The summary below examines the impact of SFO for Test 1-2, identifying the SNR levels at which throughput degradation begins to occur.

Table 1: Impact of SFO for Test 1-2 (MCS 13, NTN TDL-C)

| Residual SFO (ppm) | Drift (uS/sec) | SIM_DUR (sec) | DelayAmp (uS) | SNR (dB) |
| --- | --- | --- | --- | --- |
| 0.2 | 0.2 | 10 | 2 | Minimal to no impact |
| 0.4 | 0.4 | 10 | 4 | Minimal impact |
| 0.8 | 0.8 | 10 | 8 | 4.5 |
| 1 | 1 | 10 | 10 | 3.5 |

The above observations suggest that there is no meaningful impact to the throughput performance with residual SFO values even at 0.2ppm, indicating robust performance against SFO.

Observation 1: Throughput performance remains robust for residual SFO even at 0.2 ppm, indicating minimal impact under reasonably high SFO values.

Proposal 1: Given the demonstrated robustness to residual SFO, RAN4 may consider omitting SFO-related impact analysis on demodulation performance.

3. Test Applicability

Test applicability of requirement with applying time-varying channel model

- Proposals

  - Option 1

    - The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration.

  - Option 2

    - For Rel-19 UE, if UE can pass new NGSO requirements applying the new channel model, then it can skip all NGSO requirements defined in Rel-17/Rel-18.

    - For Rel-17/Rel-18 UE, if UE can pass new NGSO requirements applying the new channel model, then it can skip all NGSO requirements defined in Rel-17/Rel-18.

- Other options are not precluded

RAN4 discussed the test applicability for the demodulation requirements during the last meeting. The way forward is outlined below.

As presented during RAN4#116, we would like to reiterate that applying a satellite motion-based time-varying channel model to pre-Rel-19 test cases offers a practical and consistent method for evaluating device performance under realistic conditions. Furthermore, this approach can be based on DUT declaration, thereby maintaining flexibility for vendors while ensuring alignment between test conditions and actual deployment environments. It is also important to note that RF core requirements are defined based on the Rel-17 channel model; therefore, applying the Rel-19 model to pre-Rel-19 test cases may introduce inconsistencies and could be problematic.

Proposal 2: The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration.

- Further consider the implication due to the RF core requirements being defined only with Rel-17 channel model.

4. Conclusions

This paper presents our perspectives on the impact of SFO on demodulation performance and test applicability. The following proposals have been made.

Observation 1: Throughput performance remains robust for residual SFO even at 0.2ppm, indicating minimal impact under reasonably high SFO values.

Proposal 1: Given the demonstrated robustness to residual SFO, RAN4 may consider omitting SFO-related impact analysis on demodulation performance.

Proposal 2: The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration.

- Further assess the implications, given that the RF core requirements are currently defined solely based on the Rel-17 channel model.

5. References

[1] R4-2512525, “Way Forward for [116][323] NTN_testing_NGSO_channel_model_and_demod,” 3GPP TSG RAN WG4 Meeting #116, Bengaluru, India, Aug 25 – 29, 2025.