3GPP TSG-WG4 Meeting #116bis	R4-2514496

# Prague, Czech Republic, 13-17 Oct. 2025

# Title:			On Intra-Band Contiguous CA GB and Outer1,2

# Agenda Item:	4.30.1

# Source:		Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

In this contribution we share some observations on two potential issues: one is related to the equations that define the Intra-Band contiguous Carrier Aggregation (CA) configuration parameters; the second is related to the equations that define the non-contiguous RB allocation MPR. We would like to collect the views of interested companies on these issues and we offer some initial solutions to address each issue.

- Discussion

  - On minimum Guard-Band

At RAN4 meeting 116, one company presented the example of a 100MHz + 60MHz intra-band contiguous CA configuration to show that the TS 38.101-1 equations may not always guarantee that the minimum Guard-Band (minGB) of each component carrier (CC) [1]. One of the proposed solutions consists in porting the TS 38.104 equations to TS 38.101-1. In this contribution we analyze several intra-band CA configurations to check if the minGB of the lower edge CC, denoted here “minGBCC,low”, and the minGB of the upper CC, denoted here “minGBCC,high” are met, using either TS 38.101-1 equations or TS 38.104 equations.

Throughout this document, we use the following additional acronyms as summarized in Figure 1:

- GB,low / GB,high, to denote the intra-band contiguous CA guard-band on the lower edge/upper edge of the CA channel bandwidth;

- Primary CC / Secondary CC (PCC/SCC), to denote the lowermost CC/uppermost CC of an intra-band contiguous CA with a maximum of two CCs;

- NCS to denote the Nominal Channel Spacing; and

- Other related acronyms specified in clause 5 and clause 6 are re-used without any change, e.g. BWChannel_CA, Foffset,low/Foffset,high, RBStart,low/RBStart,high etc.

Figure 1: Intra-band contiguous CA “GB,low / GB,high“ and lowermost/uppermost CC “minGBCC,low / minGBCC,high.”

The CA “GB,low / GB,high“ calculated using the equations of TS 38.101-1 and equations of TS38104 are compared against each CC’s “minGBCC,low / minGBCC,high” in Figure 2 (left, right) for a small set of CA configurations, where:

- GB,high margin = GB,high - minGBCC,high and cases where “GB,high <  minGBCC,high” are highlighted in red;

- The last column calculates the difference between the aggregated BW (PCC CBW + SCC CBW) and BWChannel_CA. Green highlighted cells means that BWChannel_CA < Aggregated BW.

Figure 2: “GB,low / GB,high“ margins for the example of SCC = 35MHz CBW vs PCC CBW. Left: TS38.101-1 results, right: TS38.104 results.

We make the following observation based on the calculations for the additional configurations provided in the Annex 1 for TS 38.101-1 and in the Annex 2 for TS 38.104.

Observation 1: Based on the results collected in the Annex 1 and 2, it seems that:

- The lower edge CC “minGBCC,low“ is always met using either equation set (TS 38.101-1 or TS 38.104). The worst case being CA configurations where “GB,low” is equal to “minGBCC,low”, the CA configuration provides 0Hz margin, i.e. GB,low ≥  minGBCC,low.

- The equations from both TS often fail to meet the upper edge CC “minGBCC,high”, i.e. GB,high < minGBCC,high . The failures do not seem to exceed more than one sub-carrier (SC) of the largest common µ.

Proposal 1: Interested companies are invited to check and confirm if the issues identified in the Observation 1 based on whether or not the small CA configuration sets of Annex 1 / Annex 2 are valid.

Assuming the above issue is confirmed valid, we see two options to correct “GB,high”

- Option 1: Increase “GB,high” by decreasing the nominal channel spacing AND by increasing Foffset,high, where:

  - Fedge,low, GB,low, Foffset,low, FC,low, BWChannel_CA and Fedge,high are unchanged;

- NCS, FC,high and BWgap are reduced; and

- Foffset,high and GB,high are increased.

- Option 2: Increase “GB,high” by increasing Foffset,high, where:

  - Fedge,low, GB,low, Foffset,low, FC,low, NCS, BWgap and FC,high are unchanged; and

  - BWChannel_CA , Foffset,high and GB,high are increased by 1 SC of the largest common µ.

Option 1 is not preferred:

- It changes the dual-UL IMD emissions due to closer proximity of each CC and

- It mis-aligns the definition of NCS between UE and gNB.

Option 2 is preferred:

- It does not change the UE NCS and hence does not change the 2UL IMD emissions;

- It does not break FFT grid alignment; and

- It ensures that BWChannel_CA is always less than or equal to the aggregated BW.

It should be noted however that Option 2 may impact the outer1/outer 2 definitions for some CA configurations. This aspect is treated in the next clause of this contribution where we show there is no impact when Option 2 is combined with the solutions proposed to address outer1,2 RB MPR issues.

Proposal 2: To ensure that “GB,high ≥  minGBCC,high”, consider increasing Foffset,high by adopting the following changes:

- From: 	Foffset,high = (NRB,high*12 - 1)*SCShigh/2 + BWGB (MHz) and

- To:	Foffset,high = (NRB,high*12 + 1)*SCShigh/2 + BWGB (MHz).

An example of the Proposal 2 correction is shown in Figure 3, where:

- The GB,high margin is now zero for all cases, i.e. GB,high =  minGBCC,high , (white cells); and

- BWChannel_CA ≤ Aggregated BW (green and white cells).

Figure 3: “GB,low / GB,high“ margins for the example of SCC = 35MHz CBW vs PCC CBW. Left TS38.101-1, right Proposal 2 corrections.

The Annex 3 captures the results of proposal 2 for other intra-band contiguous CA configurations and shows that for this limited set of configurations:

- GB,high ≥  minGBCC,high;

- GB,low ≥  minGBCC,low (not impacted by proposal 2); and

- BWChannel_CA ≤ Aggregated BW.

For information purposes only, we have tentatively applied Proposal 2 to the TS 38.104 and found that this change also addresses the issues of Observation 1.

  - On inconsistent Outer1, Outer2 MPR

For the special CA configurations where the CBW of each CC are equal and equal SCS, the non-contiguous RB allocation equations seem to produce inconsistent MPR requirements for symmetrical RB allocation configurations.

Figure 4 shows one example of PCC/SCC 20MHz/20MHz (both at SCS 15kHz) where, for the example of PC3 operation and CP-OFDM QPSK waveforms we find:

- RB allocation 1RBStart=0 + 1RBStart=0 is granted the Outer 2 MPR, say 12dB MPR; and

- RB allocation 1RBStart=105 + 1RBStart=105 is granted the Outer 1 MPR, say 6.5dB MPR,

i.e. a large difference of MPR allowance for 2UL IMD emissions that are symmetrical, hence in theory should be granted the same MPR.

Figure 4: Example of inconsistent MPR requirements for non-contiguous RB allocations within an intra-band contiguous CA configuration.

In Figure 5, we expand the investigation to other similar CA configurations where:

- The term “NC_Outer1/2” denotes an non-contiguous (NC) Outer1/2 RB allocation/MPR; and

- Results are calculated for three RB allocations: 1RBStart=0 + 1RBStart=0, 1RBStart=max + 1RBStart=max, and 1RBStart=0 + 1RBStart=max.

Figure 5: Outer 1, outer 2 MPR requirements for different intra-band contiguous CA configurations with equal CBW and SCS per CC.

Observation 3: Based on the results of Figure 5:

- For CC1/CC2 CBW ≥ 20MHz (equal CBW and same SCS on each CC), the current equations provide inconsistent Outer 1/Outer 2 results for symmetrical 1RB+1RB positioned at lowest/highest position within each CC.

- For CC1=CC2 CBW ≤ 15MHz, we have consistent Outer 2 results.

- It is not clear which results are technically correct.

To lift the uncertainty on the issue reported in Observation 2, we go back to the definitions of NC inner, outer 1, outer 2 proposed in [2], which can be summarized as:

A NC-inner RB allocation is an allocation where:

  - The IMD3 falls within the allocated aggregated bandwidth BWChannel_CA; and

  - The IMD5 falls within the BWChannel_CA ranges immediately adjacent to the allocated aggregated bandwidth.

A NC-outer1 RB allocation is an allocation where:

  - Both he IMD3 and the IMD5 fall within the BWChannel_CA ranges immediately adjacent to the allocated aggregated bandwidth, a region denoted here “ACLR1” region.

A NC-outer2 RB allocation is an allocation where:

  - The IMD3 falls within the BWChannel_CA ranges immediately adjacent to the allocated aggregated bandwidth;

  - The IMD5 falls more than BWChannel_CA away from the allocated aggregated bandwidth, a region denoted here “>ACLR1” region.

We perform the IMD analysis check for each CA configuration of Figure 5. The IMD analysis shows that:

- For CC1/CC2 CBW ≥ 20MHz (equal CBW and same SCS on each CC), the NC RB allocation 1RBStart=0 + 1RBStart=0 fulfills the Outer 1 criterium and not the Outer 2. It seems the results produced by the current equations is due to the adoption of the “floor” operator.

- For CC1/CC2 CBW ≤ 15MHz (equal CBW and same SCS on each CC), the Outer 2 results are confirmed by the IMD landscape analysis. Yet it may be argued that if these allocations truly deserve being granted the outer 2 MPR due to very small IM overlap into the “>ACLR1” region.

For example, in a 10MHz + 10MHz CA configuration (both CC at SCS15kHz) we find that for:

- 1RBStart=0 + 1RBStart=0, the lower IMD5 indeed “fall” slightly into the “>ACLR1” region, but only by a small 7.5kHz overlap. This overlap represents ~0.8% of the IMD5 BW, i.e. unlikely that this small overlap requires the large outer 2 MPR allowance; and

- 1RBStart=51 + 1RBStart=51, both the IMD3 and the IMD5 “fall” into the ACLR1 region. The higher IMD5 upper edge is 7.5kHz within the higher edge of the ACLR1 high region.

It is very unlikely that such tiny IMD5 overlaps are sufficient to justify granting the Outer 2 MPR requirements for these 1RB + 1RB allocations.

Observation 4: Based on the IMD landscape analysis of each of the Figure 5 CA configurations and based on the definitions of [2] we observe that:

- For CC1/CC2 CBW ≥ 20MHz (equal CBW and same SCS on each CC), the NC RB allocation 1RBStart=0 + 1RBStart=0 fulfills the Outer 1 criterium and not the Outer 2. It seems the results produced by the current equations is due to the adoption of the “floor” operator;

- For CC1/CC2 CBW ≤ 15MHz (equal CBW and same SCS on each CC), the Outer 2 results are confirmed by the IMD landscape analysis. But in most cases, the IMD5 overlap into the “>ACLR1” region is so small relative to the IMD5 BW that it can be argued the Outer 2 MPR allowance is not justified.

Proposal 3: To correct the inconsistent outer 1/outer 2 MPR requirements for intra-band contiguous CA configurations where both CCs are configured with the same CBW and same SCS. consider adopting the following changes to RBStart,low and RBStart,high for non-contiguous RB allocations (clause 6.2A.2.1):

Change RBStart,low

from	RBStart,Low = max(1,  2 ∙ NRB_alloc – floor( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz))

to 	RBStart,Low = max(0,  2 ∙ NRB_alloc – ceiling( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz))

Change RBStart,high

from	RBStart,High = floor((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

to 	RBStart,High = ceiling((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

In the Annex 4, we capture the results of the combined Proposal 2 and Proposal 3 corrections to the entire set of CA configurations analyses used in this contribution.

In the Annex 5, we apply Proposal 2 and Proposal 4 corrections to the RAN5 TS 38.521-1 configurations specified for UL CA_n41C.

The results of these two Annexes show that implementing both  Proposal 2 and Proposal 3 corrections seem to resolve the issues captured in Observation 1 and Observation 3.

Observation 5: Based on the results captured in  Annex 4 and Annex 5, it seems that applying corrections from both Proposal 2 and Proposal 3 address the issues captured in Observations 1 and 3. They do not impact RAN5. Note that certain RAN5 configurations need to be corrected to become contiguous RB Inner allocations (Annex 5).

- Conclusion

In this contribution we propose identify two issues with the requirement specifications of intra-band contiguous CA:

- Many CA configurations fail to meet the upper CC minGB;

- For CA configurations with equal CBW and equal SCS on each CC, the MPR for outer1,2 1RB+1RB allocations are inconsistent across different configurations.

Solutions to address both issues are tentatively proposed in Proposals 2 and 3. Interested companies are invited to further check the validity of these observations and need (or not) to address these issues.

Observation 1: Based on the results collected in the Annex 1 and Annex2, it seems that:

- The lower edge CC “minGBCC,low“ is always met using either equation set (TS 38.101-1 or TS 38.104). The worst case being CA configurations where “GB,low” is equal to “minGBCC,low”, the CA configuration provides 0Hz margin, i.e. GB,low ≥  minGBCC,low.

- The equations from both TS often fail to meet the upper edge CC “minGBCC,high”, i.e. GB,high < minGBCC,high . The failures do not seem to exceed more than one sub-carrier (SC) of the largest common µ.

Proposal 1: Interested companies are invited to check and confirm if the issues identified in the Observation 1 based on the small CA configuration sets of Annex 1 / Annex 2 are valid (or not).

Proposal 2: To ensure that “GB,high ≥  minGBCC,high”, consider increasing Foffset,high by adopting the following changes:

- From: 	Foffset,high = (NRB,high*12 - 1)*SCShigh/2 + BWGB (MHz);

- To:	Foffset,high = (NRB,high*12 + 1)*SCShigh/2 + BWGB (MHz).

Observation 3: Based on the results of Figure 5:

- For CC1/CC2 CBW ≥ 20MHz (equal CBW and same SCS on each CC), the current equations provide inconsistent Outer 1/Outer 2 results for symmetrical 1RB+1RB positioned at lowest/highest position within each CC

- For CC1=CC2 CBW ≤ 15MHz, we have consistent Outer 2 results.

- It is not clear which results are technically correct.

Observation 4: Based on the IMD landscape analysis of each of the Figure 5 CA configurations and based on the definitions of [2] we observe that:

- For CC1/CC2 CBW ≥ 20MHz (equal CBW and same SCS on each CC), the NC RB allocation 1RBStart=0 + 1RBStart=0 fulfills the Outer 1 criterium and not the Outer 2. It seems the results produced by the current equations is due to the adoption of the “floor” operator.

- For CC1/CC2 CBW ≤ 15MHz (equal CBW and same SCS on each CC), the Outer 2 results are confirmed by the IMD landscape analysis. But in most cases, the IMD5 overlap into the “>ACLR1” region is so small relative to the IMD5 BW that it can be argued the Outer 2 MPR allowance is not justified.

Proposal 3: To correct the inconsistent outer 1/outer 2 MPR requirements for intra-band contiguous CA configurations where both CCs are configured with the same CBW and same SCS. consider adopting the following changes to RBStart,low and RBStart,high for non-contiguous RB allocations (clause 6.2A.2.1):

Change RBStart,low

from	RBStart,Low = max(1,  2 ∙ NRB_alloc – floor( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz))

to 	RBStart,Low = max(0,  2 ∙ NRB_alloc – ceiling( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz))

Change RBStart,high

from	RBStart,High = floor((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

to 	RBStart,High = ceiling((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

Observation 5: Based on the results captured in the Annex 4 and Annex 5, it seems that applying corrections from both Proposal 2 and Proposal 3 address the issues captured in Observations 1 and 3. They do not impact RAN5. Note that certain RAN5 configurations need to be corrected to become contiguous RB Inner allocations (annex 5).

# References

- R4-2509585 Intra-band Contiguous CA Issue and Proposed Resolution, 3GPP TSG-RAN WG4 Meeting # 116, Bengaluru, India, T-Mobile USA.

- R4-2006637 [NR FR1 ULCA] Contiguous Intra-band ULCA Allocation Definitions, 3GPP TSG-RAN WG4 Meeting # 95e, Electronic meeting, Skyworks Solutions, Inc.

# Annexes