3GPP TSG-RAN WG4 Meeting #116bis	R4-2514806

Bengaluru, India, August 25th – 29th, 2025

Title: 	WF on NR_MIMO_demod

Agenda Item:	6.12.1

Source: 	Moderator (Samsung)

Document for:	Approval

Topic 1. UE demodulation/CSI reporting performance requirements

Sub-topic 1-1 General scope

Issue 1-1-1: Clarify if PMI reporting requirements should be specified for eTypeII-r19

Agreement:

-	RAN4 shall specify PMI reporting requirements for eTypeII-r19.

Issue 1-1-2: Clarify if PMI reporting requirements should be specified for typeII-Doppler-r19

Agreement:

-	RAN4 do not specify PMI reporting requirements for typeII-Doppler-r19.

Issue 1-1-3: Clarify if UE demodulation/CSI reporting requirements should be specified for SRS port grouping

Agreement:

-	Not introduce UE demodulation/CSI reporting requirements for SRS port grouping.

Issue 1-1-5: Clarify if CSI reporting requirements should be specified for UE reporting enhancements for CJT

Agreement:

-	Not introduce CSI reporting requirements for UE reporting enhancements for CJT.

Sub-topic 1-2 Test assumptions for typeI-SinglePanel-r19

Issue 1-2-1: How to solve the very low TP issue with random precoder

Agreement:

-	Keep current values of correlation coefficients.

Issue 1-2-2: Test metric for 64 CSI-RS ports

Proposals:

-	Option 1: Modify the legacy test metric to following:

- Option 1A: X=5dB

γ=tuetrnd tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trndis the throughput measured at SNRue+[XdB] with random precoding. X could be specified to make the γ value in the reasonable range.

-	Option 2: Consider possibility to define requirement only based on SNR of 90% of the maximum throughput when PMI is followed

Way forward:

-	FFS based on simulation results provided in following meetings

Issue 1-2-3: MCS level for 64 CSI-RS ports

Agreement:

-	MCS22

Issue 1-2-4 and Issue 1-2-5: ZP-CSI-RS, CSI-IM, NZP-CSI-RS configurations for 64 CSI-RS ports

Agreement:

-	For the ZP-CSI-RS, CSI-IM and NZP-CSI-RS configurations, use below configuration

Issue 1-2-6: CQI/RI/PMI delay for FDD 64 CSI-RS ports

Agreement:

-	Configure CSI report slot offset and PMI delay as 7 slots and 11ms.

Issue 1-2-7: CQI/RI/PMI delay for TDD 64 CSI-RS ports

Agreement:

-	Configure CSI report slot offset and PMI delay as 11 slots and 9ms.

Issue 1-2-9: round-off operation of MIMO correlation matrices

Agreement:

-	Companies to provide simulation results based on either option 1 or option 2, the results based on both options will be taken into account for deriving the requirements.

-	Further discuss and decide which option to be captured in the CR in next meeting

  - Option 1: Include round-off to four digits precision operation

  - Option 2: Not include round-off to four digits precision operation

Issue 1-2-10: Scaling factors for MIMO correlation matrices

Agreement:

-	For option 1 in Issue 1-2-9, as baseline, adopt the following scaling factors for 64 CSI-RS ports XP-Medium correlation level matrices to ensure PSD matrices with a measure of numerical stability

| Array Type | Antenna Configuration | Correlation Level | Scaling Factor |
| --- | --- | --- | --- |
| Cross-polarized | 64 (8,4,2)×2 | Medium | 0.00065 |
| Cross-polarized | 64 (8,4,2)×4 | Medium | 0.00092 |

-	Further update on the scaling factor is not precluded.

Issue 1-2-11: Additional codebook mode for typeI-SinglePanel-r19?

Agreement:

-	Not specify additional test case for typeI-SinglePanel-r19, with typeI-codebookMode-r19 as Scheme B.

Issue 1-2-12: Whether to specify requirements for 128 CSI-RS ports for typeI-SinglePanel-r19?

Agreement:

-	Not specify requirements for 128 CSI-RS ports for typeI-SinglePanel-r19.

Sub-topic 1-3 Test assumptions for eTypeII-r19

Issue 1-3-1: MCS level

Agreement:

-	MCS22 (Table 1)

Issue 1-3-2: MIMO correlation matrixes

Agreement:

-	Same as typeI-SinglePanel-r19

Issue 1-3-3: Test metric for 64 CSI-RS ports

Agreement:

-	Follow the agreement for typeI-SinglePanel-r19 as in Issue 1-2-2

Note: For eType II codebook, randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A.

Topic 2. BS demodulation performance requirements

Sub-topic 2-1 General scope

Issue 2-1-1: Clarify if demodulation requirements should be specified for 3-antenna-port codebook-based UL transmissions

Agreement:

-	RAN4 should specify PUSCH demodulation requirements for 3-antenna-port codebook-based UL transmissions.

-  FFS on whether to introduce test applicability rule for 3Tx

Sub-topic 2-2 Test assumptions for PUSCH demodulation requirements

Issue 2-2-1: SCS and CHBW for FDD

Agreement:

-	15kHz SCS, 5MHz, 50MHz

Issue 2-2-2: SCS and CHBW for TDD

Agreement:

-	30kHz SCS, 10MHz, 100MHz

Topic 3. CR split

Draft CRs are expected on RAN4#117 meeting.

| Topic | Section | Company |
| --- | --- | --- |
| PMI reporting requirements for up to 128 CSI-RS ports | 6.1.1 Applicability of requirements | CATT |
|  | 6.3.2.1.X1 Single PMI with 64TX TypeI-SinglePanel Codebook | Huawei |
|  | 6.3.2.1.X2 Multiple PMI with 64TX Enhanced Type II Codebook | Huawei |
|  | 6.3.2.2.X1 Single PMI with 64TX TypeI-SinglePanel Codebook | Nokia |
|  | 6.3.2.2.X2 Multiple PMI with 64TX Enhanced Type II Codebook | Ericsson |
|  | 6.3.3.1.Y1 Single PMI with 64TX TypeI-SinglePanel Codebook | MediaTek |
|  | 6.3.3.1.Y2 Multiple PMI with 64TX Enhanced Type II Codebook | Apple |
|  | 6.3.3.2.Z1 Single PMI with 64TX TypeI-SinglePanel Codebook | QC |
|  | 6.3.3.2.Z2 Multiple PMI with 64TX Enhanced Type II Codebook | Ericsson |
|  | A.3.2.1.1 Reference measurement channels for SCS 15kHz | Samsung |
|  | A.3.2.3.3 Reference measurement channels for SCS 30kHz | Apple |
|  | B.2.3 MIMO Channel Correlation Matrices | Nokia |
|  | Big CR for 38.101-4 | Samsung |
| PUSCH demodulation requirements for 3Tx transmission (if specified) | PUSCH requirements 8.2.1.2 in 38.104 | Samsung |
|  | PUSCH requirements in 38.141-1 for FR1 conducted test | Ericsson |
|  | PUSCH requirements in 38.141-2 for FR1 OTA test | ZTE |
|  | Measurement of performance requirements and BS type 1-C and Type 1-H performance requirements in 38.141-1 | Ericsson |
|  | Measurement of performance requirements and BS type 2-O performance requirements in 38.141-2 | Ericsson |
|  | FRC for 3 ports in 38.104 | ZTE |
|  | FRC for 3 ports in 38.141-1 | Ericsson |
|  | FRC for 3 ports in 38.141-2 | CATT |
|  | Big CR for 38.104 | Samsung |
|  | Big CR for 38.141-1 | Ericsson |
|  | Big CR for 38.141-2 | ZTE |