3GPP TSG-RAN WG4 Meeting # 116bis											draftR4-2514424

Prague, CZ, October 13th – 17th, 2025

Agenda item:			4.1.3

Source:	Moderator (Ericsson)

Title:	Topic summary for [116bis][301] BSRF_Maintenance

Document for:	Information

# Introduction

The scope of this topic summary is BS RF maintenance agenda items. Topics are divided according to the agenda:

- NR FR1 7 MHz Channel Bandwidth	(4.9.2)	(No Tdocs)

- Enhancements for Air-to-ground network for NR	(4.12.2) 	(No Tdocs)

- Low-power wake-up signal and receiver for NR (LP-WUS/WUR)	(4.22.2)

- NR base station (BS) RF requirement evolution for FR1/FR2 and testing:
Expected EIRP mask for upper 6GHz	(4.28.1.1)

- NR base station (BS) RF requirement evolution for FR1/FR2 and testing: 
OTA test enhancement (co-location reference antenna) 	(4.28.1.2) 	(No Tdocs)

- Other Rel-19 non-spectrum related WIs	(4.29.2)	(No Tdocs)

- Rel-19 TEI	(4.30.3)	(Move to 11.1)

- RAN task and other topics:
Framework simplification for co-location/co-existence requirements (RP-243288)	(11.1)

# Topic #1: Low-power wake-up signal and receiver for NR (LP-WUS/WUR) (4.22.2)

## Companies’ contributions summary

Submitted CRs

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2513242 | CATT | draftCR to TR 38.774 on transmitted signal quality (clause 7.2.4) for LP-WUS Summary of change: The same change adapted to the latest approved TR RP-252465 is made |
| R4-2513933 | ZTE Corporation, Sanechips | Draft CR to TS38.104 on transmission bandwidth configuration for LP-WUS Summary of change: Introduce LP-WUS transmission bandwidth configuration of LP-WUS for both FR1 and FR2-1. |

# Topic #2: NR BS RF requirement evolution for FR1/FR2 and testing: Expected EIRP mask for upper 6GHz (4.28.1.1)

## Companies’ contributions summary

Submitted CRs

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2514111 | Ericsson | CR to TS 38.104: improvements related to Expected EIRP for band n104 Summary of change: corrections and text improvements |
| R4-2514449 | Nokia | CR to TS 38.106 with improvements for EIRP mask for NCR Summary of change: Corrections and text improvements |
| R4-2514450 | Nokia | CR to TR 38.908 with aligment for definitions and test procedure Summary of change: Corrections and text improvements |

# Topic #3: RAN task – Framework simplification for co-location/co-existence requirements (RP-243288) (11.1)

## Companies’ contributions summary

Discussion papers

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2514477 | Huawei, HiSilicon | Proposal 1: Modifications presented in Track Changes shall be reflected in all Draft CRs which are part of this RAN task. Proposal 2: Remove TS 36.143 from this RAN task and keep legacy tables. |
| R4-2513158 | CATT | Proposal 1: Update the NOTE1 for co-existence requirement as the following to solve the deleting note in note column of original table. NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in table 6.6.5.5.1.3-1 do not apply for the frequency range extending ΔfOBUE immediately outside the downlink operating band (see TS 38.104 [2], table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements. Proposal 2: RAN4 to discuss whether to update the simplified table for co-location requirement in endorsed draft CR for TS 38,141-1 as below: Frequency range of uplink operating band of the; System type to co-locate with; Basic limits (dBm/100kHz); Notes. co-located BS; WA BS; MR BS; LA BS. 824 - 849 MHz; GSM850 or CDMA850; -98; -91; -70; NOTE 1. 876 - 915 MHz; GSM900; -98; -91; -70. 1710 - 1785 MHz; DCS1800; -98; -91; -80. 1850 - 1910 MHz; PCS1900; -98; -91; -80. UL frequency range of n51, n91, n93, or UL frequency range of 51; NR or E-UTRA; N/A; N/A; -88. UL frequency range of n46, n53, or UL frequency range of 46, 53; NR or E-UTRA; N/A; -91; -88. UL frequency range of n100, n101; NR; -96; N/A; N/A. UL frequency range of n96, n102; NR; N/A; -90; -87. UL frequency range of n104; NR; -95; -90; -87. Frequency range of uplink operating band of the co-located BS; Other (NOTE 4, 5); -96; -91; -88. Proposal 3: RAN4 to discuss whether to update the simplified table for co-location requirement in endorsed draft CR for TS 37.107 as below: Frequency range of uplink operating band of the; System type to co-locate with; Basic limits (dBm/100kHz); Notes. co-located BS; WA BS; MR BS; LA BS. 824 - 849 MHz; GSM850 or CDMA850; -98; -91; -70; NOTE 1. 876 - 915 MHz; GSM900; -98; -91; -70. 1710 - 1785 MHz; DCS1800; -98; -91; -80. 1850 - 1910 MHz; PCS1900; -98; -91; -80. UL frequency range of n51, n91, n93, or UL frequency range of 49, 51; NR, or E-UTRA; N/A; N/A; -88. UL frequency range of n46, n53, or UL frequency range of 46, 53; NR, or E-UTRA; N/A; -91; -88. UL frequency range of n100, n101; NR; -96; N/A; N/A. UL frequency range of n96, n102; NR; N/A; -90; -87. UL frequency range of n104; NR; -95; -90; -87. Frequency range of uplink operating band of the co-located BS; Other (NOTE 4, 5); -96; -91; -88. NOTE: Moved from AI 4.30.3 to AI 11.1. |

Submitted CRs

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513063 | Nokia | (TEI19-BDaT_simp_improvement) CR to 36.141 on framework simplification for co-location/co-existence requirements Summary of change: Relevant Clauses are updated. |
| R4-2513064 | Nokia | (TEI19-BDaT_simp_improvement) CR to 37.104 on framework simplification for co-location/co-existence requirements Summary of change: Relevant Clauses are updated. |
| R4-2513065 | Nokia | (TEI19-BDaT_simp_improvement) CR to 38.141-1 on framework simplification for co-location/co-existence requirements Summary of change: Relevant Clauses are updated. |
| R4-2514033 | ZTE Corporation | (TEI19-BDaT_simp_improvement) draftCR to TS 37.141: spec structure simplification for co-location/co-existence requirements Summary of change: The table is simplified to include the general requirement level and exceptions. |
| R4-2514034 | ZTE Corporation | (TEI19-BDaT_simp_improvement) draftCR to TS 38.115-1 spec structure simplification for co-locationco-existence requirements Summary of change: The table is simplified to include the general requirement level and exceptions. |
| R4-2514035 | ZTE Corporation | (TEI19-BDaT_simp_improvement) CR to TS 38.176-1: spec structure simplification for co-locationco-existence requirements Summary of change: The table is simplified to include the general requirement level |
| R4-2514036 | ZTE Corporation | (TEI19-BDaT_simp_improvement) CR to TS 38.176-2: spec structure simplification for co-locationco-existence requirements Summary of change: The table is simplified to include the general requirement level |
| R4-2514108 | Ericsson | CR to TS 38.104: Simplification of tables for additional spurious emission requirements for co-existence and co-location Summary of change: The tables are restructured to include the general requirement level and exceptions. |
| R4-2514109 | Ericsson | CR to TS 37.105: Simplification of tables for additional requirements for co-existence and co-location Summary of change: The tables are restructured to include the general requirement level and exceptions. |
| R4-2514110 | Ericsson | CR to TS 36.104: Simplification of tables for additional requirements for co-existence and co-location Summary of change: The tables are restructured to include the general requirement level and exceptions. |
| R4-2514478 | Huawei, HiSilicon | (TEI19-BDaT_simp_improvement) Draft CR to TS 38-141-2: simplification of co-location and co-existence requirement tables Summary of change: Placeholder introduction for the new simplified co-ex and co-location requirement tables. |
| R4-2514479 | Huawei, HiSilicon | (TEI19-BDaT_simp_improvement) Draft CR to TS 37.145-2: simplification of co-location and co-existence requirement tables Summary of change: Placeholder introduction for the new simplified co-ex and co-location requirement tables. |
| R4-2513159 | CATT | (TEI19)CR for 38.106, on framework simplification for co-location/co-existence requirement[BDaT_simp_improvement] Summary of change: 1) Replace co-existence spurious requirement in Table 6.5.4.2.2-1 with new format table. 2) Replace co-location spurious requirement in Table 6.5.4.2.3-1 with new format table. NOTE: Moved from AI 4.30.3 to AI 11.1. |
| R4-2513160 | CATT | (TEI19)CR for 38.174, on framework simplification for co-location/co-existence requirement[BDaT_simp_improvement] Summary of change: 1) Replace co-existence spurious requirement in Table 6.6.5.2.2-1 with new format table. 2) Replace co-location spurious requirement in Table 6.6.5.2.3-1 with new format table. NOTE: Moved from AI 4.30.3 to AI 11.1. |
| R4-2513161 | CATT | (TEI19)CR for 37.145-1, on framework simplification for co-location/co-existence requirement[BDaT_simp_improvement] Summary of change: 1) Replace co-existence spurious requirement in Table 6.6.6.5.2.5-1 with new format table. 2) Replace co-location spurious requirement in Table 6.6.6.5.2.6-1 with new format table. 3) Replace blocking co-location requirement for MSR in Table 7.5.5.1.2-1 with new format table. 4) Replace blocking co-location requirement for single RAT UTRA FDD operation in Table 7.5.5.2-4 with new format table. 5) Replace blocking co-location requirement for single RAT UTRA TDD 1.28 Mcps option operation in Table 7.5.5.3.2-1 with new format table. 6) Replace blocking co-location requirement for single RAT E-UTRA operation in Table 7.5.5.4.2-1 with new format table. NOTE: Moved from AI 4.30.3 to AI 11.1. |
| R4-2513162 | CATT | (TEI19)CR for 36.106, on framework simplification for co-location/co-existence requirement[BDaT_simp_improvement] Summary of change: 1) Replace co-existence spurious requirement in Table 9.2.2.1-1 with new format table. 2) Replace co-location spurious requirement in Table 9.2.3.1-1 with new format table. NOTE: Moved from AI 4.30.3 to AI 11.1. |

## Open issues summary

### Sub-topic 3-1

Issue 2-1: Note 1 for “frequency range not applicable” for co-existence requirement

- Proposals

  - Proposal 1: Update the NOTE1 for co-existence requirement to solve the deleting note in note column of original table. (CATT, R4-2513158)

- Recommended WF

  - TBA

Issue 2-2: Missed co-location requirement in exemplified CR for TS 38.141-1 and TS 37.104

- Proposals

  - Proposal 1: RAN4 to discuss whether to update the simplified table for co-location requirement in endorsed draft CR for TS 38.141-1 and TS 37.104. (CATT, R4-2513158)

- Recommended WF

  - TBA

Issue 2-3: Further modifications based on the content captured in the WF in R4-2512581

- Proposals

  - Proposal 1: Modifications presented in Track Changes shall be reflected in all Draft CRs which are part of this RAN task. (Huawei, R4-2514477)

- Recommended WF

  - TBA

Issue 2-3: Further modifications based on the content captured in the WF in R4-2512581

- Proposals

  - Proposal 1: Remove TS 36.143 from this RAN task and keep legacy tables. (Huawei, R4-2514477)

- Recommended WF

  - TBA