3GPP TSG-RAN WG4 Meeting # 116bis											R4-2513519

Prague, Czech Republic, October 13th – October 17th, 2025

Agenda item:			6.18.1

Source:	Moderator (Nokia)

Title:	Topic summary for [116bis][213] NR_XR_Ph3_RRM

Document for:	Information

# Introduction

This summary includes the proposals and issues for discussion related to Rel-19 enhancements for extended reality for NR Phase 3, under thread [116bis][213] NR_XR_Ph3_RRM.

This summary is based on the contributions submitted to the agenda items:

- 4.24	XR for NR Phase 3	[NR_XR_Ph3]

  - 4.24.1	RRM core requirements	[NR_XR_Ph3-Core]

- 6.18	XR for NR Phase 3	[NR_XR_Ph3-Perf]

  - 6.18.1	Moderator summary and conclusions	[NR_XR_Ph3-Perf]

  - 6.18.2	RRM performance requirements	[NR_XR_Ph3-Perf]

The issues for discussion are organized in topics which are as follows:

- Topic #1: RRM core requirements

- Topic #2: RRM performance requirements

The RAN4 #116 bis meeting is the last meeting planned for RRM core requirements, therefore, the recommended priority for discussion is as follows:

- Issue 2-1-1: Scenarios for the definition of the test cases

- Issue 2-1-2: Per UE and per FR gaps

- Issue 2-1-3: Type 1 and Type 2 gaps

- Issue 2-1-4: DRX proposals

- Issue 2-1-5: Test target

- Issue 2-1-6: List of test cases

- Issue 2-1-7: Complete list of test cases for discussion

- Issue 2-4-1: Toffset between DCI and start of MG

- Issue 2-3-1: Cancelation model for XR test cases

- Issue 2-3-2: Cancelation model for XR test cases with Type 2 gaps

- Issue 2-3-3: Cancelation ratio for Type 1 gaps

- Issue 2-3-4: Cancelation ratio for Type 2 gaps

# Topic #1: Core requirements maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2514262 | Nokia, Qualcomm | Rapporteur Inputs <moderator> document for information |
| R4-2514040 | ZTE Corporation, Sanechips | Draft CR on R19 XR <moderator> Draft CR without proposals |
| R4-2514263 | Nokia | Maintenance of XR core requirements <moderator> Draft CR without proposals |
| R4-2514305 | Ericsson | draftCR38133 Measurement requirements with XR <moderator> Draft CR without proposals |

## Open issues summary

### Sub-topic 1-1 RRM core maintenance CRs

#### Issue 1-1: Draft CR R4-2514040

- Changes on clauses

  - 9.1.y.4, 8.1.2.2, 8.1.3.2, 8.5.2.2, 8.5.3.2

- Recommended WF

  - Email discussion

#### Issue 1-2: Draft CR R4-2514263

- Changes on clauses

  - 9.1.14, 9.4.2, 9.4.3, 9.15.5

- Recommended WF

  - Email discussion

#### Issue 1-3: Draft CR R4-2514305

- Changes on clauses

  - 9.1.14, 8.1.2.2, 8.1.3.2, 8.5.2.2, 8.5.3.2, 9.2.5, 9.2.6, 9.3.4, 9.3.5, 9.3.9

- Recommended WF

  - Email discussion

# Topic #2: RRM performance requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513111 | OPPO | Discussion on RRM performance requirements for XR Proposal 1: Consider both per-FR gap and per-UE gap in the test case. Proposal 2: Consider Rel-17 concurrent gaps in the test case. Proposal 3: Define test cases for: - Intra-frequency measurements with gaps - Inter-frequency measurements with gaps - Inter-RAT E-UTRAN FDD measurements - Inter-RAT E-UTRAN TDD measurements - Inter-frequency L1-RSRP with MG Proposal 4: Define the test cases for the cases without DRX and with DRX. Proposal 5: UE Tx/Rx behavior in the skipping MG occasions and the extended delay requirements should be verified in the same test case. |
| R4-2513473 | Huawei, HiSilicon | Discussion on Performance requirements for XR Phase 3 Proposal 1: RAN4 defines the test cases for both per-UE and per-FR MGs. Proposal 2: RAN4 define the test cases for both Type 1 and Type 2. Proposal 3: The test cases for XR should be defined with the non‑DRX cases as a high priority. Proposal 4: The processing time of 3ms/5ms shall be verified in the test cases. Proposal 5: RAN4 to design a fixed cancellation gap pattern for test cases, and further discuss the pattern. |
| R4-2513648 | CMCC | Discussion on RRM performance requirements for XR (eXtended Reality) Proposal 1: it is proposed to define test cases for NR-SA. Proposal 2: it is proposed to define test cases for FR1 and FR2-1. Proposal 3: it is proposed to define test cases for Type 1 measurement gap and Type 2 measurement gap ( e.g. the colliding case). Proposal 4: it is proposed to consider both the case without DRX and the case with DRX. Proposal 5: it is proposed to define test cases for intra-frequency measurement, inter-frequency measurement, and LTE inter-RAT measurement. Proposal 6: it is proposed to define test cases for both 3ms offset and 5ms offset. And applicability rules can be considered, e.g. the UE supporting 3ms offset only need to pass the test with 3ms timeline, the UE not supporting 3ms offset need to pass the test with 5ms timeline. |
| R4-2513890 | ZTE Corporation, Sanechips | Discussion on Perf part of R19 XR Proposal 1: Both FR1 NR SA and FR2-1 NR SA should be verified in the test. Proposal 2: The skipping in both Type-1 MG and Type-2 MG should be verified in different tests. Proposal 3: The MG skipping has impact on L3 & L1 measurements with gap. The impact by measurement skipping on the following types of L3&L1 measurement with gap should be verified: - Intra-frequency measurement with gap - Inter-frequency measurement with gap - EUTRAN measurement with gap - L1 inter-frequency measurement with gap Proposal 4: The MG skipping also impacts the L3 & L1 measurements without gap when these measurement occasions partially or fully overlapping with MG occasions. Such impacts should be verified as well. Proposal 5: The UE processing timeline of receiving the DCI based MG skipping indication should be verified independently or together with out test motivation. |
| R4-2514077 | vivo | On RRM performance requirements for XR Proposal 1: Define test cases to very core requirements for intra-frequency measurement with gaps, inter-frequency measurement with gaps, inter-frequency L1-RSRP measurement with gaps and inter-RAT E-UTRAN measurements with per-UE and per-FR gaps when gap cancellation is enabled. Proposal 2: Combination of gap-based measurement and per-UE/per-FR gap should be selected properly to avoid unnecessary tests. Proposal 3: In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancellation. Proposal 4: RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Proposal 5: Time offset for gap cancellation indication is 5ms in the tests. Proposal 6: Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests. |
| R4-2514246 | MediaTek inc. | Discussion on RRM performance requirements for XR Proposal 1: RAN4 RRM to define test cases for NR-SA, and FR1 and FR2-1. Proposal 2: RAN4 RRM to define test cases based on per-UE MGs configuration only. Proposal 3: RAN4 RRM to define test cases for Type-1 gap only. Proposal 4: RAN4 to define TCs that each type of test case is specified for either DRX or non-DRX but not both. Proposal 5: RAN4 to not define TCs for accuracy requirements due to MG cancellation nor TCs for UE processing timeline of receiving the DCI-based MG skipping indication. Proposal 6: RAN4 to define test cases for (i) delay extension with gap cancellation and (ii) reception/transmission without scheduling restrictions during cancelled gap occasions. Proposal 7: RAN4 to define TCs with working assumption that the gap is configured for inter-frequency measurement with gaps. Proposal 8: RAN4 RRM to define test cases for Type-1 gap only and there is no need to consider Type-2 gap in defining test cases. Proposal 9: RAN4 RRM should use a repetitive sequence where the test machine configures the UE with DCI containing MG cancellation according to this sequence. Proposal 10: RAN4 RRM define a single test case that include time offset of 5ms and 3ms with UE capability used as a condition. |
| R4-2514264 | Nokia | Discussion on XR ph3 RRM performance requirements - RAN4 to agree on developing XR measurement gap cancelation test cases provided in Table 1 below cases to ensure enough test coverage while keeping a reasonable RAN4 workload. Gap type; Intra/Inter/Inter-RAT; Frequency Range; DRX; Test type. Type 1 gap per UE; Intra-frequency; FR1; OFF; Meas procedure. FR2-1; OFF; Meas procedure. Inter-frequency; FR1; ON; Meas procedure. OFF; Meas procedure + accuracy. FR2-1; ON; Meas procedure. OFF; Meas procedure + accuracy. Inter-RAT E-UTRAN TDD; FR1 (serving); ON; Meas procedure. OFF; Meas procedure + accuracy. Type 2 gaps per UE; Intra-frequency; FR1; OFF; Meas procedure. FR2-1; OFF; Meas procedure. Inter-frequency; FR1; OFF; Meas procedure. FR2-1; OFF; Meas procedure. L1 RSRP measurements with gaps; Inter-frequency; FR1; OFF; Meas procedure. FR2-1; OFF; Meas procedure. Proposal 2: RAN4 to define test cases for NR-SA. Proposal 3: RAN4 to define test cases for FR1 and FR2-1. Proposal 4: Test cases are specified only for per-UE MGs. Proposal 5: Test cases are defined for both Type 1 and Type 2 gaps. Proposal 6: RAN4 to define the test cases for both Type 1 and Type 2 gaps but should focus on only non-DRX test cases for Type 2 MGs. Proposal 7: Test cases without DRX configuration should have higher priority than with DRX configuration. a. Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG b. Develop TCs for inter-RAT with DRX and with Type 1 MG c. Focus on non-DRX test cases for Type 2 MG Proposal 8: RRM test cases should verify a. Accuracy requirements b. Delay extension with gap cancelation c. Reception/transmission without scheduling restrictions during cancelled gap occasions d. The UE processing timeline of receiving the DCI-based MG skipping indication Proposal 9: The accuracy requirements are tested in limited number of accuracy specific test, including a. Inter-frequency measurements with Type 1 gaps, without DRX in FR1 b. Inter-frequency measurements with Type 1 gaps, without DRX in FR2-1 c. Inter-RAT E-UTRAN TDD with Type 1 gaps, without DRX Proposal 10: RAN4 should define test cases for a. Intra-frequency measurements with gaps, b. Inter-frequency measurements with gaps, c. Inter-RAT E-UTRAN TDD measurements, d. Inter-frequency L1-RSRP with MG. Proposal 11: For Type 2 MGs test cases, include 2 gaps with partial overlap Proposal 12: For Type 2 MGs test cases, measurement gap configuration of A.6.6.18.2.1 can be reused, i.e., a. Low priority MG pattern #1 with MGRP=40ms, offset=39ms b. High priority MG pattern #2 with MGRP=80ms, offset=4ms Proposal 13: Measurement gap cancelation is determined randomly by the test equipment. Proposal 14: For Type 2 gaps, the measurement gap cancelation ratio is configured independently for each configured gap pattern. Proposal 15: For Type 1 MGs test cases, RAN4 should agree on Option 1, i.e., a. Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset b. Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset Proposal 16: For Type 2 MGs test cases, RAN4 should agree on Option 1, i.e., configure cancelation ratio as: a. For MG pattern #1 with MGRP=40ms, offset=39ms, cancelation ratio of 0%. b. For MG pattern #2 with MGRP=80ms, offset=4 ms, cancelation ratio of 50% for UE supporting 5ms Toffset and 40% for UE supporting 3ms Toffset. Proposal 17: Define test cases for both 3ms offset and 5ms offset. Proposal 18: Since test case can cover both 3ms offset and 5ms offset with different configuration depending on UE capability. Proposal 19: Define RAN4 test cases using DCI 1-1 for the canceling indication. Proposal 20: RAN4 to agree that UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Further details are as follows: a) Test Option 2 in every test case. b) For testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability. |
| R4-2514306 | Ericsson | On RRM performance issues for XR Proposal 1: A subset of relevant test cases is specified by RAN4 for UE measurements under XR operation. Proposal 2: Test cases with and without DRX are specified. Proposal 3: Test cases for inter-frequency measurements with gaps are specified for Type 1 MG but not Type 2 MG. Proposal 4: RAN4 to discuss whether test cases with Type 2 MG can be deprioritized in Rel-19. Proposal 5: Test cases are specified only for per-UE MGs. Proposal 6: For measurement procedure, define test cases without SSB time index detection. Proposal 7: Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1. Proposal 8: The minimum set of measurement procedure TCs for XR is, e.g., as in the table below: TC title; FR (FR1, FR2-1); DRX or non-DRX; MG type. Inter-frequency with gap; A.6.6.2.x SA event triggered reporting without SSB time index detection in DRX under XR operation for FR1; All cells in FR1; DRX; Per-UE, Type 1. A.7.6.2.x SA event triggered reporting without SSB time index detection in DRX under XR operation for FR2-1; PCell in FR1, target cell in FR2-1; DRX; Per-UE, Type 1. Inter-RAT E-UTRAN FDD/TDD measurements; A.6.6.3.x SA NR - E-UTRAN event-triggered reporting in DRX under XR operation with FR1 serving cell; PCell in FR1, target cell in E-UTRAN FDD/TDD; DRX; Per-UE, Type 1. Proposal 9: The minimum set of measurement accuracy TCs for XR is, e.g., as in the table below: TC title; FR (FR1, FR2-1); DRX or non-DRX; MG type. Inter-frequency with gaps (SS-RSRP); A.6.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell; All cells in FR1; Non-DRX; Per-UE, Type 1. A.7.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell; PCell in FR1, target cell in FR2-1; Non-DRX; Per-UE, Type 1. Inter-frequency with gaps (SS-RSRQ); A.6.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell; All cells in FR1; Non-DRX; Per-UE, Type 1. A.7.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell; PCell in FR1, target cell in FR2-1; Non-DRX; Per-UE, Type 1. |
| R4-2514390 | Qualcomm Incorporated | RRM performance requirements for XR enhancements Proposal 1: RAN4 to consider shortlisting test cases for DCI-based Measurement Gap (MG) cancellation from the table below for FR1, aiming to cover representative scenarios across the impacted measurement types, while maintaining sufficient test coverage. Type; Title; Comments. Intra-frequency; SA event triggered reporting tests with per-UE gaps under non-DRX (e.g., A.6.6.1.3). SA event triggered reporting tests with per-UE gaps under DRX (e.g., A.6.6.1.4); Maybe. SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading (e.g., A.6.6.1.6); Yes. Inter-frequency; SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used (e.g., A.6.6.2.1); Maybe. SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used (e.g., A.6.6.2.2). SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used (e.g., A.6.6.2.5). SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used (e.g., A.6.6.2.6); Yes. Inter-frequency L1-RSRP; Inter-frequency SSB based L1-RSRP measurement with measurement gap (e.g., A.6.6.27.1); Yes. Proposal 2: RAN4 to adopt per-UE measurement gaps as the baseline configuration, with a single MG cancellation. Proposal 3: One additional test case for concurrent MG configurations may also be introduced. Proposal 4: RAN4 to use an incremental test-case design approach to specify new test cases, where only additional parameters and configurations from the baseline test case are specified along with the new test requirements. |

## Open issues summary

### Sub-topic 2-1 – Test case list

#### Issue 2-1-1: Scenarios for the definition of the test cases

- Proposals

  - Proposal 1 (CMCC): it is proposed to define test cases for NR-SA.

  - Proposal 2 (CMCC): it is proposed to define test cases for FR1 and FR2-1.

  - Proposal 3 (ZTE): Both FR1 NR SA and FR2-1 NR SA should be verified in the test.

  - Proposal 4 (MediaTek): RAN4 RRM to define test cases for NR-SA, and FR1 and FR2-1.

  - Proposal 5 (Nokia): RAN4 to define test cases for NR-SA.

  - Proposal 6 (Nokia): RAN4 to define test cases for FR1 and FR2-1.

  - Proposal 7 (Ericsson):

    - Measurement procedure test cases: FR1 (All cells in FR1) and FR2-1 (target cell in FR2-1);

    - Measurement accuracy test cases: Define at least one accuracy test case for XR in FR1 (all cells in FR1) and at least one accuracy test for XR in FR2-1 (target cell in FR2-1).

- Recommended WF:

  - All the contributions with proposals on that issue are well aligned on that topic. Discuss if the following can be agreed

    - RAN4 RRM to define test cases for NR-SA for both FR1 and FR2-1

  - Further discuss the target cells for FR1 and FR2-1:

    - FR1:

      - Option 1: all cells are in FR1

    - FR2-1:

      - Option 1: target cell in FR2-1

#### Issue 2-1-2: Per UE and per FR gaps

- Company Proposals

  - Proposal 1 (OPPO): Consider both per-FR gap and per-UE gap in the test case.

  - Proposal 2 (Huawei): RAN4 defines the test cases for both per-UE and per-FR MGs.

  - Proposal 3 (vivo): Define test cases to very core requirements for intra-frequency measurement with gaps, inter-frequency measurement with gaps, inter-frequency L1-RSRP measurement with gaps and inter-RAT E-UTRAN measurements with per-UE and per-FR gaps when gap cancellation is enabled.

  - Proposal 4 (vivo): Combination of gap-based measurement and per-UE/per-FR gap should be selected properly to avoid unnecessary tests.

  - Proposal 5 (MediaTek): RAN4 RRM to define test cases based on per-UE MGs configuration only.

  - Proposal 6 (Nokia, Ericsson): Test cases are specified only for per-UE MGs.

  - Proposal 7 (Qualcomm): RAN4 to adopt per-UE measurement gaps as the baseline configuration, with a single MG cancellation.

- Proposals merged by the moderator

  - Option 1: Test cases are specified only for per-UE MGs

  - Option 2: Test cases are specified for both per-UE and per-FR MGs

    - Option 2a: Combination of gap-based measurement and per-UE/per-FR gap

- Recommended WF

  - Discussion needed

#### Issue 2-1-3: Type 1 and Type 2 gaps

- Company Proposals

  - Proposal 1 (OPPO): Consider Rel-17 concurrent gaps in the test case.

  - Proposal 2 (Huawei, Nokia): RAN4 define the test cases for both Type 1 and Type 2.

  - Proposal 3 (CMCC): it is proposed to define test cases for Type 1 measurement gap and Type 2 measurement gap ( e.g. the colliding case).

  - Proposal 4 (ZTE): The skipping in both Type-1 MG and Type-2 MG should be verified in different tests.

  - Proposal 5 (MediaTek): RAN4 RRM to define test cases for Type-1 gap only.

  - Proposal 6 (Ericsson):

    - Test cases for inter-frequency measurements with gaps are specified for Type 1 MG but not Type 2 MG.

    - RAN4 to discuss whether test cases with Type 2 MG can be deprioritized in Rel-19.

  - Proposal 7 (Qualcomm): One additional test case for concurrent MG configurations may also be introduced.

- Merged proposals by the moderator

  - Option 1: Test cases are defined for both Type 1 and Type 2 gaps

    - Option 1a: One additional test case for Type 2 gaps is introduced

  - Option 2: Test cases are defined only for Type 1 gaps

- Recommended WF:

  - Discussion needed.

#### Issue 2-1-4: DRX proposals

- Company proposals:

  - Proposal 1 (OPPO): Define the test cases for the cases without DRX and with DRX.

  - Proposal 2 (Huawei): The test cases for XR should be defined with the non‑DRX cases as a high priority.

  - Proposal 3 (CMCC): it is proposed to consider both the case without DRX and the case with DRX.

  - Proposal 4 (MediaTek): RAN4 to define TCs that each type of test case is specified for either DRX or non-DRX but not both.

  - Proposal 5 (Nokia): RAN4 to define the test cases for both Type 1 and Type 2 gaps but should focus on only non-DRX test cases for Type 2 MGs. Test cases without DRX configuration should have higher priority than with DRX configuration.

    - Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG

    - Develop TCs for inter-RAT with DRX and with Type 1 MG

    - Focus on non-DRX test cases for Type 2 MG

  - Proposal 6 (Ericsson): Test cases with and without DRX are specified.

  - Proposal 7 (Ericsson): DRX for measurement procedure TCs and no DRX for accuracy TCs.

- Merged proposals by the moderator

  - Option 1: Define test cases with and without DRX.

  - Option 2: The test cases for XR should be defined with the non‑DRX cases as a high priority.

    - Option 2a: DRX TCs cover

      - Inter-frequency measurements with Type 1 MG

      - Inter-RAT measurements with Type 1 MG

  - Option 3: Each type of test case is specified for either DRX or non-DRX but not both.

  - Option 4: DRX for measurement procedure TCs and no DRX for accuracy TCs.

- Recommended WF:

  - Discuss the proposals having in mind the test coverage and workload.

#### Issue 2-1-5: Test target

- Company proposals

  - Proposal 1 (OPPO): UE Tx/Rx behavior in the skipping MG occasions and the extended delay requirements should be verified in the same test case.

  - Proposal 2 (Huawei): The processing time of 3ms/5ms shall be verified in the test cases.

  - Proposal 3 (ZTE): The UE processing timeline of receiving the DCI based MG skipping indication should be verified independently or together with other test motivation.

  - Proposal 4 (MediaTek): RAN4 to not define TCs for accuracy requirements due to MG cancellation nor TCs for UE processing timeline of receiving the DCI-based MG skipping indication.

  - Proposal 5 (MediaTek): RAN4 to define test cases for (i) delay extension with gap cancellation and (ii) reception/transmission without scheduling restrictions during cancelled gap occasions.

  - Proposal 6 (Nokia): RRM test cases should verify

    - Accuracy requirements

    - Delay extension with gap cancelation

    - Reception/transmission without scheduling restrictions during cancelled gap occasions

    - The UE processing timeline of receiving the DCI-based MG skipping indication

  - Proposal 7 (Nokia): The accuracy requirements are tested in limited number of accuracy specific test, including

    - Inter-frequency measurements with Type 1 gaps, without DRX in FR1

    - Inter-frequency measurements with Type 1 gaps, without DRX in FR2-1

    - Inter-RAT E-UTRAN TDD with Type 1 gaps, without DRX

  - Proposal 8 (Ericsson):

    - Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1,

    - measurement procedure TCs for at least inter-frequency and inter-RAT,

    - limited number of TCs.

- Proposals merged by the moderator:

  - RRM test cases should verify

    - Proposal 1: delay extension with gap cancellation

    - Proposal 2: Reception/transmission without scheduling restrictions during cancelled gap occasions

    - Proposal 3: The UE processing timeline of receiving the DCI-based MG skipping indication

    - Proposal 4: Proposals 1, 2, and 3 are verified in the same test case

    - Proposal 5: Accuracy requirements

    - Proposal 6: Accuracy requirements are verified in a limited number of cases

    - Proposal 7: inter-frequency requirements are verified for measurement delay

    - Proposal 8: inter-RAT requirements are verified for measurement delay

    - Proposal 9: at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1

- Recommended WF:

  - Discuss which proposals are agreeable. Discuss whether different test targets are captured in individual TCs or not, like in Proposal 3.

#### Issue 2-1-6: List of test cases

- Company proposals

  - Proposal 1 (OPPO): Define test cases for:

    - Intra-frequency measurements with gaps

    - Inter-frequency measurements with gaps

    - Inter-RAT E-UTRAN FDD measurements

    - Inter-RAT E-UTRAN TDD measurements

    - Inter-frequency L1-RSRP with MG

  - Proposal 2 (CMCC): it is proposed to define test cases for intra-frequency measurement, inter-frequency measurement, and LTE inter-RAT measurement.

  - Proposal 3 (ZTE): The MG skipping has impact on L3 & L1 measurements with gap. The impact by measurement skipping on the following types of L3&L1 measurement with gap should be verified:

    - Intra-frequency measurement with gap

    - Inter-frequency measurement with gap

    - EUTRAN measurement with gap

    - L1 inter-frequency measurement with gap

  - Proposal 4 (ZTE): The MG skipping also impacts the L3 & L1 measurements without gap when these measurement occasions partially or fully overlapping with MG occasions. Such impacts should be verified as well.

  - Proposal 5(vivo): Define test cases to very core requirements for intra-frequency measurement with gaps, inter-frequency measurement with gaps, inter-frequency L1-RSRP measurement with gaps and inter-RAT E-UTRAN measurements with per-UE and per-FR gaps when gap cancellation is enabled.

  - Proposal 6 (MediaTek): RAN4 to define TCs with working assumption that the gap is configured for inter-frequency measurement with gaps.

  - Proposal 7 (Nokia): RAN4 should define test cases for

    - Intra-frequency measurements with gaps,

    - Inter-frequency measurements with gaps,

    - Inter-RAT E-UTRAN TDD measurements,

    - Inter-frequency L1-RSRP with MG.

  - Proposal 8 (Ericsson): A subset of relevant test cases is specified by RAN4 for UE measurements under XR operation.

  - Proposal 9 (Ericsson): Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1.

  - Proposal 10 (Qualcomm): RAN4 to use an incremental test-case design approach to specify new test cases, where only additional parameters and configurations from the baseline test case are specified along with the new test requirements.

  - Proposal 11 (Ericsson): at least the following test cases are defined:

    - Inter-frequency event-triggered reporting with gaps [without SSB time index detection]

    - Inter-RAT E-UTRAN FDD/TDD event-triggered reporting with gaps

    - Inter-frequency accuracy for SS-RSRP and SS-RSRQ

- Proposals merged by the moderator

  - Define test cases for

    - Proposal 1: Intra-frequency measurements procedure with gaps

    - Proposal 2: Inter-frequency measurements procedure with gaps

    - Proposal 3: Inter-RAT E-UTRAN FDD measurements procedure with gaps

    - Proposal 4: Inter-RAT E-UTRAN TDD measurements procedure with gaps

    - Proposal 5: Inter-frequency L1-RSRP with MG

    - Proposal 6: L3 and L1 measurements without gaps

    - Proposal 7: Define at least one accuracy test case in FR1 and in FR2-1.

    - Proposal 8: Inter-frequency accuracy for SS-RSRP and SS-RSRQ

  - Discuss whether to use incremental test-case design approach for specifying new test cases with new test requirements compared to baseline test case.

- Recommended WF

  - Discuss the merged proposals, and whether reduced number of tests are defined for each of the proposals. Per UE/per FR and Type 1/Type 2 is to be discussed after agreement on the proposals above.

#### Issue 2-1-7: Complete list of test cases for discussion

- Proposals

  - Proposal 1:

    - Test case list for down selection for measurement procedure

| Gap type | Intra/Inter/Inter-RAT | Frequency Range | DRX |
| --- | --- | --- | --- |
| Type 1 gap per UE | Intra-frequency | FR1 | OFF |
|  |  |  | ON |
|  |  | FR2-1 | OFF |
|  |  |  | ON |
|  | Inter-frequency | FR1 | ON |
|  |  |  | OFF |
|  |  | FR2-1 | ON |
|  |  |  | OFF |
|  | Inter-RAT E-UTRAN TDD | FR1 (serving) | ON |
|  |  |  | OFF |
|  | Inter-RAT E-UTRAN FDD | FR1 (serving) | ON |
|  |  |  | OFF |
| Type 2 gaps per UE | Intra-frequency | FR1 | OFF |
|  |  | FR2-1 | OFF |
|  | Inter-frequency | FR1 | OFF |
|  |  | FR2-1 | OFF |
| L1 RSRP measurements with gaps | Inter-frequency | FR1 | OFF |
|  |  | FR2-1 | OFF |

    - Test case list for down selection for accuracy test cases

| Gap type | Intra/Inter/Inter-RAT | Frequency Range | DRX | Measurement type |
| --- | --- | --- | --- | --- |
| Type 1 gap per UE | Inter-frequency | FR1 | OFF | SS-RSRP |
|  |  | FR2-1 | OFF | SS-RSRP |
|  | Inter-RAT E-UTRAN TDD | FR1 (serving) | OFF | SS-RSRP |
|  | Inter-frequency | FR1 | OFF | SS-RSRQ |
|  |  | FR2-1 | OFF | SS-RSRQ |

  - Proposal 2:

    - The minimum set of measurement procedure TCs for XR is, e.g., as in the table below:

|  | TC title | FR (FR1, FR2-1) | DRX or non-DRX | MG type |
| --- | --- | --- | --- | --- |
| Inter-frequency with gap | A.6.6.2.x SA event triggered reporting without SSB time index detection in DRX under XR operation for FR1 | All cells in FR1 | DRX | Per-UE, Type 1 |
|  | A.7.6.2.x SA event triggered reporting without SSB time index detection in DRX under XR operation for FR2-1 | PCell in FR1, target cell in FR2-1 | DRX | Per-UE, Type 1 |
| Inter-RAT E-UTRAN FDD/TDD measurements | A.6.6.3.x SA NR - E-UTRAN event-triggered reporting in DRX under XR operation with FR1 serving cell | PCell in FR1, target cell in E-UTRAN FDD/TDD | DRX | Per-UE, Type 1 |

    - The minimum set of measurement accuracy TCs for XR is, e.g., as in the table below:

|  | TC title | FR (FR1, FR2-1) | DRX or non-DRX | MG type |
| --- | --- | --- | --- | --- |
| Inter-frequency with gaps (SS-RSRP) | A.6.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | All cells in FR1 | Non-DRX | Per-UE, Type 1 |
|  | A.7.7.1.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell | PCell in FR1, target cell in FR2-1 | Non-DRX | Per-UE, Type 1 |
| Inter-frequency with gaps (SS-RSRQ) | A.6.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | All cells in FR1 | Non-DRX | Per-UE, Type 1 |
|  | A.7.7.2.x SA inter-frequency measurement accuracy with FR1 serving cell and FR2-1 target cell | PCell in FR1, target cell in FR2-1 | Non-DRX | Per-UE, Type 1 |

- Recommended WF:

  - Discuss the proposals. Use the outcome of the previous issues to delete or add items to the table also depending on the outcome of other issues.

### Sub-topic 2-2 – Test case configuration

#### Issue 2-2-1: Type 2 test configurations

- Proposals

  - Proposal 1 (MediaTek, Ericsson): RAN4 RRM to define test cases for Type-1 gap only and there is no need to consider Type-2 gap in defining test cases.

  - Proposal 2 (Nokia): For Type 2 MGs test cases, include 2 gaps with partial overlap

  - Proposal 3 (Nokia): For Type 2 MGs test cases, measurement gap configuration of A.6.6.18.2.1 can be reused, i.e.,

    - Low priority MG pattern #1 with MGRP=40ms, offset=39ms

    - High priority MG pattern #2 with MGRP=80ms, offset=4ms

- Recommended WF:

  - Discuss the proposals considering the outcome of issue 2-1-3.

#### Issue 2-2-2: SSB time index detection

- Proposals

  - Proposal 1 (Ericsson): For measurement procedure, define test cases without SSB time index detection.

  - Proposal 2 (Qualcomm): Use SSB index detection for intra-/inter-frequency measurement TCs, use no SSB index detection with lower priority for intra-/inter-frequency measurement TCs.

- Recommended WF:

  - Discuss the proposals.

### Sub-topic 2-3 – MG cancellation model

#### Issue 2-3-1: Cancelation model for XR test cases: conditions for Type 1 gaps and Type 2 gaps

- Proposals

  - Proposal 1 (Huawei): RAN4 to design a fixed cancellation gap pattern for test cases and further discuss the pattern.

  - Proposal 2 (MediaTek): RAN4 RRM should use a repetitive sequence where the test machine configures the UE with DCI containing MG cancellation according to this sequence.

  - Proposal 3 (Nokia): Measurement gap cancelation is determined randomly by the test equipment.

- Proposals merged by the moderator

  - Option 1: Measurement gap cancelation is modelled as a fixed pattern for the test cases.

  - Option 2: Measurement gap cancelation is determined randomly by the test equipment.

- Recommended WF

  - Discuss the proposals.

#### Issue 2-3-2: Cancelation model for XR test cases: additional condition for Type 2 gaps

- Proposals

  - Proposal 1 (Nokia): For Type 2 gaps, the measurement gap cancelation ratio is configured independently for each configured gap pattern.

- Recommended WF

  - Discuss the proposal.

#### Issue 2-3-3: Cancelation ratio for Type 1 gaps

- Proposals

  - Proposal 1(vivo): Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests.

  - Proposal 2 (Nokia): For Type 1 MGs test cases:

    - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset

    - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset

- Recommended WF:

  - Discuss the proposals.

#### Issue 2-3-4: Cancelation ratio for Type 2 gaps

- Proposals

  - Proposal 1 (Nokia): For Type 2 MGs test cases, configure cancelation ratio as:

    - For MG pattern #1 with MGRP=40ms, offset=39ms, cancelation ratio of 0%

    - For MG pattern #2 with MGRP=80ms, offset=4ms, cancelation ratio of 50% for UE supporting 5ms Toffset and 40% for UE supporting 3ms Toffset

- Recommended WF:

  - Discuss the proposals. To be considered the outcome of Issue 2-1-3 and 2-3-3.

### Sub-topic 2-4 – DCI aspects

#### Issue 2-4-1: Toffset between DCI and start of MG

- Proposals

  - Proposal 1 (Huawei): The processing time 3ms/5ms shall be verified in the test cases.

  - Proposal 2 (CMCC): it is proposed to define test cases for both 3ms offset and 5ms offset. And applicability rules can be considered, e.g. the UE supporting 3ms offset only need to pass the test with 3ms timeline, the UE not supporting 3ms offset need to pass the test with 5ms timeline.

  - Proposal 3 (MediaTek): RAN4 RRM define a single test case that includes time offset of 5ms and 3ms with UE capability used as a condition.

  - Proposal 4 (Nokia):

    - Define test cases for both 3ms offset and 5ms offset.

    - Since test case can cover both 3ms offset and 5ms offset with different configuration depending on UE capability.

  - Proposal 5 (vivo): Time offset for gap cancellation indication is 5ms in the tests.

- Proposals merged by the moderator

  - Option 1: The processing time 3ms/5ms shall be verified in the test cases.

    - Option 1a: Define a single test case that includes time offset of 5ms and 3ms with UE capability used as a condition

  - Option 2: Define separate test cases for 3ms offset and 5ms offset.

  - Option 3: Define test case for 5ms offset only.

- Recommended WF

  - Discuss the proposals having in mind the test coverage and workload.

#### Issue 2-4-2: Types of DCI with cancellation indication

- Proposals

  - Option 1 (vivo): In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancellation.

  - Option 2 (Nokia): Define RAN4 test cases using DCI 1-1 for the canceling indication.

- Recommended WF

  - Discuss the proposals. Discuss whether different types of DCI are to be included in the tests, and how to verify them.

#### Issue 2-4-3: Behaviour of Option 1 and Option 2 for indication of value ‘0’

- Proposals

  - Proposal 1 (vivo): RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested.

  - Proposal 2 (Nokia): RAN4 to agree that UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Further details are as follows:

    - Test Option 2 in every test case.

    - For testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability.

- Recommended WF

  - Discuss the proposals having in mind the test coverage and workload.