3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514028

Prague, CZ, 13th – 17th October 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on SAN conformance testing for NR NTN Ku band

# Agenda item:	5.10.4

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some initial views about NR NTN Ku band from SAN conformance testing perspective.

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

# 2 Discussion

We make the following proposals for SAN conformance testing for NTN Ku band.

Table 2.2 General requirements for NTN Ku band

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| - 4 General test conditions and declarations |  |
| 4.1 Measurement uncertainties and test requirements | Need to update the test system uncertainty for carrier frequency f of Ku band in Table 4.1.2.2-2, 4.1.2.2-3, 4.1.2.3-2, 4.1.2.3-3. |
| 4.2 Requirement reference points | No specification impact. |
| 4.3 Satellite Access Node classes | No specification impact. |
| 4.4 Regional requirements | No specification impact. |
| 4.5 SAN configurations | No specification impact. |
| 4.6 Manufacturer declarations | No specification impact. |
| 4.7 Test configurations | Need to define the signal to be used to build NR TCs for SAN type 1-O for Ku band. |
| 4.8 Applicability of requirements | No specification impact. |
| 4.9 RF channels and test models | No specification impact. |
| 4.10 Void |  |
| 4.11 Reference coordinate system | No specification impact. |
| 4.12 Format and interpretation of tests | No specification impact. |

Table 2.2 Tx requirements for NTN Ku band

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| - 9 Radiated transmitter characteristics |  |
| 9.1 General | No specification impact. |
| 9.2 Radiated transmit power | Need to update test requirement for Ku band in Table 9.2.5-1. |
| 9.3 OTA SAN output power | The TRP accuracy in section 9.3.5 needs to be updated for Ku band. |
| 9.4 OTA output dynamics |  |
| 9.4.1 General | No specification impact. |
| 9.4.2 OTA RE power control dynamic range | No specification impact. |
| 9.4.3 OTA total power dynamic range | OTA total power dynamic range requirement in 9.4.3.5 needs to be updated for Ku bands. |
| 9.4.4 OTA NB-IoT RB power dynamic range for in-band operation | No specification impact. |
| 9.5 OTA transmit ON/OFF power | No specification impact. |
| 9.6 OTA transmitted signal quality |  |
| 9.6.1 General | No specification impact. |
| 9.6.2 OTA frequency error | No specification impact. |
| 9.6.3 OTA modulation quality | EVM window length in section 9.6.3.5.1 for FR1-NTN needs to be updated. |
| 9.7 OTA unwanted emissions |  |
| 9.7.1 General | No specification impact. |
| 9.7.2 OTA occupied bandwidth | Span and number of measurement points for OBW measurements in Table 9.7.2.4.2-1 needs to be updated for FR1-NTN Ku bands. |
| 9.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | SAN channel bandwidths in Table 9.7.3.5.1-1 and 9.7.3.5.1-2 need to be updated. |
| 9.7.4 OTA out-of-band emissions | No specification impact. |
| 9.7.5 OTA transmitter spurious emissions | OTA transmitter spurious emission limits for Ku bands needs to be defined. |
| 9.8 OTA transmitter intermodulation | No specification impact. |

Table 2.3 Rx requirements for NTN Ku band

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| - 10 Radiated receiver characteristics |  |
| 10.1 General | OTA REFSENS RoAoA absolute requirement values for FR1-NTN operating above 10GHz need to be updated. |
| 10.2 OTA sensitivity | EIS levels needs to be updated for Ku bands. |
| 10.3 OTA reference sensitivity level | OTA reference sensitivity level needs to be updated for Ku bands. |
| 10.4 OTA dynamic range | Not applicable. |
| 10.5 OTA in-band selectivity and blocking |  |
| 10.5.1 OTA adjacent channel selectivity | OTA ACS requirement for Ku bands needs to be updated. |
| 10.5.2 OTA in-band blocking | Not applicable. |
| 10.6 OTA out-of-band blocking | OTA out-of-band blocking for SAN type 1-O operating above 10GHz needs to be updated. |
| 10.7 OTA receiver spurious emissions | Not applicable. |
| 10.8 OTA receiver intermodulation | Not applicable. |
| 10.9 OTA in-channel selectivity | OTA ICS for SAN type 1-O operating above 10GHz needs to be updated. |

Proposal 1: We make the proposals as Table 2.1, 2.2 and 2.3 for SAN conformance testing for NTN Ku band.

# 3 Conclusion

In this contribution, we shared some views on SAN conformance testing for NTN Ku band and the proposal is made as follows:

Proposal 1: We make the proposals as Table 2.1, 2.2 and 2.3 for SAN conformance testing for NTN Ku band.

# 4 Reference

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat et al.