3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514036

Prague, CZ, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-2 | CR | - | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TEI19-BDaT_simp_improvement) CR to TS 38.176-2: spec structure simplification for co-locationco-existence requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the RAN task, the spec structure simplification for transmitter spurious emission in subclause 6.7.5.4.5.1 and 6.7.5.5.5.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The table is simplified to include the general requirement level |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the specification cannot be improved as requested by RAN task. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.5.4.5.1 and 6.7.5.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.7.5.4.5	Test requirement

6.7.5.4.5.1	Test requirement for IAB type 1-O

The power of any spurious emission shall not exceed the test limits in table 6.7.5.4.5-1 for a IAB where requirements for co-existence with the system listed in the first column apply. For a multi-band RIB, the exclusions and conditions in the Note column of table 6.7.5.4.5-1 apply for each supported operating band.

Table 6.7.5.4.5.1-1: IAB-DU and IAB-MT spurious emissions basic limits for co-existence with systems operating in other frequency bands

[

| System type to co-exist with | Frequency range for co-existence requirement | IAB-DU type 1-O Test limits | IAB-MT type 1-O Test limits | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 - 894 MHz | -45.4 dBm | -45.4 +Y dBm | 100kHz | NOTE 1 |
|  | 824 - 849 MHz | -49.4 dBm | -49.4 +Y dBm |  |  |
| GSM900 | 921 - 960 MHz | -45.4 dBm | -45.4 +Y dBm |  |  |
|  | 876 - 915 MHz | -49.4 dBm | -49.4 +Y dBm |  |  |
| DCS1800 | 1805 - 1880 MHz | -35.4 dBm | -35.4 +Y dBm |  |  |
|  | 1710 - 1785 MHz | -49.4 dBm | -49.4 +Y dBm |  |  |
| PCS1900 | 1930 - 1990 MHz | -35.4 dBm | -35.4 +Y dBm |  |  |
|  | 1850 - 1910 MHz | -49.4 dBm | -49.4 +Y dBm |  |  |
| Other system type | Below 3GHz downlink operating band of the BS to co-exist with (NOTE 7, 8) | -40.4 dBm | -40.4 +Y dBm | 1MHz | NOTE 1 |
|  | Below 3GHz uplink operating band of the BS to co-exist with (NOTE 8) | -37.4 dBm | -37.4 +Y dBm |  |  |
|  | 3.3 - 4.2 GHz TDD or FDD downlink operating band of the BS to co-exist with (NOTE 8) | -40 dBm | -40 +Y dBm |  |  |
|  | 3.3 - 4.2 GHz FDD uplink operating band of the BS to co-exist with (NOTE 8) | -37 dBm | -37 +Y dBm |  |  |
|  | Above 4.4 GHz TDD operating band of the BS to co-exist with (NOTE 8) | -39.5 dBm | -39.5 +Y dBm |  |  |

]

NOTE 1:	As defined in the scope for spurious emissions in this clause the co-existence requirements in table 6.7.5.4.5.1-1do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.7.5.4.5.1-1 assumes that two operating bands, where the frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:   Y = - 9 + 10log10(NTXU,OTApercell) dB

NOTE 4:	Does not apply for co-existence with standalone downlink bands (SDO) defined in TS 36.104, table 5.5-1.

NOTE 5:	Frequency range of UTRA, E-UTRA and NR bands, as described in TS 25.104, TS 36.104 and TS 38.104.

6.7.5.5.5.1	Test requirement for IAB type 1-O

These requirements may be applied for the protection of other IAB receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with a IAB Node.

The requirements assume co-location with base stations of the same class.

NOTE:	For co-location with UTRA, the requirements are based on co-location with UTRA FDD or TDD base stations.

This requirement is a co-location requirement as defined in clause 4.9, in TS 38.174 [2], the power levels are specified at the CLTA output.

The output of the CLTA of any spurious emission shall not exceed the test limit in table 6.7.5.5.5.1-1.

For a multi-band RIB, the exclusions and conditions in the notes column of table 6.7.5.5.5.1-1 apply for each supported operating band.

Table 6.7.5.5.5.1-1: IAB-DU and IAB-MT spurious emissions basic limits for co-location with BS or IAB-Node

[

| Frequency range for | System type to | Test limits (dBm/100kHz) |  |  |  |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| co-location requirement | co-locate with | WA IAB-DU | WA IAB-MT | MR IAB-DU | LA IAB-DU | LA IAB-MT |  |
| 824 - 849 MHz | GSM850 or CDMA850 | -115.9 +Y dBm | -115.9 dBm | -108.9 dBm | -87.9 +Y dBm | -87.9 +Y dBm | Note 1 |
| 876 - 915 MHz | GSM900 | -115.9 dBm | -115.9 +Y dBm | -108.9 dBm | -87.9 dBm | -87.9 +Y dBm |  |
| 1710 - 1785 MHz | DCS1800 | -115.9 dBm | -115.9 +Y dBm | -108.9 dBm | -97.9 dBm | -97.9 +Y dBm |  |
| 1850 - 1910 MHz | PCS1900 | -115.9 dBm | -115.9 +Y dBm | -108.9 dBm | -97.9 dBm | -97.9 +Y dBm |  |
| UL frequency range of n51 | E-UTRA or NR | N/A | N/A | N/A | -105.9 dBm | -105.9 +Y dBm |  |
| UL frequency range of n46, n53 | E-UTRA or NR | N/A | N/A | -108.9 dBm | -105.9 dBm | -105.9 +Y dBm |  |
| UL frequency range of n96, n102 | NR | N/A | N/A | -107.6 dBm | -104.6 dBm | -104.6 +Y dBm |  |
| UL frequency range of n104 | NR | -112.6 dBm | -112.6 +Y dBm | -107.6 dBm | -104.6 dBm | -104.6 +Y dBm |  |
| UL frequency range of band n79 | NR | -113.6 dBm | -113.6 dBm | -108.6 dBm | -105.6 dBm | -105.6 +Y dBm |  |
| UL frequency range of band n48, n77, n78 | E-UTRA or NR | -113.7 dBm | -113.7 dBm | -108.7 dBm | -105.7 dBm | -105.7 +Y dBm |  |
| Frequency range of uplink operating band of the co-located BS | E-UTRA Band 42, 43 | -113.7 dBm | -113.7 dBm | -108.7 dBm | -105.7 dBm | -105.7 +Y dBm |  |
| Frequency range of uplink operating band of the co-located BS | Other (NOTE 4, 5) | -113.9 dBm | -113.9 +Y dBm | -108.9 dBm | -105.9 dBm | -105.9 +Y dBm |  |

]

NOTE 1:	As defined in the scope for spurious emissions in this clause, the co-location requirements in table 6.6.5.2.3-1 do not apply for the frequency range extending ΔfOBUE immediately outside the transmit frequency range of a IAB-MT and IAB-DU. The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB antenna to antenna minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [15].

NOTE 2:	Table 6.6.5.2.3-1 assumes that two operating bands, where the corresponding transmit and receive frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:   Y = - 9 + 10log10(NTXU,OTApercell) dB.

NOTE 4:	Does not apply for co-location with V2X operation defined in TS 36.104, table 5.5-1.

NOTE 5:	UTRA, E-UTRA, NR as described in TS 25.104, TS 36.104 and TS 38.104.