3GPP TSG-RAN WG4 Meeting #116bis		    								               R4-2514443

Prague, Czech Republic, October 13-17, 2025

Agenda item:			7.8.1

Source:	Moderator (Apple)

Title:	Topic summary for [116bis][320] NR_low_band_CA_switching

Document for:	Information

# Introduction

This document summarizes the contributions submitted for the general and RF related considerations on the enhancement of low NR band carrier aggregation via switching work item (AI 7.8.2 and 7.8.3).

## Summary of company contributions

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513536 | Huawei, HiSilicon | General views on NR_LBCA_Sw_enh Observation 1: The WI was approved under the situation that RAN4 TU is limited and no RAN1/2 TU is available. Observation 2: Any unnecessary enhancements such as shorter switching period, etc., other than the objective in Rel-20 LB-CA produce unnecessary workload. Observation 4: Following observations are obtained on co-existence issues per scenario. Observation 5: Some issues captured in Observation 4 are not R20 specific, but rather also apply to R19. Proposal 1: RAN4 should focus on competing only the objective in the WID and avoid discussing unnecessary enhancements such as shorter switching period, etc. unless the necessity is agreed. Proposal 2: RAN4 shouldnt consider the introduction of higher order CA with the LB CA switching as one of fallback configurations and/or different device types of LB CA switching (e.g., con-current DL CA, or FDD+FDD CA switching with concurrent DL CA) as RAN4 did in Rel-19. It is noted that for CA_n28-n67, only the difference from CA_n12-n29 is captured. Proposal 3: For UE to UE co-existence (FDD UL Transmission to SDL reception), RAN4 should discuss whether or not special care is needed or not. Proposal 4: For BS to BS co-existence, RAN4 should discuss how practically this issue is resolved and if there is any restriction, e.g., guard band. |
| R4-2513769 | Nokia | Considerations for NR_LBCA_Sw_enh Observation 1: 2 DL LO solution may operate FDD DL continuously without interruptions Observation 2: 1 DL LO solution leads to switching gaps for both the FDD DL carrier and the aggregated carrier Observation 3: Regarding CA_n28A-n67A, there should be requirements set for the PCC + SCC aggregated channel bandwidth combinations. Observation 4: Assuming that there are no interferers between carriers doesnt necessarily lead to a generic feature that is future proof if only 1 DL solution is considered Observation 5: With a 1 LO solution including a wide RFFE filter to receive both carriers, even the contiguous FDD DL may be sensitive to inter-UE interference from the FDD UL. Observation 6: 2 DL LO solution can withstand interferers and is thus a future proof solution Proposal 1: For optimum network operation it would be highly beneficial to focus on scenarios and UE implementations that supports contiguous FDD DL operation without switching interruptions. Proposal 2: Requirements for the aggregated DL operation should be set in such a way that 2 DL LO and continuous FDD DL operation are not excluded. Proposal 3: For optimum network operation it would be highly beneficial to focus on scenarios where only the SDL carrier is sensitive to cross-UE interference from the FDD UL carrier, not the contiguous FDD DL carrier. |
| R4-2514294 | TELUS | Work plan for enhancement of low NR band carrier aggregation via switching |
| R4-2514405 | Apple | Draft CR for 38.768 on enhanced LB CA via switching scenarios Reason for change: The new work item on the enhancement of low NR band carrier aggregation via switching was approved in RAN #109 in RP-252953. The work item will continue to capture technical background in TR38.768. This draft CR provides the work item description for the enhancement and introduces the relevant operator-provided scenarios. Summary of change: Change 1: capture the work item description on the enhancement of low NR band carrier aggregation via switching (Clause 1) Change 2: capture the operator-provided scenarios (Clause 4) |
| R4-2513176 | CATT | Discussion on R20 low bands CA via switching Observation 1: the switching gap in SDL band is needed when the status is switching from Dual-DL FDD+SDL bands to UL/DL FDD band. Observation 2: the switching gap in UL band is needed when the status is switching from Dual-DL FDD+SDL bands to UL/DL FDD band. Observation 3: current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 cant be reused for R20 Low-Low bands inter-band CA via switching, since current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for R20 Low-Low bands inter-band CA via switching. Observation 4: for Low-Low bands inter-band CA via switching, its possible to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together. Proposal 1: The reference RF architecture for CA_n12-n29 shown in the figure 1 can be considered for the discussion. Proposal 2: RAN4 need to discuss whether UE can maintain the DL reception in DL FDD band during the switching gap from Dual-DL FDD+SDL bands to UL/DL FDD band or from UL/DL FDD band to Dual-DL FDD+SDL bands. Proposal 3: to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together. |
| R4-2513448 | Xiaomi | UE RF requirements on LB CA with switching Observation 1: From UE RF architecture perspective, similar architecture as CA via switching between {case 1 and case 2} with additional RF front-end filter/duplex is required for Dual DL case. Observation 2: Existing switching gap can be extended to cover Rel-20 LB CA via switching case Proposal 1: Introduce new separate UE capability to support LB CA via switching between {case 1, case 3} with Dual DL with multiple values for switching period to accommodate different UE implementation. Proposal 2: Tx on/off time mask requirements which is introduced in Rel-19 for LB CA switching can be applied to LB CA via switching with dual DL. |
| R4-2513537 | Huawei, HiSilicon | Views on UE RF requirements for NR_LBCA_Sw_enh Observation 1: There is no factors to shorten switching period in R20 compared to that for R19 in terms of UE RF point of view. Required Rx filter design for R20 CA_n12-n29/CA_n28-n67 is different from that for R20, but it doesnt give any reasons to shorten the switching period. Observation 2: R20 WID clearly says that reuse R19 requirements as much as possible. Observation 3: How UE to UE or BS to BS co-existence issue (SDL BS transmission to FDD BS reception), is addressed shall be clarified before going into deep detail discussion on Rx requirements as discussed in our companion paper of [R4-2513536]. Observation 4: For CA_n28-n67, if an operator holds only lower portion of n28, 5 MHz guard band is available so that it may resolve the issue, though still feasibility study is needed, e.g., whether or not 5 MHz is enough for BS filter to have sufficient attenuation, if it is feasible to keep 50 MHz(738-788 MHz) pass-band as UE Rx filter in 700 MHz frequency range, etc. Observation 5: For CA_n28-n67, if full-range of spectrum is used, UE Rx filter to cover both n67 + n28 DL(738-803MHz) wouldnt be feasible. Observation 6: For CA_n12-n29, RAN4 may be able to keep the current REFSENS or may need only limited amount of relaxation depending on further study. Observation 7: For CA_n28-n67, the discussion is affected by the conclusion on if there is any restriction in terms of spectrum usage for LB CA operation with dual DL, e.g., fullband n28 is expected to be used or only lower portion of n28 is used. Proposal 1: No new time mask for R20 LB-LB CA SW is needed on top of that for R19 LB CA SW. Proposal 2: Switching periods for R20 shall be the same as those for R19, i.e., 35s, 70s, and 140s for switching between Case 1 and Case 3 are defined as an optional UE capability. Proposal 3: RAN4 should first clarify how UE to UE and BS to BS co-existence issues are resolved and if there is any restriction such as only lower portion of n28 is used, or only some portion of n29 is used etc., before going into the deep detail. Proposal 4: Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of Proposal 3. Proposal 5: Locations of blockers for blocking requirements shall be shift depending on assumed actual Rx filter frequency range. |
| R4-2513563 | vivo | Discussion on UE RF requirements for Rel-20 LBCA Observation 1: The switching of case 1 - case 2 and case 1 - case 3 is quite similar for the Tx part. Observation 2: DL interruption need to be considered for case 3, which belongs to RRM scope. Proposal 1: Rel-19 switching delay requirements {35us, 70us, 140us} can be used as a starting point for Rel-20. Proposal 2: Reusing Rel-19 ON/OFF mask for case 1 - case 2 as starting point for Rel-20 case 1 - case 3, and confirmation may need RAN1 involvement. Proposal 3: Study interruption requirements in RRM scope. Proposal 4: Re-check the requirements related to CA applicability for LB CA. |
| R4-2513641 | Spreadtrum, UNISOC | Discussion on UE RF requirements for R20 LB-LB CA switching Proposal 1: Figure1 can be UE architecture assumption in R20 LB-LB CA switching. Reuse the switching periods (35us, 70us and 140us) for switching between Case1 and Case3 as an optional UE capability in TR 38.768. Proposal 2: the switching period location can refer to the figure2 for FDD to DL CA switch. Proposal 3: the switching period location can refer to the figure3 for DL CA to FDD switch. Proposal 4: UL time mask R19 LB-LB CA switching can be reuses in R20 LB-LB CA switching. Proposal 5: Fall back behavior for R20 DL CA via switching needs to be further discussed. Proposal 6: REFSENS and RIB,c requirements are all need to be discussed. |
| R4-2513735 | Samsung | Views on UE RF requirements for LB CA switching Observation 1: The scope of the Rel-20 WI expands beyond simple FDD/SDL switching into the dual downlink scenario, where both an FDD DL carrier and an SDL carrier may be received simultaneously under the switching framework. Observation 2: The uplink behavior under Rel-20 dual DL switching remains fundamentally identical to Rel-19. Observation 3: The WID initially describes its scope as applying to adjacent FDD DL and SDL band combinations under the condition of no-gap impact. Observation 4: The main purpose of this WI is to enable UEs with only a single low-band Rx chain to realize some of the benefits of DL CA. Observation 5: When the Rx chain expands to cover an extended span that includes both carriers, several factors could degrade performance relative to the single-carrier case. Proposal 1: No new uplink-related requirements are necessary, and the existing Rel-19 UL time mask can be directly reused without any modification. Proposal 2: A maximum allowable separation below which the two carriers are considered in conjunction with the maximum aggregated channel bandwidth (CBW) that a single Rx chain can process. Proposal 3: RAN4 needs to consider a maximum aggregated channel bandwidth between two adjacent LBs, which can guarantee the single-Rx performance with legacy Rx requirements. Proposal 4: Each downlink carrier should satisfy existing single-carrier requirements for the feature, which is following the legacy intra-band contiguous CA mechanism. Proposal 5: No new intermodulation or spurious response requirements are proposed at this stage if specific aggregated CBWs or separated gaps are later identified. Proposal 6: Restricting the scope to two-band combinations consisting of one FDD DL carrier and one SDL carrier ensures that the WI remains focused, manageable, and aligned with its original objective of UEs that cannot support conventional DL CA. |
| R4-2513810 | Skyworks Solutions, Inc. | Feasibility of LB switching FDD+SDL example combinations Observations: · n67 SDL band overlaps with n28UL (n28B side) and is adjacent to Band n28 DL · n29 SDL band has only 1MHz gap to Band n12 (and n85) UL and 1MHz gap to Band n12 DL (and adjacent to n85 DL) · In most cases: o Band n12 uses a n85 implementation o Band n28 uses a full band filter. · n29+n12DL bandwidth is smaller than current worst-case filters that are duplexer which have more demanding OOB rejections => n29+n12DL filter is feasible · n67+n28ADL bandwidth is similar than current worst-case filters that are duplexer which have more demanding OOB rejections => n67+n28ADL filter is feasible · n67+n28FDL bandwidth is 25% larger than current worst-case filters that are duplexer which have more demanding OOB rejections => although the OOB rejection is less demanding than for a duplexer the feasibility of n67+n28FDL needs to be confirmed. Proposal: · Requirements are developed based on a two-antenna architecture and a combined FDD DL+SDL filter based on example bands where the FDD DL and SDL bands are adjacent. o The post LNA FDD DL and SDL band separation is not the not the default implementation and may also need to be considered in the requirements. · For the feature to be applicable to other LBLB case, there may be a need to be another example case where the SDL band is separate from the FDD DL band and the SDL/FDD DL filters are duplexed · The n12 DL+n29 filter and n28A DL+n67 combined filters should be feasible. However, the n28F DL+n67 combined filter feasibility needs to be studied. |
| R4-2513813 | Ericsson | On low-band CA via switching with dual downlink Observation 1: the CA_n5A-n29A switching band combination can be supported with dual downlink, a triplexer supporting this combination appears feasible from a filter capability standpoint. Observation 2: a first assessment of antenna performance for the CA_n5A-n29A combination with support of dual downlink shows a degradation of n5 total antenna efficiency of 2-3 dB and 4 dB for the n29 with a single-tuning state for a single antenna as compared to an implementation with two-state antenna tuning and LB switching. Further optimization of n5 (Pcell) uplink performance with a single tuning state at the expense of n29 (Scell) downlink performance could possibly enable implementation as a standard DL-only CA configuration without the performance degradation of semi-static switching, however. Observation 3: a single Rx filter appears feasible for supporting the example band combinations CA_n12A-n29A and CA_n28A-n67A with dual downlink conditioned on a restriction to the restriction of n28 operation to the range 703-733/758-788 MHz relevant for operations in Region 1. Observation 4: the standard ACS, in-band and out-of-band blocking requirements for inter-band CA also apply for LB CA implemented via switching unless otherwise stated. Observation 5: notwithstanding the deployment constraint no in-gap interference between DL carriers is expected listed in the Rel-20 WI objectives, there may still be in-band/gap blockers (other operators) for the example band combinations. Observation 6: the selectivity for the example band combinations could be verified using requirements for intra-band non-contiguous CA for TDD bands with in-gap and out-of-gap interferers just as for EN-DC configurations for partially overlapping TDD bands for which co-located and synchronized network deployment for both carriers is also assumed. The selectivity requirements could be specified with regard to the aggregated CA bandwidth for both single-and dual Rx architectures with due account of the switching. Observation 7: verifying for conformance that the UL transient period precedes the first UL symbol after the switch and that it does not include the switching period is challenging using the existing time-mask tests (difficult to fail except for SRS even if the tests have been improved). Observation 8: that the UE transmits the UL also when the transmissions are overlapping with the switching gap of the SDL-FDD transition could be tested by setting a large TA value for a high-order UL MCS sensitive to the location of the transient period and with DL/UL slots around the switching instants scheduled; then perform an UL TP test to verify that the switching period does not overlap with the first symbols of the UL transmission. |
| R4-2513855 | Apple | Initial Views on RF requirements for LB CA via switching with 2 DLs Observation 1: The two DLs are either adjacent (CA_n28-n67) or relatively very close in frequency Observation 2: Because n29 is located within n12 band, antenna tuning may not be required when switching between n12 and n29. Likewise, antenna tuning is not required for CA_n28-n67. Proposal 1: To accommodate different UE implementations, we propose using the same switching period values used in Rel-19 (35uSec, 70uSec, and 140uSec) as a baseline to determine the periods for Rel-20. |
| R4-2513993 | OPPO | Discussion on R20 low NR band carrier aggregation via switching Observation 1: There is no impact of Tx to Rx in both of case 2 and case 3. Observation 2: One wider filter can be adopted to combine FDD DL spectrum with SDL spectrum for CA_n12A-n29A and CA_n28A-n67A, and its not needed to consider the in-gap interference due to there is no gap or only 1MHz gap between two DL carriers. Observation 3: The only difference of UE implementation between Rel-19 low-band CA via switching and R20 low-band CA via switching for CA_n12A-n29A and CA_n28A-n67A is the filter in the receiver for SDL or for FDD DL+ SDL. Proposal 1: Using one wider filter cover two bands DL spectrums for CA_n12A-n29A and CA_n28A-n67A. Proposal 2: R20 low-band CA via switching can reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching. Proposal 3: The REFSENs of FDD band in case 3 need re-evaluate based on new receiver architecture. |
| R4-2513997 | ZTE Corporation, Sanechips | Discussion on R20 low-low band CA via switching Observation 1: Some FDD+SDL low band CA can support both single DL switched and dual DL switched options Observation 2: Rel-19 LBCA switching outcomes are reused as much as possible. Observation 3: Only single carrier in each band for FDD-SDL LBCA band combination switching. Proposal 1: For dual DL switching: Proposal 2: Reuse the Tx on-off time mask requirements of Rel-19 for defined, where minor modifications maybe needed to describe the dual DL switching in the specification. Proposal 3: Inform RAN2 that RAN4 introduce new dual DL switching case for signalling updating. |
| R4-2514254 | LG Electronics | Discussion on low NR band carrier aggregation via switching Observation 1: The key difference between Rel19 and Rel20 is the support of FDD DL enabling DLDL operation, with additional CA combinations such as CA_n12n29 and CA_n28n67. Observation 2: In Rel19, the architecture used in RAN4 cannot support both bands Proposal 1: RAN4 should study whether switching period is updated or not according to architecture Proposal 2: RAN4 should decide whether to adopt a single chain or a dual chain for Rel20 LBCA as the baseline assumption for requirement. Proposal 3: RAN4 should determine whether the on/off time mask is reused or not according to the architecture Proposal 4: The switching gap should be applied to switching from, as in Rel19, to maintain control channel robustness. Proposal 5: Capablity is needed for supporting Rel-20 LBCA(Case1-Case3) |
| R4-2514371 | Google | Discussion on enhancement of low band carrier aggregation via switching Proposal 1: It is proposed to reuse switching periods 35s, 70s, and 140s for switching between Case 1 and Case 3 as an optional UE capability. Proposal 2: It is proposed that RIB,c relaxation may be needed for Case#3 and the detailed values can be further discussed. Proposal 3: It is proposed that Rx power imbalance requirements may needed for Case#3 and the detailed values can be further discussed. |

# Topic #1: General

## Summary of open issues

Issue 1-1: Work plan

- Proposal: Approve the work plan provided by the Rapporteur [R4-2514294]

| Meeting | Tasks |
| --- | --- |
| RAN4 #116-bis (RF, 0.25 TU) (RRM, 0.25 TU) | General - Approval of workplan RF (core) - Discuss reference architecture for Case 3 - Discuss switching time applicable to Case 3 RRM (core) - Preliminary analysis on RRM working scope for Rel-20 dual DL LB CA via switching scenario |
| RAN4 #117 (RF, 0.25 TU) (RRM, 0.25 TU) | RF (core) - Conclude reference architecture and switching time for Case 3 - Discuss scope of impact of Case 3 on the remaining RF requirements RRM (core) - Continue discussion and conclude on RRM working scope for Rel-20 dual DL LB CA via switching scenario - Initial discussion on - Applicability of Rel-19 LB CA RRM core requirements in Rel-20 dual DL LB CA via switching scenario - RRM core requirements impact identified for Rel-20 dual DL LB CA via switching scenario |
| RAN4 #118 (RF, 0.25 TU) (RRM, 0.5 TU) | RF (core) - Continue discussion of RF requirements and impact on the specification RRM (core) Continue discussion on - Applicability of Rel-19 LB CA RRM core requirements in Rel-20 dual DL LB CA via switching scenario - RRM core requirements impact identified for Rel-20 dual DL LB CA via switching scenario |
| RAN4 #118-bis (RF, 0.25 TU) (RRM, 0.5 TU) | RF (core) - Conclude the RF requirement discussion and endorse a draft CR RRM (core) - Continue discussion and conclude on the applicability of Rel-19 LB CA RRM core requirements in Rel-20 dual DL LB CA via switching scenario - Continue discussion on RRM core requirements impact identified for Rel-20 dual DL LB CA via switching scenario |
| RAN4 #119 (RF, 0.25 TU) (RRM, 0.5 TU) | RF (core) - Agree on a formal CR RRM (core) Continue discussion on RRM core requirements impact identified for Rel-20 dual DL LB CA via switching scenario |
| RAN4 #120 (RF, 0.25 TU) (RRM, 0.5 TU) | RRM (core) - Continue discussion on RRM core requirements impact identified for Rel-20 dual DL LB CA via switching scenario and agree on the identified impacts - Discuss spec impacts of RRM core requirements and agree on the work split of draft CR(s) |
| RAN4 #120-bis (RF, 0.25 TU) (RRM, 0.5 TU) | RRM (core) - Update or specify new RRM core requirements for Rel-20 dual DL LB CA via switching scenario - Discussion on draft CR(s) if available |
| RAN4 #121 (RF, 0.25 TU) (RRM, 0.5 TU) | RRM (core) Finalize discussion and endorse draft CR(s) on RRM core requirements for Rel-20 dual DL LB CA via switching scenario |
| RAN4 #122 (RF, 0.25 TU) (RRM, 0.25 TU): | RRM (core) - Agree on formal CR(s) - Resolve remaining issues on RRM requirements for Rel-20 dual DL LB CA via switching scenario if any and close the WI |

NOTE: TR 38.768 will continue to be used to capture relevant RF and RRM technical background during this work item. The TR is now under revision control, and change requests are encouraged via the draft CR / formal CR process.

- Recommended WF: Approve the work plan

Issue 1-2: Work scope clarification

- Option 1: RAN4 should not consider the introduction of higher order CA with the LB CA switching as one of fallback configurations and/or different device types of LB CA switching (e.g., con-current DL CA, or FDD+FDD CA switching with concurrent DL CA) as RAN4 did in Rel-19 (Huawei R4-2513536)

- Option 2: Restricting the scope to two-band combinations consisting of one FDD DL carrier and one SDL carrier ensures that the WI remains focused, manageable, and aligned with its original objective of UEs that cannot support conventional DL CA [Samsung R4-2513735]

- Recommended WF: Discuss further

Issue 1-3: Enhanced low band switching scenarios

- Proposal: draft CR on low band switching scenarios (Apple R4-2514405)

  - Change 1: capture the work item description on the enhancement of low NR band carrier aggregation via switching (Clause 1)

  - Change 2: capture the operator-provided scenarios (Clause 4)

- Recommended WF: Discuss further

# Topic #2: Reference architecture

## Summary of open issues

Issue 2-1: Summary of proposed UE reference architectures

- Option 1: Nokia R4-2513769

2 LO arrangement for aggregated reception

1 LO arrangement for aggregated reception

- Option 2: CATT R4-2513176

- Option 3: Xiaomi R4-2513448

  - The required transient period from UE transceiver chain switching between carrier 1 and carrier is depending on UE architecture which can be categorized as following:

  - Partially shared receiver chain: UE share RF front-end including antenna and LNA with separate LO/Mixer, LBF between two carriers

  - UE needs to switch duplex/RF filter in front-end between transient period with necessary RF retuning as well

  - Fully shared receiver chain: UE share whole receiver chain from antenna to ADC

  - UE needs to switch duplex/RF filter in front-end between transient period with necessary RF retuning for mixer/LO and other analogy components as well

- Option 4: Huawei R4-2513537

- Option 5: Spreadtrum R4-2513641

- Option 6: Samsung R4-2513735

Case 1: Single-Rx switching (dual DL via single Rx chain)

Case 2: Dual-Rx conventional CA (dual Rx chains)

- Option 7: Skyworks R4-2513810

RF front-end architecture to support switching CA between FDD DL+UL and adjacent FDD DL + SDL bands

RF front-end architecture to support switching CA between FDD DL+UL and separate FDD DL + SDL bands

  - In our paper we have shown that a combined band n28+n67 DL filter is challenging in terms of bandwidth (especially if full band n28 is supported). In our view the feasibility of such n28DL+n67DL filter will require further study before we can conclude whether such RFFE architecture can be considered as the reference.

- Option 8: Ericsson R4-2513813

  - the CA_n5A-n29A switching band combination can be supported with dual downlink, a triplexer supporting this combination appears feasible from a filter capability standpoint

  - a single Rx filter appears feasible for supporting the example band combinations CA_n12A-n29A and CA_n28A-n67A with dual downlink conditioned on a restriction to the restriction of n28 operation to the range 703-733/758-788 MHz relevant for operations in Region 1

- Option 9: Apple R4-2513855

- Option 10: OPPO R4-2513993

- Option 11: LG Electronics R4-2514254

a)b)

(a)candidate single chain for Rel-20 (b)candidate dual chain for Rel-20

Option 12: ZTE (R4-2513997)

- Moderator’s note: other options are not precluded, and the options are not necessarily mutually exclusive

- Recommended WF: Discuss further and decide how to make progress on the reference architecture assumption toward the next (November) meeting

# Topic #3: Switching period aspects

## Summary of open issues

Issue 3-1: Switching period for switching between Case 1 and Case 3

- Option 1: In 2 DL LO operation both LOs run at constant frequency. FDD RX may run continuously while SDL RX is activated/deactivated based on the RRC pattern [Nokia R4-2513769]

- Option 2: [CATT R4-2513176]

  - the switching gap in SDL band is needed when the status is switching from Dual-DL FDD+SDL bands to UL/DL FDD band.

  - the switching gap in UL band is needed when the status is switching from UL/DL FDD band to Dual-DL FDD+SDL bands

  - RAN4 need to discuss whether UE can maintain the DL reception in DL FDD band during the switching gap from Dual-DL FDD+SDL bands to UL/DL FDD band or from UL/DL FDD band to Dual-DL FDD+SDL bands

- Option 3: Introduce new separate UE capability to support LB CA via switching between {case 1, case 3} with Dual DL with multiple values for switching period to accommodate different UE implementation [Xiaomi R4-2513448, Huawei R4-2513537, Spreadtrum R4-2513641, Apple R4-2513855, ZTE R4-2513997, Google R4-2514371]

  - Switching period capability values {35us, 70us, 140us}

  - The capability shall be per band pair per band combination basis

- Option 4: Rel-19 switching delay requirements {35us, 70us, 140us} can be used as a starting point for Rel-20 [vivo R4-2513563]

- Recommended WF: Discuss further

Issue 3-2: Switching time mask for switching between Case 1 and Case 3

- Option 1: Consider the whole picture [CATT R4-2513176]

- Option 2: Reuse the Rel-19 ON/OFF time mask for Rel-20 LB CA switching [Xiaomi R4-2513448, Huawei R4-2513537, Spreadtrum R4-2513641, Samsung R4-2513735, ZTE R4-2513997, LG Electronics R4-2514254]

  - Option 2a: Tx on/off time mask requirements which is introduced in Rel-19 for LB CA switching can be applied to LB CA via switching with dual DL [Xiaomi]

    - For Dual DL CA switching to FDD carrier, the switching period/switching gap located out of FDD carrier slot

    - For FDD carrier switching to Dual DL carriers, the switching period/switching gap located within FDD carrier slot

    - Some editorial update expected on the description below  on/off mask figure

  - Option 2b: only define Tx on-off time mask requirements, also it seems Rel-19 Tx on-off time mask requirements can be adopted where minor modifications maybe needed to describe the dual DL switching in the specification [ZTE]

  - Option 2c: In Rel‑19, RAN4 discussed where the switching should be applied within the time‑mask definition. One reason for deciding on “switching from” was to avoid loss of PDCCH. If the switching gap were applied to “switching to,” it could harm PDCCH symbols by truncating or erasing control symbols. So, likewise, Rel‑20 could reuse the Rel‑19 approach for consistency.  The switching gap should be applied to “switching from,” as in Rel‑19, to maintain control‑channel robustness [LG Electronics]

- Option 3: Reuse Rel-19 ON/OFF mask for case 1 <-> case 2 as starting point for Rel-20 case 1 <-> case 3, and confirmation may need RAN1 involvement [vivo R4-2513563]

- Option 4: verifying for conformance that the UL transient period precedes the first UL symbol after the switch and that it does not include the switching period is challenging using the existing time-mask tests (difficult to fail except for SRS even if the tests have been improved) [Ericsson R4-2513813]

- Recommended WF: Discuss further

# Topic #4: Others

## Summary of open issues

Issue 4-1: Possible co-existence issues

- Following observations are obtained on co-existence issues per scenario [Huawei, R4-2513536]

| Scenario | Type of interference | CA_n12-n29 | CA_n28-n67 |
| --- | --- | --- | --- |
| UE to UE (UE “A” is transmitting FDD UL and UE “B” is receiving both SDL & FDD DL in dual DL operation) | Co-channel | UE “B” may face heavy co-channel interference issue if UE “B” is receiving signal at the first half part of n29 spectrum and UE “A” is transmitting signal at the upper side of n12 UL spectrum. This could be acceptable, given that n29 is supplemental carrier. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. If, however, RAN4 assumes that n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz for the LB CA, the situation can be mitigated thanks to 5 MHz guard band. This also mitigates the challenges in developing UE Rx filter to cover both the entire n28 and n67, since relative frequency ratio of around 8.4% (100*(65/770.5)) decreases to around 6.6% (100*(50/762)) |
|  | Blocking | Since Rx filter covering both n29 and n12 DL cannot cut n12 UL power out if n12 UL is used at the upper side of n12, both n29 & n12 DL of UE “B” may face a blocking issue. Given that dual DL operation assumes that n12 DL (PCell) is always available, the blocking issue should be carefully handled, though those issues including co-channel are UE to UE so that the actual impact depends on various conditions such as distance between UEs, UL power, RBs position, received DL power, etc. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to Co-channel interference, 5 MHz guard band can help UE “B” cut the power of n28 UL out |
| BS to UE / UE to BS | Given that co-location is assumed, all those wouldn’t have an issue. |  |  |
| BS to BS | Co-channel interference and blocking | Since the assumption is collocated scenario, this will cause a big impact on BS performance to receive n12 UL from UEs due to co-channel and blocking from n29 BS DL transmission, given that the guard band is only 1MHz. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to UE to UE co-existence for CA_n28-n67, 5 MHz guard band can help n28 BS cut the power of n67 BS out and n67 BS cut the interference over n28 UL out. |

  - For UE to UE co-existence (FDD UL Transmission to SDL reception), RAN4 should discuss whether or not special care is needed or not.

  - For CA_n12-n29, e.g., necessity of special care in terms of ASE and/or blocking

  - For CA_n28-n67, e.g., clarification of frequency restriction is needed

  - RAN4 should first clarify how UE to UE and BS to BS co-existence issues are resolved and if there is any restriction such as only lower portion of n28 is used, or only some portion of n29 is used etc., before going into the deep detail

- Recommended WF: Discuss further

Issue 4-2: Impact on RX requirements

- Option 1: consider practical implementation aspects [Nokia, R4-2513769]

  - Requirements for the aggregated DL operation should be set in such a way that 2 DL LO and continuous FDD DL operation are not excluded.

  - For optimum network operation it would be highly beneficial to focus on scenarios where only the SDL carrier is sensitive to cross-UE interference from the FDD UL carrier, not the contiguous FDD DL carrier

- Option 2: Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of Issue 4-1 [Huawei, R4-2513537]

  - CA_n12-n29 REFSENS: may be able to keep single band requirement or may need only limited amount of relaxation

  - CA_n28-n67 REFSENS: the discussion is affected by the conclusion on if there is any restriction in terms of spectrum usage for LB CA operation with dual DL, e.g., fullband n28 is expected to be used or only lower portion of n28 is used

  - Blocking: Locations of blockers for blocking requirements shall be shift depending on assumed actual Rx filter frequency range

- Option 3: Rx part is the main difference between Rel-20 and Rel-19, in that Rel-19 do not support DL CA while Rel-20 do in case 3. This is actually quite similar to what Rel-16 Tx switching case, in that DL interruption need to be considered. However, this is in RRM scope and would not be discussed in detail here. [vivo R4-2513563]

- Option 4: REFSENS and ΔRIB,c requirements are all need to be discussed [Spreadtrum, R4-2513641]

- Option 5: When the Rx chain expands to cover an extended span that includes both carriers, several factors could degrade performance relative to the single-carrier case [Samsung R4-2513735]

  - RAN4 needs to consider a maximum aggregated channel bandwidth between two adjacent LBs, which can guarantee the single-Rx performance with legacy Rx requirements

  - Each downlink carrier should satisfy existing single-carrier requirements for the feature, which is following the legacy intra-band contiguous CA mechanism.

  - No new intermodulation or spurious response requirements are proposed at this stage if specific aggregated CBWs or separated gaps are later identified

- Option 6: Consider selectivity: ACS and in-band blocking with in-gap interferers [Ericsson R4-2513813]

  - the standard ACS, in-band and out-of-band blocking requirements for inter-band CA also apply for LB CA implemented via switching unless otherwise stated.

  - notwithstanding the deployment constraint ‘no in-gap interference between DL carriers is expected’ listed in the Rel-20 WI objectives, there may still be in-band/gap blockers (other operators) for the example band combinations.

  - the selectivity for the example band combinations could be verified using requirements for intra-band non-contiguous CA for TDD bands with in-gap and out-of-gap interferers just as for EN-DC configurations for partially overlapping TDD bands for which ‘co-located and synchronized network deployment for both carriers’ is also assumed. The selectivity requirements could be specified with regard to the aggregated CA bandwidth for both single-and dual Rx architectures with due account of the switching

- Option 7: The REFSENs of FDD band in case 3 need re-evaluate based on new receiver architecture [OPPO R4-2513993]

- Option 8: Consider specifying RX requirements based on common LNA to accommodate these adjacent two carriers [Google R4-2514371]

  - It is proposed that ΔRIB,c relaxation may be needed for Case#3 and the detailed values can be further discussed.

  - It is proposed that Rx power imbalance requirements may needed for Case#3 and the detailed values can be further discussed.

- Option 9: current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for R20 Low-Low bands inter-band CA via switching, since current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for R20 Low-Low bands inter-band CA via switching [CATT R4-2513176]

- Moderator’s note: other options are not precluded, and the options are not necessarily mutually exclusive

- Recommended WF: Discuss further

Issue 4-3: Impact on BS requirements

- RAN4 should first clarify how UE to UE and BS to BS co-existence issues are resolved and if there is any restriction such as only lower portion of n28 is used, or only some portion of n29 is used etc., before going into the deep detail [Huawei, R4-2513536, R4-2513537]

# Annex: Approved WID (for information)

Please refer to the latest approved WID in RP-252953

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.