3GPP TSG-RAN WG4 Meeting #116bis	R4-2514612

# Prague, Czech Republic,  13th - 17th October 2025				(revision of R4-2513847)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | <draft> | rev | <#> | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Renaming NTN IOT band 256 NS_24 flag to NS_24N |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NTN IOT band 256 flag NS_24 was inherited from the terrestrial specifications, where it is defined only for the PC3 power class. Since Rel-19 enabled PC2 and PC1 power classes for both NTN NR and IOT technologies, it was decided to rename NS_24 to NS_24N with the corresponding change already implemented in the NR specification TS 38.101-5. However, the same change was not applied to the IOT track in TS 36.102. |  |  |  |  |  |  |  |  |
| N |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | NTN IOT band 256 NS_24 is renamed to NS_24N to have consistent NS flag names across NR and IOT specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be logical mismatches in NS flag naming between NTN IOT and NR specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3, 6.2B.3, 6.5A.4.4.3, 6.5B.4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2A.3	UE additional maximum output power reduction for category M1 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

The specific requirements and identified subclauses are specified in Table 6.2A.3-1 and Table 6.2A.3-1a along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2A.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE for PC3 and PC5

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |  |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | Table 5.3A-1 | N/A |  |
| NS_24N | 6.5A.4.4.3 | 256 | Table 5.3A-1 | PC3 | PC5 |
|  |  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5A.4.4.2 | 255 | Table 5.3A-1 | N/A |  |
| NS_03N | 6.5A.3.3.1, 6.5A.4.4.4 | 254 | Table 5.3A-1 | N/A |  |
| NS_04N | 6.5A.3.3.2, 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_05N | 6.5A.3.3.3, 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_06N | 6.5A.4.4.7 6.5A.3.3.1 | 252 | Table 5.3A-1 | N/A |  |
| NS_07N | 6.5A.4.4.8 6.5A.3.3.1 | 252 | Table 5.3A-1 | N/A |  |
| NS_08N | 6.5A.4.4.9 6.5A.3.3.1 | 252 | Table 5.3A-1 | N/A |  |
| NS_11N | 6.5A.3.3.2, 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_12N | 6.5A.3.3.3, 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |

Table 6.2A.3-1a: Additional Maximum Power Reduction (A-MPR) for category M1 UE for PC2

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | Table 5.3A-1 | N/A |
| NS_24N | 6.5A.4.4.3 | 256 | Table 5.3A-1 | ≤ 6.5 |
| NS_02N | 6.5A.4.4.2 | 255 | Table 5.3A-1 | N/A |

For subPRB allocation, the allowed A-MPR values specified below in Table 6.2A.3-2 and Table 6.2A.3-3 for category M1 UE are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-2: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation for PC3 and PC5

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | N/A |  |
| NS_24N | 6.5A.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5A.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5A.3.3.1, 6.5A.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5A.3.3.2, 6.5A.4.4.5 | 254 | N/A |  |
| NS_05N | 6.5A.3.3.3, 6.5A.4.4.5 | 254 | N/A |  |
| NS_06N | 6.5A.4.4.7 6.5A.3.3.1 | 252 | N/A |  |
| NS_07N | 6.5A.4.4.8 6.5A.3.3.1 | 252 | N/A |  |
| NS_08N | 6.5A.4.4.9 6.5A.3.3.1 | 252 | N/A |  |
| NS_11N | 6.5A.3.3.2, 6.5A.4.4.5 | 254 | PC3 | PC5 |
|  |  |  | ≤ 3 | 0 |
| NS_12N | 6.5A.3.3.3, 6.5A.4.4.5 | 254 | PC3 | PC5 |
|  |  |  | ≤ 3 | 0 |

Table 6.2A.3-3: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation for PC2

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | N/A |
| NS_24N | 6.5A.4.4.3 | 256 | ≤ 6.5 |
| NS_02N | 6.5A.4.4.2 | 255 | N/A |

*********** NEXT CHANGED SECTION **********

### 6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE PC3/PC5 and PC1/PC2, the specific requirements and identified subclauses are specified in Table 6.2B.3-1 and 6.2B.3-2 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_24N | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.3.3.2, 6.5B.4.4.4 | 249, 254 | N/A |  |
| NS_04N | 6.5B.3.3.3, 6.5B.4.4.5 | 249, 254 | 6.2B.3.1 |  |
| NS_05N | 6.5B.3.3.4, 6.5B.4.4.5 | 249, 254 | 6.2B.3.2 |  |
| NS_06N | 6.5B.4.4.7 6.5B.3.3.1 | 252 | N/A |  |
| NS_07N | 6.5B.4.4.8 6.5B.3.3.1 | 252 | N/A |  |
| NS_08N | 6.5B.4.4.9 6.5B.3.3.1 | 252 | N/A |  |
| NS_11N | 6.5B.3.3.3, 6.5B.4.4.5 | 249, 254 | 6.2B.3.3 |  |
| NS_12N | 6.5B.3.3.4, 6.5B.4.4.5 | 249, 254 | 6.2B.3.4 |  |

Table 6.2B.3-2: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1 and PC2

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_24N | 6.5B.4.4.3 | 256 | PC1 | PC2 |
|  |  |  | ≤ 10 | ≤ 5.5 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |

*********** NEXT CHANGED SECTION **********

##### 6.5A.4.4.3	Minimum requirement (network signalled value "NS_24N")

When "NS_24N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3-1.

Table 6.5A.4.4.3-1: Additional requirements for "NS_24N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4MHz |  |
| Band 34 | -50 | MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band 256. |  |  |

*********** NEXT CHANGED SECTION **********

##### 6.5B.4.4.3	Minimum requirement (network signalled value "NS_24N")

When "NS_24N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.4.4.3-1.

Table 6.5B.4.4.3-1: Additional requirements for "NS_24N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 200kHz |  |
| Band 34 | -50 | MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band 256. |  |  |