3GPP TSG-RAN WG4 Meeting #116bis	R4-2514791

Prague, Czech Republic, October 13 ‒ October 17, 2025

Title: 	WF on NonCol_intraB_ENDC_NR_CA_Ph2_demod

Agenda Item:	6.2.1

Source: 	Ericsson

Document for:	Approval

# Topic #1: Work plan

## Sub-topic 1-1	Work plan

Agreement: Work plan

- RAN4#116bis, October 2025 (0.25 TU for Demod/RRM performance)

  - Discuss the scope of UE demodulation requirements.

  - Agree with the initial simulation assumption.

- RAN4#117, November 2025 (0.25 TU for Demod/RRM performance)

  - Update the simulation assumption if necessary.

  - Collect the initial simulation results.

  - Decide the CR work split.

- RAN4#118, February 2026 (0.25 TU for Demod/RRM performance)

  - Collect the simulation results.

  - Agree with the draft CRs for TS38.101-4.

  - Agree with the big CR for TS38.101-4 (email approval).

# Topic #2: UE demodulation requirements

## Sub-topic 2-1	Test scope

Agreement: Test scenario

Define PDSCH demodulation requirements for Type 4b UE with 2CC NR-CA scenario.

- Specify FR1 TDD + FR1 TDD only.

Agreement: Received time difference between two carriers

Configure the timing offset of the SCell from the PCell is 33 µs.

Agreement: Channel bandwidth and SCS

Configure CBW=40MHz and SCS=30kHz for both CCs.

## Sub-topic 2-2	Simulation assumption

Way forward: MCS/rank

Down-select the MCS for the stronger CC based on the simulation results

- For weaker CC:

  - Rank 2 + QPSK 0.3 (MCS4 in MCS index table 1)

- For stronger CC:

  - Option 1: Rank 4 + 256QAM 0.69 (MCS21 in MCS index table 2)

  - Option 2: Rank 4 + 256QAM 0.74 (MCS22 in MCS index table 2)

  - Other options are not precluded.

Interested companies are encouraged to provide the simulation results (both alignment and impairment results) in the RAN4#117 to decide the rank/MCS.

Agreement: Channel model and antenna configuration

Use the static channel model specified in TS 38.101-4 B.1.1.

- 2x4 for CC with rank 2

- 4x4 for CC with rank 4

Agreement: How to choose MCS/rank

The final choice of MCS and Rank should consider the maximum SNR difference of 25dB, where the SNRs are average of companies’ impairment results, plus additional margin (0.5dB for QPSK, 0.8dB for 256QAM).

Agreement: Tx EVM assumption

Assume Tx EVM=3% for both CCs.

Way forward: Antenna connection

Discuss whether to capture the antenna connection for Type 4b UE test in TS 38.101-4 5.1.1.

- Example:

  - For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx, 4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration.

## Sub-topic 2-3	Applicability

Agreement: Separation of two CCs

Send LS to RAN5 as same as Rel-18 NonCol_intraB_ENDC_NR_CA UE demodulation performance part in RAN4#117.

Qualcomm volunteers to send LS to RAN5.

Agreement: Test applicability

Type 4b UE demodulation requirement is applicable for UEs with the capability of:

- Supporting ‘intraBandNR-CA-non-collocated-r19’,

- Providing ‘nonCollocatedTypeNR-CA-v1900’ with value ‘type4’.