3GPP TSG-RAN WG4 Meeting #116bis	R4-2514795

# Prague, Czech Republic, Oct. 13-17, 2025

Title: 	WF on NR_ATG_enh_RRM

Agenda Item:	6.7.1

Source: 	CMCC

Document for:	Approval

<Topic 1> RRM perf requirement

Issue 2-1-1: Test scope - separate tests for neighbour cell skipping/not skipping

Agreement:

- Introduce Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle (no signalling for neighbour cell skipping)

- FFS whether and how to introduce Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms or 160ms SCell measurement cycle (with signalling for neighbour cell skipping)

Issue 2-1-3: Detail test configuration and test procedure - Event triggered reporting on SCC with deactivated SCell

Agreement:

- For Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle (no signalling for neighbour cell skipping), the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except that:

  - No highSpeedMeasCA-Scell-r17 indication configured

  - the DRX configuration shall be OFF

  - the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A

  - the propagation condition shall follow R18 ATG test cases. PCell shall be continuously scheduled in DL

  - The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2, and include the interruption requirement

- For Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle (with signalling for neighbour cell skipping), if the test will be introduced, the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except that:

  - No highSpeedMeasCA-Scell-r17 indication configured

  - the DRX configuration shall be OFF

  - the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A

  - the measCycleSCell shall be configured as 160ms

  - the neighbour cell measurement skipping indication shall be configured

  - the propagation condition shall follow R18 ATG test cases

  - The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2.

  - Measurement report triggering event: FFS

  - Note: whether this test will be introduced is depending on Issue 2-1-1.

Issue 2-1-4: Detail test configuration and test procedure - Interruption requirement at SRS antenna port switching

Agreement: Take proposal 1 as the starting point

- Proposal 1:

  - For Interruption requirement at SRS antenna port switching, the test in Clause A.6.5.2.3 and A.6.5.2.4 TS38.133 can be taken as the reference respectively, except that:

    - only DL interruption will be considered in test environment and test requirement

    - Uplink initial BWP configuration and Uplink dedicated BWP configuration for SCell shall be N/A

    - SRS configuration for SCell shall be N/A

    - propagation condition shall follow R18 ATG test cases

Issue 2-1-5: Detail test configuration and test procedure - Direct SCell Activation at Handover

Agreement: Use Proposal 1 as starting point, and further check the details in CR stage.

- Proposal 1:

  - Update the test scenario for Direct SCell Activation at Handover test to:

    - At the start of time duration T1, the UE is in connected mode with PCell (Cell 1) and UE is reporting CQI for PCell. The UE is configured to measure RF channel 2 in measurement gaps. During T1, the UE detects and measures Cell 2 on RF channel 2, and sends a measurement report containing Cell 2 to the test equipment. After having received a measurement report containing Cell 2, the test equipment deconfigures the measurement gaps and thereafter sends a RRC connection reconfiguration message to the UE. The time between reception of the last measurement report carrying SCell and transmission of the RRC connection reconfiguration message directly activating SCell is kept short enough to allow the SCell to remain known to the UE.

    - Time period T2 starts when UE receives a handover command to PCell (Cell 3) that also activates SCell 1 (Cell 2). This is done using an RRCReconfiguration message with parameter sCellState set to activated for the SCell 1 (Cell 2). The message is sent from the test equipment to the UE and is received in a subframe # denoted n at the UE antenna connector. The UE shall accomplish the activation of the SCell no later than subframe (n + Ndirect).

    - Time period T3 starts at (n + Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell 1.

Issue 2-1-6: Detail test configuration and test procedure - TRS based SCell Activation of SSB-less SCell

Agreement: Take proposal 1 as the starting point

- Proposal 1:

  - For TRS based SCell Activation of SSB-less SCell, the existing test case defined in Clause A.6.5.3.15 can be taken as the reference, except that propagation condition shall follow R18 ATG test cases and an additional applicability rule shall be added that only applicable for UE support one common Rx beam.

Issue 2-1-7: Detail test configuration and test procedure - Others

Agreement: Take proposal 1 as the starting point

- Proposal 1:

  - For normal SCell activation and deactivation, Direct SCell activation at SCell addition, fast SCell activation, and SCell activation with valid L3 measurement results test case, the existing test case defined in Clause A.6.5.3.1, A.6.5.3.2, A.6.5.3.3, A.6.5.3.4, A.6.5.3.10, A.6.5.3.11 and A.6.5.3.14 can be taken as the reference respectively, except that propagation condition shall follow R18 ATG test cases.

CR work split

| Test number | Title | Company |
| --- | --- | --- |
| 1-1 | A.19.4.6 A.19.4.6.1 Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot | Huawei |
| 1-2 | A.19.4.6.2 Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot | Huawei |
| 2-1 | A.19.4.7 A.19.4.7.1 SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification) | ZTE |
| 2-2 | A.19.4.7.2 SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification) | ZTE |
| 2-3 | A.19.4.7.3 SCell Activation and deactivation of unknown SCell in FR1 in non-DRX (together with interruption requirement verification) | Ericsson |
| 2-4 | A.19.4.7.4 Direct SCell activation at SCell addition of known SCell in FR1 (together with interruption requirement verification) | Apple |
| 2-5 | A.19.4.7.5 Direct SCell activation at handover with known SCell in FR1 | Apple |
| 2-6 | A.19.4.7.6 Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | LGE |
| 2-7 | A.19.4.7.7 Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | LGE |
| 2-8 | A.19.4.7.8 SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle | Ericsson |
| 2-9 | A.19.4.7.9 TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX | CATT |
| 3-1 | A.19.4.2.3 Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG | CATT |
| 4-1 | A.19.5.1.5 Event triggered reporting on SCC with deactivated SCell test under non-DRX, with measurement cycle of no smaller than 640ms | CMCC |
| 4-2 | FFS: Event triggered reporting on SCC with deactivated SCell test under non-DRX, with measurement cycle of no smaller than 160ms | CMCC |