# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2514799

# Prague, Czech, October 13th – 17th, 2025

Agenda item:			6.10

Source:	Moderator (Ericsson)

Title:	WF on RRM requirements for NR_RRM_Ph5_Part1 and NR_RRM_Ph5_Part2

Document for:	Approval

# Topic #1: Performance requirements of SCell activation

#### Issue 2-1-1: Scope of test cases

Agreement in RAN4#115: - Scenario: - Normal SCell activation for FR1 and FR2 - Direct SCell activation at SCell addition for FR1 - PUCCH SCell activation for FR1 and FR2 - FFS: - Direct SCell activation at SCell addition for FR2 - UE capabilities and network configurations: - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - FFS: - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

- Proposals

  - Direct SCell activation at SCell addition for FR2

    - Consider this scenario: OPPO, CATT, CT, Huawei, CMCC, ZTE, Ericsson, Nokia

    - Do not consider this scenario: N/A

  - Configuration 3

    - Consider this configuration: OPPO, CATT, CT, Huawei, CMCC, ZTE, vivo, Ericsson, Nokia

    - Do not consider this configuration: Apple

Agreements:

- Direct SCell activation at SCell addition for FR2

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 has not expired at the moment of initiation of RRC state transition to CONNECTED mode

#### Issue 2-1-2: Applicability rules and number of test cases

- Proposals

  - Option 1: (CATT)

    - No applicability rules are required if the following test case list is agreed. Otherwise, option 1 (i.e., option 2 in this issue) can be a good starting point.

| TC No. | Scenario | Comment |
| --- | --- | --- |
| 1 | Normal SCell activation + FR1 + Configuration 1 | Non-DRX |
| 2 | Normal SCell activation + FR2 + Configuration 1 | N/A |
| 3 | Normal SCell activation + FR1 + Configuration 2 | N/A |
| 4 | Normal SCell activation + FR2 + Configuration 2 | DRX |
| 5 | Normal SCell activation + FR1 + Configuration 3 | Non-DRX |
| 6 | Normal SCell activation + FR2 + Configuration 3 | N/A |
| 7 | Direct SCell activation at SCell addition + FR1 + Configuration 1 | N/A |
| 8 | Direct SCell activation at SCell addition + FR2 + Configuration 1 | DRX |
| 9 | Direct SCell activation at SCell addition + FR1 + Configuration 2 | Non-DRX |
| 10 | Direct SCell activation at SCell addition + FR2 + Configuration 2 | N/A |
| 11 | Direct SCell activation at SCell addition + FR1 + Configuration 3 | N/A |
| 12 | Direct SCell activation at SCell addition + FR2 + Configuration 3 | DRX |
| 13 | PUCCH SCell activation + FR1 + Configuration 1 | Non-DRX |
| 14 | PUCCH SCell activation + FR2 + Configuration 1 | N/A |
| 15 | PUCCH SCell activation + FR1 + Configuration 2 | N/A |
| 16 | PUCCH SCell activation + FR2 + Configuration 2 | DRX |
| 17 | PUCCH SCell activation + FR1 + Configuration 3 | N/A |
| 18 | PUCCH SCell activation + FR2 + Configuration 3 | Non-DRX |

  - Option 2a: (Apple)

    - Introduce some applicability rule to reduce testing effort.

    - Agree the following testing applicability rules:

      - Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped.

      - Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped.

      - Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped.

  - Option 2b: (Huawei)

    - UE supporting two PUCCH groups, UE is required to pass PUCCH SCell activation TC and skip normal SCell activation TC.

  - Option 2c: (vivo, Qualcomm)

    - If a UE supports Direct SCell Activation or PUCCH SCell Activation, the Normal SCell Activation test case can be skipped.

  - Option 3: (OPPO)

    - For normal SCell activation and PUCCH SCell activation, choose only one set of legacy configurations (e.g., non-DRX, 640ms SCell measurement cycle) to define the test case for fast SCell activation.

  - Option 4: (OPPO, CT)

    - RAN4 to define some applicability rules to reduce the testing efforts.

  - Option 5a: (Ericsson)

    - It is preferred to have a unified test case for all Scell activation type.

  - Option 5b: (Qualcomm)

    - RAN4 to consolidate test cases for EMR capability-related configurations into unified test cases based on SCell activation types (Normal SCell activation, Direct SCell activation, and PUCCH SCell activation)

      - Within each consolidated test case, adapt the test flow to accommodate the specific UE capabilities.

      - Use conditional logic or branching within the test case to handle capability-specific behaviors.

  - Option 6: (Nokia)

    - Consider applicability only if core UE functionality between two test cases / scenarios is the same:

      - measIdleValidityDuration-r18 and measReselectionValidityDuration-r18 use different capabilities and carrier lists

      - MAC CE based activation and RRC based activation is good to test separately

      - Different test cases are needed where the core requirement delays differ

    - Discuss which combinations of EMR configurations are tested with each SCell scenario.

  - RAN4 to define the following 6 TCs for EMR based SCell activation:

    - In each TC, all three configurations are included with branching method used.

| TC No. (CR No.) | Scenario |
| --- | --- |
| 1 | Normal SCell activation + FR1 + Configuration 1 |
|  | Normal SCell activation + FR1 + Configuration 2 |
|  | Normal SCell activation + FR1 + Configuration 3 |
| 2 | Normal SCell activation + FR2 + Configuration 1 |
|  | Normal SCell activation + FR2 + Configuration 2 |
|  | Normal SCell activation + FR2 + Configuration 3 |
| 3 | Direct SCell activation at SCell addition + FR1 + Configuration 1 |
|  | Direct SCell activation at SCell addition + FR1 + Configuration 2 |
|  | Direct SCell activation at SCell addition + FR1 + Configuration 3 |
| 4 | Direct SCell activation at SCell addition + FR2 + Configuration 1 |
|  | Direct SCell activation at SCell addition + FR2 + Configuration 2 |
|  | Direct SCell activation at SCell addition + FR2 + Configuration 3 |
| 5 | PUCCH SCell activation + FR1 + Configuration 1 |
|  | PUCCH SCell activation + FR1 + Configuration 2 |
|  | PUCCH SCell activation + FR1 + Configuration 3 |
| 6 | PUCCH SCell activation + FR2 + Configuration 1 |
|  | PUCCH SCell activation + FR2 + Configuration 2 |
|  | PUCCH SCell activation + FR2 + Configuration 3 |

  - Discuss the option 2a as a starting point for applicability rules:

    - Agree the following testing applicability rules:

      - Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped.

      - Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped.

      - Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped.

    - Note: Any agreed testing applicability rules will be captured in clause A.3.X in 38.133.

Discussions:

QC: no specific frequencies are listed in the table.

CATT: add note that PCell and target cell shall be in same frequency range.

MTK: All in table are target SCells.

CATT: UE only needs to test some (not all) cases according to rules.

OPPO: EMR is our test target. For each type of SCell activation, choose one is sufficient.

Ericsson: depends on UE capability.

MTK: FR1 and FR2 shall be tested separately?

Ericsson: Different for FR1 and FR2.

OPPO: TC 5, 6 may be not needed.

Ericsson: no overlapping

Agreements:

- Further discussion applicability rules and test configurations, taking the below table containing TCs for EMR based SCell activation as reference.

| TC No. (CR No.) | Scenario |
| --- | --- |
| 1 | Normal SCell activation + FR1 + Configuration 1 |
|  | Normal SCell activation + FR1 + Configuration 2 |
|  | Normal SCell activation + FR1 + Configuration 3 |
| 2 | Normal SCell activation + FR2 + Configuration 1 |
|  | Normal SCell activation + FR2 + Configuration 2 |
|  | Normal SCell activation + FR2 + Configuration 3 |
| 3 | Direct SCell activation at SCell addition + FR1 + Configuration 1 |
|  | Direct SCell activation at SCell addition + FR1 + Configuration 2 |
|  | Direct SCell activation at SCell addition + FR1 + Configuration 3 |
| 4 | Direct SCell activation at SCell addition + FR2 + Configuration 1 |
|  | Direct SCell activation at SCell addition + FR2 + Configuration 2 |
|  | Direct SCell activation at SCell addition + FR2 + Configuration 3 |
| 5 | PUCCH SCell activation + FR1 + Configuration 1 |
|  | PUCCH SCell activation + FR1 + Configuration 2 |
|  | PUCCH SCell activation + FR1 + Configuration 3 |
| 6 | PUCCH SCell activation + FR2 + Configuration 1 |
|  | PUCCH SCell activation + FR2 + Configuration 2 |
|  | PUCCH SCell activation + FR2 + Configuration 3 |

#### Issue 2-1-3: Whether to consider DRX configuration in test cases

- Proposals

  - Option 1a: (Huawei)

    - The non-DRX mode is able to be considered in the test case for normal SCell activation with Configuration 1.

  - Option 1b: (Ericsson)

    - There is no need to configure DRX cycle for all the test cases.

  - Option 1c: (Qualcomm)

    - RAN4 to not define test cases for DRX.

Agreements:

- No DRX is configured in all TCs for EMR based fast SCell activation.

# Topic #1: Maintenance of core requirements of NR_RRM_Ph5

#### Issue 1-3-1: Detailed UE capability for SCell activation based on EMR

- Prerequisite feature groups. Discuss the options as follow:

  - Option 1: The UE shall support 39-8 or 39-9 in R18 RAN4 feature list or 18-7 (idleInactiveNR-MeasReport-r16) and on top of such features, idleInactiveNR-MeasBeamReport-r16 is also needed.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-4 | Fast SCell activation based on early measurement report | Supports fast SCell activation based on early measurement reports as specified in TS38.133 clause 8.3.2A |  | Yes | N/A | UE cannot meet fast SCell activation requirements as specified in TS38.133 clause 8.3.2A | Per UE | NA | Yes | N/A |  | Optional with capability signaling |

Discussions:

Nokia: Is SSB beam index in reporting supported?

Oppo: beam reporting is not needed for FR1 to support fast SCell activation.

Agreements:

- For prerequisites:

  - The UE shall support 39-8 or 39-9 in R18 RAN4 feature list or 18-7 (idleInactiveNR-MeasReport-r16)

  - Further discuss if idleInactiveNR-MeasBeamReport-r16 is the prerequisites.

- Further discuss the texts in ‘Consequence if the feature is not supported by the UE’.

  - Option 1: NW assumes that the SCell activation timeline follows the behavior defined prior to Release 19.

  - Option 2: UE cannot meet fast SCell activation requirements as specified in TS38.133 clause 8.3.2A

  - Option 3: Fast SCell activation requirements based on early measurement report (as specified in TS 38.133, clause 8.3.2A, 8.3.4, 8.3.12) are not applicable.

# Topic #1: Performance requirements of CSSF solution

#### Issue 1-2-1: Inter-frequency MOs without a measurement gap supported by CSSF solution 3

- Option 1 (Huawei, Apple, QC): Not apply CSSF solution 3 for the case configuring inter-frequency MOs without a measurement gap.

  - Option 1a (MTK): Discuss whether to exclude the 3-searcher enhancement for inter-frequency measurement without MG scenario.

  - Concern: Nokia

- Option 2 (Ericsson, Nokia, ZTE, OPPO): Apply CSSF solution 3 for the case configuring inter-frequency MOs without a measurement gap.

  - Concern: Apple

- Option 3: Not update the core requirements. Meanwhile, not define the test case for: CSSF solution 3 for the case configuring inter-frequency MOs without a measurement gap (OPPO, Nokia, E///, ZTE)

  - Concern:

- Option 4: Not apply CSSF solution 3 for the case configuring inter-frequency MOs without a measurement gap, for some cases. CR: R4-2513478. (HW)

Agreements:

- Apply CSSF solution 3 for the case configuring inter-frequency MOs without a measurement gap as listed in CR: R4-2513478.

#### Issue 1-1-2: Multi-band supported by FBS

- Option 1 (OPPO, Huawei, Ericsson): Do not define the requirements when multiple bands are involved and UE reports different FBS values for different bands. (the serving carrier and target carrier are on the same band)

- Option 2 (CATT, Nokia, Apple): For inter-frequency measurements and when serving carrier and target carrier are in different bands, use the FBS value reported for the band of the target carrier in the requirements.

  - Option 2-1: For the inter-band case, the serving carrier can be a FR1 or FR2-1 carrier.

  - Option 2-2: For the inter-band case, the serving carrier is a FR2-1 carrier. (Nokia, QC - compromise, E///, CATT, ZTE)

    - Not support: OPPO

  - Option 2-3: For the inter-band case, the serving carrier can be a FR1 carrier. (Apple)

- Option 3 (QC): For inter-frequency measurements and when serving carrier and target carrier are in different bands, use maximum value of the FBS values reported for the two bands.

  - Option 3-1: For the inter-band case, the serving carrier can be a FR1 or FR2-1 carrier.

  - Option 3-2: For the inter-band case, the serving carrier is a FR2-1 carrier. (QC, OPPO)

    - Not support: Nokia

Agreements:

- The serving carrier shall be a FR2-1 carrier.

- For inter-frequency measurement, when serving carrier and target carrier are in different bands, use the FBS value reported for the band of the target carrier in the requirements.

#### Issue 2-1-3: Cell configurations in TCs for CSSF solution 3

- Proposal 1 (Huawei): Test set for CSSF solution 3 with the cell configurations including four cells: one PCell, one neighbour Cell of PCC or one inter-RAT E-UTRAN Cell, and two deactivated SCells.

Agreements:

- Continue discussions on the test case when one inter-RAT E-UTRAN Cell configured and consider the impacts to core requirements.

#### Issue 2-1-1: Cell configurations in TCs for CSSF solution 1

- Proposal 1 (Nokia): Test set for CSSF solution 1 with the cell configurations including:

  - At least two carrier frequencies in one FR2 band

  - Event triggered measurement reporting on a neighbour cell on the same frequency with PSCell (in EN-DC) or PCell (in FR2 SA).

Agreements:

- Apply the below test configurations without change:

      - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC

      - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC