# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2514800

# Prague, Czech, October 13th – 17th, 2025

Agenda item:			6.10.1

Source:	Moderator (Ericsson)

Title:	WF on RRM requirements for NR_RRM_Ph5_Part1

Document for:	Information

# Topic #1: Maintenance of core requirements of NR_RRM_Ph5 (4.15)

### Issue 1-1: issues related with FBS

Issue 1-1-1: Condition of re-entering FSB after exiting

<Way Forward>:

- Further discuss in next meeting considering the below options:

  - Option1: Define T2 for re-evaluation after the UE exits FBS mode.

  - Option2: Not to define T2 for re-evaluation after the UE exits FBS mode.

    - Option 2.1: Need to update core requirements.

    - Option 2.2: Not need to update core requirements.

Issue 1-1-2: Multi-band supported by FBS

<Agreement>:

- The serving carrier shall be a FR2-1 carrier.

- For inter-frequency measurement, when serving carrier and target carrier are in different bands, use the FBS value reported for the band of the target carrier in the requirements.

Issue 1-1-3: Measurement gap types supported for FBS

<Agreement>:

- FBS applies for inter-frequency measurement with both per-FR measurement gap and per-UE measurement gap.

Issue 1-1-4: Applicability of FBS when the target and serving carrier frequency are on different

<Way Forward>:

- Further discuss the options:

  - Option 1: When the target and serving carrier frequency are different, FBS applies to target carrier only.

  - Option 2: When the target and serving carrier frequency are different, FBS applies to the serving carrier and target carrier.

  - Option 3:  Introduce UE capability indicating UE can support FBS on both target and serving carrier frequency.

### Issue 1-2: issues related with CSSF enhancement

Issue 1-2-1: Inter-frequency MOs without a measurement gap supported by CSSF solution 3

<Agreement>:

- Apply CSSF solution 3 for the case configuring inter-frequency MOs without a measurement gap, as listed in CR: R4-2513478.

### Issue 1-3: issues related with SCell activation

Issue 1-3-1: Detailed UE capability for SCell activation

<Agreement>:

- UE capability for SCell activation is defined as follow:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-4 | Fast SCell activation based on early measurement report | Supports fast SCell activation based on early measurement reports as specified in TS38.133 clause 8.3.2A | Both 1) and 2): 1) 39-8 or 39-9 in R18 RAN4 feature list or 18-7 (idleInactiveNR-MeasReport-r16) 2) idleInactiveNR-MeasBeamReport-r16 | Yes | N/A | UE may not fulfill fast SCell activation requirements as specified in TS38.133 clause 8.3.2A but fulfills SCell activation requirements defined prior to Release 19. | Per UE | N/A | Yes | N/A |  | Optional with capability signaling |

# Topic #2: RRM performance requirements of FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC (6.10.2.2)

### Issue 2-1: Test case design for FR2-1 L3 measurement delay reduction by optimizing CSSF

Issue 2-1-1: Cell configurations in TCs for CSSF solution 1

<Agreement>:

- Apply the below test configurations without change from the agreed TCs as listed in Annex:

      - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC

      - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC

Issue 2-1-2: Network indication in TCs for CSSF solution 1

<Agreement>:

- Keep the test configurations unchanged as listed in Annex.

Issue 2-1-3: Cell configurations in TCs for CSSF solution 3

<Way Forward>:

- Continue discussions on the test case when one inter-RAT E-UTRAN Cell configured and consider the impact to core requirements.

# Annex

TC list for CSSF optimization:

TC list for CSSF optimization solution 1 - Agreement TC index; Scenario; CA/DC Operation mode; Configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them; - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC; Nokia. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them; - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC; Ericsson. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them; - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC; ZTE. Note: network indication is up to the other RAN2/4 core part discussion. In case network indication for a specific SCC measurement is replaced by other new solution (e.g., SCC selected by UE implementation), use the new solution for CA mode test.

TC list for CSSF optimization solution 3 Agreement on Monday: TC index; Aspects; CA/DC mode; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them.; Without DRX and without SSB index reporting; Ericsson. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them.; Without DRX and without SSB index reporting; OPPO.