3GPP TSG-RAN WG4 Meeting #116-bis                                                          R4-2514816

Prague, CZ, 13th - 17th Oct 2025

Agenda item:			6.17.1

Source:	China Telecom

Title:	WF on RRM requirements for NR_Mob_Ph4_Part2

Document for:	Approval

Notes

In this document,

- < Agreement > represents the decisions made in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other proposals can be proposed.

Topic #1: RRM discussion

CLTM

Issue 1-1: Test case list for CLTM

< Agreement >

- Define the following test cases for FR1:

      - #1: RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

      - #2: RACH-based inter-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

      - #3: RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1

      - #4: RACH-less intra-frequency PCell L3 triggered CLTM cell switch from FR1 to FR1

UE to pass one test case for FR1 based on the supported UE capability. FFS the detailed applicability rule.

- Define the following test cases for FR2:

      - #1: RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2.

      - #2: RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2.

UE to pass one test case for FR2 based on the supported UE capability. FFS the detailed applicability rule.

| Test case | Baseline test case | Test case | Company |
| --- | --- | --- | --- |
| 1 | LTM R18 A.6.3.4.1 | RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1 | China Telecom |
| 2 | LTM R18 A.6.3.4.2 | RACH-based inter-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1 | Huawei |
| 3 | LTM R18 A.6.3.4.3 | RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR1 to FR1 | Nokia |
| 4 | LTM R18 A.6.3.4.4 | RACH-less intra-frequency PCell L3 triggered CLTM cell switch from FR1 to FR1 | Ericsson |
| 5 | LTM R18 A.7.3.4.1 | RACH-based intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2 | ZTE |
| 6 | LTM R18 A.7.3.4.2 | RACH-less intra-frequency PCell L1 triggered CLTM cell switch from FR2 to FR2. | vivo |

Issue 1-2: frequency range

- This issue is merged into issue 1-1 and this is closed.

Issue 1-3: intra/inter-frequency

- This issue is merged into issue 1-1 and this is closed.

Issue 1-4: L1/L3 based CLTM

- This issue is merged into issue 1-1 and this is closed.

Issue 1-5: RACH-less and RACH-based CLTM

- This issue is merged into issue 1-1 and this is closed.

Issue 1-6: PCell/PSCell

- This issue is merged into issue 1-1 and this is closed.

Issue 1-7: early DL sync

< Way forward >

- Option 1: With early DL sync.

- Option 2: With and without early DL sync.

- Option 3: For each FR, one test case with and without early DL sync, with early DL sync only for other test cases.

Issue 1-8: PDCCH ordered RACH

- This issue is merged into issue 1-1 and this is closed.

Subsequent CLTM

Issue 2-1: Test case list for Subsequent CLTM

< Way forward >

- Option 1: Define test cases for subsequent CLTM.

      - For FR1:

      - If UE support 2 or more test cases for FR1, UE to pass one test case for subsequent CLTM which is different from CLTM.

      - If UE support 1 test case for FR1, UE is only required to pass the test case for CLTM, and not required to pass the test case for subsequent CLTM.

      - For FR2: the same proposal as FR1.

- Option 2: Not define test case for subsequent CLTM as CLTM.

Issue 2-2: Others

- This issue is merged into issue 2-1 and this is closed.