3GPP TSG-RAN WG4 Meeting # 116bis											R4-2514818

Prague, Czech Republic, October 13th – October 17th, 2025

Agenda item:			6.18.1

Source:	Moderator (Nokia)

Title:	WF on NR_XR_Ph3_RRM

Document for:	Approval

# Topic #1: RRM performance requirements

## Sub-topic 1-1 – Test case list

### Issue 1-1-1: Scenarios for the definition of the test cases

<Agreement>

- At least for measurement procedure RAN4 RRM to define test cases for NR-SA for both FR1 and FR2-1

  - For FR1 TCs,

    - all cells are in FR1

  - For FR2 TCs

    - all cells in FR2-1

    - FFS if all TCs also cover FR2-1

- Test cases are specified only for per-UE MGs

### Issue 1-1-2: Type 1 and Type 2 gaps

<Agreement>

- Test cases are defined only for Type 1 gaps

### Issue 1-1-3: DRX proposals

<Agreement>

- Each type of test case is specified for either DRX or non-DRX but not both.

### Issue 1-1-4: Test target

<Agreement>

- RRM test cases should verify in the same test case

  - Measurement delay extension with gap cancellation

  - Reception/transmission without scheduling restrictions during cancelled gap occasions

<Way forward>

Further discuss on the following proposals

- Proposal 1: Further discuss on the UE processing timeline of receiving the DCI-based MG skipping indication

- Proposal 2: Accuracy requirements

- Proposal 3: Cancellation of scheduling restrictions due to L3 and L1 measurements without gaps during cancelled gaps

### Issue 1-1-5: List of test cases

<Agreement>

- Define test case for:

  - Inter-frequency measurements procedure with gaps

  - Define test case for: Inter-frequency L1-RSRP measurements procedure with MG for FR1 and FR2-1

  - FFS whether to define at least one accuracy test case in FR1 and in FR2-1

### Issue 1-1-6: List of CRs

<Agreement>

| Draft CR | Volunteering company |
| --- | --- |
| FFS: A.3.x Model for measurement gap cancellation | Nokia |
| A.6.6.2.x SA event triggered reporting with measurement gap cancellation for FR1under non-DRX | Ericsson |
| A.7.6.2.x SA event triggered reporting with measurement gap cancellation for FR2-1 under DRX | ZTE |
| A.6.6.x LTM Inter-frequency L1-RSRP measurement with measurement gap cancellation in FR1 | vivo |
| A.6.6.x LTM Inter-frequency L1-RSRP measurement with measurement gap cancellation in FR2-1 | Nokia |

## Sub-topic 1-2 – Test case configuration

### Issue 1-2-1: SSB time index detection

<Way forward>

Proposals

- Proposal 1: For measurement procedure, define test cases without SSB time index detection.

- Proposal 2: Use SSB index detection for intra-/inter-frequency measurement TCs, use no SSB index detection with lower priority for intra-/inter-frequency measurement TCs.

## Sub-topic 1-3 – MG cancellation model

### Issue 1-3-1: Cancelation model for XR test cases: conditions for Type 1 gaps and Type 2 gaps

<Way forward>

Proposals for discussion

- Option 1: Measurement gap cancelation is modelled as a fixed pattern for the test cases.

- Option 2: Measurement gap cancelation is determined randomly by the test equipment.

### Issue 2-3-3: Cancelation ratio

<Way forward>

Proposals for discussion

- Proposal 1: Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests.

- Proposal 2: For Type 1 MGs test cases:

  - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset

  - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset

## Sub-topic 1-4 – DCI aspects

### Issue 1-4-1: Toffset between DCI and start of MG

<Way forward>

Proposals for discussion

- Option 1: The processing time 3ms/5ms shall be verified in the test cases.

  - Option 1a: Define a single test case that includes time offset of 5ms and 3ms with UE capability used as a condition

- Option 2: Define test case for 5ms offset only.

### Issue 1-4-2: Types of DCI with cancellation indication

<Way forward>

Proposals for discussion

- Option 1: In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancellation.

- Option 2: Define RAN4 test cases using DCI 1-1 for the canceling indication.

### Issue 1-4-3: Behaviour of Option 1 and Option 2 for indication of value ‘0’

<Way forward>

Proposals for discussion

- Proposal 1: RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested.

- Proposal 2: RAN4 to agree that UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Further details are as follows:

  - Test Option 2 in every test case.

  - For testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability.