3GPP TSG-RAN WG4 Meeting #116bis	R4-2515053

Prague, Czech Republic, October 13 ‒ October 17, 2025

Title: 	Way Forward for [116bis][328] Rel-19 Demodulation_Part3

Agenda Item:	6.23

Source: 	Ericsson

Document for:	Approval

# NTN testing for NGSO

Channel models

Agreement: Eccentricity of satellite orbit

- No need to capture “The method specified in this sub clause is applicable to cases that Eccentricity (e) in Step 1-4 is more than zero” in the specification, because e≠0 for the initial ephemeris information developed in Rel-19 RAN4.

Agreement: Additional margin to IoT-NTN UE demodulation requirements when the time-varying Doppler shift and propagation delay model is applied

- For NB-IoT, add 0.5dB to the existing NPDSCH requirements.

- For eMTC, add 1.0dB to the existing PDSCH requirements.

Agreement: Impact of sampling frequency offset

- RAN4 does not need to consider the impact of sampling frequency offset for UE demodulation requirements.

Way forward: How to ensure the satellite elevation angles more than 30 degrees during the test

Background: RAN5 conformation test procedure specifies to run multiple subtest to measure the average throughput for a duration sufficient to achieve statistical significance (See TS 38.521-5 8.2.1.2.2.1.1_1.3.2).

- Interested companies are encouraged to study how TE ensures to apply the same Doppler shift and propagation delay model for each subtest. FFS how to capture in the specification.

  - Option 1: For each subtest, TE should start from the same initial ephemeris information (i.e., positionX, positionY, positionZ, velocityVX, velocityVY, velocityVZ) = (-2654249, 4386991, 1594205, 14581, -34487, 120182).

Way forward: Applicability of Rel-19 tests with time-varying Doppler shift and propagation delay model

Background In Rel-17 RAN4 defined four PDSCH demodulation requirements for NR NTN (1-1, 1-2, 1-3, and 1-4). In this WI, RAN4 has agreed to add 2 new test cases (1-5 and 1-6) by applying the time-varying Doppler shift and propagation delay model for NGSO to tests 1-1 and 1-2, as follows. Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 0.3. 1-2; R.PDSCH.1-2.1 FDD; 10 / 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; 7.6. 1-3; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLC5-200; 1x2, ULA Low; 70; -0.4. 1-4; R.PDSCH.1-1.1 FDD(1); 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; 1.1. 1-5; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; [0.3+0.5]. 1-6; R.PDSCH.1-2.1 FDD; 10 / 15; 16QAM, 0.48; NTN-TDLC5-200; 1x2, ULA Low; 70; [7.6+0.5]. Note 1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. Note 2: For Tests 1-5 and 1-6, the time-varying Doppler shift and propagation delay model, specified in Annex E, is applied..

For UE declaring Rel-19 UE,

- Option 1

  - Applicable tests are 1-5, 1-6, 1-3 and 1-4.

- Option 2

  - Applicable tests are 1-5, 1-6.

For UE declaring Rel-17/18 UE,

- Option 1a:

  - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6, 1-3 and 1-4.

  - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4.

- Option 1b:

  - If UE declares to meet the requirements with time-varying Doppler shift and propagation delay, applicable tests are 1-5, 1-6.

  - Otherwise, applicable tests are 1-1, 1-2, 1-3 and 1-4.

- Option 2:

  - Applicable tests are 1-1, 1-2, 1-3 and 1-4.

Note the same test applicability rule is used for IoT-NTN UE demodulation requirements.