3GPP TSG-RAN WG4 Meeting #117	R4-2520020

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.1.2

Source:	MediaTek Inc.

Title:	Simulation results on 6Rx UE demodulation performance requirements

Document for:	Information

# Introduction

In this contribution, we provide simulation results for 6Rx PDSCH, SDR and CQI requirements.

- Simulation results for PDSCH demodulation performance

Based on assumptions in [1], we provide simulation results for 4T6R4L and 8T6R6L PDSCH requirements.

4 layers results

The simulation results for 4 layers case are provided in Figure 1 and SNR values achieving 70% of maximum throughput are summarized in Table 1.

Figure 1. Simulation results for 4T6R4L with MCS13 (Table 1) and MIMO correlation, α = 0 and β = 0.07179.

Table 1. Summary for the required SNR achieving 70% maximum throughput (4T6R4L)

| Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Rank 4 | 40 / 30 | MCS13 (MCS Table 1) | TDLA30-10 | 4x6 ULA with α = 0 and β = 0.07179 | 70 | 16.32 |

6 layers results

The simulation results for 6 layers case are provided in Figure 2 and SNR values achieving 70% of maximum throughput are summarized in Table 2.

Figure 2. Simulation results for 8T6R6L with MCS17 (Table 1) and zero MIMO correlation, α = 0 and β = 0.

Table 2. Summary for the required SNR achieving 70% maximum throughput (8T6R6L)

| Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Rank 6 | 40 / 30 | MCS17 (MCS Table 1) | TDLA30-10 | 8x6 ULA with α = 0 and β = 0 | 70 | 18.83 |

# Simulation results for SDR requirements

We provide simulation results based on assumptions in [1]. It is noted that there is no user data scheduled on slot #0, 20 and 21 within 20 ms for SCS 30 kHz and no user data scheduled on special slots according to Clause 5.5A.1 in TS38.101-4. The SNR values achieving 85% of maximum throughput are summarized in Table 4.

Table 3. Summary for the required SNR achieving 85% maximum throughput

| MCS | SNR(dB)@85% of maxTP |  |  |
| --- | --- | --- | --- |
|  | MCS Table 1 8T6R6L Fixed precoder with i1,1=i1,2=i2=0 | MCS Table 2 8T6R6L Fixed precoder with i1,1=i1,2=i2=0 | MCS Table 4 2T6R2L Random precoder |
| 28 | 21.2 |  |  |
| 27 | 19.2 | 26.2 |  |
| 26 | 18.1 | 25.1 |  |
| 25 | 17.1 | 24.1 | 31.6 |
| 24 | 16.1 | 23.1 | 27.1 |
| 23 | 15.0 | 22.0 | 24.8 |
| 22 |  | 20.3 |  |

- Simulation results for CQI requirements

In table 4, we provide simulation results for 6Rx CQI requirements under static channel based on assumptions in [1].

Table 4. Simulation results for 6Rx CQI requirements under static channel

| SNR [dB] | Median CQI | Percentage of {median CQI-1, median CQI, median CQI+1} | BLER at Median CQI -1 for | BLER at median CQI | BLER at Median CQI +1 |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 100% | 0 | 0 | 1 |
| 2 | 3 | 100% | 0 | 0 | 1 |
| 3 | 4 | 100% | 0 | 1 | 1 |
| 4 | 4 | 100% | 0 | 0.48 | 1 |
| 5 | 5 | 100% | 0 | 1 | 1 |
| 6 | 5 | 100% | 0 | 0.42 | 1 |
| 7 | 6 | 100% | 0 | 1 | 1 |
| 8 | 6 | 100% | 0 | 0.43 | 1 |
| 9 | 7 | 100% | 0 | 1 | 1 |
| 10 | 7 | 100% | 0 | 0.31 | 1 |
| 11 | 8 | 100% | 0 | 1 | 1 |
| 12 | 8 | 100% | 0 | 0.15 | 1 |
| 13 | 9 | 100% | 0 | 1 | 1 |

# Reference

- R4-2514788, Way Forward for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon