3GPP TSG-RAN WG4 Meeting #117		   				 		  							R4-2510039

Dallas, USA, 17th – 21th Nov., 2025

Agenda Item:	7.6.4.2

Source:	MediaTek Inc.

Title:	Discussion on RRM performance requirements for NR NTN Ku bands

Document for:	Discussion

# Introduction

Our view on outstanding issues in WF [2] for HD-FDD Phase 1 (Ku bands with FR1 numerology) as in WID [1].

# RRM performance requirements for NR NTN Ku bands

#### Issue 2-1-1: RRM requirements for HD-FDD with FR1 numerology

Agreement

- Define the HD-FDD VSAT with FR1 numerology RRM requirements based on the Rel-19 Ku-band NTN FDD requirements, and incorporate updates to address HD-FDD impacts, using the RedCap NTN approach as the baseline.

- The following RRM requirements scope for HD-FDD VSAT in Ku band with FR1 numerology

  - Cell reselection, including paging reception

  - Handover and CHO

  - Satellite switching

  - Random access

  - UE UL timing accuracy

  - RLM, including scheduling availability and L1 indication

  - intra/inter-frequency measurements, including scheduling availability

  - L1-RSRP measurement, including scheduling availability

The VSAT RRM requirements to be updated to include HD-FDD with FR1 numerology are agreed in WF [2] as below.

Table 1 summarizes specification impacts based on NTN RedCap experience. In general, the same approach is applicable.

Table 1 RRM impact of HD-FDD VSAT

| Requirements | VSAT requirement in 38.133 | HD-FDD VSAT impact |
| --- | --- | --- |
| Cell reselection | 4.2C Cell Re-selection for NR UE for Satellite Access | Need clarification such as in 4.2E.2.5 Maximum interruption in paging reception - at least 1 SSB is available at the UE in the serving cell during the last 160 ms duration. |
| Handover | 6.1C.1.3 NR SAN FR2-NTN – NR SAN FR2-NTN Handover 6.1C.2.4 NR SAN FR2-NTN – NR SAN FR2-NTN conditional handover | No impact - as no need UL during HO interruption |
| Satellite switching | 6.1C.3.3 NR SAN Satellite switching with re-synchronization for FR2-NTN | Need clarification |
| RRC Re-establishment | 6.2C.1.2.2 UE Re-establishment delay requirement for VSAT | Need clarification |
| UE UL timing accuracy | 7.1C UE transmit timing for Satellite Access | No impact. - As already condition of available SSB during 160 ms |
| RLM | 8.1C Radio Link Monitoring for Satellite Access | Need clarification such as in 8.1E.3 Requirements for CSI-RS based radio link monitoring - at least one RLM-RS sample must fall with DL occasion within an indication period. |
| inter-/intra-frequency | 9.2C NR intra-frequency measurements for SAN 9.3C NR inter-frequency measurements for SAN | Need clarification |
| L1-RSRP | 9.5C L1-RSRP measurements for Reporting for satellite access | Need clarification |

The RRM requirements for HD-FDD Phase 1 (Ku bands with FR1 numerology) appear sufficiently mature to prepare CR. Thus, we propose to have core part CR split as the follows.

Proposal 1: CR work split for RRM core requirement for HD-FDD Phase 1 (Ku bands with FR1 numerology):

| VSAT requirement in 38.133 | Volunteer |
| --- | --- |
| Cell reselection, including paging reception |  |
| Handover and CHO |  |
| Satellite switching |  |
| Random access |  |
| UE UL timing accuracy |  |
| RLM, including scheduling availability and L1 indication |  |
| intra/inter-frequency measurements, including scheduling availability |  |
| L1-RSRP measurement, including scheduling availability |  |

# Summary

Our observations and proposals in this paper are summarized as below:

Proposal 1: CR work split for RRM core requirement for HD-FDD Phase 1 (Ku bands with FR1 numerology):

| VSAT requirement in 38.133 | Volunteer |
| --- | --- |
| Cell reselection, including paging reception |  |
| Handover and CHO |  |
| Satellite switching |  |
| Random access |  |
| UE UL timing accuracy |  |
| RLM, including scheduling availability and L1 indication |  |
| intra/inter-frequency measurements, including scheduling availability |  |
| L1-RSRP measurement, including scheduling availability |  |

# References

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated), RAN#109

- R4-2515082, Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2, MediaTek, Oct. 2025.