3GPP TSG-RAN WG4 Meeting #117                        R4-2520050                                                                                                       Dallas, Texas, USA, 17 – 21 November 2025

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for 6Rx

Agenda Item:	6.1.2

Document for:	Discussion

Introduction

The RAN4#114bis is the first meeting we discuss the UE demodulation performance requirements for 6Rx. According to the WID, RAN4 will define 6-layer performance requirements for FWA and 4-layer performance requirements for FWA and handheld UE devices. During RAN4#116bis, the Way Forward is approved in [1]

In this contribution, our views on the remaining open issues are given.

Discussion

Issue 1-2: Test applicability rule for 6Rx 6 layers performance requirements

Agreement: - Agree to define the applicability rules that 6 MIMO layers performance requirements are applicable for UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6. Proposals: - Discuss whether to explicitly capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4

As discussed in the last meeting, there is clear conclusion in the previous meeting: it will be captured in 38.101-4 that ‘6 layers requirements are only applicable for FWA UE’.

Therefore, we are fine to include such information in the specification, although it gives limited information for UE conformance testing which is based on UE declaration.

We propose to only capture such information in the test applicability part to minimize the spec impact.

Combining with the test applicability rule agreed in the last meeting, we propose the following test applicability rules for 6Rx requirements:

Proposal 1: Capture the information ‘6 layers requirements are only applicable for FWA UE’ only in the test applicability part as below to minimize the spec impact.

Proposal 2: Define the following applicability rules for 6Rx requirements.

Table 5.1.1.3-1: Requirements applicability for optional UE features UE feature/capability [14]; Test type; Test list; Applicability notes. Supported maximum number of 6 PDSCH MIMO layers for FWA UE (maxNumberMIMO-LayersPDSCH-v1900); FR1 TDD; PDSCH; Clause xxx (new 6 Layer requirements). Table 5.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling UE feature/capability [14]; Test type; Test list; Applicability notes. Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH); FR1 FDD; PDSCH; Clause 5.2.2.1.1 (Tests 2-1, 2-2, 2-3, 3-1) Clause 5.2.2.1.2 Clause 5.2.3.1.1 (Tests 2-1, 2-2, 3-1, 4-1, 5-1, 4-2) Clause 5.2.3.1.2 Clause 5.2.4.1.1 (Tests 1-1, 2-1, 3-1, 4-1, 5-1); The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed UE PDSCH MIMO layers capability. FR1 TDD; PDSCH; Clause 5.2.2.2.1 (Tests 2-1, 2-2, 2-3, 3-1) Clause 5.2.2.2.2 Clause 5.2.3.2.1 (Tests 2-1, 2-2, 3-1, 4-1, 5-1, 4-2) Clause 5.2.3.2.2 Clause 5.2.4.2.1 (Tests 1-1, 2-1, 3-1, 4-1, 5-1) Clause xxx (new 4 Layer requirements).

Test requirement values for CQI test for 6Rx

According to [1], the SNR values for 6Rx CQI test under AWGN is still in ‘TBD’.

Currently, SNR requirement values of 8/9/14/15dB are defined for 2Rx, and 5/6/11/12dB are defined for 4Rx considering a clear 3dB performance improvement under AWGN.

For 8Rx CQI requirements, SNR values of 4/5/10/11dB are defined.

Compared with 4Rx requirements, we think lower SNR values should be used considering UE supports more antenna ports while it may be not reasonable to have even lower SNR values than 8Rx. Therefore we propose to use SNR values of 4/5/10/11dB for 6Rx requirements.

Observation 1: SNR requirement values of 8/9/14/15dB are defined for 2Rx, and 5/6/11/12dB are defined for 4Rx, 4/5/10/11dB are defined for 8Rx.

Proposal 2: Use SNR values of 4/5/10/11dB for 6Rx CQI reporting requirements under AWGN.

Conclusion

Proposal 1: Capture the information ‘6 layers requirements are only applicable for FWA UE’ only in the test applicability part as below to minimize the spec impact.

Proposal 2: Define the following applicability rules for 6Rx requirements.

Table 5.1.1.3-1: Requirements applicability for optional UE features UE feature/capability [14]; Test type; Test list; Applicability notes. Supported maximum number of 6 PDSCH MIMO layers for FWA UE (maxNumberMIMO-LayersPDSCH-v1900); FR1 TDD; PDSCH; Clause xxx (new 6 Layer requirements). Table 5.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling UE feature/capability [14]; Test type; Test list; Applicability notes. Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH); FR1 FDD; PDSCH; Clause 5.2.2.1.1 (Tests 2-1, 2-2, 2-3, 3-1) Clause 5.2.2.1.2 Clause 5.2.3.1.1 (Tests 2-1, 2-2, 3-1, 4-1, 5-1, 4-2) Clause 5.2.3.1.2 Clause 5.2.4.1.1 (Tests 1-1, 2-1, 3-1, 4-1, 5-1); The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed UE PDSCH MIMO layers capability. FR1 TDD; PDSCH; Clause 5.2.2.2.1 (Tests 2-1, 2-2, 2-3, 3-1) Clause 5.2.2.2.2 Clause 5.2.3.2.1 (Tests 2-1, 2-2, 3-1, 4-1, 5-1, 4-2) Clause 5.2.3.2.2 Clause 5.2.4.2.1 (Tests 1-1, 2-1, 3-1, 4-1, 5-1) Clause xxx (new 4 Layer requirements).

Observation 1: SNR requirement values of 8/9/14/15dB are defined for 2Rx, and 5/6/11/12dB are defined for 4Rx, 4/5/10/11dB are defined for 8Rx.

Proposal 2: Use SNR values of 4/5/10/11dB for 6Rx CQI reporting requirements under AWGN.

Reference

R4-2514788, Way Forward for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx, 3GPP TSG RAN4 Meeting #116bis