3GPP TSG-RAN WG4 Meeting #117                                                                                                      R4-2520061

Dallas, USA, 17th –21st November, 2025

Source: 	CATT

Title: 	Further discussion on SAN demodulation performance requirements for NR NTN Ku-band

Agenda item:	5.10.3

Document for:	Approval

- Introduction

In RAN4#116bis meeting, A Way Forward for [116bis][312] Rel-19 Demodulation_Part2 was agreed [1]. It included the agreement for Ku-band SAN demodulation requirement. But there is also some issues such as AWGN level for Ku-band SAN demodulation needs to be discussed.

In this contribution, we provide our views on SAN demodulation performance requirements for Ku-band for NR NTN.

- Discussion

SAN demodulation requirement for Ku-band

As per the WF [1], the agreement concerning Ku-band SAN demodulation is shown as below:

## Sub-Topic 1-1: Ku Band SAN Demodulation #### Whether to define requirements Agreement Do not define SAN demodulation requirements

For clarification, the word “new” is missing in above agreement. The correct agreement is “do not define new SAN demodulation requirements for Ku band SAN demodulation”, since the SAN demodulation for Ku-band is necessary.

Proposal 1: For clarification, do not define new SAN demodulation requirement, reuse existing SAN demodulation requirements for SAN type 1-O for FR1-NTN Ku-band, and reuse existing SAN demodulation requirements for SAN type 2-O for FR2-NTN Ku-band.

AWGN power level at the SAN input

The AWGN power level at the SAN input for PUSCH demodulation requirement in Table 11.2.1.4.2-2 of TS 38.181 is copied as below:

Table 11.2.1.4.2-2: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| 1-O (Note 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1 NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

The AWGN power level at the SAN input for PUCCH demodulation requirement in Table 11.3.1.4.2-2 of TS 38.181 is copied as below:

Table 11.3.1.4.2-2: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| SAN type 1-O (Note 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| SAN type 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

The AWGN power level at the SAN input for PRACH demodulation requirement in Table 11.4.1.4.2-1 of TS 38.181 is copied as below:

Table 11.4.1.4.2-1: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| SAN type 1-O (Note 2) | 15 | 5 | -83.5 - ΔOTAREFSENS dBm / 4.5MHz |
|  | 30 | 10 | -80.6 - ΔOTAREFSENS dBm / 8.64MHz |
| SAN type 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NNOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS receiver target reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

From above Table 11.2.1.4.2-2, Table 11.3.1.4.2-2 and Table 11.4.1.4.2-1, AWGN power level at the SAN input for SAN type 1-O is defined based on ΔOTAREFSENS, which is not used for FR1-NTN Ku-band OTA reference sensitivity level. The FR1-NTN Ku-band OTA reference sensitivity level(for SAN type 1-O operating above 10GHz) is defined based on EISREFSENS_50M and ΔFR1_REFSENS like that for SAN type 2-O. So, the existing AWGN power level at the SAN input for SAN type 1-O is not applicable for FR1-NTN Ku-band.

Observation 1: The existing AWGN power level at the SAN input for SAN type 1-O is not applicable for FR1-NTN Ku-band.

Comparing the AWGN power level for PUSCH, PUCCH and PRACH in above three tables, we also can find the methods for defining of AWGN power level for SAN type 1-O and SAN type 2-O are different. The AWGN power level for SAN type 1-O is defined based on noise power level plus 16dB offset, while the AWGN power level for SAN type 2-O is defined based on reference sensitivity level + 15dB offset. Generally, reference sensitivity level is equal to noise power level + IM +SNR, since we typically use IM=2 dB and SNR=-1 dB for calculation of reference sensitivity level, so reference sensitivity level = noise power level +1.

For example, In Table 11.2.1.4.2-2, the AWGN power level for SAN type 1-O for 15kHz SCS/5MHz CBW is -86.5 - ΔOTAREFSENS dBm / 4.5 MHz, where -86.5 is equal to -174+10*log10(4.5*1e6)+NF+16 = -86.4679, where NF=5dB (Note, for NTN, the NF use TN NF 5dB instead of NTN NF 7.4dB for GEO in TR 38.863). AWGN power level for TN is directly used for that for NTN. So -86.5 is defined based on noise power level plus 16dB offset, and noise power level is  -174+10*log10(4.5*1e6)+NF = -102.4679.

In Table 11.2.1.4.2-2, for the AWGN power level for SAN type 2-O is EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz,  where EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm is  defined based on EISREFSENS_50M + ΔFR2_REFSENS plus 15dB offset.

So we can draw the following conclusion:

For definition of conducted AWGN power level at SAN input, if noise power level is used, the 16dB offset is needed, and if reference sensitivity level is used, the 15dB offset is needed. Conducted AWGN power level = noise power level +16dB = reference sensitivity level +15dB.

Observation 2: For definition of conducted AWGN power level at SAN input, if noise power level is used, the 16dB offset is needed, and if reference sensitivity level is used, the 15dB offset is needed. Conducted AWGN power level = noise power level +16dB = reference sensitivity level +15dB.

For “-O” (1-O, or 2-O) SAN type, the antenna gain is needed to be included in AWGN power level, So methods for defining of AWGN power level for SAN type 1-O and SAN type 2-O are equivalent.

Observation 3: Methods for defining of AWGN power level for SAN type 1-O and SAN type 2-O are equivalent.

Since method for OTA reference sensitivity level for FR1-NTN Ku-band(SAN type 1-O operating above 10GHz) is the same as that for SAN type 2-O, both are defined based on  EISREFSENS_50M, so the method for AWGN power level for SAN type 2-O can be reused for FR1-NTN Ku-band(SAN type 1-O operating above 10GHz).

From above Table 11.2.1.4.2-2, the AGWN power level for SAN type 2-O for 120kHz SCS/50MHz CBW is EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz as below:

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |

Since the EISREFSENS_50M is defined based on 47.52MHz(60kHz/66RBs(50MHz)). So OTA reference sensitivity level for 46.08 MHz is equal to EISREFSENS_50M + ΔFR2_REFSENS + 10*log10(46.08/47.52) = EISREFSENS_50M + ΔFR2_REFSENS -0.1336, so AWGN power level for 46.08 MHz is EISREFSENS_50M + ΔFR2_REFSENS -0.1336 +15dBm, 15dB offset is needed. -0.1336dB can be ignored. So, the AWGN power level for 46.08 MHz is EISREFSENS_50M + ΔFR2_REFSENS +15dBm / 46.08 MHz.

For FR1-NTN Ku-band(SAN type 1-O operating above 10GHz), AWGN power level can be defined as below:

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| 1-O operating above 10GHz | 15 | 5 | EISREFSENS_50M + ΔFR1_REFSENS +10*log10(4.5 MHz/47.52MHz)+ 15 dBm / 4.5 MHz |
|  | 30 | 10 | EISREFSENS_50M + ΔFR1_REFSENS +10*log10(8.64MHz/47.52MHz)+ 15 dBm / 8.64 MHz |

Furthe calculation:

The 10*log10(4.5 MHz/47.52MHz) = -10.2366 ≈-10 (ignoring the decimal part)

The 10*log10(8.64MHz/47.52MHz)= -7.4036  ≈ -7(ignoring the decimal part)

So, using above calculation resut, AWGN power level for SAN type 1-O operating above 10GHz can be defined as below:

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| 1-O operating above 10GHz | 15 | 5 | EISREFSENS_50M + ΔFR1_REFSENS + 5 dBm / 4.5 MHz |
|  | 30 | 10 | EISREFSENS_50M + ΔFR1_REFSENS + 8 dBm / 8.64 MHz |

Comparing the AWGN power level for PUSCH, PUCCH and PRACH in above three tables, we can find that for SAN  type 1-O, AWGN power level for PUSCH, PUCCH are same, and they is 3dB less than AWGN power level for PRACH. While for SAN type 2-O, AWGN power level for PUSCH, PUCCH and PRACH are same. This difference indicates that for PRACH, for 15kHz SCS/5MHz CBW, and 30kHz SCS/10MHz CBW, the 3dB improvement for AWGN power level for PRACH is needed.

So, for PRACH for SAN type 1-O operating above 10GHz, AWGN power level can be defined as below:

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| 1-O operating above 10GHz | 15 | 5 | EISREFSENS_50M + ΔFR1_REFSENS + 8 dBm / 4.5 MHz |
|  | 30 | 10 | EISREFSENS_50M + ΔFR1_REFSENS + 11dBm / 8.64 MHz |

So, based on above discussion, we propose,

Proposal 2: Add AWGN power level at the SAN input for PUSCH demodulation requirement for SAN type 1-O operating above 10GHz in Table 11.2.1.4.2-2 of TS 38.181d as below:

Table 11.2.1.4.2-2: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| 1-O operating below 10GHz (Note 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| 1-O operating above 10GHz (Note 5) | 15 | 5 | EISREFSENS_50M + ΔFR1_REFSENS + 5 dBm / 4.5 MHz |
|  | 30 | 10 | EISREFSENS_50M + ΔFR1_REFSENS + 8 dBm / 8.64 MHz |
| 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1 NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 6: ΔFR1_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. |  |  |  |

Proposal 3: Add the AWGN power level at the SAN input for PUCCH demodulation requirement for SAN type 1-O operating above 10GHz in Table 11.3.1.4.2-2 of TS 38.181 as below:

Table 11.3.1.4.2-2: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| SAN type 1-O operating below 10GHz (Note 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| SAN type 1-O operating above 10GHz (Note 5) | 15 | 5 | EISREFSENS_50M + ΔFR1_REFSENS + 5 dBm / 4.5 MHz |
|  | 30 | 10 | EISREFSENS_50M + ΔFR1_REFSENS + 8 dBm / 8.64 MHz |
| SAN type 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 6: ΔFR1_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. |  |  |  |

Proposal 4: Add the AWGN power level at the SAN input for PRACH demodulation requirement for SAN type 1-O operating above 10GHz in Table 11.4.1.4.2-1 of TS 38.181 as below:

Table 11.4.1.4.2-1: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| SAN type 1-O operating below 10GHz (Note 2) | 15 | 5 | -83.5 - ΔOTAREFSENS dBm / 4.5MHz |
|  | 30 | 10 | -80.6 - ΔOTAREFSENS dBm / 8.64MHz |
| SAN 1-O operating above 10GHz (Note 5) | 15 | 5 | EISREFSENS_50M + ΔFR1_REFSENS + 8 dBm / 4.5 MHz |
|  | 30 | 10 | EISREFSENS_50M + ΔFR1_REFSENS + 11dBm / 8.64 MHz |
| SAN type 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NNOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS receiver target reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 6: ΔFR1_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. |  |  |  |

- Conclusion

This contribution provides analysis on SAN demodulation for NR NTN Ku-band, and the following proposal is concluded as follows:

Proposal 1: For clarification, do not define new SAN demodulation requirement, reuse existing SAN demodulation requirements for SAN type 1-O for FR1-NTN Ku-band, and reuse existing SAN demodulation requirements for SAN type 2-O for FR2-NTN Ku-band.

Observation 1: The existing AWGN power level at the SAN input for SAN type 1-O is not applicable for FR1-NTN Ku-band.

Observation 2: For definition of conducted AWGN power level at SAN input, if noise power level is used, the 16dB offset is needed, and if reference sensitivity level is used, the 15dB offset is needed. Conducted AWGN power level = noise power level +16dB = reference sensitivity level +15dB.

Observation 3: Methods for defining of AWGN power level for SAN type 1-O and SAN type 2-O are equivalent.

Proposal 2: Add AWGN power level at the SAN input for PUSCH demodulation requirement for SAN type 1-O operating above 10GHz in Table 11.2.1.4.2-2 of TS 38.181d as below:

Table 11.2.1.4.2-2: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| 1-O operating below 10GHz (Note 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| 1-O operating above 10GHz (Note 5) | 15 | 5 | EISREFSENS_50M + ΔFR1_REFSENS + 5 dBm / 4.5 MHz |
|  | 30 | 10 | EISREFSENS_50M + ΔFR1_REFSENS + 8 dBm / 8.64 MHz |
| 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1 NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 6: ΔFR1_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. |  |  |  |

Proposal 3: Add the AWGN power level at the SAN input for PUCCH demodulation requirement for SAN type 1-O operating above 10GHz in Table 11.3.1.4.2-2 of TS 38.181 as below:

Table 11.3.1.4.2-2: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| SAN type 1-O operating below 10GHz (Note 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| SAN type 1-O operating above 10GHz (Note 5) | 15 | 5 | EISREFSENS_50M + ΔFR1_REFSENS + 5 dBm / 4.5 MHz |
|  | 30 | 10 | EISREFSENS_50M + ΔFR1_REFSENS + 8 dBm / 8.64 MHz |
| SAN type 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 6: ΔFR1_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. |  |  |  |

Proposal 4: Add the AWGN power level at the SAN input for PRACH demodulation requirement for SAN type 1-O operating above 10GHz in Table 11.4.1.4.2-1 of TS 38.181 as below:

Table 11.4.1.4.2-1: AWGN power level at the SAN input

| SAN type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| SAN type 1-O operating below 10GHz (Note 2) | 15 | 5 | -83.5 - ΔOTAREFSENS dBm / 4.5MHz |
|  | 30 | 10 | -80.6 - ΔOTAREFSENS dBm / 8.64MHz |
| SAN 1-O operating above 10GHz (Note 5) | 15 | 5 | EISREFSENS_50M + ΔFR1_REFSENS + 8 dBm / 4.5 MHz |
|  | 30 | 10 | EISREFSENS_50M + ΔFR1_REFSENS + 11dBm / 8.64 MHz |
| SAN type 2-O (Note 5) | 120 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NNOTE 3: ΔFR2_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS receiver target reference direction (as declared in D.43 in table 4.6-1) is used for testing. NOTE 4: EISREFSENS_50M as declared in D.53 in table 4.6-1. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 6: ΔFR1_REFSENS = -3 dB as described in clause 10.1, since the OTA REFSENS reference direction (as declared in D.43 in table 4.6-1) is used for testing. |  |  |  |

- References

- R4-2515160, Way Forward for [116bis][312] Rel-19 Demodulation_Part2, Moderator (Nokia), RAN4#116bis

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#107