3GPP TSG RAN WG4 Meeting #117		R4-2520091

Dallas, Texas, United States, Nov 17th – 21st , 2025

Title: 	Discussion on 3MHz (e)RedCap UE

Agenda Item:	7.4.3.1

Source: 	CATT

Document for:	Approval

1 Introduction

In the RAN#109 meeting, a new WID [1] for 3MHz (e)RedCap UE was approved as one of R20 RAN4 5G-A package. The objectives are listed below.

The core part of WI: - Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD

In the RAN4#116bis meeting, RAN4 made a good progress with most of agreements on the identified issues in the approved WF [2]. There are some open issues listed below. And we will share our views on them in this paper.

Issue 3-5: REFSENS RB allocation configuration for eRedcap FFS how to capture RB allocation configuration in spec for eRedcap Issue 4-1: Whether 3MHz UE CBW should be optional or mandatory in Rel-20 for (e)Redcap FFS whether 3MHz should be optional or mandatory in Rel-20 for (e)Redcap Issue 4-2: Release independent of 3MHz UE CBW for (e)Redcap It is agreed 3MHz UE CBW is release independent from Rel-18 for (e)RedCap UE. Issue 4-3: UE capability FFS in next meeting what UE capabilities of (e)Redcap need to be updated to support 3MHz and sent LS to RAN2.

2 Discussion

- UE capability:

After checking the latest TS 38.306 v19.0.0, the following UE capabilities should be updated.

The restriction about “This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18” for the following Rel-18 WI ‘less than 5MHz’ can be removed from R20.

support3MHz-ChannelBW-Symmetric-r18

support3MHz-ChannelBW-Asymmetric-r18

support12PRB-CORESET0-r18

support12PRB-CORESET0-GSCN-41637-r18

Proposal 1: RAN4 send LS to RAN2 to remove the restriction about “This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18” for the following UE capabilities from R20.

- Optional or mandatory in Rel-20 for (e)Redcap supporting 3MHz

As the following UE capabilities are optional for normal UE, if these UE capacities will be reused for R20 (e)Redcap supporting 3MHz CBW, it’s very hard to mandatorily force R20 (e)Redcap to support 3MHz CBW.

Observation 1: As the following UE capabilities are optional for normal UE, if these UE capacities will be reused for R20 (e)Redcap supporting 3MHz CBW, it’s very hard to mandatorily force R20 (e)Redcap to support 3MHz CBW.

support3MHz-ChannelBW-Symmetric-r18

support3MHz-ChannelBW-Asymmetric-r18

support12PRB-CORESET0-r18

support12PRB-CORESET0-GSCN-41637-r18

- Release independent of 3MHz UE CBW for (e)Redcap

Although RAN4 agreed that 3MHz UE CBW is release independent from Rel-18 for (e)RedCap UE, if the restriction about “This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18” for the following Rel-18 WI ‘less than 5MHz’ can be removed from R18 TS 38.306, it’s impossible to support 3MHz UE CBW Rel-18 for (e)RedCap UE being release independent from Rel-18.

Proposal 2: RAN4 to review the previous ‘release independent’ agreement if the restriction about “This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18” for the following Rel-18 WI ‘less than 5MHz’ can be removed from R18 TS 38.306

Issue 4-2: Release independent of 3MHz UE CBW for (e)Redcap It is agreed 3MHz UE CBW is release independent from Rel-18 for (e)RedCap UE. (In the WF R4-2515101)

- REFSENS RB allocation configuration for eRedcap

Based on the WID, only 15RBs can be used for eRedCap supporting 3MHz, which is less than the BB capability of eRedCap. Thus, there is no need to restrict RB allocation configuration for eRedCap when testing REFSENS.

Proposal 3: the following text proposals can be considered for 3MHz channel bandwidth REFSENS requirements of eRedCap UE.

For UE supporting IE supportOfERedCap-r18, the 3MHz channel bandwidth REFSENS requirements for RedCap UE in clause 7.3I.2 apply.

3 Summary

Proposal 1: RAN4 send LS to RAN2 to remove the restriction about “This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18” for the following UE capabilities from R20.

support3MHz-ChannelBW-Symmetric-r18

support3MHz-ChannelBW-Asymmetric-r18

support12PRB-CORESET0-r18

support12PRB-CORESET0-GSCN-41637-r18

Observation 1: As the following UE capabilities are optional for normal UE, if these UE capacities will be reused for R20 (e)Redcap supporting 3MHz CBW, it’s very hard to mandatorily force R20 (e)Redcap to support 3MHz CBW.

support3MHz-ChannelBW-Symmetric-r18

support3MHz-ChannelBW-Asymmetric-r18

support12PRB-CORESET0-r18

support12PRB-CORESET0-GSCN-41637-r18

Proposal 2: RAN4 to review the previous ‘release independent’ agreement if the restriction about “This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18” for the following Rel-18 WI ‘less than 5MHz’ can be removed from R18 TS 38.306

support3MHz-ChannelBW-Symmetric-r18

support3MHz-ChannelBW-Asymmetric-r18

support12PRB-CORESET0-r18

support12PRB-CORESET0-GSCN-41637-r18

Issue 4-2: Release independent of 3MHz UE CBW for (e)Redcap It is agreed 3MHz UE CBW is release independent from Rel-18 for (e)RedCap UE. (In the WF R4-2515101)

Proposal 3: the following text proposals can be considered for 3MHz channel bandwidth REFSENS requirements of eRedCap UE.

For UE supporting IE supportOfERedCap-r18, the 3MHz channel bandwidth REFSENS requirements for RedCap UE in clause 7.3I.2 apply.

Reference

[1] RP-252955, “WID: NR Radio Resource Management (RRM) Phase 6”, Moderator (Apple)

[2] R4-2515101, “WF for [116bis][323] NR_less_than_5MHz_RedCap_UERF”, Sony