3GPP TSG RAN WG4 Meeting #117		R4-2520094

Dallas, Texas, United States, Nov 17th – 21st , 2025

Title: 	Discussion on Sensing/ISAC for 6G study

Agenda Item:	8.10

Source: 	CATT

Document for:	Approval

1 Introduction

In the RAN#109 meeting, a 6G RAN WG SI [1] was updated with the following objectives related to sensing.

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

Based on the RAN4 Chair’s guideline [2] on the RAN4 6G discussion and meeting arrangement in Rel-20 and the draft of RAN4#116bis meeting agenda, the general scope for UE RF requirements include the following parts.

- Sensing/ISAC [FS_6G_Radio] # NOTE: The scope includes RAN4 related PHY functions and procedure, RF, coexistence and testability

According to the WF[10] in the last meeting, RAN4 reached some consensus that some warm-up discussion about ISAC in RAN4 is needed.

Before April 2026, the agenda for sensing will be kept. RAN4 discussion will primarily focus on the following aspect - Sensing related regulatory status and requirement survey - Potential architecture consideration based on the RAN plenary use case study - views sharing on the potential RAN4 scope based on the identified use cases out of RAN plenary study - Identify less RAN1 design dependent RAN4 aspects if any

Based on the WF approved in the RAN4#116bis meeting, there are four aspects to be discussed in RAN4 before next April under the sensing/ISAC agenda. We’d like to provide our initial views about 6G Sensing/ISAC as one important function of multi-functional RAN at the 6G stage.

2 Discussion for 6G Sensing/ISAC

2.1 Scenario, Use case and KPI for 6G Sensing/ISAC

Referring to the latest TR 38.914 [9], the following scenarios, use cases and Key Performance Indicator (KPI) were summarized below.

4 Deployment scenarios Editor note: Sensing related aspects will be included in section 4 4.0 General 10 4.1 Indoor hotspot 10 4.2 Dense urban 12 4.3 Rural 14 4.4 Urban macro 15 4.5 Sub-Urban macro 16 4.6 High speed 16 4.7 Extreme long distance coverage in low density area 17 4.8 Urban coverage for massive connection 17 4.9 Air-to-Ground Scenario 17 4.10 Non-Terrestrial Network 17 4.11 [Urban grid] 18 4.12 [Highway Scenario] 18

Clause 5.1 Key performance indicators of TR 38.914 ### 5.1.14 Positioning Positioning is the ability to estimate the position of connected devices. The positioning accuracy is the [90%] point of the CDF of the device positioning error for [both] horizontal [and vertical] direction[s]. Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude). [Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude)]. The minimum requirements for the device positioning accuracy for various [test environments] are summarized in Table 5.1.14-1. Table 5.1.14-1: Positioning accuracy [Test Environment]; Horizontal Accuracy; Vertical Accuracy. Indoor Factory - ISAC. [Urban Macro – ISAC / Urban – ISAC]. ### …… ### 5.1.17 Sensing Sensing-related capabilities are measured in terms of the following technical performance requirements: • Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. • Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error. • Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

### Clause 5.4 Requirements of New and Existing Services of TR 38.914 ### 5.4.4 Sensing Editor note: More sensing use cases can be included depending on further discussion. The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

Observation 1: the use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs, are supported by 6GR and 6G RAN architecture referring to the latest TR 38.914.

Observation 2-1: For positioning, the positioning accuracy is considered as the KPI, which include Horizontal positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude) and [Vertical positioning accuracy in the vertical direction (i.e., z-axis, or altitude)].

Observation 2-2: For Sensing, Detection Probability, False Alarm Probability, Horizontal/Vertical Localization Accuracy and Velocity Accuracy are considered as the KPI.

RAN1/RAN2 haven’t started to discuss sensing/ISAC, but RAN1 has started to discuss the R20 SI on ISAC for 5G-A considering UAV use cases. As RAN4 has reached some consensus on the warm-up discussion on 6G ISAC, RAN4 can start to do warm-up discussion on 6G ISAC assuming UAV case and BS [/UE] mono-static as an example initially before next April.

Proposal 1: RAN4 can start to do warm-up discussion on 6G ISAC assuming UAV case and BS [/UE] mono-static as an example initially before next April.

2.2 Targeted Frequency

Although Frequencies from 0.5 to 52.6 GHz are the primary focus during the Rel-19 study on the channel modelling for NR ISAC.

But the following factors should be considered when the targeted centre frequency (fc) /wave length (λ=c/fc) is chosen.

1) The form factor or size of physical object to be detected vs the wave length (λ) of sensing signal.

Firstly, it should be guaranteed that the wave length (λ) of sensing signal is much less than the size (a) of object, so that the RCS (Radar Cross Section) can be located at the Optical Region. Where is in the optical region, the RCS (Radar Cross Section) of physical object will not be changes for different centre frequency. As shown below, 2πa/λ should be larger than 10 where is Optical Region. In other words, size (a) of physical object is equal to or larger than three times wave length (λ).

| Centre frequency (fc) | 2GHz | 4GHz | 6GHz | 30GHz |
| --- | --- | --- | --- | --- |
| Wave length (λ) | 15cm | 7.5cm | 5cm | 1cm |
| Minimum size of physical object | Around 45cm | Around 22.5cm | 15cm | 3cm |

| Centre frequency (fc) | 2.1GHz | 2.6GHz | 3.5GHz | 4.9GHz | 7GHz | 30GHz |
| --- | --- | --- | --- | --- | --- | --- |
| DL Bands | band n1 | Band n7/n41 | n78/n77 | Band n79 | n104 | TDD FR2 |
| Wave length (λ) | 14.3cm | 11.5cm | 8.6cm | 6.1cm | 4.3cm | 1cm |
| Minimum size of physical object | Around 42.9cm | 34.5cm | 25.8cm | Around 18.3cm | 12.9cm | 3cm |

For the agreed use cases of 6GR sensing , i.e. UAVs, human, vehicles and AGV, all of them can meet the RCS requirements of Optical Region for the centre frequency equal to or larger than 2GHz. For use cases about whether objects (Whether Radar), e.g. turbulence for detecting wind speed, raindrop, cloud and mist, the wave length for sensing signal should be chosen carefully considering the size of physical object to be detected.

Observation 3: For the agreed use cases of 6GR sensing , i.e. UAVs, human, vehicles and AGV, all of them can meet the RCS requirements of Optical Region for the centre frequency equal to or larger than 2GHz.

In R20 SI on ISAC, RAN1 assume UAV with small size (0.3m x 0.4m x 0.2m) [11], so only centre frequency larger than 3.5/4.9GHz can be used to meet the RCS requirements of Optical Region for UAV use cases.

Proposal 2: For UAV case, RAN4 can start to do warm-up discussion on 6G ISAC assuming 3.5GHz/4.9GHz/7GHz as exemplary centre frequency initially before next April.

2.3 BS antenna configuration for BS monostatic mode and UAV case

According to the agreements [11] achieved in R20 SI on ISAC, the following BS antenna configurations are considered.

Parameters; Assumptions. BS antenna configuration; For 4GHz, 4.9GHz: - Option 1 as optional - Tx: (12,16, 2,1,1;2,16) - Rx: (12,16,2,1,1;2,16) - Option 2 - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8); For 6GHz: - Option 1 as optional - Tx: (16,16, 2,1,1;4,16) - Rx: (16,16,2,1,1;4,16) - Option 2: - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8). Mandatory: dH,dV = (0.5, 0.8)λ, +45°/-45° polarization Optional: dH,dV = (0.5, 0.5)λ, +45°/-45° polarization.

According to the TR 38.922 [12], the following BS antenna configurations for communication are considered.

|  | 3.5GHz/4.9GHz (M, N, P, Mg, Ng) | 7GHz (M, N, P, Mg, Ng) |
| --- | --- | --- |
| BS antenna configuration | Tx/Rx array: (4, 8, 2, 3,1) | Tx/Rx array: (8,16, 2, 4,1) |
| dH,dV | (0.5, 2.1)λ | (0.5, 2.1)λ |

Observation 4: RAN1 assumed the two separate Tx and Rx antenna arrays for BS supporting ISAC in R20 5G-A ISAC, but RAN4 only assumed one shared antenna array for both Tx and Rx in TR 38.922 supporting communication function only.

Proposal 3: RAN4 should discuss how the BS antenna arrays are used for BS supporting ISAC.

P3-1: discuss whether the existing TDD BS antenna arrays for communications can be directly reused for BS to support ISAC.

(P3-1) Possible Answer: the purpose for BS supporting ISAC is to reuse the existing TDD BS hardware / antenna arrays as much as possible.

P3-2: If BS has to support BS monostatic ISAC for UAV use case, it’s unclear whether additional BS antenna array need to be implemented, e.g. additional Rx antenna array which is separated from Tx antenna array.

(P3-2) Possible Answer: it depends on whether Pulse Wave or Continues Wave is used as the ISAC waveform.

For Pulse Wave ISAC BS, the existing shared antenna array between Tx and Rx can be reused since the operation of transmission and receiving for Pulse Wave ISAC BS is in the Time Division Duplex (TDD).

For Continues Wave ISAC BS, additional Rx antenna array is need to provide better isolation between BS transmitter and receiver. Thus, for this case, the BS antenna arrays supporting SBFD can be reused for Continues Wave ISAC BS.

P3-3: Apart from Tx and Rx antenna array, it’s unclear whether the antenna elements and the vertical spacing dV should be adjusted, for example: more elements/BB chain is needed, adjust dV from 0.7/2.1λ to 0.5λ.

(P3-3) Possible Answer: it depends on the targeted scenarios and the performance of azimuth/elevation angular resolution.

The Radar azimuth/elevation angular resolution vs the wave length (λ) of sensing signal.

Generally, the Radar azimuth/elevation angular resolution is relative to the wave length (λ) of sensing signal and antenna size (D). The formula is shown below.

θ3dB or φ3dB=k∙λD

Where, k is between 0.89 and 1.02 according to the experience. In other words, for the same size of antenna, the less wave length is, the less Radar azimuth/elevation angular resolution is.

Thus, we can get the following azimuth/elevation angular resolutions for different centre frequency and different BS antenna configurations.

|  | TR 38.922 [12] |  |  | RAN1 assumption [11] in R20 5G-A SI |  |
| --- | --- | --- | --- | --- | --- |
| fc | 3.5GHz | 4.9GHz | 7GHz | 4.9GHz | 7GHz |
| BS antenna configuration (M, N, P, Mg, Ng) | Tx/Rx array: (4, 8, 2, 3,1) | Tx/Rx array: (4, 8, 2, 3,1) | Tx/Rx array: (8,16, 2, 3,1) | - Option 1 as optional - Tx: (12,16, 2,1,1;2,16) - Rx: (12,16,2,1,1;2,16) - Option 2 - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) | For 6GHz: - Option 1 as optional - Tx: (16,16, 2,1,1;4,16) - Rx: (16,16,2,1,1;4,16) - Option 2: - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) |
| dH,dV | (0.5, 2.1)λ | (0.5, 2.1)λ | (0.5, 2.1)λ | Mandatory: dH,dV = (0.5, 0.8)λ, +45°/-45° polarization Optional: dH,dV = (0.5, 0.5)λ, +45°/-45° polarization |  |
| azimuth angular resolution (φ3dB) 0.89λDH=0.89dH/λ (N-1) =0.890.5 (N-1) | 0.25rad = 14.5o | 0.25rad = 14.5o | 0.12rad = 6.8o | Option 1: 0.12rad = 6.8o Option 2: 0.25rad = 14.5o | Option 1: 0.12rad = 6.8o Option 2: 0.25rad = 14.5o |
| elevation angular resolution (θ3dB) 0.89λDV= 0.89dV/λ (M-1) | For dv/λ=0.7, M=4*3=12 0.12rad = 6.6o For dv/λ=2.1, M=4 0.14rad = 8.1o | For dv/λ=0.7, M=4*3=12 0.12rad = 6.6o For dv/λ=2.1, M=4 0.14rad = 8.1o | For dv/λ=0.7, M=8*3=24 0.06rad = 3.2 o For dv/λ=2.1, M=8 0.06rad = 3.5o | Option 1: (0.8λ ) 0.10rad = 5.8o (0.5λ ) 0.16rad = 9.3o Option 2: (0.8λ ) 0.16rad = 9.1o (0.5λ ) 0.25rad = 14.6o | Option 1: (0.8λ ) 0.07rad = 4.2o (0.5λ ) 0.12rad = 6.8o Option 2: (0.8λ ) 0.16rad = 9.1o (0.5λ ) 0.25rad = 14.6o |

Currently, the candidate azimuth angular resolution and elevation angular resolution are shown below for different BS antenna configurations.

Observation 5: the candidate azimuth angular resolution and elevation angular resolution are summarised below for different assumptions BS antenna configurations

| azimuth angular resolution | elevation angular resolution |  |
| --- | --- | --- |
| case 1: (N=16, dHλ=0.5) 0.12rad = 6.8o case 2: (N=8, dHλ=0.5) 0.25rad = 14.5o | case 0-1: (M=8) (2.1λ ) 0.06rad = 3.5o case 0-2: (M=4) (2.1λ ) 0.14rad = 8.1o | case 1: (M=16) (0.8λ ) 0.07rad = 4.2o (0.5λ ) 0.12rad = 6.8o case 2: (M=12) (0.8λ ) 0.10rad = 5.8o (0.5λ ) 0.16rad = 9.3o Option 3: (M=8) (0.8λ ) 0.16rad = 9.1o (0.5λ ) 0.25rad = 14.6o |

Proposal 4: RAN4 should discuss if the summarized azimuth / elevation angular resolution above can meet the requirements or not for UAV use case.

In addition, RAN1 agreed some simulation assumptions for the scenarios to be evaluated. Some important information is highlighted below.

Simulation assumptions Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Scenario; UMa-AV, Optional RMa-AV. Carrier frequency; Mandatory: one value either 4 GHz or 4.9GHz. optional for FR1: 6 GHz [optional for FR2: 28 GHz]. System bandwidth; 100 MHz. Numerology; SCS = 30kHz. BS Layout; Hexagonal grid, 7 macro sites, 3 sectors per site. See Note1 3 sectors with 30, 150, 270 degrees. Inter-BS (2D) distance; Uma-AV: 500m, optional 1000m, RMa-AV: 1732m. BS antenna height; 25m for UMa-AV, 35m for RMa-AV. Wrap-round; No wrap-round. Note1: target(s) are dropped only in the center site, and inter-BS interference is not modelled. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Target type; UAV with small size (0.3m x 0.4m x 0.2m). Target distribution when target(s) are dropped; N = 5 targets per sector in the center site. Optional: N is uniformly distributed from 1 to 10. Horizontal plane: uniformly distributed in a sector Vertical plane: Uniformly distributed between 25m and 300m, optionally distributed between 1.5m and 300m. Mobility; horizontal speed: uniformly distributed between 0 and 180km/h vertical speed: 0km/h. Note: N=0 will be discussed in a later proposal in relation to false alarm. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Minimum BS-target 3D distance; 10 m. Minimum target-target (3D) distance; 10 m. Target outdoor/indoor proportion; 100% outdoor. LOS/NLOS; LOS and NLOS. Orientation; Random in horizontal domain. RCS model; RCS model 1 for UAV with small size.

Considering the assumed scenarios for UAVs, we illustrated the relationship between the BS antenna performance and the targeted scenarios in the figure 1 and figure 2.

Figure 1 the illustration on UAV scenarios from horizontal profile

Figure 2 the illustration on UAV scenarios from vertical profile

For UAV scenarios from horizontal profile, current range of horizontal electronic scan is -60≤φescan≤60 deg, which can meet the necessity.

For UAV scenarios from vertical profile, current range of vertical electronic scan is 90≤θescan≤100 deg referring to TR 38.922, which will result in some Blind Spot. In addition, 3dB of antenna element is 65 deg, which will also result in some Blind Spot.

Observation 6:

For UAV scenarios from horizontal profile, current range of horizontal electronic scan is -60≤φescan≤60 deg, which can meet the necessity.

For UAV scenarios from vertical profile, current range of vertical electronic scan is 90≤θescan≤100 deg referring to TR 38.922, which will result in some Blind Spot. In addition, 3dB of antenna element is 65 deg, which will also result in some Blind Spot.-60≤φescan≤60

Proposal 5: RAN4 should discuss how to detect the UAV objectives in the Blind Spot if we reuse the existing performance of vertical antenna array and element. Alternatively, RAN4 can discuss whether the configurations of vertical antenna array and element can be adjusted for ISAC BS.

2.4 Bandwidth of Sensing signal

The range resolution for sensing signal is derived based on the following formula:

ΔRange=cτ2=c2B

Where, c is the velocity of light, 3×108 m/s, τ (second) is the Pulse duration for Pulse Radar or B (Hz) is the Bandwidth of sensing signal.

No matter what kind of Radar system we choose, the bandwidth of sensing signal will determine the range resolution of sensing signal.

| Bandwidth of Sensing Signal (MHz) | 5MHz | 10MHz | 20MHz | 30MHz | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The range resolution (m) | 30m | 15m | 7.5m | 5m | 3m | 2m | 1m | 0.5m |

Observation 5: No matter what kind of Radar system we choose, the bandwidth of sensing signal will determine the range resolution of sensing signal.

Proposal 6: For UAV use case, RAN4 should discuss if the 5m range resolution and above can meet the requirements or not.

2.5 sensing signal: Continuous Wave (CW) / Pulse Wave (PW)

According to the existing Radar system, there are two kinds of sensing signal/Radar System, i.e. Pulse Wave (PW) Radar and Continuous Wave (CW) Radar. As we discussed above, Pulse Wave (PW) Radar and Continuous Wave (CW) Radar will lead to different ISAC BS implementation. We’d like to compare these two kinds of Radar System considering the following aspects.

|  | Pulse Wave (PW) Radar | Continuous Wave (CW) Radar |
| --- | --- | --- |
| Time Domain characteristic | Or |  |
| Frequency Domain characteristic |  |  |
| Time-Bandwidth Product (TBP) | τ×B=1 | τ×B>1 |
| Operating State of the transmitter PA | Saturation State for larger output power | Saturation State or linear State due to large TBP |
| Duplex Mode | The operation of transmission and receiving is in the Time Division Duplex (TDD). | The operation of transmission and receiving is in the Full Duplex at the same time and frequency (bandwidth). |
| Self-interference | As TDD is used for transmission and receiving, there is no self-interference issue. | There is a strong receiving self-interference coupled from transmitter. Generally, the methods of moving target detection technology should be used to eliminate the self-interference, e.g. self-heterodyne, Pulse-Doppler Method. |
| Range Blind Zone | cτ/2 | If Full Duplex Operation between transmission and receiving is used, there is no Range Blind Zone for CW Radar. |
| Pulse Compression | Not supported | Supported. For example, matched filter is used to complete the Pulse Compression for LMF signal, but the side lobe of strong Object may interfere the detection of the small Object. |

Proposal 7: the characteristics above for Pulse Wave (PW) Radar and Continuous Wave (CW) Radar can be considered as the part of RAN4 6G ISAC study.

Proposal 8: RAN4 should active work closely with RAN1 on design of sensing signal from system performance impacts and coexistence perspective.

3 Summary

Current status for the studies of NR sensing/ISAC

2.1 Scenario, Use case and KPI for 6G Sensing/ISAC

Observation 1: the use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs, are supported by 6GR and 6G RAN architecture referring to the latest TR 38.914.

Observation 2-1: For positioning, the positioning accuracy is considered as the KPI, which include Horizontal positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude) and [Vertical positioning accuracy in the vertical direction (i.e., z-axis, or altitude)].

Observation 2-2: For Sensing, Detection Probability, False Alarm Probability, Horizontal/Vertical Localization Accuracy and Velocity Accuracy are considered as the KPI.

Proposal 1: RAN4 can start to do warm-up discussion on 6G ISAC assuming UAV case and BS [/UE] mono-static as an example initially before next April.

2.2 Targeted Frequency

Observation 3: For the agreed use cases of 6GR sensing , i.e. UAVs, human, vehicles and AGV, all of them can meet the RCS requirements of Optical Region for the centre frequency equal to or larger than 2GHz.

Proposal 2: For UAV case, RAN4 can start to do warm-up discussion on 6G ISAC assuming 3.5GHz/4.9GHz/7GHz as exemplary centre frequency initially before next April.

2.3 BS antenna configuration for BS monostatic mode and UAV case

Observation 4: RAN1 assumed the two separate Tx and Rx antenna arrays for BS supporting ISAC in R20 5G-A ISAC, but RAN4 only assumed one shared antenna array for both Tx and Rx in TR 38.922 supporting communication function only.

Proposal 3: RAN4 should discuss how the BS antenna arrays are used for BS supporting ISAC.

P3-1: discuss whether the existing TDD BS antenna arrays for communications can be directly reused for BS to support ISAC.

(P3-1) Possible Answer: the purpose for BS supporting ISAC is to reuse the existing TDD BS hardware / antenna arrays as much as possible.

P3-2: If BS has to support BS monostatic ISAC for UAV use case, it’s unclear whether additional BS antenna array need to be implemented, e.g. additional Rx antenna array which is separated from Tx antenna array.

(P3-2) Possible Answer: it depends on whether Pulse Wave or Continues Wave is used as the ISAC waveform.

For Pulse Wave ISAC BS, the existing shared antenna array between Tx and Rx can be reused since the operation of transmission and receiving for Pulse Wave ISAC BS is in the Time Division Duplex (TDD).

For Continues Wave ISAC BS, additional Rx antenna array is needed to provide better isolation between BS transmitter and receiver. Thus, for this case, the BS antenna arrays supporting SBFD can be reused for Continues Wave ISAC BS.

P3-3: Apart from Tx and Rx antenna array, it’s unclear whether the antenna elements and the vertical spacing dV should be adjusted, for example: more elements/BB chain is needed, and/or adjust dV from 0.7/2.1λ to 0.5λ.

(P3-3) Possible Answer: it depends on the targeted scenarios and the performance of azimuth/elevation angular resolution.

Observation 5: the candidate azimuth angular resolution and elevation angular resolution are summarised below for different assumptions BS antenna configurations

| azimuth angular resolution | elevation angular resolution |  |
| --- | --- | --- |
| case 1: (N=16, dHλ=0.5) 0.12rad = 6.8o case 2: (N=8, dHλ=0.5) 0.25rad = 14.5o | case 0-1: (M=8) (2.1λ ) 0.06rad = 3.5o case 0-2: (M=4) (2.1λ ) 0.14rad = 8.1o | case 1: (M=16) (0.8λ ) 0.07rad = 4.2o (0.5λ ) 0.12rad = 6.8o case 2: (M=12) (0.8λ ) 0.10rad = 5.8o (0.5λ ) 0.16rad = 9.3o Option 3: (M=8) (0.8λ ) 0.16rad = 9.1o (0.5λ ) 0.25rad = 14.6o |

Proposal 4: RAN4 should discuss if the summarized azimuth / elevation angular resolution above can meet the requirements or not for UAV use case.

Observation 6:

For UAV scenarios from horizontal profile, current range of horizontal electronic scan is -60≤φescan≤60 deg, which can meet the necessity.

For UAV scenarios from vertical profile, current range of vertical electronic scan is 90≤θescan≤100 deg referring to TR 38.922, which will result in some Blind Spot. In addition, 3dB of antenna element is 65 deg, which will also result in some Blind Spot.

Proposal 5: RAN4 should discuss how to detect the UAV objectives in the Blind Spot if we reuse the existing performance of vertical antenna array and element. Alternatively, RAN4 can discuss whether the configurations of vertical antenna array and element can be adjusted for ISAC BS.

2.4 Bandwidth of Sensing signal

Observation 5: No matter what kind of Radar system we choose, the bandwidth of sensing signal will determine the range resolution of sensing signal.

Proposal 6: For UAV use case, RAN4 should discuss if the 5m range resolution and above can meet the requirements or not.

2.5 sensing signal: Continuous Wave (CW) / Pulse Wave (PW)

Proposal 7: the characteristics above for Pulse Wave (PW) Radar and Continuous Wave (CW) Radar can be considered as the part of RAN4 6G ISAC study.

|  | Pulse Wave (PW) Radar | Continuous Wave (CW) Radar |
| --- | --- | --- |
| Time Domain characteristic | Or |  |
| Frequency Domain characteristic |  |  |
| Time-Bandwidth Product (TBP) | τ×B=1 | τ×B>1 |
| Operating State of the transmitter PA | Saturation State for larger output power | Saturation State or linear State due to large TBP |
| Duplex Mode | The operation of transmission and receiving is in the Time Division Duplex (TDD). | The operation of transmission and receiving is in the Full Duplex at the same time and frequency (bandwidth). |
| Self-interference | As TDD is used for transmission and receiving, there is no self-interference issue. | There is a strong receiving self-interference coupled from transmitter. Generally, the methods of moving target detection technology should be used to eliminate the self-interference, e.g. self-heterodyne, Pulse-Doppler Method. |
| Range Blind Zone | cτ/2 | If Full Duplex Operation between transmission and receiving is used, there is no Range Blind Zone for CW Radar. |
| Pulse Compression | Not supported | Supported. For example, matched filter is used to complete the Pulse Compression for LMF signal, but the side lobe of strong Object may interfere the detection of the small Object. |

Proposal 8: RAN4 should active work closely with RAN1 on design of sensing signal from system performance impacts and coexistence perspective.

Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple)

[3] TS 22.137, “Service requirements for Integrated Sensing and Communication”, SA1

[4] TR 22.837, Feasibility Study on Integrated Sensing and Communication, SA1

[5] TR 38.901-v19.0.0, Study on channel model for frequencies from 0.5 to 100 GH, RAN1

[6] RP-242348, Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, Xiaomi, AT&T

[7] RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR, Xiaomi, China Telecom

[8] RP-252200, SR on Integrated Sensing And Communication (ISAC) for NR, Xiaomi, China Telecom

[9] RP-252882, TR 38.914 v0.2.0, “Study on 6G Scenarios and Requirements”, RAN

[10] R4-2514592, WF on 6G sensing, ZTE (moderator)

[11] R1-2507427, Summary #5 on evaluations for NR ISAC, Moderator (Xiaomi)

[12] TR 38.922, Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz, RAN4

Annex

Large scale path loss vs the wave length (λ) of sensing signal.

As the centre frequency is increasing (the wave length of sensing signal is decreasing), the large scale path loss will increase.

For example, the basic monostatic (Pulse Wave) Radar Function for Point Physical Point is shown below.

The smaller wave length (λ) of sensing signal represents the smaller receiving power of echo signal.

4) Doppler frequency vs the wave length (λ) of sensing signal.

Generally, the Doppler frequency caused by relative velocity is calculated based on the following formula.

fdoppler=2v∙cosθλ

Proposal 2: When the targeted centre frequency is discussed, the following factors should be considered together, e.g. the size (a) of physical object to be detected, Radar azimuth/elevation angular resolution (θ3dB or φ3dB), the Radar Function / the targeted receiving Power of echo signal and the Doppler frequency/Maximum velocity of object.