3GPP TSG-RAN WG4 Meeting # 117                                                              R4-2520132

Dallas, USA, Nov. 17-21, 2025

Title: 	Discussion on the feasibility of adjacent channel coexistence between SBFD BS and SBFD BS

Source: 	CATT

Agenda item:	7.2.4

Document for:	Discussion

- Background

In WF[2] of RAN4 116bis meeting, configuration of aggressor SBFD BS and victim SBFD BS has three options, we analyse feasibility of all options in this contribution.

- Feasibility analysis

Configuration options of aggressor SBFD BS and victim SBFD BS are shown following, we analyse all options feasibility in this paper.

  - SBFD BS to SBFD BS adjacent channel coexistence

For next meeting Agreements:

- further consider following options, with the intention to down-select:

Option 1: UD (Operator 1) and DU (Operator 2)

Option 2: DUD (Operator 1 and 2)

Option 3: UD and DUD

Provide more description on each specific case.

If the SBFD BSs of Operator 1 and Operator 2 have the same DL and UL sub-band configuration, the mutual interference between them will be identical. Otherwise, the mutual interference will differ and requires separate analysis.

Proposal 1: When the SBFD BSs of Operator 1 and Operator 2 have the same DL and UL sub-band configuration, the mutual interference between them only needs to be analysed in one direction. Otherwise, mutual interference needs to be analysed separately.

  - Tx feasibility analysis

Figure 1 Schematic diagram of suppression from RF BPF (used after PA)

Table 1: Tx feasibility analysis of SBFD BS co-location

| Parameters | Value |
| --- | --- |
| Output power of aggressor SBFD-capable BS(dBm) | 49 |
| BW(MHz) | 100 |
| Degradation of victim SBFD BS sensitivity(dB) | [1] |
| Required total isolation(dB) | 144 |

For FR1 aggressor WA SBFD BS, the output power is 49dBm, if assume the channel bandwidth is 100MHz and assume the degradation of victim SBFD BS sensitivity is 1dB, the required total isolation between the two BSs is 144dB, that is, the sum of ACLR, co-location coupling loss, and the suppression from the BPF after the PA in the UL sub-band of the victim BS must be greater than 144dB.

Proposal 2: Under the assumption of 1dB sensitivity degradation, the sum of ACLR, co-location coupling loss, and the suppression from the BPF after the PA in the UL sub-band of the victim BS must be greater than 144dB.

Table 2: For the BPF used after PA, the required suppression levels

| Parameters | 2.6GHz | 4.9GHz | 7GHz |
| --- | --- | --- | --- |
| 1st ACLR(dB) | 45 | 45 | [45] |
| Coupling loss of co-location (dB) | 30 | [51] | [55] |
| For the BPF used after PA, the required suppression levels (dB) | 69 | 48 | 44 |

According to WID [1], the applicable bands focus on band n41, n79 and n104, it is assumed that the co-location coupling losses for these operating bands are the values shown in [4]. ACLR is 45dB, but for 7GHz, 45 dB ACLR can be achieved within a 100 MHz bandwidth. However, for a 400 MHz bandwidth, the DPD optimization capability might not be as effective as it is with lower bandwidths, potentially leading to degradation in achievable ACLR performance. For the BPF used after PA, the required suppression levels on the UL sub-band of the victimized BS are shown in Table 2. If the value of the co-located coupling loss and 7GHz 1st ACLR is updated later, the BPF suppression requirements should be adjusted accordingly.

Proposal 3:  For the BPF used after PA, the required suppression levels on the UL sub-band of the victimized BS are shown in table 2.

  - Rx feasibility analysis

Figure 2 Schematic diagram of suppression from RF BPF (used in UL sub-band)

Table 3:  Rx feasibility analysis of SBFD BS co-location

| Output power of aggressor SBFD-capable BS(dBm) | 49 |
| --- | --- |
| Existing in-band blocking of NR BS (dBm) | -43 |
| Required total isolation(dB) | 92 |

If the in-band blocking of victim SBFD-capable BS is -43dBm, the required total isolation between transmission of aggressor SBFD-capable BS and the reception of victim SBFD-capable BS is 92dB. The total isolation, comprising the co-location coupling loss and the rejection provided by the victim BS's uplink RF BPF against the interfering BS's downlink sub-band, shall exceed 92dB.

Proposal 4: Under the assumption of -43dBm in-band blocking, the sum of the co-location coupling loss and the rejection provided by the victim BS's uplink RF BPF against the interfering BS's downlink sub-band, shall exceed 92dB.

Table 4: For the RF BPF used in victim BS UL link, the required suppression levels

| Parameters | 2.6GHz | 4.9GHz | 7GHz |
| --- | --- | --- | --- |
| Required total isolation(dB) | 92 | 92 | 92 |
| Coupling loss of co-location (dB) | 30 | [51] | [55] |
| Interference signal power level at TAB of victim SBFD-capable BS (dBm) | 18 | 5 | -6 |
| For the RF BPF used in victim BS UL link, required suppression on interference signal (dB) | 61 | 46 | 37 |

In table 4, it is assumed that the co-location coupling losses at 2.6GHz, 4.9GHz, and 7GHz are the values shown in [3]. If the co-location coupling loss values are updated later, the BPF suppression requirements shall be adjusted accordingly.

For FR1 aggressor WA SBFD-capable BS, the output power is 49dBm. If the co-location blocking is 16dBm, the coupling loss of co-location must lager than 33dB. The co-location coupling losses at 4.9GHz and 7GHz are both greater than 33dB，but the co-site coupling loss at 2.6GHz is 2dB less than the required value, this 2dB difference can be effectively compensated by the RF BPF used in the UL link.

Proposal 5: For the RF BPF used in UL link of victim BS, the required suppression levels on the DL sub-band of the aggressor BS are shown in table 4.

  - Other analysis

Based on the analysis above, the optimal spectrum configuration for SBFD BSs with two operators is {UD} and {DU}. In the case of three operators, the optimal spectrum configuration becomes {UD}, {DUD}, and {DU}.

Proposal 6: The optimal spectrum configuration for SBFD BSs with two operators is {UD} and {DU}. In the case of three operators, the optimal spectrum configuration becomes {UD}, {DUD}, and {DU}.

- Summary

In this contribution, we have the following proposals.

Proposal 1: When the SBFD BSs of Operator 1 and Operator 2 have the same DL and UL sub-band configuration, the mutual interference between them only needs to be analysed in one direction. Otherwise, mutual interference needs to be analysed separately.

Proposal 2: Under the assumption of 1dB sensitivity degradation, the sum of ACLR, co-location coupling loss, and the suppression from the BPF after the PA in the UL sub-band of the victim BS must be greater than 144dB.

Proposal 3: For the BPF used after PA, the required suppression levels on the UL sub-band of the victimized BS are shown in table 2.

Proposal 4: Under the assumption of -43dBm in-band blocking, the sum of the co-location coupling loss and the rejection provided by the victim BS's uplink RF BPF against the interfering BS's downlink sub-band, shall exceed 92dB.

Proposal 5: For the RF BPF used in UL link of victim BS, the required suppression levels on the DL sub-band of the aggressor BS are shown in table 4.

Proposal 6: The optimal spectrum configuration for SBFD BSs with two operators is {UD} and {DU}. In the case of three operators, the optimal spectrum configuration becomes {UD}, {DUD}, and {DU}.

- Reference

[1] RP-252905, “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, RAN#109

[2] R4-2508657, “Way-Forward for NR_BS_RF_Ph2”, Ericsson, RAN4#116bis

[3] R4-2417852, “Discussion on modification of existing Tx requirements for SBFD operation”, CATT, RAN4#113

[4] TR 37.941