3GPP TSG RAN WG4 Meeting #117		R4-2520134

Dallas, Texas, United States, Nov 17th – 21st , 2025

Title: 	Discussion on enhanced OTA test methodology for VSAT device

Agenda Item:	7.3.3

Source: 	CATT

Document for:	Discussion

1	Introduction

In RAN4#116bis meeting, the work plan for Rel-20 OTA WI was agreed in [1] and WF was agreed in [2]. According to [1], the work will concentrate on making progress on NTN test methodology during RAN4#117 meeting. In this paper, we share our views on the extension of existing test methodologies for NR NTN Ka and Ku bands for VSAT device.

2	Discussion

2.1	Performance requirements and metrics for NTN VSAT

The performance requirements for Ka and Ku bands are already defined in clause 9 and clause 10 in [3] for NTN VSAT devices. And the corresponding test metrics are given at the same time in [3]. The requirements and metrics can be summarized as in table 1:

Table 1 Radiated performance requirements and test metrics for Ku-band and Ka-band

|  | Characteristics | Test metric |
| --- | --- | --- |
| Tx | maximum output power: EIRP, TRP | EIRP(Link=Tx beam peak direction, Meas=Link angle), TRP |
|  | Transmit OFF power | TRP (Link=TX beam peak direction, Meas=TRP grid) |
|  | Transmit ON/OFF time mask | EIRP(Link=TX beam peak direction, Meas=Link angle) |
|  | Frequency Error |  |
|  | Error vector magnitude |  |
|  | Occupied bandwidth |  |
|  | Spectrum emission mask | TRP (Link=TX beam peak direction, Meas=TRP grid) |
|  | Adjacent channel leakage ratio |  |
|  | Spurious Emissions |  |
|  | On-axis spurious requirement | EIRP(Link=TX beam peak direction, Meas=Link angle) |
|  | Off-axis spurious requirement |  |
|  | “Carrier-on” and “Carrier-off” states |  |
| Rx | OTA reference sensitivity level | EIS (Link=RX beam peak direction, Meas=Link Angle) |
|  | Maximum input level |  |
|  | Adjacent channel selectivity |  |
|  | Blocking characteristics |  |

It can be seen that the current test metrics in [3] fall into three categories: EIRP(Link=Tx beam peak direction, Meas=Link angle), TRP (Link=TX beam peak direction, Meas=TRP grid), and EIS(Link=RX beam peak direction, Meas=Link Angle).

The example measurement grids for EIRP and EIS testing are illustrated in [3] (Figure 9.2.1.2-1 and Figure 10.3.2-1), reproduced as below Figure 1:

Figure 1. Example measurement grid for EIRP and EIS given in [3]

The classical measurement procedure have been defined clause 5.2.3.3.2 for peak EIRP and clause 5.2.3.3.4 for peak EIS in [4] for Indirect far field (IFF) system RAN4 could consider adopt these measurement procedures together with the measurement grid defined in [3].

Proposal 1:	For the defined performance requirements with EIRP test metrics in TS 38.101-5, RAN4 could consider adopt the classical EIRP measurement procedures in TR 38.810 clause 5.2.3.3.2 as baseline, together with the measurement grid for EIRP in TS 38.101-5 Figure 9.2.1.2-1.

Proposal 2:	For the defined performance requirements with EIS test metrics in TS 38.101-5, RAN4 could consider adopt the classical EIS measurement procedures in TR 38.810 clause 5.2.3.3.4 as baseline, together with the measurement grid for EIS in TS 38.101-5 Figure 10.3.2-1.

As for TRP test metric, the example measurement grids for TRP are not defined in [3]. In this way, we can assume the classical TRP measurement grid in [4] will be used. In clause G.1.2 in [4], two different measurement grid types for TRP are considered:

- constant step size grid with  =  =15° Figure 2: Distribution of measurement grid points in 3D for a constant step size grid with ==15o (266 unique measurement points) - constant density grid Figure 3: Distribution of measurement grid points in 3D for a constant density grid type with 266 unique measurement points

Both classical types have 266 measurement points evenly distributed across the whole sphere.

As for NTN VSAT device which provides coverage in the upper hemisphere, only the upper hemisphere or even less zone matters. Using classical 266 measurement points will cause a waste of test time. Only integrate the radiated power from the measurement grids across the upper hemisphere may be sufficient,

Proposal 3:	For TRP-based requirements with TRP metrics, RAN4 consider integrating the radiated power only over the upper hemisphere for NTN VSAT device.

If proposal 3 is not acceptable, in order to reduce test time cost, one alternative approach is to consider applying different measurement grid density in both  and for upper hemisphere and lower hemisphere. For example, perform the TRP measurement with constant step size grid  =  =15° in the upper hemisphere and  =  =30°in the lower hemisphere.

Proposal 4:	If Proposal 3 is not agreed, consider apply coarse measurement grid for the lower hemisphere for TRP measurement to reduce test time. For example, adopt constant step size grid  =  =15° in the upper hemisphere and  =  =30°in the lower hemisphere.

3	Conclusion

In this document, we share our views on the test methodologies for Ka-band and Ku-band for NTN VSAT device. Following proposals are submitted:

Proposal 1:	For the defined performance requirements with EIRP test metrics in TS 38.101-5, RAN4 could consider adopt the classical EIRP measurement procedures in TR 38.810 clause 5.2.3.3.2 as baseline, together with the measurement grid for EIRP in TS 38.101-5 Figure 9.2.1.2-1.

Proposal 2:	For the defined performance requirements with EIS test metrics in TS 38.101-5, RAN4 could consider adopt the classical EIS measurement procedures in TR 38.810 clause 5.2.3.3.4 as baseline, together with the measurement grid for EIS in TS 38.101-5 Figure 10.3.2-1.

Proposal 3:	For TRP-based requirements with TRP metrics, RAN4 consider integrating the radiated power only over the upper hemisphere for NTN VSAT device.

Proposal 4:	If Proposal 3 is not agreed, consider apply coarse measurement grid for the lower hemisphere for TRP measurement to reduce test time. For example, adopt constant step size grid  =  =15° in the upper hemisphere and  =  =30°in the lower hemisphere.

Reference

[1]	R4-2513570 Workplan of Rel-20 OTA WI

[2]	R4-2514820 WF on NR_UE_OTA_Enh

[3]	TS 38.101-5 User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 19)

[4]	TR 38.810 Study on test methods