3GPP TSG RAN WG4 Meeting #117		R4-2520135

Dallas, Texas, United States, Nov 17th – 21st , 2025

Title: 	Discussion on 6G OTA and testability

Agenda Item:	8.11

Source: 	CATT

Document for:	Discussion

1	Introduction

In RAN4#116bis meeting, RAN4 initiated the discussion for 6G OTA and testability topic and reached some general agreements. Meantime, many issues require further discussion. In this paper we are sharing our views on 6G OTA methodologies and potential testability issues.

2	Discussion

In RAN4#116bis meeting, a WF for 6G Testability and OTA was agreed in [1]. In this WF, following aspects were discussed and reached some early agreements.

Issue 1-2-1: Whether it is valuable to improve some conducted requirements to be verified via OTA approach Agreements: - RAN4 can further study the testability limitations of some conducted test cases, consider the following aspects as starting point, e.g., - FR1 conducted test cases already consider antenna-performance impacts but not verified via radiated approach in a case-by-case manner, e.g. MSD - FR1 test case simplification with both conducted and radiated considered, e.g., spurious emission - Discuss which conducted conformance testing could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern. Issue 4-1-1: OTA testability applicability for different UE types in 6GR day-1 Agreements: - RAN4 consider the study of OTA test system to better accommodate different UE types, new feature/functionalities, new performance metrics and test cases. - The UE types and form factor considered in testability can be further discussed in this SI. - RAN4 study the feasibility of developing a single system to cover above aspects. Issue 6-1-1: Harmonized OTA test methodologies for TN and NTN Agreements: - Targeting a harmonized test system/test method for TN and NTN devices for FR1.

In this paper: the MSD testability issue, development of single OTA test system, harmonized test method for TN and NTN, as well as the OTA testing method for FR1+FR2 will be discussed in following sub-clauses.

2.1	MSD testability issue

The drawbacks of FR1 MSD conducted testing was mentioned during RAN4#116bis meeting. The key point is that FR1 conducted test method cannot capture the influence of antenna coupling. The real performance can’t be correctly verified via conducted test method. To solve this MSD testability issue, verifying MSD via OTA methodology instead of conducted method is a good approach.

Given that OTA testing requires longer test time and cost, RAN4 could also consider improving current conducted test approach. For example, the TE vendor could add a signal simulator that emulates antenna-coupling products and other interferers, and provide the downlink signal together with the simulated interferes to the UE Rx connectors.

Proposal 1：	To improve FR1 MSD testing, consider one of the following approaches:

- Apply OTA test method to replace conducted test method

- Improve the existing conducted test setup by introducing signal simulators that provide emulated antenna-coupling products and other interferers to UE Rx connectors

2.2	Single OTA test system development

During RAN4#116bis meeting, it is agreed that RAN4 should study the feasibility of developing a single system to cover the testing for different UE types/ features/ performance metrics and test cases. In our point of view, an integrated OTA system in 6G should at least consider following aspects:

- Different UE types have different requirement for the phantom. For example, smartphone testing requires hand phantom, head-worn XR device requires head phantom. And it hasn’t been studied whether some other UE form factors, i.e. laptops or FWA devices, require a phantom or not. Therefore, the single OTA test system in 6G era should support flexible phantom configuration.

Observation 1:	the single OTA test system in 6G era should support flexible phantom configuration.

- Different UEs may have different requirements for QZ size and AC size. For example, FR1 smartphones are appropriately tested using DFF test method, while FR2 UEs typically require IFF testing. Similar situation applies to NTN handheld UEs and NTN VSAT devices. Therefore, the single OTA test system in 6G era should support both IFF and DFF test methods to accommodate different device types.

Observation 2:	the single OTA test system in 6G era should support both IFF and DFF test methods to accommodate different device types.

- Reducing testing time has always been an important aspect in the development of OTA test methodology. Applying different measurement grid density for different testing scenarios is worth exploring in 6G. Hence, the single OTA system should consider supporting adaptable grid-density deployment.

Observation 3:	the single OTA test system in 6G era should consider supporting adaptable deployment of measurement grid density.

Proposal 2:	The single OTA test system in 6G era should consider supporting flexible phantom configuration; both IFF and DFF test methods, and adaptable deployment of measurement grid density to accommodate different device types and new features.

2.3	Harmonized OTA test methodologies for TN and NTN

In Rel-19, Partial Radiated Metrics (PRP/PRS) are defined for NTN handheld UE scenario 2 in [2]. As for TN handheld UE, basically Total Radiated Metrics (TRP/TRS) are applied. With the development of NTN, both PRP/PRS and TRP/TRS will be important performance metrics in OTA testing.

The harmonized OTA test methodologies for TN and NTN could consider supporting the verification of both TRS/TRS and PRP/PRS metric as a starting point.

Proposal 3：	The harmonized OTA test methodologies for TN and NTN could consider supporting the verification of both TRP/TRS and PRP/PRS metric as a starting point.

2.4	Pure OTA testing methodology for mixed FR1-FR2 operation

In 5G era, there have been some considerations on the testing of combined FR1 and FR2 configuration, such as providing FR1 link to the DUT when verifying the performance of the FR2 carrier. Taking DFF method as an example, following is the extract of the DFF description in 5.2.1.1 of [5]:

The key aspects of the DFF setup are: - Far-field measurement system in an anechoic chamber …… - For setups intended for measurements in NR CA mode with FR1 and FR2 inter-band NR CA, test setup provides NR FR1 link to the DUT. The NR FR1 link has a stable and noise-free signal without precise path loss or polarization control.

But the fact is that the conformance test for FR1+FR2 band configurations has never been executed in practice. The main reason is the absence of an applicable single test system that can simultaneously perform conducted measurements in FR1 and OTA measurements in FR2. Since FR2 bands is unable to be tested via conducted method, to solve such legacy testability issues, studying a pure OTA test methodology for mixed FR1-FR2 operation is required.

In 6G era, UL/DL spectrum decoupling is highly likely to be considered. Considering that FR1 UL carrier below 2.2G can guarantee the UL capability and provide good UL coverage, while FR2 DL carrier can provide enhanced throughput due to their larger bandwidth. A concurrent operation strategy with FR1 UL carrier and FR2 DL carrier may be supported by UE in the 6G era. From this perspective, a pure OTA test methodology for mixed FR1-FR2 operation is worth investigation.

Based on the consideration from both aspects mentioned above, it’s worthwhile to study the pure OTA test methodology for UE configuring mixed FR1-FR2 operation in 6G era.

Proposal 4:	RAN4 to study pure OTA test methodology for UE configuring mixed FR1-FR2 operation in 6G era.

3	Conclusion

In this paper, we submit following proposals on the consideration of OTA test methodology and testability issues in 6G:

Proposal 1：	To improve FR1 MSD testing, consider one of the following approaches:

- Apply OTA test method to replace conducted test method

- Improve the existing conducted test setup by introducing signal simulators that provide emulated antenna-coupling products and other interferers to UE Rx connectors

Observation 1:	the single OTA test system in 6G era should support flexible phantom configuration.

Observation 2:	the single OTA test system in 6G era should support both IFF and DFF test methods to accommodate different device types.

Observation 3:	the single OTA test system in 6G era should 1)	consider supporting adaptable grid-density deployment.

Proposal 2:		The single OTA test system in 6G era should consider supporting flexible phantom configuration; both IFF and DFF test methods, and adaptable deployment of measurement grid density to accommodate different device types and new features.

Proposal 3：	The harmonized OTA test methodologies for TN and NTN could consider supporting the verification of both TRP/TRS and PRP/PRS metric concurrently as a starting point.

Proposal 4:	RAN4 to study pure OTA test methodology for UE configuring mixed FR1-FR2 operation in 6G era.

Reference

[1]	R4-25114640 WF for [116bis][110] 6G testability and OTA

[2]	TR 38.870 Technical Specification Group Radio Access Network; Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) (Release 19)