3GPP TSG-RAN WG4 Meeting #117				                                                R4-2520141

Dallas, USA, November 17th – November 21st, 2025

Title:	Discussion on performance requirements of FR2-1 L3 measurement delay reduction by reducing Rx BSF

Source:	CATT

Agenda Item:	6.10.2.1

Document for:	Discussion

1 Introduction

In last meeting, RAN4 discussed the open issues on performance requirements of FR2-1 SSB based measurement delay reduction by optimizing Rx BSF. The agreements were captured in the approved WF [1].

In this contribution, we would like to discuss remaining issues and present our views.

2 Discussion

Issue 1: Whether to test FBS quitting

Proposals for whether to test FBS quitting were brought up in lasting meeting but no agreement was achieved. In our understanding, whether UE is still in FBS mode or quits from FBS mode depends on UE implementation after 90 s since the triggering conditions are met. Therefore we think it is okay not to test FBS quitting in the test.

Proposal 1: RAN4 not to test FBS quitting in the test.

Issue 2: Design of TCs for FBS

In the test for SSB based L3 measurements with FBS, one issue is how long should the test run. One option is the test time duration is longer than 90 s which verifies UE measurement behaviour within 90 s, and this aligns with the applicability requirements. However, in the legacy test cases for L3 measurements, the Event A3 is used to trigger the measurement reporting and no reportInterval is configured, which means UE only needs to report one measurement result. Moreover, the candidate values for reportInterval are shown below:

ReportInterval information element

-- ASN1START

-- TAG-REPORTINTERVAL-START

ReportInterval ::=                  ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960,

min1,min6, min12, min30 }

ReportInterval-v1900 ::=            ENUMERATED {ms20, ms60, ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240,

ms20480, ms40960, min1,min6, min12, min30 }

-- TAG-REPORTINTERVAL-STOP

-- ASN1STOP

Even configuring reportInterval with 120 ms in the test for FBS, it seems difficult to distinguish whether UE uses FBS or not for the measurements except for the first reporting. Therefore we suggest using short time duration that can cover the measurement with FBS and the first reporting for the FBS tests.

Proposal 2: Use short time duration that can cover the measurement with FBS and the first reporting for the FBS tests.

3 Conclusion

In this paper, we discussed some maintenance issues on performance requirements of FR2-1 SSB based measurement delay reduction by optimizing Rx BSF and the following proposals are provided:

Proposal 1: RAN4 not to test FBS quitting in the test.

Proposal 2: Use short time duration that can cover the measurement with FBS and the first reporting for the FBS tests.

4 References

- R4-2514801, WF on NR_RRM_Ph5_Part2, Moderator (CATT), RAN4#116bis.