3GPP RAN WG4 Meeting #117	R4-2520147

Dallas, USA, November 17th – November 21st, 2025

Title: 	Discussion on the RRM core requirements of LBCA enhancements

Source: 	CATT

Agenda item:	7.8.4

Document for:	Discussion

Introduction

In RAN#109 meeting, RAN approved a new work item on enhancement of low NR band CA via switching to enable dual downlink case [1]. The objectives are reproduced here:

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

RAN4 discussed this WI in RAN4#116bis meeting and some agreements were captured in the way forward [2]. In this contribution we will present our views of the impacts of enhanced LB-CA on RRM core requirements.

Discussion

Impacts on RRM requirements

According to the WID, the same deployment constraints as Rel-19 LB-CA WID are assumed, and the requirements defined in RRM and RF and the semi-static switching mechanism should be reused as much as possible. The new things here are more band combinations are considered for support of this dual downlink case, Case 3. The example band combinations are shown below as specified in TS 38.101-1, which are all FDD+SDL configurations:

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL19 |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n67 | N/A | 738 MHz – 758 MHz | SDL19 |

In order to identify the impacts of switching between Case 1 and Case 3, we would like to analyse the difference between Case 2 and Case 3. In Figure 1, the only difference between {case 1, case 2} switching and {case 1, case 3} switching is when UE stays on SDL carrier, UE can still perform DL receptions from FDD DL carrier. Therefore compared with {Case 1, Case 2} switching, {Case 1, Case 3} switching has less impacts on PCell-related behaviour.

Figure 1 - Difference between {case 1, case 2} switching and {case 1, case 3} switching

In last meeting, RAN4 agreed to reuse the RRM requirements defined for Rel-19 LB-CA WI as much as possible:

RAN4#116bis: Agreement: - It is RAN4 common understanding that the following RRM requirements are defined in Rel-19 LB-CA switching SDL SCell related RRM requirement; Index; Requirements. 1; Deactivated SDL SCell measurement. 2; Interruption due to deactivated SCell measurement. 3; Activated SDL SCell L3 measurement. 4; BFD/CBD for SDL SCell. 5; L1-RSRP/SINR for SDL SCell. 6; SDL SCell activation/deactivation. 7; TCI state switch delay. FDD PCell related RRM requirement; 8; PCell L3 measurement. 9; BFD/CBD for PCell. 10; L1-RSRP/SINR for PCell. 11; RLM for PCell. 12; TCI state switch delay. 13; PL-RS switch delay for PCell. - For Rel-20, following the WID, RAN4 to Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

As mentioned above, UE can still perform DL receptions from FDD DL carrier when it stays on SDL carrier. Therefore the following RRM requirements for PCell defined in Rel-19 LB-CA WI do not apply for the UE with dual downlink reception capability and the legacy requirements for non-LB-CA WI apply:

| FDD PCell related RRM requirement | 8 | PCell L3 measurement |
| --- | --- | --- |
|  | 9 | BFD/CBD for PCell |
|  | 10 | L1-RSRP/SINR for PCell |
|  | 11 | RLM for PCell |
|  | 12 | TCI state switch delay |
|  | 13 | PL-RS switch delay for PCell |

Proposal 1: The RRM requirements for PCell defined in Rel-19 LB-CA WI do NOT apply for the UE with dual downlink reception capability, e.g.,

- PCell L3 measurement

- BFD/CBD for PCell

- L1-RSRP/SINR for PCell

- RLM for PCell

- TCI state switch delay

- PL-RS switch delay for PCell

Proposal 2: The legacy requirements for non-LB-CA UE apply for PCell requirements in Rel-20 LB-CA WI.

For the RRM requirements for SDL SCell, the following requirements may be impacted and analysis is required one by one:

| SDL SCell related RRM requirement | Index | Requirements |
| --- | --- | --- |
|  | 1 | Deactivated SDL SCell measurement |
|  | 2 | Interruption due to deactivated SCell measurement |
|  | 3 | Activated SDL SCell L3 measurement |
|  | 4 | BFD/CBD for SDL SCell |
|  | 5 | L1-RSRP/SINR for SDL SCell |
|  | 6 | SDL SCell activation/deactivation |
|  | 7 | TCI state switch delay |

In last meeting, RAN4 has agreed that no new RRM requirement is expected for deactivated SCell measurement period requirements since the switching pattern does not apply for the deactivated SCell. For other requirements, considering that UE cannot receive reference signals from SDL SCell when UE stays on PCell, the requirements defined for Rel-19 LB-CA SCell can be reused.

Proposal 3: For Rel-20 LB-CA SCell, RAN4 to reuse the following RRM requirements defined for Rel-19 LB-CA SCell:

- Interruption due to deactivated SCell measurement

- Activated SDL SCell L3 measurement

- BFD/CBD for SDL SCell

- L1-RSRP/SINR for SDL SCell

- SDL SCell activation/deactivation

- TCI state switch delay

Conclusions

This contribution discussed the impacts of LBCA on RRM performance requirements and some proposals are provided:

Proposal 1: The RRM requirements for PCell defined in Rel-19 LB-CA WI do NOT apply for the UE with dual downlink reception capability, e.g.,

- PCell L3 measurement

- BFD/CBD for PCell

- L1-RSRP/SINR for PCell

- RLM for PCell

- TCI state switch delay

- PL-RS switch delay for PCell

Proposal 2: The legacy requirements for non-LB-CA UE apply for PCell requirements in Rel-20 LB-CA WI.

Proposal 3: For Rel-20 LB-CA SCell, RAN4 to reuse the following RRM requirements defined for Rel-19 LB-CA SCell:

- Interruption due to deactivated SCell measurement

- Activated SDL SCell L3 measurement

- BFD/CBD for SDL SCell

- L1-RSRP/SINR for SDL SCell

- SDL SCell activation/deactivation

- TCI state switch delay

References

[1] RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Moderator (Huawei), RAN#109.

[2] R4-2514824, WF for [116bis][224] NR_LBCA_Sw_enh_RRM, Moderator (Samsung), RAN4#116bis.