3GPP RAN WG4 Meeting #117	R4-2520148

Dallas, USA, November 17th – November 21st, 2025

Title: 	Discussion on inter-vendor training collaboration for two-sided AIML models

Source: 	CATT

Agenda item:	7.9.3

Document for:	Discussion

Introduction

In RAN#109 meeting, RAN approved the new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 [1] with the following objectives for RAN4:

… Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Data collection [RAN2]: - Further study and specify if deemed necessary, standards-based UE data collection at least for UE-side model training in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 based on SA WG feedback and SA2 study on data collection Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In RAN4#116bis meeting, RAN4 have started to discuss this WI and agreements are captured in the WF [2]. In this contribution, we will further discuss the open issues on inter-vendor training collaboration and present our views.

Discussion

In RAN4#116bis meeting, RAN4 discussed the relation between RAN1 Direction C and RAN4 Option 3 and reached the agreement that they are essentially the same:

Agreement: Relation between RAN1 and RAN4: • The test decoder of option 3 is the same as the reference decoder of direction C - Test decoder of option 3 will be specified by RAN4 - If RAN1 develops the reference decoder for direction C which is different from RAN4’s, further discussion across RAN4 and RAN1 is needed. • The reference encoder is the same for both option 3 and direction C • The option 3 reference encoder structure is the same as the encoder structure for direction A option 3a-1. - Reference encoder of option 3 will be specified in RAN4.

In addition, a detailed plan to develop test decoder and reference encoder is provided for guidance:

Agreement Steps to develop the test decoder and reference encoder: - Step 1: Create dataset - Step 1-1: Align on configurations for mixed dataset construction, taking factors such as scalability over Tx port number and subband size and payload size into account - Step 1-2 Decide how align and on which metrics can be used for evaluating dataset alignment, if any - Step 1-3 Companies create datasets - Step 1-4 Identify which company datasets to include in the mixed dataset(s) - Step 1-5 Create new dataset(s) - Step 2: Align on the model backbone, model hyperparameters and the scalability solution. - Step 2-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution. - This can be in parallel with step 1. - Transformer is agreed as baseline for backbone - Same backbone for test decoder and reference encoder - Step 2-2: Preliminary selection of structures, taking into account complexity - [Step 2-2a Align on training parameters] - Step 2-3: Agree on metrics and steps for evaluation - Step 2-4: Check on performance alignment -> see simulation results from contributing companies. - Step 2-5: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed. - Step 2-6: Select one or more model structures considering complexity and robustness and decide how many models. - Step 2-7: Select one or more scalability solutions. This step can be done in parallel with Steps 2-2 to 2-4. - Step 3: Specify the model for the following work in WID. Where and how to specify may need further discussion. Note: At this stage, models using the agreed structures, dataset, scalability have already been created by most companies - Step 3-1 Decide how to select from the trained model(s) from each company - Step 3-2: Freeze model(s) - Step 3-3: Agree on how and where to capture the models, the dataset and relevant information

According to the plan, RAN4 need to discuss how to create a new mixed dataset that takes scalability into account and the backbone of the test decoder.

Dataset creation

In study phase, RAN4 created a mixed dataset to evaluate the performances of encoder-decoder pair with different backbones, such as CNN (normal and simplified complexity), MLP and transformer. The procedure used to create that mixed dataset is summarized below in a high-level manner:

Step 1: Align system simulation assumptions [3] to generate dataset by each company.

Step 2: Compare the simulation results contributed by each company and identify any misalignment among the datasets contributed by companies;

Step 3: The dataset(s) that are aligned are adopted to get the mixed dataset.

In general, the above procedure can be reused to generate the mixed dataset with scalability considered. The only issue is how to consider scalability. Regarding the aspects that need to be scalable, RAN1 reached some agreements in RAN1#120 meeting:

RAN1#120: Agreement: For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values. - Number of Tx ports - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports) - [N1, N2, P] = [4,4,2] (32 CSI-RS ports) - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports) - CSI feedback payload size - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z. - Bandwidth & subband size - All subbands - A subset of all the subbands.

According to RAN1 discussion, scalability of number of Tx ports, CSI feedback payload size and bandwidth & subband size need to be considered. Therefore a straightforward way of generating the dataset is to change the configurations of number of Tx ports and bandwidth & subband size, and under each configuration some data are obtained. On the candidate values for the number of Tx ports and subband size, RAN4 can select a few typical values to start the dataset creation procedure. For example, the following two configurations can be used for Tx ports:

[N1, N2, P] = [4, 4, 2] (32 CSI-RS ports), [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports).

And 6 subbands and 12 subbands can be considered for subband size.

Proposal 1: Reuse the aligned simulation assumption in R4-2414447 for obtaining training dataset as much as possible.

Proposal 2: Data obtained under different configurations are mixed first when companies create their dataset. Configurations include:

- Number of Tx ports;

- Bandwidth & subband size.

Proposal 3: For generating scalable dataset, consider the following configurations as a starting point:

- For Tx ports: [N1, N2, P] = [4, 4, 2] (32 CSI-RS ports), [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports).

- For subband size, 6 and 12 subbands.

Regarding the scalability of CSI feedback payload, it has impacts on output layer of encoder and input layer of decoder but it has nothing to do with system-level simulation assumption. Therefore we will discuss the scalability of CSI feedback payload in “Test decoder backbone” section.

Test decoder backbone

In last meeting, RAN4 has agreed transformer is assumed as the baseline but the detailed structure needs further discussion. Considering that RAN1 has studied transformer in SI phase and in order to leverage the RAN1 outcomes as much as possible, we think the example transformer structure illustrated in RAN1#120 meeting chair note [4] for Case 0 with spatial-frequency domain inputcan be used as a starting point:

RAN1#120: Agreement For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure as one example structure for study purpose, Encoder description: Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer); Number of tokens Feature dimension of each token Output dimension dmodel per token. Positional encoding; Number of tokens positions Dimension of positional encoding for each token position: dmodel. Transformer blocks; Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice. Output linear layer; Reshape the matrix to a vector Output dimension zdim. Quantization; Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg - Total payload size for Vector quantization = Number of segments * bits per segment.

Proposal 4: RNA4 to use the RAN1 example transformer structure for Case 0 with spatial-frequency domain input specified in RAN1#120 meeting can be used as a starting point.

In order to accommodate the scalability of input/output data, compared with the CNN structure used in SI phase, some adjustments need to be made to the input layer and output layer.

Regarding the input layer, data with different number of Tx ports and subband sizes will be fed into encoder. There are two options to fulfill the purpose of accepting data with different values in some dimensions:

Option 1: Adopt adaption layer.

Option 2: Adopt zero padding.

RAN4 can further discuss which option can be used to accommodate the scalability.

Proposal 5: To accommodate the scalability of input data, RAN4 consider two options for the input layer:

Option 1: Adopt adaption layer.

Option 2: Adopt zero padding.

As mentioned above, the scalability of CSI feedback payload has impacts on output layer of encoder and input layer of decoder. At first, the supported CSI feedback payload bits should be decided by RAN1 without final agreement, but RAN4 can select a few candidate values to start the test decoder development. For example, one value in small, medium, and large payload regions respectively, e.g., 64, 128, 256 bits, can be considered.

Observation 1: Discussion on supported CSI feedback payload bits is in RAN1 scope and no final agreement made yet.

Proposal 6: RAN4 to select one value in small, medium, and large payload regions respectively to start the test decoder development, e.g., 64, 128, 256 bits.

Similarly, adjustment to output layer of encoder should also be made to accommodate the scalability of CSI feedback payload. In our opinion, adaptation layer can be a good option, i.e., multiple outlays are used to generate CSI feedback with corresponding bit lengths.

Proposal 7: Use adaptation layers to generate CSI feedback with corresponding bit lengths.

Conclusions

This contribution discussed the issues on inter-vendor training collaboration for AI/ML enabled CSI compression, and some observations and proposals are provided:

Proposal 1: Reuse the aligned simulation assumption in R4-2414447 for obtaining training dataset as much as possible.

Proposal 2: Data obtained under different configurations are mixed first when companies create their dataset. Configurations include:

- Number of Tx ports;

- Bandwidth & subband size.

Proposal 3: For generating scalable dataset, consider the following configurations as a starting point:

- For Tx ports: [N1, N2, P] = [4, 4, 2] (32 CSI-RS ports), [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports).

- For subband size, 6 and 12 subbands.

Proposal 4: RNA4 to use the RAN1 example transformer structure for Case 0 with spatial-frequency domain input specified in RAN1#120 meeting can be used as a starting point.

Proposal 5: To accommodate the scalability of input data, RAN4 consider two options for the input layer:

Option 1: Adopt adaption layer.

Option 2: Adopt zero padding.

Observation 1: Discussion on supported CSI feedback payload bits is in RAN1 scope and no final agreement made yet.

Proposal 6: RAN4 to select one value in small, medium, and large payload regions respectively to start the test decoder development, e.g., 64, 128, 256 bits.

Proposal 7: Use adaptation layers to generate CSI feedback with corresponding bit lengths.

References

[1] RP-252445, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, Qualcomm, RAN#109.

[2] R4-2514614, WF on R20 NR_AIML_air_Ph2, Ericsson, RAN4#116bis.

[3] R4-2414447, Alignment of model training and data set parameters for AI/ML-based CSI compression, Nokia, RAN4#112.

[4] Chair notes RAN1#120 eom1, RAN1#120.