3GPP RAN WG4 Meeting #117	R4-2520149

Dallas, USA, November 17th – November 21st, 2025

Title: 	Discussion on the RRM core requirements of AIML air ph2

Source: 	CATT

Agenda item:	7.9.4

Document for:	Discussion

Introduction

In RAN#109 meeting, RAN approved the new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 [1] with the following objectives for RAN4:

… Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Data collection [RAN2]: - Further study and specify if deemed necessary, standards-based UE data collection at least for UE-side model training in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 based on SA WG feedback and SA2 study on data collection Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In RAN4#116bis meeting, RAN4 discussed this WI and agreements are captured in the way forward [2]. In this contribution, we will discuss the potential impacts of AI/ML based CSI compression on RRM core requirements and present our views.

Discussion

Regarding the potential impacts on RRM requirements, the discussions and agreements in Rel-19 AIML air ph1 can be referred to. For example, RAN4 agreed to define AI/ML functionality activation delay requirement for BM and the agreement is as follows [3]:

RAN4#116: Issue 2-4: Activation delay Agreement: - UE has to be ready to start measurements for inference after sending RRC reconfiguration complete - Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay - This applies to all reporting schemes

In RAN4#116bis meeting, the recommended WF [4] is provided for further discussion:

Issue 3-1-2: Functionality activation delay - Recommended WF - Discuss whether the following can be agreed: - There is a need to wait for RAN2 design before making a final decision. - Assuming the same RAN2 design as for CSI prediction, then UE has to be ready to start measurements for inference after sending RRC reconfiguration complete.

Therefore if RAN2 agree to reuse the signalling framework for applicability report in Rel-19 AI/ML based BM, then the corresponding agreement can also apply for AI/ML based CSI compression. But definitely the assumption in RAN4 can be revisited if RAN2 design a different signalling framework for applicability report.

Proposal 1: For AI/ML functionality activation delay, the following agreement achieved in R19 AI/ML BM applies if RAN2 agree to reuse the signalling framework for applicability report:

-	UE has to be ready to start measurements for inference after sending RRC reconfiguration complete.

Note: This agreement can be revisited if RAN2 design a different signalling framework for applicability report.

In addition, RAN4 may need to define LCM requirements for performance monitoring for this use case, but this needs more RAN1/2 design. Therefore, RAN4 can discuss how to define requirements for performance monitoring in a later phase. But in last meeting, a set of LCM requirements is provided to facilitate discussion:

Issue 3-1-1: LCM requirement - Recommended WF - Discuss the following: - Should the Rel-20 CSI prediction LCM requirements be based on Rel-19 as a baseline ? - Rel-19 Beam Management, or Rel-19 CSI prediction ? - For performance monitoring, should requirements be placed on LCM decisions ? - Should RAN4 discuss performance metrics for monitoring ? - Set of potential requirements for discussion: - Activation - Deactivation -> Deprioritize ? - Switching - Fallback to non-AI -> Deprioritize ? - Performance monitoring - Reporting delay - Accuracy - Considering possibly averaging and connection with reporting time

Regarding the baseline for CSI compression, we do not have strong preference since each use case has its dedicated procedures, and therefore analysis is required and adaption will be made in the end. On the performance metrics for monitoring, based on the discussion situation in Rel-19, performance metrics for monitoring will be discussed in RAN1.

Proposal 2: RAN4 do not discuss the performance metrics for monitoring which will be discussed by RAN1.

For the listed potential requirements for discussion, we generally agree to discuss how to define requirements for:

      - Activation

      - Switching

      - Performance monitoring

        - Reporting delay

        - Accuracy

And AI/ML functionality deactivation is deprioritized. However, for the fallback to non-AI procedure, i.e., when AI/ML model performance is worse than a certain level and UE is indicated by network to deactivate AI/ML models and fallback to non-AI methods, we think UE needs to be ready to report CSI using legacy method in specified time duration.

Proposal 3: RAN4 to discuss how to define requirements for:

      - Activation

      - Switching

      - Performance monitoring

        - Reporting delay

        - Accuracy

And AI/ML functionality deactivation is deprioritized.

Proposal 4: RAN4 to discuss how to define LCM requirements for fallback to non-AI procedure to ensure that UE is ready to report CSI using legacy methods in specified time duration.

Conclusions

This contribution discussed the potential impacts of AI/ML based CSI compression on RRM core requirements and some observations and proposals are provided:

Proposal 1: For AI/ML functionality activation delay, the following agreement achieved in R19 AI/ML BM applies if RAN2 agree to reuse the signalling framework for applicability report:

-	UE has to be ready to start measurements for inference after sending RRC reconfiguration complete.

Note: This agreement can be revisited if RAN2 design a different signalling framework for applicability report.

Proposal 2: RAN4 do not discuss the performance metrics for monitoring which will be discussed by RAN1.

Proposal 3: RAN4 to discuss how to define requirements for:

      - Activation

      - Switching

      - Performance monitoring

        - Reporting delay

        - Accuracy

And AI/ML functionality deactivation is deprioritized.

Proposal 4: RAN4 to discuss how to define LCM requirements for fallback to non-AI procedure to ensure that UE is ready to report CSI using legacy methods in specified time duration.

References

[1] RP-252445, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, Qualcomm, RAN#109.

[2] R4-2514614, WF on R20 NR_AIML_air_Ph2, Ericsson, RAN4#116bis.

[3] R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated, RAN4#116.

[4] R4-2514530, Topic summary for [116bis][123] NR_AIML_air_Ph2, Moderator (Ericsson), RAN4#116bis.