3GPP TSG-RAN WG4 Meeting #117				                                              R4-25120154

Dallas, USA, November 17th – November 21st, 2025

Title:	Discussion on 6G AIML based RAN4 use cases

Source:	CATT

Agenda Item:	8.8.2

Document for:	Discussion

1 Introduction

In RAN4#116bis meeting, RAN4 driven AI/ML use cases for 6G radio were discussed and the agreements are captured in the way forward [1]. In this contribution, we will continue to discuss the potential RAN4-led AI/ML use cases in 6G radio.

2 Discussion

As artificial intelligence (AI) and machine learning (ML) have been developing rapidly, they are applied in more and more industries to improve efficiency and reduce operation cost, such as AI-assisted driving in industrial areas and intelligent robots/drones in factories. Hence, it seems necessary to introduce AI/ML functions into the design of 6G wireless communication system. In Rel-18, 3GPP RAN firstly studied AI/ML based use cases and sequentially in Rel-19 spec some physical mechanisms and performance requirements are specified for AI/ML based CSI prediction, beam management and positioning. However, the convergence of AI/ML and wireless communication system started in Rel-18 in NR, which was quite late since the framework of NR system was already stable and was not easy to be optimized for adapting AI/ML-assisted use cases. In 6G radio, AI/ML functions should be considered at the very beginning of the new framework design.

From RAN4 perspective, it is worthy of considering AI/ML in each technical scope as much as possible, i.e., in RRM, RF and Demod scope. In RAN4#116bis meeting, multiple use cases were brought up for each RAN4 scope and in this contribution we will analyse those use cases and present our views.

  - AI/ML based RRM

In last meeting, more than ten AI/ML based RRM use cases were proposed as summarized in the table below:

Table 1 AI/ML based RRM use cases proposed in RAN4#116bis

| Index | use cases | Model location (Training method) |
| --- | --- | --- |
|  | L1 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Spatial domain (inter-cell) - Time and spatial domain (inter-cell) | NW and UE sided model (Offline training) |
|  | L3 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Time and freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) - Freq. domain prediction for N carriers single cell | NW and UE sided model (Offline training) |
|  | L3 cell measurement: - Time domain (intra-cell) - Freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) | NW and UE sided model (Offline training) |
|  | Multi-Domain L3 level Prediction for Measurement-Gap Reduction | UE sided model (Offline training) |
| 5a (5+6 combined) | Dynamic Adaptation of Measurement Procedure | UE sided model |
| 7a (7+8 combined) | Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model |
|  | AI/ML based RLM/BFD/CBD, LTM by measurement prediction | UE-sided model |
|  | AI/ML based SCell related enhancement by measurement prediction | UE-sided model |
|  | AI-based prediction for enhanced s-measure mechanism | UE-sided model |
|  | AI/ML based AGC through temporal/frequency domain prediction | UE-sided model |
|  | AI/ML based DL sync through temporal/frequency domain prediction | UE-sided model |
|  | AI abstraction of the wireless environment to facilitate data scheduling and network power saving | NW-sided model |

According to the use cases in the table, it seems that many use cases were brought up based on the current 5G RRM measurement framework (e.g., differentiation of L1/L3 measurements) and mechanisms, such as L1/L3 measurement, LTM and SCell management, etc. However, there is a dedicated agenda for 6G RRM discussion where a new 6G RRM measurement framework that is different from 5G one may be designed. Similarly, the framework of SCell management in 6G is also unknown at current stage.

Observation 1: A dedicated agenda is set for 6G RRM, where a new 6G RRM measurement framework that is different from 5G may be designed.

Observation 2: SCell management in 6G is studied in RAN1 which may also be different from 5G.

With these observations, we think it is too early to agree on the use cases assuming 6G RRM measurement framework and SCell management scheme are the same as the ones in 5G.

Proposal 1: It is too early to agree on the use cases assuming 6G RRM measurement framework and SCell management scheme are the same as 5G.

But we also found a few attractive use cases among the proposed ones. Regarding use case 4) and 5a), the motivation is to reduce the impacts of measurement gap with multi-domain prediction or dynamic adaptation. As we all know, there are more than 20 measurement gap patterns and different measurement gap types are supported, e.g., pre-configured measurement gap and network controlled small gap, in NR RRM spec, which makes both RRM spec and the UE RRM measurement behaviour quite complicated. Therefore it is beneficial to find some solutions to simplify measurement gap based RRM measurement requirements and AI/ML could be a good candidate. From our perspective, though more details about these two AI/ML based use cases are required, it is meaningful to follow the motivation of these two use cases for further analysis.

| 4) | Multi-Domain L3 level Prediction for Measurement-Gap Reduction | UE sided model (Offline training) |
| --- | --- | --- |
| 5a (5+6 combined) | Dynamic Adaptation of Measurement Procedure | UE sided model |

Observation 3: Measurement gaps and gap-based RRM measurements in 5G are complicated and simplification is required in 6G.

Proposal 2: RAN4 further analyse the pros and cons of use case 4)/5a) which may help reduce the complexity of measurement gap-based RRM measurements and impacts on system performance.

Regarding use case 14) which is high-level idea, we would like to provide more details to elaborate the idea and the potential use case(s) that align with this high-level idea.

| 14) | AI abstraction of the wireless environment to facilitate data scheduling and network power saving | NW-sided model |
| --- | --- | --- |

One use case is AI based handover. When UE deployed with AI/ML models moves in a cell, UE can measure serving and neighbour cells and predict if handover will happen in a near future. If handover will happen with a relative high probability, UE can report the handover with target cell information (e.g., ID) to network. The model input can be measurement results of serving and neighbour cells and the model output can be target cell information (e.g., ID).

Proposal 3: RAN4 to study the AI/ML based handover.

Another use case is that gNB with AI/ML models can leverage the information of wireless environment to facilitate data scheduling and reduce measurement efforts/air interface overhead.

In legacy RATs including NR, RRM highly relies on the interworking between base stations and UEs, in which case the amount of measurement and reporting performed by UEs is quite high, consuming much UE power and also leading to high overheads for both measurement configuration and reporting in air interface.

Observation 4: In legacy RATs, DL/UL scheduling/handover highly depends on the UE measurement reports.

Leveraging the AI/ML capabilities, 6G base stations, which have fixed locations, can model the wireless environment in the proximity area to themselves by learning from the daily information they obtain during communications. Meanwhile, the AI/ML model can continuously develop and update itself to adapt to a changing environment and thereby polish the model of wireless environment. Based on this model of wireless environment, 6G base stations can schedule UEs with optimized schemes, instead of highly relying on lots of measurement results reported by UEs. Ideally, one AI/ML model can be used to assist in the scheduling decisions of both downlink and uplink directions.

In addition, it is possible that new RRM requirements/performance metrics can also be defined based on the outputs of this AI/ML model.

Observation 5: Outputs abstracted by AI/ML models from wireless environment help reduce the overheads of measurement configuration and UE measurement reporting in the air interface.

Furthermore, abstraction of the wireless environment, i.e., the outputs of AI/ML model, in different time instances may be different in busy time duration, or they could be quite stable during some idle time from a system perspective, in which case AI/ML models deployed in base stations do not need frequently online/offline retraining or online fine-tuning. Base stations can still implement both downlink and uplink scheduling with stable schemes based on the output results of the AI/ML models with non-degraded or even improved system-level performance. Meanwhile, base stations can stop the online retraining or fine-tuning of AI/ML models to save energy, thereby reducing the power consumptions in the 6G radio. Therefore we suggest studying the potential usages of AI/ML based wireless environment abstraction in the scenario of data scheduling and network power saving in the 6G radio study in RAN4.

Observation 6: Outputs abstracted by AI/ML models from wireless environment could be beneficial to reduce the network power consumption while maintaining or improving the system-level performance.

Proposal 4: RAN4 to study the abstraction of the wireless environment by base station AI/ML models which can be used to facilitate data scheduling and network power saving in 6G radio.

  - AI based RF

AI/ML-DPD/DPoD

In many scenarios, increasing transmitting power will bring up many benefits, such as enlarging coverage and providing a high SNR which is high enough to enable higher order modulation schemes. However, this method probably would cause a consequence that power amplifiers work in the nonlinear region. When RF power amplifiers operate in near maximum output (within nonlinear region), both amplitude distortions and phase distortion increase, leading to severe adjacent channel interference and elevated error vector magnitude. These issues in return will force the system to reduce power levels, sacrificing the overall performance. Currently it is difficult for conventional solutions to solve this issue with balanced complexity and performance. To overcome this issue, AI/ML-based compensation techniques, e.g., AI/ML based DPD/DPoD, can be considered to improve the link performance.

In RAN4#116bis meeting, AI/ML based DPD/DPoD has been initially discussed and the following agreements were achieved to facilitate discussion:

Agreement: - The scope of potential RAN4-led AI use case, AI-DPD/DPoD, will be further discussed in the next meeting. The candidate sub-use cases include - UL: - UE: AI-DPD - NW: AI-DPoD - DL: - NW: AI-DPD - UE: AI-DPoD - To facilitate the 6G AI scope discussion, the interested companies are encouraged to timely provide more analysis, including the performance benefit, spec impact, the assumption behind their analysis, on the specific sub-use case(s).

At first, a clarification is required, which is even for the same link direction, the DPD and DPoD are decoupled, i.e., distortion processing at one side can be totally transparent to the other side. Therefore there are four options to be considered for study.

In Figure 1, we provided preliminary evaluation results, i.e., ACPR, for both non-DPD and AI-DPD in DL direction, which shows some performance improvements.

Figure 1 Evaluation performance comparison between non-DPD and AI-DPD

However, regarding how to select options for study among four options, we think a high-level principle is whether the selected option has spec impacts. If no spec impacts are identified, we do not think the corresponding option needs to be discussed in RAN4 even if performance gain is observed.

Proposal 5: RAN4 should consider the spec impacts of the selected AI based RF use cases when making down- selection. Use cases have no spec impacts do not need discussion in RAN4.

  - AI based Demod

In last meeting, three AI/ML-based demodulation use cases were proposed and summarized below:

Use Case#2: AI-based channel estimation Sub-use case; Model location; Views. AI-based channel estimation with DMRS reduction; NW-sided model for PUSCH UE-sided model for PDSCH; Companies show interests, with numerical results (1) Samsung (NW-sided) Companies show interests, without numerical results (2) Ericsson (both UE and NW-sided), Nokia (UE-sided). AI-based channel estimation with CSI-RS reduction; UE-sided model; Companies show interests, without numerical results (2) Ericsson, Huawei. Use Case#3: AI-SRS-assisted channel reconstruction Sub-use case; Model location; Views. AI-SRS-assisted channel reconstruction for (a) SRS IL imbalance, (b) SRS aging; Case 1: NW-side model-based Case 2: Two-sided model-based approach; Companies show interests, with numerical results (2) Huawei, Samsung (Case-1 and Case-2) Companies show interests, without numerical results (1) OPPO. Use Case#4: AI-enabled PRACH receiver Sub-use case; Model location; Views. AI-enabled PRACH receiver; NW-side model; Companies show interests, with numerical results (1) Samsung.

Regarding use case#2 AI-based channel estimation, the common understanding is this use case should be studied in RAN1, and therefore RAN4 work should be triggered by RAN1 via LS. Before any LS(s) from RAN1 are received, no discussion is expected in RAN4.

For use case#3 and use case #4, it seems that both use cases can be further discussed. However, the principle of use case selection is that the balance between cost and performance gain should be ensured. In other words, the performance improvements provided by selected use cases deserve the efforts of introducing AI/ML function into Demod.

Proposal 6: Balance between the efforts on study/specification and performance gain should be ensured when RAN4 select AI based Demod.

3 Conclusion

In this paper, we provided our views on potential RAN4 driven AI/ML use cases in 6G radio, and the following observations and proposals are given:

AI/ML based RRM

Observation 1: A dedicated agenda is set for 6G RRM, where a new 6G RRM measurement framework that is different from 5G may be designed.

Observation 2: SCell management in 6G is studied in RAN1 which may also be different from 5G.

Proposal 1: It is too early to agree on the use cases assuming 6G RRM measurement framework and SCell management scheme are the same as 5G.

Observation 3: Measurement gaps and gap-based RRM measurements in 5G are complicated and simplification is required in 6G.

Proposal 2: RAN4 further analyse the pros and cons of use case 4)/5a) which may help reduce the complexity of measurement gap-based RRM measurements and impacts on system performance.

Proposal 3: RAN4 to study the AI/ML based handover.

Observation 4: In legacy RATs, DL/UL scheduling highly depends on the UE measurement reports.

Observation 5: Outputs abstracted by AI/ML models from wireless environment help reduce the overheads of measurement configuration and UE measurement reporting in the air interface.

Observation 6: Outputs abstracted by AI/ML models from wireless environment could be beneficial to reduce the network power consumption while maintaining or improving the system-level performance.

Proposal 4: RAN4 to study the abstraction of the wireless environment by base station AI/ML models which can be used to facilitate data scheduling and network power saving in 6G radio.

AI/ML based RF

Proposal 5: RAN4 should consider the spec impacts of the selected AI based RF use cases when making down- selection. Use cases have no spec impacts do not need discussion in RAN4.

AI based Demod

Proposal 6: Balance between the efforts on study/specification and performance gain should be ensured when RAN4 select AI based Demod.

- References

[1] R4-2514637, WF on 6G AI, Samsung, RAN4#116bis.

[2] R4-2514636, Topic Summary for [116bis][107] 6G AI, Moderator (Samsung), RAN4#116bis.