3GPP TSG-RAN4 Meeting #117	R4-2520194

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 3053 | rev | - | Current version: | 19.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to R19 38.101-1 to introduce HPUE MSD simplification for UL configurations including intra-band ULCA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Solutions Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Complete HPUE MSD simplification for the missing 1UL2CC, 2UL3CC configurations when the UL band with the intra-band ULCA configuration is a TDD band |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding section 7.3A.2.3.3 to cover delta MSD for 1UL 2CC HPUE MSD Adding section 7.3A.2.3.4 to cover delta MSD for 2UL 3CC HPUE MSD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HPUE MSD simplification is not complete and analysis in TP/TRs will still be needed for the missing 1UL2CC, 2UL3CC configurations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

##### 7.3A.2.3.2	PC2 and PC1.5 MSD requirements with look-up tables for two-band or three-band DL CA with two-band UL CA with one CC per UL band

The PC2 and the PC1.5 MSD requirements with look-up tables for two-band and three-band DL CA reference sensitivity exceptions (MSD) due to 2UL CA with one CC per UL band intermodulation interference shall apply when the following criteria’s are met:

-	A PC3 reference sensitivity exception requirement is specified either in Table 7.3A.5-1, or in Table 7.3A.5-2, and,

-	PC2 or PC1.5 two-band UL CA for a total of 2Tx or 3Tx and 1CC in each UL band is specified as a valid two-band UL CA configuration in Table 5.5A.3.1-1a ~ Table 5.5A.3.1-1o, or in Table 5.5A.3.2-1a ~ Table 5.5A.3.2-1c, and,

-	The PC2 or PC1.5 MSD is caused by the same uplink/downlink configurations as in case of PC3 MSD, and,

-	One of the constituent uplink bands is neither band n46, band n96 nor band n102.

For these cases, and where in the following PCx denotes either PC2 or PC1.5, the PCx MSD due to 2UL CA intermodulation interference is specified as:

PCx MSD = PC3 MSD + MSD

where,

-	PCx MSD is the reference sensitivity exception specified for PC2 or PC1.5 with 2UL band CA for a total of 2Tx or 3Tx and with 1UL CC in each UL band,

-	PC3 MSD is the reference sensitivity exception specified for PC3 in Table 7.3A.5-1, or in Table 7.3A.5-2,

-	MSD values are specified in Table 7.3A.2.3.2-1 output columns denoted “MSDmax 6, 9, 12, 15, 18, 24, 30”. These apply for the same uplink/downlink configurations as those specified for the minimum PC3 MSD requirements in Table 7.3A.5-1, or in Table 7.3A.5-2, and,

-	The correspondence between the MSDmax specified in Table 7.3A.2.3.2-1, the IMD order and PCx MSD is specified in Table 7.3A.2.3.2-2,

Table 7.3A.2.3.2-1: MSD per MSDmax look-up table for MSD due to 2UL CA intermodulation interference

| PC3 MSD (dB) | MSDmax / MSD(dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 0.1≤ PC3 MSD <0.5 | 1.2 | 2.4 | 3.9 | 5.8 | 7.9 | 13.2 | 18.8 |
| 0.5≤ PC3 MSD <1.0 | 1.7 | 3.5 | 5.4 | 7.5 | 10.0 | 15.6 | 21.4 |
| 1.0≤ PC3 MSD <2.0 | 2.8 | 4.8 | 7.1 | 9.6 | 12.4 | 18.0 | 24.1 |
| 2.0≤ PC3 MSD <3.0 | 3.5 | 5.7 | 8.2 | 10.9 | 13.7 | 19.5 | 25.5 |
| 3.0≤ PC3 MSD <4.0 | 3.9 | 6.3 | 8.9 | 11.7 | 14.5 | 20.5 | 26.5 |
| 4.0≤ PC3 MSD <5.0 | 4.3 | 6.8 | 9.4 | 12.4 | 15.2 | 21.2 | 27.2 |
| 5.0≤ PC3 MSD <6.0 | 4.5 | 7.1 | 9.9 | 12.8 | 15.8 | 21.7 | 27.7 |
| 6.0≤ PC3 MSD <7.0 | 4.8 | 7.6 | 10.4 | 13.3 | 16.2 | 22.3 | 28.2 |
| 7.0≤ PC3 MSD <8.0 | 5.0 | 7.8 | 10.7 | 13.6 | 16.6 | 22.5 | 28.5 |
| 8.0≤ PC3 MSD <9.0 | 5.2 | 8.1 | 11.0 | 14.0 | 16.9 | 22.8 | 28.8 |
| PC3 MSD ≥9.0 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |

Table 7.3A.2.3.2-2: MSDmax correspondence look-up table for IMD order and PCx MSD

| IMD order | MSDmax |  |
| --- | --- | --- |
|  | PC2 MSD | PC1.5 MSD |
| IMD2 | 6 | 12 |
| IMD3 | 9 | 18 |
| IMD4 | 12 | 24 |
| IMD5 | 15 | 30 |

As an exception, for cases where:

-	The PC2 MSD is specified in Table 7.3A.5-1a or in Table 7.3A.5-2a, and,

-	The PC3 MSD is not specified in Table 7.3A.5-1 or in Table 7.3A.5-2, and,

-	The PC1.5 MSD is not specified in Table 7.3A.5-1b or in Table 7.3A.5-2b, and,

-	PC2 or PC1.5 two-band UL CA for a total of 2Tx or 3Tx and 1CC in each UL band is specified as a valid two-band UL CA configuration in Table 5.5A.3.1-1a ~ Table 5.5A.3.1-1o, or in Table 5.5A.3.2-1a ~ Table 5.5A.3.2-1c, and,

-	One of the constituent uplink bands is neither band n46, band n96 nor band n102,

then the PC1.5 MSD is specified as:

PC1.5 MSD = PC2 MSD + MSD,

where,

-	In the Table 7.3A.2.3.2-1, MSD is specified with output columns denoted “MSDmax 6, 9, 12, 15” and where the input column uses the specified PC2 MSD specified in Table 7.3A.5-1a or in Table 7.3A.5-2a instead of the PC3 MSD, and

-	In the Table 7.3A.2.3.2-2, the correspondence between the MSDmax and the IMD order is specified using the column specified for “PC2 MSD”, and

-	These PC1.5 MSD requirements apply for the same uplink/downlink configurations as those specified in the PC2 MSD requirements of Table 7.3A.5-1a or Table 7.3A.5-2a.

In all cases, the MSD requirements specified in Table 7.3A.2.3.2-1 and in Table 7.3A.2.3.2-2 do not apply to 2UL band CA configurations with 3UL CCs, e.g. a combination of intra-band and inter-band carrier aggregation.

##### 7.3A.2.3.3	PC2 and PC1.5 MSD requirements with look-up tables for two-band DL CA with 1UL band with two CCs

The PC2 and the PC1.5 MSD requirements with look-up tables for two-band DL CA reference sensitivity exceptions (MSD) due to 1UL band with two contiguous or non-contiguous UL CC intermodulation interference shall apply when the following criteria’s are met:

-	The aggressor intra-band contiguous or non-contiguous UL CA band is a TDD band, and,

-	A PC3 reference sensitivity exception requirement is specified in Table 7.3A.5-1, and,

-	PC2 or PC1.5 power class is specified in Table 6.2A.1.1-1, Table 6.2A.1.2-1 or Table 6.2H.1.1-1

-	A reference sensitivity exception requirement is not specified in Table 7.3A.5-1a or 7.3A.5-1b for PC2 or PC1.5 respectively

-	The PCx aggressor NR UL band is the same aggressor UL band as in case of PC3 MSD, and,

For these cases, and where in the following PCx denotes either PC21Tx, PC22Tx or PC1.52Tx, the PCx MSD due to 1UL band with two contiguous or non-contiguous UL CC intermodulation interference is specified as:

PCx MSD = PC3 MSD + MSD

where,

-	The reference sensitivity exception specified for PC21Tx, PC22Tx, or PC1.52Tx is PCx MSD, and,

-	PC3 MSD is the reference sensitivity exception specified for PC3 in Table 7.3A.5-1,

-	MSD values for MSD due to odd order IMD products is 0dB

-	MSD values for MSD due to even order IMD products are specified in Table 7.3A.2.3.2-1 output columns denoted “MSDmax 9, 12, 18”. These apply to the same uplink/downlink configurations as those specified for the minimum PC3 MSD requirements in Table 7.3A.5-1,

-	MSDmax specified in Table 7.3A.2.3.2-1, the source of interference and PCx MSD is specified in Table 7.3A.2.3.3-1 for MSD due to even order IMD products

-	The correspondence between the MSDmax specified in Table 7.3A.2.3.2-1, the source of interference and PCx MSD is specified in Table 7.3A.2.3.3-1 for MSD due to even order IMD products

Table 7.3A.2.3.3-1: MSDmax correspondence look-up table for IMD order and PCx MSD

| IMD order | MSDmax |  |
| --- | --- | --- |
|  | PC2 MSD | PC1.5 MSD |
| IMD4 | 9 | 12 |
| IMD6 | 12 | 18 |

As an exception, for TDD one UL band with two CC cases where:

-	The PC2 MSD is specified in Table 7.3A.5-1a, and,

-	The PC3 MSD is not specified in Table 7.3A.5-1, and,

-	The PC1.5 MSD is not specified in Table 7.3A.5-1b, and,

-	PC1.5 power class is specified in Table 6.2A.1.1-1, Table 6.2A.1.2-1 or Table 6.2H.1.1-1

then the PC1.5 MSD is specified as:

PC1.5 MSD = PC2 MSD + MSD,

where,

-	For MSD due to odd order IMD interference, MSD = 0, and

-	For MSD due to even order IMD interference, MSD is derived from the output columns denoted “MSDmax 3” column in Table 7.3A.2.3.1-1, where the PC3MSD value entry on the first input column is replaced by the PC2MSD value.

In all cases, the MSD requirements specified in Table 7.3A.2.3.3-1 do not apply to 1UL FDD band CA configurations with two UL CCs, and in this case, MSD shall be specified in Table 7.3A.5-1a or 7.3A.5-1a for PC2 or PC1.5 respectively.

##### 7.3A.2.3.4	PC2 and PC1.5 MSD requirements with look-up tables for two-band or three-band DL CA with two-band UL CA with two contiguous CC in one UL band

The PC2 and the PC1.5 MSD requirements with look-up tables for two-band and three-band DL CA reference sensitivity exceptions (MSD) due to 2UL CA 3rd order intermodulation interference from one CC in one UL band and two contiguous CC in the other UL band shall apply when the following criteria’s are met:

-	The UL band with and intra-band UL CA configuration is a TDD band, and,

-	A PC3 reference sensitivity exception requirement is specified either in Table 7.3A.5-1, or in Table 7.3A.5-2, and,

-	PC2 or PC1.5 two-band UL CA with two contiguous UL CCs in one UL band for a total of 2Tx or 3Tx is specified in Table 6.2A.1.3-1 or Table 6.2H.3.1-1

-	A reference sensitivity exception requirement is not specified in Tables 7.3A.5-1a or 7.3A.5-2a or Tables 7.3A.5-1b or 7.3A.5-2b for PC2 or PC1.5 respectively

-	The PCx aggressor NR UL band is the same aggressor UL band as in case of PC3 MSD, and,

For these cases, and where in the following PCx denotes either PC21Tx, PC22Tx or PC1.52Tx, the PCx MSD due to 2UL band with two contiguous UL CC in one band  intermodulation interference is specified as:

PCx MSD = PC3 MSD + MSD

where,

-	PCx MSD is the reference sensitivity exception specified for PC2 or PC1.5 with 2UL band CA for a total of 2Tx or 3Tx and with 1UL CC in one UL band and 2UL contiguous CCs in the other band,

-	PC3 MSD is the reference sensitivity exception specified for PC3 in Table 7.3A.5-1, or in Table 7.3A.5-2,

-	MSD values are specified in Table 7.3A.2.3.2-1 output columns denoted “MSDmax 6, 12”. These apply for the same uplink/downlink configurations as those specified for the minimum PC3 MSD requirements in Table 7.3A.5-1, or in Table 7.3A.5-2, and,

-	The correspondence between the MSDmax specified in Table 7.3A.2.3.2-1, the IMD order and PCx MSD is specified in Table 7.3A.2.3.4-1,

Table 7.3A.2.3.2-2: MSDmax correspondence look-up table for IMD order and PCx MSD

| IMD order | MSDmax |  |
| --- | --- | --- |
|  | PC2 MSD | PC1.5 MSD |
| IMD3 | 6 | 12 |

As an exception, for cases where:

-	The PC2 MSD is specified in Table 7.3A.5-1a or in Table 7.3A.5-2a, and,

-	The PC3 MSD is not specified in Table 7.3A.5-1 or in Table 7.3A.5-2, and,

-	The PC1.5 MSD is not specified in Table 7.3A.5-1b or in Table 7.3A.5-2b, and,

-	PC1.5 two-band UL CA for a total of 2Tx or 3Tx and with 1CC in one UL band and two CCs in the other UL band is specified in Table 6.2A.1.3-1 or Table 6.2H.3.1-1,

then the PC1.5 MSD is specified as:

PC1.5 MSD = PC2 MSD + MSD,

where,

-	For MSD due to three CC IMD3 interference, MSD is derived from the output columns denoted “MSDmax 6” column in Table 7.3A.2.3.2-1, where the PC3MSD entry value on the first input column is replaced by the PC2MSD value.

==============End of change==============