3GPP TSG-RAN WG4 # 117	R4-2520229

Dallas, USA, 17th November 2025 – 21st November 2025

Agenda item:	8.7

Source:	MediaTek Inc.

Title:	Discussion on 6G RAN4 driven non-AI demod topics

Document for:	Discussion

# Introduction

This contribution provides an overview of our recommendations on technology directions and key pain points to be addressed for 6G RAN4‑driven non‑AI demodulation and CSI topics to fulfil the 6G demodulation and performance related study objectives. Selected open issues listed in the previous WF are discussed in this contribution.

# Discussion

Study on 6G radio work item description [1] gives high level objective for demodulation and performance aspects.

- Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS

- Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

When evaluating existing 5G demodulation, our high‑level observation is that test configurations are often set to the minimum settings in terms of the number of transmit antennas, using random precoding. This approach may not be a good indicator of performance in real deployments, but it can still provide a solid foundation for verifying basic “day‑1” functionality and performance.

In the existing CSI reporting tests, the test metrics and requirements are designed to be decoupled from demodulation performance. In our view, this principle often results in unnecessarily complicated test procedures. Furthermore, we believe that the applied metrics are not necessarily good indicators of overall performance.

General performance requirement aspects - Agreement - In 6G SI, RAN4 will study how to strive to develop a demod/test performance requirement framework for the minimum performance to better reflect real field conditions than 5G, if necessary, by taking into consideration of the requirement/test coverage/alignmentablity and the test feasibility, complexity and cost/resources.

Our high‑level objective for the 6G demodulation and CSI reporting requirements study is to challenge the traditional testing approach. As agreed in the previous meeting, we should define requirements that better represent the real‑world performance of DUTs. In real deployments, overall radio performance is a combination of demodulation performance and CSI reporting performance, with additional impact from transmitter behaviour. Therefore, rather than limiting the test scope, we encourage greater use of integrated tests that jointly assess demodulation and CSI reporting under scenarios that better reflect real‑world radio conditions and configurations.

Observation #1: In real-world scenarios, link performance is the result of CSI reporting and demodulation operating jointly.

## 2.1 General aspects

Demodulation specification principles - Proposals - Option 1: For 6G Demodulation specification structures, take TS38.101-4 as a starting point. - Option 2: For 6G Demodulation specification drafting principles, the descriptions of test parameters should be aligned with RAN1/RAN2 descriptions as much as possible, in order to avoid ambiguous understanding. - Option 3: For FRCs in 6G Demodulation specification, prefer to use a formula-based or pseudo-code-based definition for FRCs instead of table-based approach listing every parameter combination. - Option 3A: RAN4 needs to discuss how to specify FRC table in the specification for both BS and UE demodulation performance, considering the discussion in SI modernization of specification format and procedures for 6G.

We consider that, for the 6G demodulation specification structure, the existing 5G TS 38.101‑4 structure can be used as a starting point. However, we need to retain the possibility of further modifications if the current split between decoupled demodulation and CSI tests is changed to a more combined approach, such as demodulation with link adaptation. This could potentially require some restructuring of the specification.

Proposal #1: Use the 5G TS38.101-4 structure as the starting point for the 6G demodulation specification structure.

We also consider it worthwhile to study a formula‑based or pseudo‑code‑based definition for FRCs instead of the current table‑based approach. However, it should be confirmed whether this study should be conducted within the 6G demod thread or the 6G specification enhancement thread.

Proposal #2: Study a formula-based or pseudo-code-based definition for FRCs.

Proposal #3: Confirm which thread should address the FRC study.

Broadcast and feedback-less channels/signals testing - Proposals - Option 1: Study whether broadcast and feedback-less channels/signals can be assumed testable. RAN4 to check whether to recommend to RAN5 to define needed test solutions.

We are open to studying whether broadcast and feedback‑less channels/signals can be considered testable. Our general view is that, in many use cases, it may be possible to implement a test mode in the DUT to enable performance measurement during testing; however, feasibility in all cases is not guaranteed and may require further confirmation.

Since a test mode requires additional implementation effort for certain UE behaviours that will never occur in the real field, the threshold for adoption should be set high. For example, a test mode may only be considered when it can dramatically reduce test costs (e.g., testing time).

A more critical issue in testing broadcast and feedback‑less channels/signals is the strict use of valid RAN1 configurations.

Proposal #4: Study whether broadcast and feedback-less channels/signals can be considered testable.

Proposal #5: Testing of broadcast and feedback‑less channels/signals shall strictly use valid RAN1 configurations.

ISAC study - Proposals - Option 1: Study the demodulation for ISAC for 6G.

We propose postponing this discussion in the 6G demod thread until there is further progress in the 6G sensing thread. This will allow a better understanding of the different testability issues and whether there are any issues that the 6G demod thread should address.

Proposal #6: Postpone ISAC discussion in 6G demod until 6G sensing has more progress.

## 2.2 Channel models

### 2.2.1 Channel type

6G channel model preparations have already started in RAN4 Rel19 spatial channel model study item, and its outcome is documented in TR38.753 [5]. Additionally, RAN1 has updated TR38.901 [6] with 6G related updates. Also, RAN4 is going to continue spatial channel model study in Rel20 with extended scope from Rel19 work.

RAN4 TR38.753 considers following channel models.

- Legacy TDL (The current baseline channel for all 5G requirements in TS38.101)

- Extended TDL (Multi-cluster variant of legacy TDL)

- CDL (TR38.901 based link-level CDL with randomness reduction for better alignment)

Observation #2: After Rel-19 study RAN4 has 3 channel model options available for 6G requirements.

The current TR38.753 considers all 3 channel model options feasible and can be aligned but do not give recommendations for specific use cases.

- Proposals - Option 1: Clarify use of TDL and SCM models for 6G - Option 1A: Use rCDL baseline for MIMO. - Option 1B: Use CDL/rCDL as baseline for 6G. - Option 1C: Maintain TDL and CDL. - Option 1D: Continue TDL for simplicity. - Option 1E: Include xTDL. - Option 1F: Define default CDL propagation models. - Option 1G: Adoption of SCM should be justified for each test purpose. - Option 1H: Study practical MIMO correlation matrices for TDL. - Option 1I: Use rCDL baseline for MIMO and a limited number of TDL requirements for MIMO features with single layer transmission. - Option 1J: select one channel model (either TDL or CDL) for one specific feature. - Option 1Ja: The criteria of selection should be clarified and applied for all features.

Based on the proposals from the last meeting, we suggest a starting point for defining use cases. We consider the CDL channel to more realistically model MIMO effects and to be the preferred option. Thus, to simplify channel model usage, we propose using the CDL channel as the baseline assumption in all cases.

On the other hand, we recognize that there is not adequate proof of the applicability of CDL models to all test cases. The 5G requirements employ TDL models, and converting all of them to using CDL models may be an impossible task. Therefore, we propose that TDL and xTDL channels are considered as fallback solutions for any requirements if the CDL channel presents testability or alignment issues.

Finally, we recommend that this use case guideline for selecting the channel type should not be considered mandatory in 6G work items. The final selection should be determined during the work item process.

Proposal #7: Use the CDL channel as the baseline to all requirements.

Proposal #8: Keep TDL and xTDL channels as fallback solutions for any requirements.

Proposal #9: Treat these as 6G guidelines and leave final selection to work item stage.

The legacy TDL channel antenna correlation model in its current form does not scale well for large antenna arrays. For example, for the one-dimensional linear gNB antenna array, parameter α specifies the correlation between the first and the last antenna element of the array. As a result, for a fixed correlation parameter, increasing the number of antenna elements does not increase the multiplexing gain of the channel. However, if we assume that a typical antenna element separation at gNB is half a wavelength (as we do for CDL models), multiplexing gain scales with the array size. For TDL models, it would make sense to define the correlation model in a similar manner. More specifically, we should define the spatial correlation as a function of antenna element distance instead of antenna element index. Furthermore, it is not clear whether the current correlation function itself corresponds to physical environments. We propose to study alternatives to find more realistic correlation functions.

Observation #3: The legacy TDL channel antenna correlation definition is not well suited for large antenna arrays.

Proposal #10: Re-specify spatial correlation for TDL models as function of antenna element distance instead of antenna element index.

Proposal #11: Further study how to define antenna correlation function to better resemble physical environments.

### 2.2.2 Frequency related aspects of channel model

In the Rel‑20 NR demodulation performance Phase 6 work item, it was agreed to define the requirements using the same channel model for all FR1 frequencies in that WI, thereby preserving the frequency agnostic approach of the channel model, as with the legacy TDL channels.

Frequency generalization for rCDL channel modeling (Rel-20 NR demod Phase 6) - Agreement: - Define the requirements based on the same channel model for all different FR1 frequencies for this WI - The maximum Doppler fD should be fixed as X Hz - As baseline for Rel-20 demod requirement, X =10 - FFS if other fD can be considered - On the note to Maximum Doppler: - The ratio between the UE speed v, i.e., magnitude of the velocity vector, and the wavelength is normalized to be X Hz (maximum Doppler).

Frequency related aspects of channel model - Proposals - Option 1: Study new frequency range. - Option1A: Study new frequency ranges of 6G - Derive CDL for 7-15 GHz. - Option1B: Study FR2. - Option 1C: Evaluate necessity and study spatial channel model for other frequency ranges in 6GR. - Option 2: Study if channel model is agnostic to different carrier frequencies or separate models are needed. - Option 3: Discuss the number of antennas required for different frequency ranges.

In this study, we need to ensure that channel model solutions are ready for use with any 6G carrier frequencies that can be assumed at this phase of the work. Therefore, we must consider whether it is feasible to generally apply the same frequency agnostic approach to the CDL channel for all bands, or whether different channels should be defined for different frequency ranges or even specific frequency bands.

Our initial preference is to treat the CDL channel model as agnostic to different carrier frequencies.

Proposal #12: Study whether the channel model is agnostic to different carrier frequencies or whether separate models are required.

Proposal #13: Consider the channel model as agnostic to different carrier frequencies.

### 2.2.3 Uplink aspects of channel model

Uplink aspects of channel model - Proposals - Option 1: Study UL CDL for BS variants. - Option 1a: Study UL CDL for BS variants if key issues in DL CDL study are settled.

To our understanding, the UL CDL channel can be assumed to be the exact reverse of the DL CDL channel. We believe this principle could already be agreed upon. Otherwise, we suggest first finalizing all DL CDL issues and then conducting selected trial UL CDL simulations to verify whether the UL simulation results can be aligned.

Proposal #14: Confirm that the UL CDL channel is the exact reverse of DL CDL channel.

Proposal #15: Conduct selected trial UL CDL simulations to confirm alignment.

### 2.2.4 PMI bias

In the Rel-20 NR demodulation performance Phase 6 work item, PMI bias issue will be discussed in the next meetings.

PMI bias (Rel-20 NR demod Phase 6) - Candidate options: - Option 1: RAN4 shall exploit approaches to address the PMI bias issue - Option 1a: Apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3 - Option 1b: time variant mean angles (AOA/AOD/ZOA/ZOD) - Other options are not precluded - Option 2: No need to consider time-wise evolution for PMI reporting tests - Option 3: Defer a consideration of a comprehensive long-term solutions to the 6G Radio Study Item

PMI bias - Proposals - Option 1: Consideration of CDL modeling in 6GR for PMI report evaluation should follow the identification of robust countermeasures to prevent overfitting to deterministic channel behavior.

We suggest postponing the discussion of the PMI bias issue in the 6G study until the Rel‑20 work item has concluded a solution for 5G. After that decision, we can analyse whether the same solution is adequate for 6G or whether further study would be required.

Proposal #16: Postpone the PMI bias discussion until Rel-20 WI has concluded a solution for 5G.

## 2.3 Receiver assumptions

Receiver assumption for UE - Proposals - Option 1: MMSE-IRC as a baseline receiver. - Option 2: MMSE-IRC and R-ML as baseline receivers. - CMCC: with the prerequisite that the receiver is transparent to the network and does not require any PHY layer modification and additional assistance information. - Option 3: Cover advanced receivers (R-ML, soft-IC) - CT: Study the required information for advanced Rec for MU-MIMO. - Option 4: Study baseline and simplified structures. - Option 5: Study widely linear MMSE-IRC.

In 5G UE downlink demodulation and CSI requirements the default baseline receiver is assumed as MMSE linear receiver. Additionally, it can be assumed in include noise whitening capability (IRC). In TR38.878 [2], where study of advanced receiver to cancel inter-user interference for MU-MIMO was documented, more advanced receiver types like E-MMSE-IRC and R-ML are also documented.

Observation #4: 5G UE downlink demodulation and CSI requirements the default baseline receiver is assumed as MMSE linear receiver.

Observation #5: 5G UE downlink demodulation and CSI requirements can assume other advanced receiver types for specific tests.

In scenarios where the form factor of a device is not a limiting factor, such as Fixed Wireless Access (FWA) devices, it is feasible to equip the device with multiple antennas featuring low correlation to support high number of MIMO layers. However, as the number of MIMO layers increases, the UE processing complexity and power consumption rise significantly, posing challenges for efficient operation. Recognizing this, in Rel-18 and Rel-19, RAN4 has already considered the concept of an 8Rx FWA simplified receiver, which leverages two joint 4Rx MIMO detectors within an 8Rx device to reduce complexity. Building on this approach, in 6G, RAN4 could also explore the requirements of low-complexity receiver designs that balance performance with reduced UE processing demands and power consumption.

Observation #6: When the UE is equipped with large number of antennas, it is possible to use several small number Rx MIMO detectors for data processing, e.g., 8Rx UE can use two joint 4Rx MIMO detectors instead of one 8Rx MIMO detector for data processing.

Proposal #17: Study baseline and simplified receiver structures.

Several important dependencies remain unresolved before a baseline can be set. Key RAN1 PHY parameters, including 200 MHz and 400 MHz carrier bandwidths, FFT sizes, and SCS/CP choices, will affect receiver architecture, complexity, memory bandwidth, and DMRS density and tracking. Device capability classes and day‑1 target use cases (FWA, Handheld, RedCap, IoT, NTN) are not yet finalized. The baseline UE structure, including receiver chain count, maximum layers per UE, and MU‑MIMO usage, has not been evaluated in RAN4. Layer and rank assumptions, together with bandwidth and FFT choices, will define computational and memory profiles, and premature baselines risk locking in unfavourable cost and power points. Processing latency for aggregated component carriers over 200 MHz or 400 MHz CBW with advanced receivers is also unstudied.

Therefore, we recommend deferring the decision on day‑1 baseline receiver assumptions until RAN1 parameters and device capability envelopes are defined.

Observation #7: Several key RAN1 PHY parameters that determine receiver architecture are still to be defined.

Proposal #18: Postpone the decision on day‑1 baseline receiver assumptions until the dependencies on RAN1 parameters and device capability envelopes are clarified.

Receiver assumption for BS - Proposals - Option 1: MMSE-IRC as a baseline receiver. - Option 2: Study feasibility of considering higher than 8Rx scenarios.

In the last meeting, all proposals indicated that the only practical baseline receiver assumption for base stations is the MMSE‑IRC receiver. If this reflects the consensus view of all infrastructure vendors, we are prepared to confirm MMSE‑IRC as the baseline receiver for base stations.

Proposal #19: We are fine to confirm MMSE-IRC as the baseline receiver for base stations.

## 2.4 TxEVM and SNR

TxEVM aspects - Proposals - Option 1: Study what EVM simulations assumptions should be used in demodulation and CSI requirements. - Option 1A: Study impact of TX EVM for higher modulation order/ MIMO layers on Demodulation requirements. - Option 1B: Abandon the SNR operating point limitations via fixed 20dB rule, or fixed TE TxEVM assumptions, and adopt a SNR limitation derivation based on actual TDRA/FDRA configuration. - Option 1C: Study whether the TxEVM requirements for the base station, at least in the simulations for deriving the demodulation requirements, could be tightened. SNR aspects - Proposals - Option 1: To ensure good field coverage, the SNR values in the demodulation requirements should follow the SSB SNR values that can be measured in existing 5G NR scenarios and those SSB SNR values that can be expected in upcoming 6G deployments. - Option 2: Study whether the coverage range for relevant field scenarios can be extended by defining demodulation requirements for larger SNR values as currently being used in 5G NR. - Option 3: Study whether the coverage range for relevant field scenarios can be extended for carrier aggregation. It should also be studied whether the SNR requirement should be dependent on the number of component carriers. - Option 4: Abandon the SNR operating point limitations via fixed 20dB rule, or fixed TE TxEVM assumptions, and adopt a SNR limitation derivation based on actual TDRA/FDRA configuration.

There has been discussion to revisit simulation assumption of EVM in several 5G work items.

The 5G TX EVM UE downlink simulation assumptions are listed in Table 2.1.2‑1. The 5G EVM requirements for BS type 1‑C and BS type 1‑H carriers, as specified in TS 38.104 [3], are listed in Table 2.1.2‑2 (corresponding to Table 6.5.2.2‑1 in TS 38.104 [3]). The 5G UE downlink simulation assumptions are not well aligned with the 5G BS EVM requirements. However, the typical TX EVM observed by a UE can be assumed to be significantly lower than the values specified in the BS RF requirements.

Observation #8: 5G TX EVM UE downlink simulation assumptions are not aligned with 5G BS EVM requirements.

Table 2.1.2-1: UE downlink simulation assumptions of EVM

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 6 % |
| 16QAM | 6 % |
| 64QAM | 6 % |
| 256QAM | 3 % |
| 1024QAM | 2.5 % |

Table 2.1.2-2: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |
| 1024QAM | 2.5 %1 2.8 %2 |
| Note1: This requirement is applicable for frequencies equal to or below 4.2 GHz. Note 2: This requirement is applicable for frequencies above 4.2 GHz. |  |

Furthermore, as discussed in Views on 6G BS TX EVM requirement framework paper submitted in RAN4#117 meeting [4], modulation specific EVM requirement is fine only when the number of MIMO layers is not very high. However, when the number of MIMO layers is higher, such an approach becomes problematic. With higher MIMO ranks, even with the same modulation order, the required SNR will increase correspondingly. However, if we keep the same EVM value, the maximum achievable SNR will be upper bounded, as well as the throughput.

Observation #9: With the EVM values based on current TS 38.104, the max throughput cannot be achieved with more MIMO layers, and the degradation compared to EVM 0% can be huge in some scenarios.

We should study which TX EVM simulation assumptions should be applied in demodulation and CSI requirements. Ideally, RAN4 should be able to define these requirements under SNR conditions that can be observed in real field scenarios. A good starting point would be to collect observed SNR values from field logs to determine the maximum achievable SNR. Secondly, we should consult TE vendors to identify the highest achievable SNR at a reasonable device cost, as this would represent a trade‑off.

Proposal #20: Study what EVM simulations assumptions should be used in demodulation and CSI requirements.

Proposal #21: Collect observed SNR values from field logs to determine the maximum achievable SNR.

Proposal #22: Consult TE vendors to identify the highest achievable SNR at a reasonable device cost.

## 2.5 Interference modelling aspects

Interference profile - Proposals - Option 1: Study the interference profile for 6G DL/UL intra-cell and inter-cell interference scenarios. - Option 1A: RAN4 further evaluate interference profiles for intra-cell/interference cell scenarios: gNB and UE configuration e.g., power class, antenna configuration Homogenous and heterogenous scenarios Asynchronization TDD or dynamic TDD scenario Semi-static/Dynamic SBFD operation in gNB. - Option 1B: For 6G Demodulation with interference modelling, further discussion, and analysis on the modelling of directions, INRs, modulation orders of interference(s), number of layers from interference(s) are needed. - Option 2: Study possible interference caused by MRSS, by interference cancellation or mitigation.

It is considered worthwhile to evaluate interference profiles for 6G downlink and uplink in both intra‑cell and inter‑cell interference scenarios. However, some simulation assumptions may require further knowledge of the 6G PHY design to better align with the new radio system. As preparation, we should begin collecting assumptions that are outdated in the 5G context, as they may potentially remain issues in 6G.

Proposal #23: Study the interference profile for 6G DL/UL intra-cell and inter-cell interference scenarios.

Proposal #24: Start collecting updated interference assumptions based on 5G learnings.

## 2.6 Performance testing and requirement

### 2.6.1 Demodulation requirements with fixed Rank and MCS

Demodulation testing - Proposals - Option 1: For 6G demodulation study, use FRC style, MCS value, fixed rank, fixed channel bandwidth, fixed subframe configuration as a starting point. - Option 3: Discuss the additional margins and measurements uncertainty for requirements definition of 6GR. - Option 4: Discuss the SNR derivation procedure for 6GR, the span of ideal results span is <= [X] dB.

In the 5G UE downlink demodulation requirements, the default test point is defined as 70 % of the maximum throughput. In addition, there is a rare exception at 30 % of the maximum throughput, used to test HARQ. UE reporting in link adaptation is defined to target an operation point of less than 10 % BLER, corresponding to more than 90 % of the maximum throughput.

Observation #10: There is a misalignment between the operation points in the demodulation requirements and the actual system target.

A common claim regarding the demodulation requirement test point is that it is easier to align at 70 % of the maximum throughput than at 90 %. This claim is typically based on legacy demodulation test assumptions using the legacy TDL channel and random precoding. Furthermore, with the introduction of new channel models and potentially more representative test configuration parameters, it is necessary to verify whether the legacy assumptions on implementation margin and SNR derivation remain valid for 6G.

Observation #11: New channel models may impact demodulation alignment.

Proposal #25: Study whether a 10 % BLER operation point would be feasible instead of the legacy 30 % BLER.

Proposal #26: Study whether legacy implementation margins remain applicable under new 6G assumptions.

Proposal #27: Study whether the legacy SNR derivation procedure remains applicable under new 6G assumptions.

### 2.6.2 Demodulation requirements with link adaptation

Demodulation testing - Proposals - Option 2: Study extending scope of demodulation tests with link adaptation.

In the legacy demodulation tests, there are few demodulation tests with full link adaptation in addition to fixed reference channel testing. In these tests number of layers, precoding, and MCS are selected based on UE CSI reports. In 5G test coverage of these tests is very limited because of 2 antenna port configuration causing maximum of number of layers to be just 2 and precoder selection is trivial.

Observation #12: In 5G there is very limited set of demodulation requirements with link adaptation.

The existing demodulation requirements with link adaptation utilize HARQ for retransmission for more practical system behaviour. Also, as TE is strictly following reported CQI, that should correspond to 10% BLER, we can assume operation point to be close to normal system operation point.

We consider demodulation requirements with link adaptation to be a more realistic and comprehensive measure of performance. Therefore, implementing such tests on a larger scale and across a more complete set of configurations could help reduce the number of simple tests with a limited scope, such as those focusing solely on demodulation or CSI reporting.

We propose studying the extension of demodulation tests with link adaptation for broader use. To make the study more concrete, we recommend conducting a simulation‑alignment trial using the 5G PHY with extended configurations. The number of TX antennas could be set to commonly deployed values of 8 and 32, and the rank should be studied at least up to 4. This trial would help identify potential alignment issues in more complex configurations that are already used in legacy 5G requirements.

Proposal #28: Study the extension of demodulation tests with link adaptation.

Proposal #29: Evaluate replacing a number of simple demodulation or CSI tests with demodulation tests incorporating link adaptation.

Proposal #30: Conduct a simulation‑alignment trial using the 5G PHY with extended configurations (NumTx = 8 and 32, Rank ≤ 4) to assess alignment feasibility.

### 2.6.3 CSI reporting test methodologies

In this section, we discuss CSI test methodologies corresponding to CQI, PMI, and RI reporting in 5G. We recognise that the 6G CSI reporting framework may ultimately diverge from the 5G framework, so some observations and proposals grounded in the legacy framework may be superseded. Nonetheless, we can adopt the legacy framework as a baseline until the new framework is finalised.

- Proposals - Option 1: Streamline CQI reporting testing into 1-step approach and setting requirements in terms of throughput/SNR and BLER limits. - Option 2: Study the necessity of CQI reporting requirements in addition to combined demodulation and link adaptation testing. - Option 3: Simplify the PMI reporting testing process and setting requirements directly in terms of throughput/SNR instead of measuring γ. - Option 4: Study the necessity of PMI reporting requirements in addition to combined demodulation and link adaptation testing. - Option 5: Study RI reporting requirements test metrics and test methodologies. - Option 5A: Investigate alternative metric that measures consistency and accuracy of Rank Indicator (RI).

#### 2.6.3.1 CQI reporting requirement related test methodologies

In the legacy CQI reporting requirements test metrics are defined as

- Testing under AWGN conditions

  - The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90 % of the time.

  - If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to BLER target, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than BLER target. If the PDSCH BLER using the transport format indicated by the median CQI is greater than BLER target, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to BLER target.

- Testing under fading conditions (assuming wideband reporting)

  - A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time where α% is specified in Reference Table;

  - The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Reference Table;

  - When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

The test procedure in AWGN and fading conditions assume two sets of predefined SNR points with 1dB difference. UE is assumed to pass requirements at least in one SNR in both SNR sets. Also, the test procedure measures median CQI as the first step and with measured median CQI + 1 evaluates BLER in the second step.

Observation #13: CQI reporting requirements use 2-step approach to evaluate all pass conditions.

We believe the current test methodology for CQI reporting is unnecessarily complicated. Therefore, we propose to consider a simpler and more realistic 1-step approach. We believe it suffices to test only in follow-CQI mode in a number of SNR points and set requirements for throughput and BLER.  More specifically, the pass criterion could be that a minimum throughput requirement is achieved so that BLER remains within allowed limits. The allowed BLER limits could depend on the channel. For example, in AWGN, BLER should fall between 0% and 10%, while in fast fading BLER could fall between 2% and 20%. Alternatively, lower limit for BLER could even be omitted since the throughput requirement should be impossible to reach by reporting too low CQI values.

Proposal #31: Streamline CQI reporting testing into 1-step approach and setting requirements in terms of throughput/SNR and BLER limits.

Finally, we propose to study the necessity of dedicated CQI reporting requirements in addition to combined demodulation and link adaptation testing.

Proposal #32: Study the necessity of CQI reporting requirements in addition to combined demodulation and link adaptation testing.

#### 2.6.3.2 PMI reporting requirement related test methodologies

In the legacy PMI reporting, test metrics are defined as

- γ = tfollow / trnd

where in the definition of throughput ratios γ, tfollow is 90 % of the maximum throughput obtained at SNRUE using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRUE with random precoding. In addition, frequency domain granularity of random precoding depends on codebook under testing, and random precoding used always Single Panel Type I codebook.

We see that the intention to define throughput ratio γ as test metric was to exclude demodulation performance in PMI testing. On the other hand, when the channel has high spatial selectivity, demodulation with random precoding may perform poorly in SNRUE operation point that can make throughput ratio γ unreliable test metric.

Observation #14: Likely original intention of throughput ratio γ test metric was to make testing independent of demodulation performance.

Observation #15: Throughput ratio γ can be unreliable test metric in highly spatially selective channels.

We are wondering if throughput ratio γ based test metric is the best method to test precoding reporting. We propose to study alternative test metrics like directly using 90% of the maximum throughput obtained at SNRUE as the test metric. It would also shorten the testing time when random precoding reference would not be needed.

Proposal #33: Simplify the PMI reporting testing process and setting requirements directly in terms of throughput/SNR instead of measuring γ.

Finally, we propose to study the necessity of dedicated PMI reporting requirements in addition to combined demodulation and link adaptation testing.

Proposal #34: Study the necessity of PMI reporting requirements in addition to combined demodulation and link adaptation testing.

#### 2.6.3.3 RI reporting requirement related test methodologies

In the legacy RI reporting requirements test metrics are defined as

- The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ γ1;

- The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ γ2;

There are just four 4RX RI requirements for FDD and TDD each and γ requirements are as {0.9, 1.05, 0.9, 0.9} per test. This means that in 3 tests out of 4 rank reporting can perform worse than fixed rank and still pass the test. This testing methodology does not seem very credible and beneficial. Also, 3 or out 4 tests use 2 AP Single Panel Type I codebook, and the last test uses 4 AP Single Panel Type I codebook. Maximum number of layers tested is just 2.

Observation #16: There are very few legacy rank reporting tests.

Observation #17: Maximum number of layers tested is just 2 in legacy rank reporting tests.

Observation #18: Test requirements are very loose in legacy rank reporting tests.

We propose to study alternative test metrics and test methodologies to define rank reporting requirements.

Proposal #35: Study RI reporting requirements test metrics and test methodologies.

Finally, we propose to study the necessity of dedicated RI reporting requirements in addition to combined demodulation and link adaptation testing.

Proposal #36: Study the necessity of RI reporting requirements in addition to combined demodulation and link adaptation testing.

## 2.7 New TE functionalities

### 2.7.1 OLLA with link adaptation

- Feature Lead notes - Option 1 discussion has been captured in Chairman and Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on OLLA in the next meeting as several companies see value to study OLLA as part of 6G demod SI. - Proposals - Option 1: Study to include OLLA in ATP requirements. - Option 1A: Use proposed OLLA model from R4-2300703 as a starting point. - Option 1B: Study the feasibility to include OLLA in ATP requirements.

The existing demodulation requirements with link adaptation utilize HARQ retransmissions to achieve more practical system behaviour. As the test equipment (TE) strictly follows the reported CQI - which should correspond to a 10% BLER - we can assume the operating point to be close to that of a normal system.

However, in real networks, the system does not fully rely on UE CQI reports alone; instead, it employs Outer Loop Link Adaptation (OLLA). During discussions on 5G PDSCH absolute physical layer throughput requirements, some companies provided reference baseline OLLA implementations.

Observation #19: The current 5G demodulation requirements with link adaptation do not include OLLA.

Observation #20: A possible reference baseline OLLA implementation has been shared in the 5G WI.

We propose studying the addition of OLLA to improve the realism of demodulation requirements. The earlier OLLA implementation proposal [7] can serve as a starting point for TE implementation. We encourage other companies to review the proposed model and suggest modifications as needed. We are also open to evaluating alternative OLLA models.

To assess the feasibility and robustness of proposed OLLA models, simulation assumptions must be defined. We propose that companies share their preferred simulation assumptions at the next meeting.

Proposal #37: Study adding OLLA to demodulation tests with link adaptation.

Proposal #38: Use OLLA model from R4-2300703 as the starting point.

Proposal #39: Companies to consider and propose simulation assumptions for OLLA model evaluation at the next meeting.

### 2.7.2 SRS based precoding

- Feature Lead notes - Option 1 discussion has been captured in Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on SRS based precoding in the next meeting as several companies see value to study SRS based precoding as part of 6G demod SI. - Proposals - Option 1: Study whether and how to define baseline SRS based precoding procedure in TE to enable aligned simulation assumptions.

For 6G requirements, three channel model options will be available, which may impact earlier alignment observations. The Spatial Channel Model study item has been working to develop more realistic channel models that correspond to physical environments observed in the field. We believe it is not meaningful to combine realistic channel models with unrealistic transmission schemes. Therefore, instead of random precoding, more realistic and practical precoding should be applied.

The legacy demodulation requirements use test assumptions with a legacy TDL channel and random precoding. In the case of spatial channel models, random precoding is less meaningful, and more practical and realistic precoding should be applied. One missing option for RAN4 is SRS‑based precoding.

Observation #21: SRS‑based precoding would enable more realistic precoding in demodulation tests with spatial channel models without requiring PMI feedback.

Observation #22: SRS‑based precoding would represent a new methodology in RAN4.

Observation #23: SRS‑based precoding would require a baseline definition of precoding processing in TE to enable aligned simulation assumptions.

We see at least two options for implementing SRS‑based precoding in TE. The first option assumes that the DUT transmits SRS, which experiences the same uplink channel as is used in the downlink. The TE would perform channel estimation on the SRS to determine the channel coefficients and, based on these, calculate a precoder that matches the channel properties. The second option is emulated SRS‑based precoding, where the TE directly uses the channel coefficients of the generated channel for the downlink and based on these, calculates a precoder that matches the channel properties. In this case, the TE can add AWGN to the channel information to emulate channel estimation error and introduce an additional delay before applying the updated precoder to emulate processing delay.

We propose that TE vendors conduct an initial feasibility study to confirm whether the proposed options are practical to implement before proceeding to the detailed definition of each option.

Proposal #40: Discuss SRS-based precoding procedure options: real SRS-based and emulated SRS-based.

Proposal #41: Conduct an initial feasibility study of SRS-based precoding procedure options in TE.

### 2.7.3 Time/frequency/phase offset precompensation

- Feature Lead notes - Discussion of Option 1 has been captured in Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on time/frequency/phase offset precompensation in the next meeting as several companies see value to study time/frequency/phase offset precompensation as part of 6G demod SI. - Proposals - Option 1: Study inclusion of higher layer aspects in demodulation requirements via dynamic TE decisions using known algorithms, e.g., applying timing offset reports (CJT).

In the Rel‑19 NR MIMO Phase 5 work item, there was an attempt to introduce requirements for the CJT scenario with non‑synchronous TRPs, where reported time, frequency, and phase offsets would be precompensated and demodulation performance measured. However, such requirements were not defined, and the exact procedure for conducting pre‑compensation of the different properties was left unspecified.

There are other use cases where time and frequency precompensation could be applied, such as HST and NTN scenarios. Therefore, we consider it worthwhile to define precompensation procedures for TE to enable the implementation of such test scenarios. We encourage network vendors to help defining time and frequency precompensation use cases, signalling, and procedures.

Proposal #42: Network vendors to define time precompensation use cases, signalling, and procedures.

Proposal #43: Network vendors to define frequency precompensation use cases, signalling, and procedures.

### 2.7.4 Other new TE functionalities

- Feature Lead notes - Discussion of following options has been captured in Ad-hoc minutes. - Proposals - Option 1: Study inclusion of higher layer aspects in demodulation requirements via increased and dynamic application of DUT feedback in the TE. - Option 2: Study inclusion of higher layer aspects in demodulation requirements via dynamic TE decisions using known algorithms, e.g., SU/MU scheduling, dynamic resource allocation/slots. - Option 3: Explore utilizing test equipment algorithms to evaluate features that rely on network-side processing, enabling realistic UE performance assessment without limiting network implementation flexibility. - Option 4: Identify features dependent on network-side processing and aim to define corresponding performance requirements, where feasible, using test equipment-based evaluation methods.

We have interest to study TE related higher layer aspect and dynamic resource allocation in time and frequency domain in different conditions. We encourage proponents to bring more detailed suggestions what kind of procedures could be studied.

Proposal #44: Study TE related higher layer aspect and dynamic resource allocation in time and frequency domain.

## 2.8 UE classification and applicability

UE classification - Proposals - Option 1: Study demod requirement handling for UE classifications and agree on questions such as a baseline set of requirements for all devices vs. individual requirements for each UE classification. Device types - Proposals - Option 1: Wait for further clarification in RAN and RAN1 what device types may get defined. Afterwards discuss how device types can be covered in the test framework of RAN4.

We are open to studying UE classification and device‑type‑related issues in relation to demodulation and CSI requirements. However, we consider that further clarification may first be needed from RAN and RAN1 before RAN4 can effectively proceed with discussions.

Proposal #45: Postpone the UE classification and device type discussion until RAN/RAN1 has made further progress.

# Summary

In this paper we provided our views to 6G RAN4 driven non-AI demodulation and CSI topics. The following observations and proposals are made.

Observation #1: In real-world scenarios, link performance is the result of CSI reporting and demodulation operating jointly.

Proposal #1: Use the 5G TS38.101-4 structure as the starting point for the 6G demodulation specification structure.

Proposal #2: Study a formula-based or pseudo-code-based definition for FRCs.

Proposal #3: Confirm which thread should address the FRC study.

Proposal #4: Study whether broadcast and feedback-less channels/signals can be considered testable.

Proposal #5: Testing of broadcast and feedback-less channels/signals shall strictly use valid RAN1 configurations.

Proposal #6: Postpone ISAC discussion in 6G demod until 6G sensing has more progress.

Observation #2: After Rel-19 study RAN4 has 3 channel model options available for 6G requirements.

Proposal #7: Use the CDL channel as the baseline to all requirements.

Proposal #8: Keep TDL and xTDL channels as fallback solutions for any requirements.

Proposal #9: Treat these as 6G guidelines and leave final selection to work item stage.

Observation #3: The legacy TDL channel antenna correlation definition is not well suited for large antenna arrays.

Proposal #10: Re-specify spatial correlation for TDL models as function of antenna element distance instead of antenna element index.

Proposal #11: Further study how to define antenna correlation function to better resemble physical environments.

Proposal #12: Study whether the channel model is agnostic to different carrier frequencies or whether separate models are required.

Proposal #13: Consider the channel model as agnostic to different carrier frequencies.

Proposal #14: Confirm that the UL CDL channel is the exact reverse of DL CDL channel.

Proposal #15: Conduct selected trial UL CDL simulations to confirm alignment.

Proposal #16: Postpone the PMI bias discussion until Rel-20 WI has concluded a solution for 5G.

Observation #4: 5G UE downlink demodulation and CSI requirements the default baseline receiver is assumed as MMSE linear receiver.

Observation #5: 5G UE downlink demodulation and CSI requirements can assume other advanced receiver types for specific tests.

Observation #6: When the UE is equipped with large number of antennas, it is possible to use several small number Rx MIMO detectors for data processing, e.g., 8Rx UE can use two joint 4Rx MIMO detectors instead of one 8Rx MIMO detector for data processing.

Proposal #17: Study baseline and simplified receiver structures.

Observation #7: Several key RAN1 PHY parameters that determine receiver architecture are still to be defined.

Proposal #18: Postpone the decision on day-1 baseline receiver assumptions until the dependencies on RAN1 parameters and device capability envelopes are clarified.

Proposal #19: We are fine to confirm MMSE-IRC as the baseline receiver for base stations.

Observation #8: 5G TX EVM UE downlink simulation assumptions are not aligned with 5G BS EVM requirements.

Observation #9: With the EVM values based on current TS 38.104, the max throughput cannot be achieved with more MIMO layers, and the degradation compared to EVM 0% can be huge in some scenarios.

Proposal #20: Study what EVM simulations assumptions should be used in demodulation and CSI requirements.

Proposal #21: Collect observed SNR values from field logs to determine the maximum achievable SNR.

Proposal #22: Consult TE vendors to identify the highest achievable SNR at a reasonable device cost.

Proposal #23: Study the interference profile for 6G DL/UL intra-cell and inter-cell interference scenarios.

Proposal #24: Start collecting updated interference assumptions based on 5G learnings.

Observation #10: There is a misalignment between the operation points in the demodulation requirements and the actual system target.

Observation #11: New channel models may impact demodulation alignment.

Proposal #25: Study whether a 10 % BLER operation point would be feasible instead of the legacy 30 % BLER.

Proposal #26: Study whether legacy implementation margins remain applicable under new 6G assumptions.

Proposal #27: Study whether the legacy SNR derivation procedure remains applicable under new 6G assumptions.

Observation #12: In 5G there is very limited set of demodulation requirements with link adaptation.

Proposal #28: Study the extension of demodulation tests with link adaptation.

Proposal #29: Evaluate replacing a number of simple demodulation or CSI tests with demodulation tests incorporating link adaptation.

Proposal #30: Conduct a simulation-alignment trial using the 5G PHY with extended configurations (NumTx = 8 and 32, Rank ≤ 4) to assess alignment feasibility.

Observation #13: CQI reporting requirements use 2-step approach to evaluate all pass conditions.

Proposal #31: Streamline CQI reporting testing into 1-step approach and setting requirements in terms of throughput/SNR and BLER limits.

Proposal #32: Study the necessity of CQI reporting requirements in addition to combined demodulation and link adaptation testing.

Observation #14: Likely original intention of throughput ratio γ test metric was to make testing independent of demodulation performance.

Observation #15: Throughput ratio γ can be unreliable test metric in highly spatially selective channels.

Proposal #33: Simplify the PMI reporting testing process and setting requirements directly in terms of throughput/SNR instead of measuring γ.

Proposal #34: Study the necessity of PMI reporting requirements in addition to combined demodulation and link adaptation testing.

Observation #16: There are very few legacy rank reporting tests.

Observation #17: Maximum number of layers tested is just 2 in legacy rank reporting tests.

Observation #18: Test requirements are very loose in legacy rank reporting tests.

Proposal #35: Study RI reporting requirements test metrics and test methodologies.

Proposal #36: Study the necessity of RI reporting requirements in addition to combined demodulation and link adaptation testing.

Observation #19: The current 5G demodulation requirements with link adaptation do not include OLLA.

Observation #20: A possible reference baseline OLLA implementation has been shared in the 5G WI.

Proposal #37: Study adding OLLA to demodulation tests with link adaptation.

Proposal #38: Use OLLA model from R4-2300703 as the starting point.

Proposal #39: Companies to consider and propose simulation assumptions for OLLA model evaluation at the next meeting.

Observation #21: SRS-based precoding would enable more realistic precoding in demodulation tests with spatial channel models without requiring PMI feedback.

Observation #22: SRS-based precoding would represent a new methodology in RAN4.

Observation #23: SRS-based precoding would require a baseline definition of precoding processing in TE to enable aligned simulation assumptions.

Proposal #40: Discuss SRS-based precoding procedure options: real SRS-based and emulated SRS-based.

Proposal #41: Conduct an initial feasibility study of SRS-based precoding procedure options in TE.

Proposal #42: Network vendors to define time precompensation use cases, signalling, and procedures.

Proposal #43: Network vendors to define frequency precompensation use cases, signalling, and procedures.

Proposal #44: Study TE related higher layer aspect and dynamic resource allocation in time and frequency domain.

Proposal #45: Postpone the UE classification and device type discussion until RAN/RAN1 has made further progress.

# References

[1] RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO

[2] TR38.878, “NR demodulation performance evolution (Release 18), V18.4.0 (2024-09)”

[3] TS38.104, “Base Station (BS) radio transmission and reception (Release 18), V18.10.0 (2025-06)”

[4] R4-2520967, “Views on 6G BS Tx EVM requirement framework”, MediaTek inc.

[5] TR38.753, “Study on spatial channel model for demodulation performance Requirements for NR (Release 19), V2.0.0 (2025-09)”

[6] TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz (Release 19), V19.1.0 (2025-09)”

[7] R4-2300703, “On Absolute physical layer throughput requirements with link adaptation”, Nokia, Nokia Shanghai Bell